3GPP TSG-CT WG4 Meeting #131	C4-254144

# Sophia Antipolis, France; 13rd – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.380 | CR | 0132 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on P-CSCF triggering SMF/PGW-C failure checking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-09-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in 3GPP TR 29.866, the P-CSCF sends a Rx Authorization-Authentication-Request (AAR) with the Specific-Action AVP set as a new value indicating that it's a test to check whether there is a failure in EPC/5GC network. While in 3GPP TS 23.380, the Specific-Action AVP is set to ACCESS_NETWORK_INFO_REPORT, which is inconsistent with 3GPP TR 29.866 and unable to trigger the failure checking. Thus, the CR proposes to correct that the P-CSCF should send the Rx AAR with the Specific-Action AVP set as a new value other than ACCESS_NETWORK_INFO_REPORT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction on the value of Specific-Action AVP in Rx AAR sent by P-CSCF. - Clean up red highlight in Figure 7.2.2-1. - Editorial correction in clause 8.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support P-CSCF triggering SMF/PGW-C failure checking procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2, 8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.2.2	P-CSCF triggering SMF/PGW-C failure checking

The P-CSCF triggering SMF/PGW-C failure checking is an optional mechanism. If supported, the mechanism shall be executed when the terminating UE does not respond to the initial INVITE request due to an EPC/5GC NF failure.

The following requirements shall be supported for the procedure specified in this clause:

-	P-CSCF supports to check whether the SMF/PGW-C is normal when timeout by sending an Rx Authorization-Authentication-Request (AAR) with the Specific-Action AVP setas a new value or invoke the Npcf_PolicyAuthorization_Create service to PCF/PCRF to indicate starting health check.

-	P-CSCF supports to send a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF after receiving the Rx Authorization-Authentication-Answer (AAA) with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) as specified in IETF RFC 6733 [25] or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065) as specified in 3GPP TS 29.214 [7], clause 5.5.1, or the Npcf_PolicyAuthorization_Create service response.

-	S-CSCF supports to start the HSS based P-CSCF Restoration procedure specified in clause 5.4.2.1 in EPC network or the UDM/HSS based P-CSCF Restoration procedure specified in clause 5.8.4 in 5GC network, after receiving a 503 Service Unavailable response with the P-CSCF Restoration indication.

-	PCF/PCRF support to send a Gx Re-Auth-Request (RAR) with Event-Trigger AVP set to ACCESS_NETWORK_INFO_REPORT (45) as specified in 3GPP TS 29.212 [6] to the SMF/PGW-C or invoke the Npcf_SMPolicyControl_UpdateNotify service operation after receiving an Rx AAR with the Specific-Action AVP set as a new value from the P-CSCF.

-	PCF/PCRF support to return a Rx AAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) specified in IETF RFC 6733 [25] or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065) specified in 3GPP TS 29.214 [7] to the P-CSCF when a response with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) is returned by the SMF/PGW-C. PCF/PCRF also support to return the Npcf_PolicyAuthorization_Create response to the P-CSCF when received the Npcf_SMPolicyControl_UpdateNotify response sent by the SMF/PGW-C.

-	SMF/PGW-C support to return a Gx RAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) specified in IETF RFC 6733 [25] or return the Npcf_SMPolicyControl_UpdateNotify response to the PCF/PCRF when the SMF/PGW-C is not reachable.

Figure 7.2.2-1 illustrates the quick restoration procedure of IMS terminating session in the EPC/5GC network below.

Figure 7.2.2-1: Quick restoration procedure of IMS terminating session

1-3.	The I-/S-CSCF in the terminating network receives an incoming INVITE request and routes it to the P-CSCF serving UE B. Then the P-CSCF routes the INVITE request to UE B.

4-5.	The P-CSCF does not receive any response from UE B after some time out, so the P-CSCF determines that the SIP signalling has already attempted the corresponding re-transmissions or retrials. Therefore, the P-CSCF sends a Rx AAR with the Specific-Action AVP set as a new value to PCRF or invokes the Npcf_PolicyAuthorization_Create service operation to the PCF to indicate starting health check. This action serves as a test to determine whether a failure exists in the EPC/5GC network.

6-7.	Upon receiving the AAR or Npcf_PolicyAuthorization_Create request, if the PCF/PCRF has identified that the SMF/PGW-C is reachable, then it sends a Gx RAR with Event-Trigger AVP set to ACCESS_NETWORK_INFO_REPORT (45) or invokes the Npcf_SMPolicyControl_UpdateNotify service operation to the SMF/PGW-C.

8-9.	If the SMF/PGW-C finds that there is no PDU session context of User B, it returns an Gx RAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) or the Npcf_SMPolicyControl_UpdateNotify service response to the PCF/PCRF.

10.	The PCF/PCRF returns either an Rx AAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065), or the Npcf_PolicyAuthorization_Create service response to the P-CSCF.

11-12.	The P-CSCF sends a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF. Then the I-/S-CSCF starts the HSS based P-CSCF Restoration procedure in EPC network specified in clause 5.4.2.1, or the UDM/HSS based P-CSCF Restoration procedure in 5GC network specified in clause 5.8.4 by sending a Cx SAR to the HSS, including a P-CSCF Restoration indication.

UE B will re-attach/re-registered to the EPC/5GC and register into the IMS quickly as specified in the HSS based or UDM/HSS based P-CSCF restoration procedure. Then UE B will receive the next incoming session for it has re-connected to the EPC/5GC network successfully. After the EPS bearer/QoS flow for IMS signalling is established, UE A initiates the initial registration procedure which specified in 3GPP TS 23.228 [27].

* * * Next Changes * * * *

8.2.2	Establishment of EPS bearer/QoS flow used for IMS signalling with PCRF/PCF bypassing

Before an IMS subscriber initiates an IMS registration procedure on an IMS network, an EPS bearer/QoS flow used for IMS signalling must be established. Figure 8.2.2-1 below illustrates the establishment procedure of EPS bearer/QoS flow for the IMS signalling in the case of all the PCRF/PCF instances failure.

Figure 8.2.2-1: EPS bearer/QoS flow establishment procedure for IMS signalling of PCRF/PCF bypass

1-2.	UE A sends a PDN Connectivity Request message or PDU Session Establishment Request to the MME/AMF to request a PDN connection. The MME/AMF allocates a bearer ID and sends a Create Session Request message to the PGW-C/SMF or invokes the Nsmf_PDUSession_CreateSMContext Request to SMF.

3.	The PGW-C/SMF shall provision a PDR containing an application ID to match RTCP RR Packets and associate it with a URR to request the UPF to report upon detecting those packets. If the PGW-C/SMF detects that all the PCRF/PCF instances in the network fail, then it uses local PCC policy to establish an EPS bearer/QoS flow for IMS signalling (QCI/5QI=5) for IMS signalling.

4.	The subsequent procedure of the EPS bearer/QoS flow establishment proceeds.

5.	After the EPS bearer/QoS flow for IMS signalling is established, UE A initiates the initial registration procedure which specified in 3GPP TS 23.228 [27].

* * * End of Changes * * * *