3GPP TSG-CT WG4 Meeting #131	C4-254186

Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 1002 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS Monitoring per QoS flow interworking with the NG-RAN |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_QME |  |  |  |  |  | Date: |  |  |  | 2025-10-03 |  |
| Category: | F |  |  |  |  |  |  | Release: |  |  |  | Rel-19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |
| Reason for change: | When QoS Monitoring for packet delay measurement per QoS Flow of a PDU session is required, as defined in 3GPP TS 23.501 Chapter 5.33.3, the SMF will instruct the NG-RAN by including QoS Monitoring Request in the QoS Flow Level QoS Parameters as below (see also 3GPP TS 38.413): QoS Monitoring Request; O; ENUMERATED (UL, DL, Both, …, stop); Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring.; YES. So, there is no instructions to the NG-RAN on how QoS Monitoring shall be performed. However, there are two distinct options are specified, which implies different requirements on the NG-RAN and PSA UPF (and I/V-UPF if the GTP GTP-U path-based solution is used): Option 1: using QoS Monitoring Packet together with various timestamps inserted into the PDU session container of the GTP-U packets as described in TS 23.501, clause 5.33.3.2, the PSA UPF does the final calculation using the inserted TimeStamps. Option 2: GTP-U path measurement based solution, as specified in clause 5.33.3.3 of TS 23.501, where RAN measures and provides the RAN part of UL/DL packet delay towards UPF in the UL PDU session container of the GTP-U packets of the respective QoS Flow via N3 and the UPF calculates the UL/DL packet delay by combining the received measurements of RAN part with the measurements of N3/N9 interface (when N9 is applicable) and report to SMF the accumulated delay value. See below the figure cited from clause 5.5.2.2 of 3GPP TS 38.415, where the yellow highlighted bits/fields are used for timestamp based solution (option1), and cyan highlighted bits/fields are used by accumulating packet delays from RAN delay, N3/N9 delay as reported in UL GTP-U packets (option 2) Correspondingly, the NG-RAN will provide the UPF distinct information based on their implementation of QoS Monitoring. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=1); QMP; DL Delay Ind.; UL Delay Ind.; SNP; 1. N3/N9 Delay Ind.; New IE Flag; QoS Flow Identifier; 1. DL Sending Time Stamp Repeated; 0 or 8. DL Received Time Stamp; 0 or 8. UL Sending Time Stamp; 0 or 8. DL Delay Result; 0 or 4. UL Delay Result; 0 or 4. UL QFI Sequence Number; 0 or 3. N3/N9 Delay Result; 0 or 4. New IE flag 7(E); New IE Flag 6; New IE Flag 5; New IE Flag 4; New IE Flag 3; New IE Flag 2; New IE Flag 1; New IE Flag 0; 0 or 1 New IE Flags Octet. Spare; D1 UL PDCPDelay Result Ind; 0 or 1. UL Congestion Information; 0 or 2. DL Congestion Information; 0 or 2. Padding; 0-3. In Core network, when the SMF requests the UPF to perform QoS monitoring for packet delay for a QoS flow of the PDU session, there are corresponding distinct instructions as specified in clause 5.24.4 Per QoS Flow Per UE QoS Monitoring and clause 5.24.5.3 QoS monitoring of a PDU session based on GTP-U path monitoring in 29.244, where for the latter option (GTP-U path based solution), the SMFwill indicate to the UPF (PSA) and for any intermediate UPF in the path of the PDU session that QoS monitoring is performed based on GTP-U path monitoring by setting the GTPUM flag to 1 in the Requested QoS Monitoring IE and include the Transport Delay Reporting IE in the UL PDR(s) associated to the corresponding QoS flow to be monitored. Pre-Rel19, it is assumed the SMF is locally configured how QoS monitoring is performed and if QoS monitoring is supported in the NG-RAN. In Rel-19, TEI19_QME specifies new requirements in 23.501, clause 5.45.1 as below that the SMF will learn if QoS monitoring for packet delay is supported for NG-RAN. Based on configuration, the AMF may know whether the QoS monitoring feature (i.e. each of Packet delay monitoring and/or Congestion monitoring and/or Available Bitrate monitoring) is supported by all NG RAN nodes in an area. The AMF shall, when configured, inform the SMF about the support of the QoS monitoring feature by sending the parameter NG RAN QoS monitoring capability during PDU Session establishment, PDU Session modification, Service request and UE mobility procedures. The SMF determines whether QoS monitoring is possible or not for the PDU Session based on whether the NG RAN QoS monitoring capability is received (or not) from the AMF and the QoS monitoring capability of the UPF. However, it is not sufficent for the SMF to determine which option shall be used to instruct correctly to the UPF (including PSA UPF and I-UPF (if applicable) to perform the QoS monitoring for packet delay. Though the AMF may be further configured which option the NG-RAN supports, this will increase further OPEX, which is suboptimal. It is propose to introduce a mechanism, that the SMF instructs the PSA UPF to perform QoS monitoring for QoS flow using timestamp based option, and in addition, the SMF requests the PSA UPF to report if it receive the timestamps or UL/DL delay results, or none of them, based on this report, the SMF may request the PSA UPF and I-UPF if applicable, as well as the I-SMF if applicable, to perform QoS monitoring for packet delay properly. |
| Summary of change: | Speciy a mechanism and relevant protocol impact to request the PSA UPF to report if it receive the timestamps or UL/DL delay results, or none of them. Based on this report, the SMF may request the PSA UPF and I-UPF if applicable, as well as the I-SMF if applicable, to perform QoS monitoring for packet delay properly |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | QoS monitoring for packet delay for QoS flow(s) of a PDU session may not work if the SMF is not correctly configured with supported QoS monitoring methods for all the NG-RAN entities, and such local configuration largely increases the OPEX. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 5.24.4.x, 7.5.2.9, 7.4.8.6, 8.1.2, 8.2.25, 8.2.231, 8.2.xxx |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 5.24.4.x	Packet Delay Measurement Information Reporting

To reduce the configuration on the support of QoS monitoring for packet dalay in the NG-RANs, the SMF and the PSA UPF may optionally support the procedure specified in this clause.

The UPF shall set the RPDMI flag in the UP Function Features IE if it supports to report the Packet Delay Measurement Information when requested (see clause 8.2.25). If so, when the SMF instructs the UPF to perform per QoS flow QoS monitoring for packet delay as specified in clause 5.24.4.2 if it is required, the SMF may in addition request the UPF to report the Packet Delay Measurement Information by setting the RPDMI flag in the Measurement Indication IE in the QoS Monitoring per QoS flow Control Information.

The UPF shall then include the Packet Delay Measurement Information as part of the QoS Monitoring Report IE, where the Packet Delay Measurement Information indicates if the PSA UPF has received the timeStamps, i.e. the DL Sending Time Stamp Repeated, the DL Received Time Stamp and the UL Sending Time Stamp, or the delay results, i.e. the DL Delay Result and the UL Delay Result, or neither timestamps nor delay results received, in the PDU Session Container extension header of the UL GTP-U packets. (See also clause 5.5.2.2 of 3GPP TS 38.415 [34]. The UPF shall include the Packet Delay Measurement Information in the subsequent QoS Monitoring Reports when there is a change, e.g. during an inter NG-RAN mobility procedures.

Upon receiving the Packet Delay Measurement Information, the SMF may determine:

-	the QoS Monitoring for packet delay is not supported in the NG-RAN if the UPF reports that neither timestamps nor delay results are received; and

-	the QoS monitoring for packet delay based on GTP-U path monitoring as specified in clause 5.24.5.3 shall be requested towards the PSA UPF and I-UPF (possibly via the I-SMF if applicable) if the UPF reports that it has received the delay results; and

-	to continue the QoS Monitoring for packet delay based on the timestamps as specified in clause 5.24.4 if the UPF reports that it has received the timestamps.

# *** Next Change ***

#### 7.5.2.9	Create SRR IE within PFCP Session Establishment Request

The Create SRR grouped IE shall be encoded as shown in Figure 7.5.2.9-1.

Table 7.5.2.9-1: Create SRR IE within PFCP Session Establishment Request

| Octet 1 and 2 |  |  | Create SRR IE Type = 212 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| SRR ID | M | This IE shall uniquely identify the SRR among all the SRRs configured for this PFCP session. |  | - | - | - | X | - | SRR ID |
| Access Availability Control Information | C | This IE shall be present if the UPF needs to report when an access type becomes available or not available (see clause 5.20.4.2). |  | - | - | - | X | - | Access Availability Control Information |
| QoS Monitoring per QoS flow Control Information | C | This IE shall be present if per QoS Flow QoS monitoring reporting is triggered. Several IEs within the same IE type may be present to represent a list of QoS Monitoring per QoS flow Control Information for different QoS flows, or for different QoS parameters of a same QoS flow (see clause 5.39.2). |  | - | - | - | X | - | QoS Monitoring per QoS flow Control Information |
| Direct Reporting Information | C | This IE shall be present if the UPF indicated support of the DRQOS feature and if the QoS monitoring events shall be reported by the UPF directly to the consumer (e.g. to the Local NEF or AF), or both to the SMF and the consumer (e.g. to the Local NEF/AF) (see clause 5.33.5). |  | - | - | - | X | - | Direct Reporting Information |
| Traffic Parameter Measurement Control Information | C | This IE shall be present if the SMF requests the UPF to perform Traffic Parameter measurement for one or more QoS Flows. See clauses 5.37 and 5.38.4. Several IEs within the same IE type may be present to represent a list of Traffic Parameter Measurement Control Information for different QoS flows or different traffic parameters. |  | - | - | - | X | - | Traffic Parameter Measurement Control Information |
| Reporting Control Information | C | This IE shall be present if any specific reporting control information shall apply for the SRR, e.g. when per QoS flow QoS monitoring reporting is activated for UE level measurements. |  | - | - | - | X | - | Reporting Control Information |

The Access Availability Control Information IE shall be encoded as shown in Table 7.5.2.9-2.

# Table 7.5.2.9-2: Access Availability Control Information

| Octet 1 and 2 |  |  | Access Availability Control Information = 216 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Requested Access Availability Information | M | This IE shall indicate the requested information to be reported. |  | - | - | - | X | - | Requested Access Availability Information |

The QoS Monitoring per QoS flow Control Information IE shall be encoded as shown in Table 7.5.2.9-3.

# Table 7.5.2.9-3: QoS Monitoring per QoS flow Control Information

| Octet 1 and 2 |  |  | QoS Monitoring per QoS flow Control Information = 242 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| QFI | M | This IE shall identify the QoS Flow Identifier for which QoS monitoring is required. Several IEs within the same IE type may be present to represent different QoS flows. |  | - | - | - | X | - | QFI |
| Requested QoS Monitoring | M | This IE shall indicate the QoS parameter(s) to be measured. For packet delay monitoring, it shall indicate whether the uplink, downlink and/or round trip packet delay between the UE and the UPF (PSA) shall be monitored, and whether QoS monitoring is performed based on GTP-U path monitoring. For congestion information monitoring, it shall indicate whether the uplink and/or downlink congestion information shall be monitored. For data rate monitoring, it shall indicate whether the uplink and/or downlink data rate shall be monitored. For available bitrate monitoring, it shall indicate whether the uplink and/or downlink available bitrate shall be monitored. |  | - | - | - | X | - | Requested QoS Monitoring |
| Reporting Frequency | M | This IE shall indicate the type of reporting, i.e. event triggered or periodic QoS monitoring reporting. |  | - | - | - | X | - | Reporting Frequency |
| Packet Delay Thresholds | C | This IE shall be present if the Requested QoS Monitoring indicates packet delay monitoring, event triggered QoS monitoring reporting is used and reporting is required upon reaching a delay threshold. When present, it shall indicate the packet delay after which the UP function shall report packet delay monitoring result for this SRR. |  | - | - | - | X | - | Packet Delay Thresholds |
| Minimum Wait Time | C | This IE shall be present if event triggered QoS monitoring reporting is required. When present, it shall indicate the minimum waiting time between two consecutive reports after which the UP function may report new QoS monitoring result for this SRR. |  | - | - | - | X | - | Minimum Wait Time |
| Measurement Period | C | This IE shall be present if Periodic QoS monitoring reporting or, for packet delay monitoring, if Event Triggered QoS monitoring reporting is required. When present, it shall indicate the period for generating and reporting QoS monitoring reports (if periodic QoS monitoring reporting is required) and, for packet delay monitoring, the period used for detecting and reporting a QoS monitoring measurement failure (if Periodic QoS monitoring reporting or Event Triggered QoS monitoring reporting is required). (NOTE 1) (NOTE 2) |  | - | - | - | X | - | Measurement Period |
| Reporting Suggestion Info | C | This IE should be present if the SMF receives it in a subscription to UPF QoS Monitoring event sent by the consumer via SMF. When present, it shall indicate whether the event report can be delayed, i.e. it is delay tolerant. |  | - | - | - | X | - | Reporting Suggestion Info |
| Measurement Indication | C | This IE shall be included if at least one of the flags is set to "1". - DQFI (Default QoS Flow Indication): this bit shall be set to "1" if the QoS flow to be monitored is associated with the default QoS rule and the UPF shall include the same indication in the corresponding QoS Monitoring Measurement report. See also clause 4.15.4.5.1 of 3GPP TS 23.502 [29]. - RPDMI (Report Packet Delay Measurement Information): this bit shall be set "1" if the SMF requests the UPF to report the Packet Delay Measurement Information as specified in clause 5.24.4.x. |  | - | - | - | X | - | Measurement Indication |
| Reporting Thresholds | C | This IE shall be present if the Reporting Frequency indicates event triggered QoS monitoring reporting is used and the Requested QoS Monitoring indicates UL and/or DL congestion information monitoring or UL and/or DL data rate monitoring. When present, it shall indicate the Reporting threshold to be used by the UPF for sending event reports. |  | - | - | - | X | - | Reporting Thresholds |
| Remaining Data Reporting Indication | O | When present, this IE shall include the indication on how to handle any QoS monitoring result not yet reported when the related N4 session is released. |  | - | - | - | X | - | Remaining Data Reporting Indication |
| NOTE 1: For packet delay monitoring, if no time stamp is received in uplink packet for a delay exceeding the Measurement Period (i.e. no measurement result is available in the UPF within the reporting period), the UP function shall generate a QoS monitoring report indicating a packet delay measurement failure to the SMF or Local NEF or AF. NOTE 2: For packet delay monitoring, SMF implementations complying with an earlier version of the specification may not support providing the Measurement Period if the Reporting Frequency IE is set to Event Triggered QoS monitoring reporting. If so, it is implementation specific whether/how the UPF generates QoS monitoring measurement failure report. |  |  |  |  |  |  |  |  |  |

The Direct Reporting Information IE shall be encoded as shown in Table 7.5.2.9-4.

# Table 7.5.2.9-4: Direct Reporting Information

| Octet 1 and 2 |  |  | Direct Reporting Information = 295 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Event Notification URI | M | This IE shall contain the notification URI to be used for sending the UPF event notifications. |  | - | - | - | X | - | Event Notification URI |
| Notification Correlation ID | C | This IE shall indicate the notification correlation id to be included in the UPF event notifications. It shall be present if it is available. |  | - | - | - | X | - | Notification Correlation ID |
| Reporting Flags | C | This IE shall be included if at least one of the flags is set to "1". - DUPL (Duplicate notification): this IE shall be set to "1" if event notifications shall be sent over N4 and directly to the notification URI indicated in the Event Notification URI IE. When this flag is set to "0", this indicates that the UPF shall only send the event notifications directly to the notification URI indicated in the Event Notification URI IE. |  | - | - | - | X | - | Reporting Flags |

The Traffic Parameter Measurement Control Information IE shall be encoded as shown in Table 7.5.2.9-5.

Table 7.5.2.9-5.: Traffic Parameter Measurement Control Information IE

| Octet 1 and 2 |  |  | Traffic Parameter Measurement Control Information IE Type = 323 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| QFI | M | This IE shall identify the QoS Flow Identifier for which traffic parameter is requested to be measured. Several IEs within the same IE type may be present to represent different QoS flows. |  | - | - | - | X | - | QFI |
| Traffic Parameter Measurement Indication | M | This IE shall be included to indicate the requested traffic parameter measurement(s), as follows: - ULPMI (UL Periodicity Measurement Indication): this bit shall be set to "1" to request the UPF to measure the UL Periodicity if the SMF does not receive the UL Periodicity from the PCF. - DLPMI (DL Periodicity Measurement Indication): this bit shall be set to "1" to request the UPF to measure the DL Periodicity if the SMF does not receive the DL Periodicity from the PCF. - N6JMI (N6 Jitter Measurement Indication): this bit shall be set to "1" to request the UPF to measure N6 Jitter. |  | - | - | - | X | - | Traffic Parameter Measurement Indication |
| Measurement Period | C | This IE shall be present if Periodic traffic parameter measurement reporting is required. When present, it shall indicate the period for generating and reporting traffic parameter measurement reports. (NOTE) |  | - | - | - | X | - | Measurement Period |
| DL Periodicity | C | This IE shall be present, if N6 jitter measurement is requested and if the DL Periodicity is available. When available, it shall contain the DL Periodicity to perform Jitter measurement. |  | - | - | - | X | - | DL Periodicity |
| Traffic Parameter Threshold | C | This IE shall be present if Traffic Parameter measurement reporting is required only upon reaching a threshold. When present, and if N6 jitter measurement is requested, it shall contain the Downlink N6 Jitter threshold after which the UP function shall report N6 Jitter measurement to the SMF. (NOTE) |  | - | - | - | X | - | Traffic Parameter Threshold |
| NOTE: Either a Measurement Period or a Traffic Parameter Threshold shall be present. |  |  |  |  |  |  |  |  |  |

# *** Next Change ***

#### 7.5.8.6	Session Report IE within PFCP Session Report Request

The Session Report grouped IE shall be encoded as shown in Table 7.5.8.6-1.

Table 7.5.8.6-1: Session Report IE within PFCP Session Report Request

| Octet 1 and 2 |  |  | Session Report IE Type = 214 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| SRR ID | M | This IE shall identify the SRR for which usage is reported. |  | - | - | - | X | - | SRR ID |
| Access Availability Report | C | This IE shall be present if change of access availability needs to be reported. When present, this IE shall indicate an access type and whether the access type has become available or unavailable. |  | - | - | - | X | - | Access Availability Report |
| QoS Monitoring Report | C | This IE shall be present if there is at least a QoS Monitoring Report that needs to be reported. Several IEs within the same IE type may be present to represent a list of QoS Monitoring Reports, e.g. for different QoS flows. |  | - | - | - | X | - | QoS Monitoring Report |
| Traffic Parameter Measurement Report | C | This IE shall be present if there is at least a Traffic Parameter Measurement Report that needs to be reported. Several IEs within the same IE type may be present to represent a list of Traffic Parameter Measurement Reports, e.g., for different QoS flows. |  | - | - | - | X | - | Traffic Parameter Measurement Report |

The Access Availability Report IE shall be encoded as shown in Table 7.5.8.2-2.

Table 7.5.8.6-2: Access Availability Report IE

| Octet 1 and 2 |  |  | Access Availability Report IE Type = 218 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Access Availability Information | M | This IE shall indicate an access type and whether the access type has become available or not available. Up to two IEs with the same IE type may be present to report access availability changes for 3GPP and non-3GPP accesses. |  | - | - | - | X | - | Access Availability Information |

The QoS Monitoring Report IE shall be encoded as shown in Table 7.5.8.6-3.

Table 7.5.8.6-3: QoS Monitoring Report IE

| Octet 1 and 2 |  |  | QoS Monitoring Report IE Type = 247 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| QFI | M | This IE shall identify the QoS Flow Identifier for which QoS monitoring is reported. |  | - | - | - | X | - | QFI |
| QoS Monitoring Measurement | M | This IE shall contain the measured packet delay(s) or the indication of packet delay measurement failure. |  | - | - | - | X | - | QoS Monitoring Measurement |
| Time Stamp | M | This IE shall provide the timestamp when the collection of the information in this report was generated. |  | - | - | - | X | - | Time Stamp |
| Start Time | O | When present, this IE shall provide the timestamp when the collection of the information in this report was started. |  | - | - | - | X | - | Start Time |
| Packet Delay Measurement Information | C | This IE shall be present to report the Packet Delay Measurement Information if the SMF has requested it by setting the RPDMI flag in the Measurement Indication IE to "1". See also clause 5.24.4.x. |  | - | - | - | X | - | Packet Delay Measurement Information |

The Traffic Parameter Measurement Report IE shall be encoded as shown in Table 7.5.8.6-4.

Table 7.5.8.6-4: Traffic Parameter Measurement Report IE

| Octet 1 and 2 |  |  | Traffic Parameter Measurement Report Type = 324 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| QFI | M | This IE shall identify the QoS Flow Identifier for which traffic parameter measurement is reported. |  | - | - | - | X | - | QFI |
| N6 Jitter Measurement | C | This IE shall be present when N6 Jitter measurement available. The UPF shall include DL Periodicity field in the N6 Jitter Measurement IE. |  | - | - | - | X | - | N6 Jitter Measurement |
| UL Periodicity | C | The IE shall be included if the SMF requested to measure UL Periodicity and if this information is available. |  | - | - | - | X | - | UL Periodicity |
| Time Stamp | M | This IE shall provide the timestamp when the collection of the information in this report was generated. |  | - | - | - | X | - | Time Stamp |
| Start Time | O | When present, this IE shall provide the timestamp when the collection of the information in this report was started. |  | - | - | - | X | - | Start Time |

# *** Next Change ***

### 8.1.2	Information Element Types

A PFCP message may contain several IEs. In order to have forward compatible type definitions for the PFCP IEs, all of them shall be TLV (Type, Length, Value) coded. PFCP IE type values are specified in the Table 8.1.2-1.

The 3rd column of this table specifies if the IE is either Extendable or has a variable length or a fixed length and a reference to the clause where the IE is specified:

-	Fixed Length: the IE has a fixed set of fields, and a fixed number of octets;

-	Variable Length: the IE has a fixed set of fields, and has a variable number of octets.
For example, the last octets may be numbered similar to "5 to (n+4)". In this example, if the value of the length field, n, is 0, then the last field is not present;

-	Extendable: the IE has a variable number of fields, and has a variable number of octets.
The last fields are typically specified with the statement: "These octet(s) is/are present only if explicitly specified". The legacy receiving entity shall ignore the unknown octets.

The 4th column of this table indicates the number of fixed Octets the IE contained when the IE was first defined in the specification, which shall be an integer value reflecting the minimum length of fixed octets defined for the IE.

An IE of any of the above types may have a null length as specified in clause 5.6.3. This shall not be considered as an error by the receiving PFCP entity.

In order to improve the efficiency of troubleshooting, it is recommended that the IEs should be arranged in the signalling messages as well as in the grouped IEs, according to the order the IEs are listed in the message definition table or grouped IE definition table in clause 7. However the receiving entity shall be prepared to handle the messages with IEs in any order.

Within IEs, certain fields may be described as spare. These bits shall be transmitted with the value set to "0". To allow for future features, the receiver shall not evaluate these bits.

# Table 8.1.2-1: Information Element Types

| IE Type value (Decimal) | Information elements | Comment / Reference | Number of Fixed Octets |
| --- | --- | --- | --- |
| 0 | Reserved |  |  |
| 1 | Create PDR | Extendable / Table 7.5.2.2-1 | Not Applicable |
| 2 | PDI | Extendable / Table 7.5.2.2-2 | Not Applicable |
| 3 | Create FAR | Extendable / Table 7.5.2.3-1 | Not Applicable |
| 4 | Forwarding Parameters | Extendable / Table 7.5.2.3-2 | Not Applicable |
| 5 | Duplicating Parameters | Extendable / Table 7.5.2.3-3 | Not Applicable |
| 6 | Create URR | Extendable / Table 7.5.2.4-1 | Not Applicable |
| 7 | Create QER | Extendable / Table 7.5.2.5-1 | Not Applicable |
| 8 | Created PDR | Extendable / Table 7.5.3.2-1 | Not Applicable |
| 9 | Update PDR | Extendable / Table 7.5.4.2-1 | Not Applicable |
| 10 | Update FAR | Extendable / Table 7.5.4.3-1 | Not Applicable |
| 11 | Update Forwarding Parameters | Extendable / Table 7.5.4.3-2 | Not Applicable |
| 12 | Update BAR (PFCP Session Report Response) | Extendable / Table 7.5.9.2-1 | Not Applicable |
| 13 | Update URR | Extendable / Table 7.5.4.4 | Not Applicable |
| 14 | Update QER | Extendable / Table 7.5.4.5 | Not Applicable |
| 15 | Remove PDR | Extendable / Table 7.5.4.6 | Not Applicable |
| 16 | Remove FAR | Extendable / Table 7.5.4.7 | Not Applicable |
| 17 | Remove URR | Extendable / Table 7.5.4.8 | Not Applicable |
| 18 | Remove QER | Extendable / Table 7.5.4.9 | Not Applicable |
| 19 | Cause | Fixed / Clause 8.2.1 | 1 |
| 20 | Source Interface | Extendable / Clause 8.2.2 | 1 |
| 21 | F-TEID | Extendable / Clause 8.2.3 | 1 |
| 22 | Network Instance | Variable Length / Clause 8.2.4 | Not Applicable |
| 23 | SDF Filter | Extendable / Clause 8.2.5 | 2 |
| 24 | Application ID | Variable Length / Clause 8.2.6 | Not Applicable |
| 25 | Gate Status | Extendable / Clause 8.2.7 | 1 |
| 26 | MBR | Extendable / Clause 8.2.8 | 10 |
| 27 | GBR | Extendable / Clause 8.2.9 | 10 |
| 28 | QER Correlation ID | Extendable / Clause 8.2.10 | 4 |
| 29 | Precedence | Extendable / Clause 8.2.11 | 4 |
| 30 | Transport Level Marking | Extendable / Clause 8.2.12 | 2 |
| 31 | Volume Threshold | Extendable /Clause 8.2.13 | 1 |
| 32 | Time Threshold | Extendable /Clause 8.2.14 | 4 |
| 33 | Monitoring Time | Extendable /Clause 8.2.15 | 4 |
| 34 | Subsequent Volume Threshold | Extendable /Clause 8.2.16 | 1 |
| 35 | Subsequent Time Threshold | Extendable /Clause 8.2.17 | 4 |
| 36 | Inactivity Detection Time | Extendable /Clause 8.2.18 | 4 |
| 37 | Reporting Triggers | Extendable /Clause 8.2.19 | 2 |
| 38 | Redirect Information | Extendable /Clause 8.2.20 | 3 |
| 39 | Report Type | Extendable / Clause 8.2.21 | 1 |
| 40 | Offending IE | Fixed / Clause 8.2.22 | 2 |
| 41 | Forwarding Policy | Extendable / Clause 8.2.23 | 1 |
| 42 | Destination Interface | Extendable / Clause 8.2.24 | 1 |
| 43 | UP Function Features | Extendable / Clause 8.2.25 | 1 |
| 44 | Apply Action | Extendable / Clause 8.2.26 | 1 |
| 45 | Downlink Data Service Information | Extendable / Clause 8.2.27 | 1 |
| 46 | Downlink Data Notification Delay | Extendable / Clause 8.2.28 | 1 |
| 47 | DL Buffering Duration | Extendable / Clause 8.2.29 | 1 |
| 48 | DL Buffering Suggested Packet Count | Variable / Clause 8.2.30 | Not Applicable |
| 49 | PFCPSMReq-Flags | Extendable / Clause 8.2.31 | 1 |
| 50 | PFCPSRRsp-Flags | Extendable / Clause 8.2.32 | 1 |
| 51 | Load Control Information | Extendable / Table 7.5.3.3-1 | Not Applicable |
| 52 | Sequence Number | Fixed Length / Clause 8.2.33 | 4 |
| 53 | Metric | Fixed Length / Clause 8.2.34 | 1 |
| 54 | Overload Control Information | Extendable / Table 7.5.3.4-1 | Not Applicable |
| 55 | Timer | Extendable / Clause 8.2 35 | 1 |
| 56 | PDR ID | Extendable / Clause 8.2 36 | 2 |
| 57 | F-SEID | Extendable / Clause 8.2 37 | 9 |
| 58 | Application ID's PFDs | Extendable / Table 7.4.3.1-2 | Not Applicable |
| 59 | PFD context | Extendable / Table 7.4.3.1-3 | Not Applicable |
| 60 | Node ID | Extendable / Clause 8.2.38 | 1 |
| 61 | PFD contents | Extendable / Clause 8.2.39 | 2 |
| 62 | Measurement Method | Extendable / Clause 8.2.40 | 1 |
| 63 | Usage Report Trigger | Extendable / Clause 8.2.41 | 2 |
| 64 | Measurement Period | Extendable / Clause 8.2.42 | 4 |
| 65 | FQ-CSID | Extendable / Clause 8.2.43 | 1 |
| 66 | Volume Measurement | Extendable / Clause 8.2.44 | 1 |
| 67 | Duration Measurement | Extendable / Clause 8.2.45 | 4 |
| 68 | Application Detection Information | Extendable / Table 7.5.8.3-2 | Not Applicable |
| 69 | Time of First Packet | Extendable / Clause 8.2.46 | 4 |
| 70 | Time of Last Packet | Extendable / Clause 8.2.47 | 4 |
| 71 | Quota Holding Time | Extendable / Clause 8.2.48 | 4 |
| 72 | Dropped DL Traffic Threshold | Extendable / Clause 8.2.49 | 1 |
| 73 | Volume Quota | Extendable / Clause 8.2.50 | 1 |
| 74 | Time Quota | Extendable / Clause 8.2.51 | 4 |
| 75 | Start Time | Extendable / Clause 8.2.52 | 4 |
| 76 | End Time | Extendable / Clause 8.2.53 | 4 |
| 77 | Query URR | Extendable / Table 7.5.4.10-1 | Not Applicable |
| 78 | Usage Report (Session Modification Response) | Extendable / Table 7.5.5.2-1 | Not Applicable |
| 79 | Usage Report (Session Deletion Response) | Extendable / Table 7.5.7.2-1 | Not Applicable |
| 80 | Usage Report (Session Report Request) | Extendable / Table 7.5.8.3-1 | Not Applicable |
| 81 | URR ID | Extendable / Clause 8.2.54 | 4 |
| 82 | Linked URR ID | Extendable / Clause 8.2.55 | 4 |
| 83 | Downlink Data Report | Extendable / Table 7.5.8.2-1 | Not Applicable |
| 84 | Outer Header Creation | Extendable / Clause 8.2.56 | 2 |
| 85 | Create BAR | Extendable / Table 7.5.2.6-1 | Not Applicable |
| 86 | Update BAR (Session Modification Request) | Extendable / Table 7.5.4.11-1 | Not Applicable |
| 87 | Remove BAR | Extendable / Table 7.5.4.12-1 | Not Applicable |
| 88 | BAR ID | Extendable / Clause 8.2.57 | 1 |
| 89 | CP Function Features | Extendable / Clause 8.2.58 | 1 |
| 90 | Usage Information | Extendable / Clause 8.2.59 | 1 |
| 91 | Application Instance ID | Variable Length / Clause 8.2.60 | Not Applicable |
| 92 | Flow Information | Extendable / Clause 8.2.61 | 3 |
| 93 | UE IP Address | Extendable / Clause 8.2.62 | 1 |
| 94 | Packet Rate | Extendable / Clause 8.2.63 | 1 |
| 95 | Outer Header Removal | Extendable / Clause 8.2.64 | 1 |
| 96 | Recovery Time Stamp | Extendable / Clause 8.2.65 | 4 |
| 97 | DL Flow Level Marking | Extendable / Clause 8.2.66 | 1 |
| 98 | Header Enrichment | Extendable / Clause 8.2.67 | 1 |
| 99 | Error Indication Report | Extendable / Table 7.5.8.4-1 | Not Applicable |
| 100 | Measurement Information | Extendable / Clause 8.2.68 | 1 |
| 101 | Node Report Type | Extendable / Clause 8.2.69 | 1 |
| 102 | User Plane Path Failure Report | Extendable / Table 7.4.5.1.2-1 | Not Applicable |
| 103 | Remote GTP-U Peer | Extendable / Clause 8.2.70 | 1 |
| 104 | UR-SEQN | Fixed Length / Clause 8.2.71 | 4 |
| 105 | Update Duplicating Parameters | Extendable / Table 7.5.4.3-3 | Not Applicable |
| 106 | Activate Predefined Rules | Variable Length / Clause 8.2.72 | Not Applicable |
| 107 | Deactivate Predefined Rules | Variable Length / Clause 8.2.73 | Not Applicable |
| 108 | FAR ID | Extendable / Clause 8.2.74 | 4 |
| 109 | QER ID | Extendable / Clause 8.2.75 | 4 |
| 110 | OCI Flags | Extendable / Clause 8.2.76 | 1 |
| 111 | PFCP Association Release Request | Extendable / Clause 8.2.77 | 1 |
| 112 | Graceful Release Period | Extendable / Clause 8.2.78 | 1 |
| 113 | PDN Type | Extendable / Clause 8.2.79 | 1 |
| 114 | Failed Rule ID | Extendable / Clause 8.2.80 | 1 |
| 115 | Time Quota Mechanism | Extendable / Clause 8.2.81 | 1 |
| 116 | Reserved |  |  |
| 117 | User Plane Inactivity Timer | Extendable /Clause 8.2.83 | 4 |
| 118 | Aggregated URRs | Extendable / Table 7.5.2.4-2 | Not Applicable |
| 119 | Multiplier | Fixed / Clause 8.2.84 | 12 |
| 120 | Aggregated URR ID | Fixed / Clause 8.2.85 | 4 |
| 121 | Subsequent Volume Quota | Extendable / Clause 8.2.86 | 1 |
| 122 | Subsequent Time Quota | Extendable / Clause 8.2.87 | 4 |
| 123 | RQI | Extendable / Clause 8.2.88 | 1 |
| 124 | QFI | Extendable / Clause 8.2.89 | 1 |
| 125 | Query URR Reference | Extendable / Clause 8.2.90 | 4 |
| 126 | Additional Usage Reports Information | Extendable / Clause 8.2.91 | 2 |
| 127 | Create Traffic Endpoint | Extendable / Table 7.5.2.7 | Not Applicable |
| 128 | Created Traffic Endpoint | Extendable / Table 7.5.3.5 | Not Applicable |
| 129 | Update Traffic Endpoint | Extendable / Table 7.5.4.13 | Not Applicable |
| 130 | Remove Traffic Endpoint | Extendable / Table 7.5.4.14 | Not Applicable |
| 131 | Traffic Endpoint ID | Extendable / Clause 8.2.92 | 1 |
| 132 | Ethernet Packet Filter | Extendable / Table 7.5.2.2-3 | Not Applicable |
| 133 | MAC address | Extendable / Clause 8.2.93 | 1 |
| 134 | C-TAG | Extendable / Clause 8.2.94 | 3 |
| 135 | S-TAG | Extendable / Clause 8.2.95 | 3 |
| 136 | Ethertype | Extendable / Clause 8.2.96 | 2 |
| 137 | Proxying | Extendable / Clause 8.2.97 | 1 |
| 138 | Ethernet Filter ID | Extendable / Clause 8.2.98 | 4 |
| 139 | Ethernet Filter Properties | Extendable / Clause 8.2.99 | 1 |
| 140 | Suggested Buffering Packets Count | Extendable / Clause 8.2.100 | 1 |
| 141 | User ID | Extendable / Clause 8.2.101 | 1 |
| 142 | Ethernet PDU Session Information | Extendable / Clause 8.2.102 | 1 |
| 143 | Ethernet Traffic Information | Extendable / Table 7.5.8.3-3 | Not Applicable |
| 144 | MAC Addresses Detected | Extendable / Clause 8.2.103 | 7 |
| 145 | MAC Addresses Removed | Extendable / Clause 8.2.104 | 7 |
| 146 | Ethernet Inactivity Timer | Extendable / Clause 8.2.105 | 4 |
| 147 | Additional Monitoring Time | Extendable / Table 7.5.2.4-3 | Not Applicable |
| 148 | Event Quota | Extendable / Clause 8.2.112 | 4 |
| 149 | Event Threshold | Extendable / Clause 8.2.113 | 4 |
| 150 | Subsequent Event Quota | Extendable / Clause 8.2.106 | 4 |
| 151 | Subsequent Event Threshold | Extendable / Clause 8.2.107 | 4 |
| 152 | Trace Information | Extendable / Clause 8.2.108 | 7 |
| 153 | Framed-Route | Variable Length / Clause 8.2.109 | Not Applicable |
| 154 | Framed-Routing | Fixed Length / Clause 8.2.110 | 4 |
| 155 | Framed-IPv6-Route | Variable Length / Clause 8.2.111 | Not Applicable |
| 156 | Time Stamp | Extendable / Clause 8.2.114 | 4 |
| 157 | Averaging Window | Extendable /Clause 8.2.115 | 4 |
| 158 | Paging Policy Indicator | Extendable / Clause 8.2.116 | 1 |
| 159 | APN/DNN | Variable Length / Clause 8.2.117 | Not Applicable |
| 160 | 3GPP Interface Type | Extendable / Clause 8.2.118 | 1 |
| 161 | PFCPSRReq-Flags | Extendable / Clause 8.2.119 | 1 |
| 162 | PFCPAUReq-Flags | Extendable / Clause 8.2.120 | 1 |
| 163 | Activation Time | Extendable / Clause 8.2.121 | 4 |
| 164 | Deactivation Time | Extendable / Clause 8.2.122 | 4 |
| 165 | Create MAR | Extendable / Table 7.5.2.8-1 | Not Applicable |
| 166 | 3GPP Access Forwarding Action Information | Extendable / Table 7.5.2.8-2 | Not Applicable |
| 167 | Non-3GPP Access Forwarding Action Information | Extendable / Table 7.5.2.8-3 | Not Applicable |
| 168 | Remove MAR | Extendable / Table 7.5.4.15-1 | Not Applicable |
| 169 | Update MAR | Extendable / Table 7.5.4.16-1 | Not Applicable |
| 170 | MAR ID | Extendable / Clause 8.2.123 | 2 |
| 171 | Steering Functionality | Extendable / Clause 8.2.124 | 1 |
| 172 | Steering Mode | Extendable / Clause 8.2.125 | 1 |
| 173 | Weight | Fixed / Clause 8.2.126 | 1 |
| 174 | Priority | Extendable / Clause 8.2.127 | 1 |
| 175 | Update 3GPP Access Forwarding Action Information | Extendable / Table 7.5.4.16-2 | Not Applicable |
| 176 | Update Non 3GPP Access Forwarding Action Information | Extendable / Table 7.5.4.16-3 | Not Applicable |
| 177 | UE IP address Pool Identity | Extendable / Clause 8.2.128 | 2 |
| 178 | Alternative SMF IP Address | Extendable / Clause 8.2.129 | 1 |
| 179 | Packet Replication and Detection Carry-On Information | Extendable / Clause 8.2.130 | 1 |
| 180 | SMF Set ID | Extendable / Clause 8.2.131 | 1 |
| 181 | Quota Validity Time | Extendable / Clause 8.2.132 | 4 |
| 182 | Number of Reports | Fixed / Clause 8.2.133 | 2 |
| 183 | PFCP Session Retention Information (within PFCP Association Setup Request) | Extendable / Table 7.4.4.1-2 | 1 |
| 184 | PFCPASRsp-Flags | Extendable / Clause 8.2.134 | 1 |
| 185 | CP PFCP Entity IP Address | Extendable / Clause 8.2.135 | 1 |
| 186 | PFCPSEReq-Flags | Extendable / Clause 8.2.136 | 1 |
| 187 | User Plane Path Recovery Report | Extendable / Table 7.4.5.1.3-1 | Not Applicable |
| 188 | IP Multicast Addressing Info within PFCP Session Establishment Request | Extendable / Clause 7.5.2.2-4 | Not Applicable |
| 189 | Join IP Multicast Information IE within Usage Report | Extendable / Table 7.5.8.3-4 | Not Applicable |
| 190 | Leave IP Multicast Information IE within Usage Report | Extendable / Table 7.5.8.3-5 | Not Applicable |
| 191 | IP Multicast Address | Extendable / Clause 8.2.137 | 1 |
| 192 | Source IP Address | Extendable / Clause 8.2.138 | 1 |
| 193 | Packet Rate Status | Extendable / Clause 8.2.139 | 1 |
| 194 | Create Bridge/Router Info | Extendable / Clause 8.2.140 | 1 |
| 195 | Created Bridge/Router Info | Extendable / Table 7.5.3.6-1 | Not Applicable |
| 196 | Port Number | Fixed Length / Clause 8.2.141 | 4 |
| 197 | NW-TT Port Number | Fixed Length / Clause 8.2.142 | 4 |
| 198 | 5GS User Plane Node ID | Extendable / Clause 8.2.143 | Not Applicable |
| 199 | TSC Management Information IE within PFCP Session Modification Request | Extendable / Table 7.5.4.18-1 | Not Applicable |
| 200 | TSC Management Information IE within PFCP Session Modification Response | Extendable / Table 7.5.5.3-1 | Not Applicable |
| 201 | TSC Management Information IE within PFCP Session Report Request | Extendable / Table 7.5.8.5-1 | Not Applicable |
| 202 | Port Management Information Container | Variable Length / Clause 8.2.144 | Not Applicable |
| 203 | Clock Drift Control Information | Extendable / Table 7.4.4.1.2-1 | Not Applicable |
| 204 | Requested Clock Drift Information | Extendable / Clause 8.2.145 | 1 |
| 205 | Clock Drift Report | Extendable / Table 7.4.5.1.4-1 | Not Applicable |
| 206 | Time Domain Number | Extendable / Clause 8.2.146 | 1 |
| 207 | Time Offset Threshold | Extendable / Clause 8.2.147 | 8 |
| 208 | Cumulative rateRatio Threshold | Extendable / Clause 8.2.148 | 4 |
| 209 | Time Offset Measurement | Extendable / Clause 8.2.149 | 8 |
| 210 | Cumulative rateRatio Measurement | Extendable / Clause 8.2.150 | 4 |
| 211 | Remove SRR | Extendable/ Table 7.5.4.19-1 | Not applicable |
| 212 | Create SRR | Extendable/ Table 7.5.2.9-1 | Not applicable |
| 213 | Update SRR | Extendable/ Table 7.5.4.21-1 | Not applicable |
| 214 | Session Report | Extendable / Table 7.5.8.7-1 | Not Applicable |
| 215 | SRR ID | Extendable / Clause 8.2.151 | 1 |
| 216 | Access Availability Control Information | Extendable / Table 7.5.2.9-2 | Not applicable |
| 217 | Requested Access Availability Information | Extendable / Clause 8.2.152 | 1 |
| 218 | Access Availability Report | Extendable / Table 7.5.8.6-2 | Not applicable |
| 219 | Access Availability Information | Extendable / Clause 8.2.153 | 1 |
| 220 | Provide ATSSS Control Information | Extendable / Table 7.5.2.10-1 | Not Applicable |
| 221 | ATSSS Control Parameters | Extendable / Table 7.5.3.7-1 | Not Applicable |
| 222 | MPTCP Control Information | Extendable / Clause 8.2.154 | 1 |
| 223 | ATSSS-LL Control Information | Extendable / Clause 8.2.155 | 1 |
| 224 | PMF Control Information | Extendable / Clause 8.2.156 | 1 |
| 225 | MPTCP Parameters | Extendable / Table 7.5.3.7-2 | Not Applicable |
| 226 | ATSSS-LL Parameters | Extendable / Table 7.5.3.7-3 | Not Applicable |
| 227 | PMF Parameters | Extendable / Table 7.5.3.7-4 | Not Applicable |
| 228 | MPTCP Address Information | Extendable / Clause 8.2.157 | 4 |
| 229 | Link-Specific Multipath IP Address | Extendable / Clause 8.2.158 | 1 |
| 230 | PMF Address Information | Extendable / Clause 8.2.159 | 1 |
| 231 | ATSSS-LL Information | Extendable / Clause 8.2.160 | 1 |
| 232 | Data Network Access Identifier | Variable Length / Clause 8.2.161 | Not applicable |
| 233 | UE IP address Pool Information | Extendable / Table 7.4.4.1-3 | Not Applicable |
| 234 | Average Packet Delay | Extendable / Clause 8.2.162 | 4 |
| 235 | Minimum Packet Delay | Extendable / Clause 8.2.163 | 4 |
| 236 | Maximum Packet Delay | Extendable / Clause 8.2.164 | 4 |
| 237 | QoS Report Trigger | Extendable / Clause 8.2.165 | 1 |
| 238 | GTP-U Path QoS Control Information | Extendable / Table 7.4.4.1.3-1 | Not Applicable |
| 239 | GTP-U Path QoS Report (PFCP Node Report Request) | Extendable / Table 7.4.5.1.5-1 | Not Applicable |
| 240 | QoS Information in GTP-U Path QoS Report | Extendable / Table 7.4.5.1.6-1 | Not Applicable |
| 241 | GTP-U Path Interface Type | Extendable / Clause 8.2.166 | 1 |
| 242 | QoS Monitoring per QoS flow Control Information | Extendable / Table 7.5.2.9-3 | Not applicable |
| 243 | Requested QoS Monitoring | Extendable / Clause 8.2.167 | 1 |
| 244 | Reporting Frequency | Extendable / Clause 8.2.168 | 1 |
| 245 | Packet Delay Thresholds | Extendable / Clause 8.2.169 | 1 |
| 246 | Minimum Wait Time | Extendable / Clause 8.2.170 | 4 |
| 247 | QoS Monitoring Report | Extendable / Table 7.5.8.6-3 | Not applicable |
| 248 | QoS Monitoring Measurement | Extendable / Clause 8.2.171 | 1 |
| 249 | MT-EDT Control Information | Extendable / Clause 8.2.172 | 1 |
| 250 | DL Data Packets Size | Extendable / Clause 8.2.173 | 2 |
| 251 | QER Control Indications | Extendable / Clause 8.2.174 | 1 |
| 252 | Packet Rate Status Report | Extendable / Table 7.5.7.1-2 | Not applicable |
| 253 | NF Instance ID | Fixed / Clause 8.2.175 | 16 |
| 254 | Ethernet Context Information | Extendable / Table 7.5.4.21-1 | Not Applicable |
| 255 | Redundant Transmission Parameters | Extendable / Table 7.5.2.2-5, Table 7.5.2.3-4 | Not Applicable |
| 256 | Updated PDR | Extendable / Table 7.5.5.5-1 | Not Applicable |
| 257 | S-NSSAI | Fixed Length / Clause 8.2.176 | 4 |
| 258 | IP version | Extendable / Clause 8.2.177 | 1 |
| 259 | PFCPASReq-Flags | Extendable / Clause 8.2.178 | 1 |
| 260 | Data Status | Extendable / Clause 8.2.179 | 1 |
| 261 | Provide RDS configuration information | Extendable / Table 7.5.2.11-1 | Not Applicable |
| 262 | RDS configuration information | Extendable / Clause 8.2.180 | 1 |
| 263 | Query Packet Rate Status IE within PFCP Session Modification Request | Extendable / Table 7.5.4.22-1 | Not Applicable |
| 264 | Packet Rate Status Report IE within PFCP Session Modification Response | Extendable / Table 7.5.5.4-1 | Not Applicable |
| 265 | Multipath Applicable Indication | Extendable / Clause 8.2.181 | 1 |
| 266 | User Plane Node Management Information Container | Variable Length / Clause 8.2.182 | Not Applicable |
| 267 | UE IP Address Usage Information | Extendable / Table 7.4.4.3.1-1 | Not Applicable |
| 268 | Number of UE IP Addresses | Extendable / Clause 8.2.183 | 1 |
| 269 | Validity Timer | Extendable / Clause 8.2.184 | 2 |
| 270 | Redundant Transmission Forwarding Parameters | Extendable / Table 7.5.2.3-4 | Not Applicable |
| 271 | Transport Delay Reporting | Extendable / Table 7.5.2.2-6 | Not Applicable |
| 272 | Partial Failure Information | Extendable / Table 7.5.3.1-2 | Not Applicable |
| 273 | Spare |  |  |
| 274 | Offending IE Information | Variable Length / Clause 8.2.185 | Not Applicable |
| 275 | RAT Type | Extendable / Table 8.2.186 | 1 |
| 276 | L2TP Tunnel Information | Extendable / Table 7.5.2.1-2 | Not Applicable |
| 277 | L2TP Session Information | Extendable / Table 7.5.2.1-3 | Not Applicable |
| 278 | L2TP User Authentication | Extendable / Clause 8.2.187 | 1 |
| 279 | Created L2TP Session | Extendable / Table 7.5.3.1-3 | Not Applicable |
| 280 | LNS Address | Extendable / Clause 8.2.188 | 4 |
| 281 | Tunnel Preference | Fixed Length / Clause 8.2.189 | 3 |
| 282 | Calling Number | Variable Length / Clause 8.2.190 | Not Applicable |
| 283 | Called Number | Variable Length / Clause 8.2.191 | Not Applicable |
| 284 | L2TP Session Indications | Extendable / Clause 8.2.192 | 1 |
| 285 | DNS Server Address | Extendable / Clause 8.2.193 | 4 |
| 286 | NBNS Server Address | Extendable / Clause 8.2.194 | 4 |
| 287 | Maximum Receive Unit | Fixed Length / Clause 8.2.195 | 2 |
| 288 | Thresholds | Extendable / Clause 8.2.196 | 1 |
| 289 | Steering Mode Indicator | Extendable / Clause 8.2.197 | 1 |
| 290 | PFCP Session Change Info | Extendable / Table 7.4.7.1-2 | Not Applicable |
| 291 | Group ID | Variable Length / Clause 8.2.198 | Not Applicable |
| 292 | CP IP Address | Variable / Clause 8.2.199 | Not Applicable |
| 293 | IP Address and Port number Replacement | Extendable / Clause 8.2.200 | 2 |
| 294 | DNS Query/Response Filter | Extendable / Clause 8.2.201 | 2 |
| 295 | Direct Reporting Information | Extendable / Table 7.5.2.9-4 | Not applicable |
| 296 | Event Notification URI | Variable Length / Clause 8.2.202 | Not Applicable |
| 297 | Notification Correlation ID | Variable Length / Clause 8.2.203 | Not Applicable |
| 298 | Reporting Flags | Extendable / Clause 8.2.204 | 1 |
| 299 | Predefined Rules Name | Variable Length / Clause 8.205 | Not Applicable |
| 300 | MBS Session N4mb Control Information | Extendable / Table 7.5.2.1-4 | Not Applicable |
| 301 | MBS Multicast Parameters | Extendable / Table 7.5.2.3-5 | Not Applicable |
| 302 | Add MBS Unicast Parameters | Extendable / Table 7.5.2.3-6 | Not Applicable |
| 303 | MBS Session N4mb Information | Extendable / Table 7.5.3.1-4 | Not Applicable |
| 304 | Remove MBS Unicast Parameters | Extendable / Table 7.5.4.3-4 | Not Applicable |
| 305 | MBS Session Identifier | Extendable Length / Clause 8.2.206 | 1 |
| 306 | Multicast Transport Information | Extendable Length / Clause 8.2.207 | 5 |
| 307 | MBSN4mbReq-Flags | Extendable Length / Clause 8.2.208 | 1 |
| 308 | Local Ingress Tunnel | Extendable / Clause 8.2.209 | 1 |
| 309 | MBS Unicast Parameters ID | Extendable / Clause 8.2.210 | 2 |
| 310 | MBS Session N4 Control Information | Extendable / Table 7.5.2.1-5 | Not Applicable |
| 311 | MBS Session N4 Information | Extendable / Table 7.5.3.1-5 | Not Applicable |
| 312 | MBSN4Resp-Flags | Extendable / Clause 8.2.211 | 1 |
| 313 | Tunnel Password | Variable Length / Clause 8.2.212 | Not Applicable |
| 314 | Area Session ID | Fixed Length / Clause 8.2.213 | 2 |
| 315 | Peer UP Restart Report | Extendable / Clause 7.4.5.1.7 | Not Applicable |
| 316 | DSCP to PPI Control Information | Extendable / Table 7.5.2.1-6 | Not Applicable |
| 317 | DSCP to PPI Mapping Information | Extendable / Clause 8.2.214 | 2 |
| 318 | PFCPSDRsp-Flags | Extendable / Clause 8.2.215 | 1 |
| 319 | QER Indications | Extendable / Clause 8.2.216 | 1 |
| 320 | Vendor-Specific Node Report Type | Extendable / Clause 8.2.217 | 3 |
| 321 | Configured Time Domain | Extendable / Clause 8.2.218 | 1 |
| 322 | Metadata | Variable / Clause 8.2.219 | Not Applicable |
| 323 | Traffic Parameter Measurement Control Information | Extendable / Table 7.5.2.9-5 | Not Applicable |
| 324 | Traffic Parameter Measurement Report | Extendable / Table 7.5.8.6-4 | Not Applicable |
| 325 | Traffic Parameter Threshold | Extendable / Clause 8.2.220 | 1 |
| 326 | DL Periodicity | Extendable / Clause 8.2.221 | 5 |
| 327 | N6 Jitter Measurement | Extendable / Clause 8.2.222 | 1 |
| 328 | Traffic Parameter Measurement Indication | Extendable / Clause 8.2.223 | 1 |
| 329 | UL Periodicity | Extendable / Clause 8.2.224 | 5 |
| 330 | MPQUIC Control Information | Extendable / Clause 8.2.225 | 1 |
| 331 | MPQUIC Parameters | Extendable / Table 7.5.3.7-5 | Not Applicable |
| 332 | MPQUIC Address Information | Extendable / Clause 8.2.226 | 4 |
| 333 | Transport Mode | Extendable / Clause 8.2.227 | 1 |
| 334 | Protocol Description | Extendable / Table 7.5.2.2-7 | Not Applicable |
| 335 | Reporting Suggestion Info | Extendable / Clause 8.2.229 | 1 |
| 336 | TL-Container | Variable Length / Clause 8.2.230 | Not Applicable |
| 337 | Measurement Indication | Extendable / Clause 8.2.231 | 1 |
| 338 | HPLMN S-NSSAI | Fixed Length / Clause 8.2.232 | 4 |
| 339 | Media Transport Protocol | Extendable / Clause 8.2.233 | 1 |
| 340 | RTP Header Extension Information | Extendable / Table 7.5.2.2-8 | Not Applicable |
| 341 | RTP Payload Information | Extendable / Table 7.5.2.2-9 | Not Applicable |
| 342 | RTP Header Extension Type | Fixed Length / Clause 8.2.234 | 1 |
| 343 | RTP Header Extension ID | Fixed Length / Clause 8.2.235 | 1 |
| 344 | RTP Payload Type | Fixed Length / Clause 8.2.236 | 1 |
| 345 | RTP Payload Format | Fixed Length / Clause 8.2.237 | 1 |
| 346 | Extended DL Buffering Notification Policy | Extendable / Clause 8.2.238 | 1 |
| 347 | MT-SDT Control Information | Extendable / Clause 8.2.239 | 1 |
| 348 | Reporting Thresholds | Extendable / Clause 8.2.240 | 1 |
| 349 | RTP Header Extension Additional Information | Extendable / Clause 8.2.241 | 2 |
| 350 | Mapped N6 IP Address | Extendable / Clause 8.2.242 | 1 |
| 351 | N6 Routing Information | Extendable / Clause 8.2.243 | 1 |
| 352 | URI | Variable Length / Clause 8.2.244 | Not Applicable |
| 353 | UE Level Measurements Configuration | Extendable / Clause 8.2.245 | 2 |
| 354 | N6 Delay Measurement Protocols | Extendable / Clause 8.2.246 | 1 |
| 355 | N6 Delay Measurement Control Information | Extendable / Table 7.4.4.1.4-1 | Not Applicable |
| 356 | N6 Delay Measurement Report (PFCP Node Report Request) | Extendable / Table 7.4.5.1.8-1 | Not Applicable |
| 357 | N6 Delay Measurement Information | Extendable / Clause 8.2.247 | 1 |
| 358 | Measurement Endpoint Address | Extendable / Clause 8.2.248 | 1 |
| 359 | Operator Configurable UPF Capability | Variable / Clause 8.2.249 | Not Applicable |
| 360 | Packet Inspection functionality | Extendable / Clause 8.2.250 | 1 |
| 361 | Header Handling Control Rule | Extendable / Table 7.5.2.3-7 | Not Applicable |
| 362 | Header Handling Reporting Control Info | Extendable / Table 7.5.2.3-8 | Not Applicable |
| 363 | Header Handling Control information | Extendable / Table 7.5.2.3-9 | Not Applicable |
| 364 | Header Detection Reference | Variable Length / Clause 8.2.251 | Not Applicable |
| 365 | Header Detection Support Information | Variable Length / Clause 8.2.252 | Not Applicable |
| 366 | Reporting Endpoint ID | Fixed / Clause 8.2.253 | 1 |
| 367 | Header Handling Control Reference | Variable Length / Clause 8.2.254 | Not Applicable |
| 368 | Header Handling Action | Fixed Length / Clause 8.2.255 | 1 |
| 369 | Header Information | Variable Length / Clause 8.2.256 | Not Applicable |
| 370 | Header Value | Variable Length / Clause 8.2.257 | Not Applicable |
| 371 | Header Handling Condition | Fixed / Clause 8.2.258 | 1 |
| 372 | Header Handling Control ID | Fixed / Clause 8.2.259 | 1 |
| 373 | Header Handling Control Rule ID | Fixed / Clause 8.2.260 | 1 |
| 374 | On-path N6 Connection Information | Extendable / Clause 8.2.261 | 1 |
| 375 | Measurement Reporting Type | Extendable / Clause 8.2.262 | 1 |
| 376 | N6 Delay Measurement Failure Information | Extendable / Clause 8.2.263 | 1 |
| 377 | N6 Delay Measurement Control Information ID | Fixed / Clause 8.2.264 | 2 |
| 378 | Protocol Specific Configuration Parameters | Extendable / Table 7.4.4.1.4-2 | 1 |
| 379 | Measurement Endpoint Port Number | Fixed Length / Clause 8.2.265 | 2 |
| 380 | Header Handling Reporting Indication | Extendable / Clause 8.2.266 | 1 |
| 381 | Can be used |  |  |
| 382 | SMF Change Reason | Extendable / Clause 8.2.267 | 1 |
| 383 | Extended Transport Level Marking | Extendable / Table 7.5.2.3-10 | Not Applicable |
| 384 | PDU Set Importance | Fixed / Clause 8.2.268 | 2 |
| 385 | MoQ Control Information | Extendable / Clause 8.2.269 | 1 |
| 386 | MoQ Information | Extendable / Table 7.3.5.1-6 | Not Applicable |
| 387 | MoQ Relay IP Address | Extendable / Clause 8.2.270 | 1 |
| 388 | Media Related Information Transfer Info | Extendable / Clause 8.2.271 | 1 |
| 389 | Reporting Control Information | Extendable / Clause 8.2.272 | 1 |
| 390 | Security Mode (STAMP) | Fixed / Clause 8.2.273 | 1 |
| 391 | HMAC Key (STAMP) | Variable Length / Clause 8.2.274 | Not Applicable |
| 392 | Security Mode (OWAMP or TWAMP) | Fixed / Clause 8.2.275 | 4 |
| 393 | Key ID and Shared Secret (OWAMP or TWAMP) | Extendable / Clause 8.2.276 | 1 |
| 394 | Remaining Data Reporting Indication | Extendable / Clause 8.2.277 | 1 |
| 395 | Expedited Transfer Indication | Fixed / Clause 8.2.278 | 1 |
| 396 | Session Reflector Mode (STAMP) | Fixed / Clause 8.2.279 | 1 |
| 397 | PFD Partial Failure Information | Extendable / Table 7.4.3.2-2 | Not Applicable |
| 398 | Transport Level Marking Indications | Fixed / Clause 8.2.280 | 1 |
| aaa | Packet Delay Measurement Information | Extendable / Clause 8.2.xxx | 1 |
| bbb to 32767 | Spare. For future use. |  |  |
| 32768 to 65535 | Reserved for vendor specific IEs |  |  |

# *** Next Change ***

### 8.2.25	UP Function Features

The UP Function Features IE indicates the features supported by the UP function. It is coded as depicted in Figure 8.2.25-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 43 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 to 6 | Supported-Features |  |  |  |  |  |  |  |  |
|  | 7 to 8 | Additional Supported-Features 1 |  |  |  |  |  |  |  |  |
|  | 9 to 10 | Additional Supported-Features 2 |  |  |  |  |  |  |  |  |
|  | 11 to 12 | Additional Supported-Features 3 |  |  |  |  |  |  |  |  |
|  | 13 to 14 | Additional Supported-Features 4 |  |  |  |  |  |  |  |  |
|  | 15 to 16 | Additional Supported-Features 5 |  |  |  |  |  |  |  |  |
|  | 17 to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.25-1: UP Function Features

The UP Function Features IE takes the form of a bitmask where each bit set indicates that the corresponding feature is supported. Spare bits shall be ignored by the receiver. The same bitmask is defined for all PFCP interfaces.

The following table specifies the features defined on PFCP interfaces and the interfaces on which they apply.

Table 8.2.25-1: UP Function Features

| Feature Octet / Bit | Feature | Interface | M/O | Description |
| --- | --- | --- | --- | --- |
| 5/1 | BUCP | Sxa, N4 | O (NOTE 2) | Downlink Data Buffering in CP function is supported by the UP function. |
| 5/2 | DDND | Sxa | O | The buffering parameter 'Downlink Data Notification Delay' is supported by the UP function. |
| 5/3 | DLBD | Sxa, N4 | O | The buffering parameter 'DL Buffering Duration' in PFCP Session Report Response is supported by the UP function. |
| 5/4 | TRST | Sxb, Sxc, N4 | O | Traffic Steering is supported by the UP function. |
| 5/5 | FTUP | Sxa, Sxb, N4 | M | F-TEID allocation / release in the UP function is supported by the UP function. |
| 5/6 | PFDM | Sxb, Sxc, N4 | O | The PFD Management procedure is supported by the UP function. |
| 5/7 | HEEU | Sxb, Sxc, N4 | O | Header Enrichment of Uplink traffic is supported by the UP function. |
| 5/8 | TREU | Sxb, Sxc, N4 | O (NOTE 3) | Traffic Redirection Enforcement in the UP function is supported by the UP function. |
| 6/1 | EMPU | Sxa, Sxb, N4 | M | Sending of End Marker packets supported by the UP function. |
| 6/2 | PDIU | Sxa, Sxb, Sxc, N4, N4mb | O (NOTE 4) | Support of PDI optimised signalling in UP function (see clause 5.2.1A.2). |
| 6/3 | UDBC | Sxb, Sxc, N4 | O | Support of UL/DL Buffering Control |
| 6/4 | QUOAC | Sxb, Sxc, N4 | O | The UP function supports being provisioned with the Quota Action to apply when reaching quotas. |
| 6/5 | TRACE | Sxa, Sxb, Sxc, N4 | O | The UP function supports Trace (see clause 5.15). |
| 6/6 | FRRT | Sxb, N4 | O | The UP function supports Framed Routing (see IETF RFC 2865 [37] and IETF RFC 3162 [38]). |
| 6/7 | PFDE | Sxb, N4 | O | The UP function supports a PFD Contents including a property with multiple values. |
| 6/8 | EPFAR | Sxa, Sxb, Sxc, N4 | O | The UP function supports the Enhanced PFCP Association Release feature (see clause 5.18). |
| 7/1 | DPDRA | Sxb, Sxc, N4 | O | The UP function supports Deferred PDR Activation or Deactivation. |
| 7/2 | ADPDP | Sxa, Sxb, Sxc, N4 | O | The UP function supports the Activation and Deactivation of Pre-defined PDRs (see clause 5.19). |
| 7/3 | UEIP | Sxb, N4 | O (NOTE 5) | The UP function supports allocating UE IP addresses or prefixes (see clause 5.21). |
| 7/4 | SSET | N4, N4mb | O | (MB-)UPF support of PFCP sessions successively controlled by different (MB-)SMFs of a same (MB-)SMF Set (see clause 5.22). |
| 7/5 | MNOP | Sxa, Sxb, Sxc, N4 | O | The UP function supports measurement of number of packets which is instructed with the flag 'Measurement of Number of Packets' in a URR. See also clause 5.2.2.2.1. |
| 7/6 | MTE | N4 | O | UPF supports multiple instances of Traffic Endpoint IDs in a PDI. |
| 7/7 | BUNDL | Sxa, Sxb, Sxc, N4, N4mb | O | PFCP messages bundling (see clause 6.5) is supported by the UP function. |
| 7/8 | GCOM | N4 | O | UPF support of 5G VN Group Communication. (See clause 5.23) |
| 8/1 | MPAS | N4, N4mb | O | (MB-)UPF support for multiple PFCP associations to the (MB-)SMFs in an (MB-)SMF set (see clause 5.22.3). |
| 8/2 | RTTL | N4 | O | UPF supports redundant transmission at transport layer. |
| 8/3 | VTIME | Sxb,N4 | O | UP function support of quota validity time feature. |
| 8/4 | NORP | Sxa, Sxb, Sxc, N4 | O | UP function support of Number of Reports as specified in clause 5.2.2.2. |
| 8/5 | IPTV | N4 | O | UPF support of IPTV service (see clause 5.25) |
| 8/6 | IP6PL | N4 | O | UPF supports: - UE IPv6 address(es) allocation with IPv6 prefix length other than default /64 (including allocating /128 individual IPv6 addresses), as specified in clause 4.6.2.2 of of 3GPP TS 23.316 [57]; and - multiple UE IPv6 addresses allocation using multiple instances of the UE IP Address IE in a same PDI or Traffic Endpoint, or using multiple PDIs or Traffic Endpoints with a different UE IP Address as specified in clause 5.21.1. |
| 8/7 | TSN | N4 | O | Integration of 5GS into a TSN data network is supported by the UPF (see clause 5.26.1). |
| 8/8 | MPTCP | N4 | O | UPF support of MPTCP Proxy functionality (see clause 5.20) |
| 9/1 | ATSSS-LL | N4 | O | UPF support of ATSSS-LL steering functionality (see clause 5.20) |
| 9/2 | QFQM | N4 | O | UPF support of per QoS flow per UE QoS monitoring for packet delay (see clause 5.24.4). |
| 9/3 | GPQM | N4 | O | UPF support of per GTP-U Path QoS monitoring (see clause 5.24.5). |
| 9/4 | MT-EDT | Sxa | O | SGW-U support of reporting the size of DL Data Packets. (see clause 5.2.4.1). |
| 9/5 | CIOT | Sxb, N4 | O | UP function support of CIoT feature, e.g. small data packet rate enforcement. (see 5.4.15) |
| 9/6 | ETHAR | N4 | O | UPF support of Ethernet PDU Session Anchor Relocation (see clause 5.13.6). |
| 9/7 | DDDS | N4 | O | UPF support of reporting the first buffered / first discarded downlink data after buffering / directly dropped downlink data for downlink data delivery status notification. |
| 9/8 | RDS | Sxb, N4 | O | UP function support of Reliable Data Service (see clause 5.29). |
| 10/1 | RTTWP | N4 | O | UPF support of RTT measurements towards the UE Without PMF. |
| 10/2 | QUASF | Sxb, Sxc, N4 | O | The UP function supports being provisioned in a URR with an Exempted Application ID for Quota Action or an Exempted SDF Filter for Quota Action which is to be used when the quota is exhausted. See also clauses 5.2.2.2.1 and 5.2.2.3.1. |
| 10/3 | NSPOC | Sxa, Sxb, N4 | O | UP function supports notifying start of Pause of Charging via user plane. |
| 10/4 | L2TP | Sxb, N4 | O | UP function supports the L2TP feature as described in clause 5.31. |
| 10/5 | UPBER | N4 | O | UP function supports the uplink packets buffering during EAS relocation. |
| 10/6 | RESPS | Sxb, N4 | O | UP function supports Restoration of PFCP Sessions associated with one or more PGW-C/SMF FQ-CSID(s), Group Id(s) or CP IP address(es) (see clause 5.22.4) |
| 10/7 | IPREP | N4 | O | UP function supports IP Address and Port number replacement (see clauses 5.33.3, 5.33.4 and 5.33.6). |
| 10/8 | DNSTS | N4 | O | UP function support DNS Traffic Steering based on FQDN in the DNS Query message (see clause 5.33.4) |
| 11/1 | DRQOS | N4 | O | UP function supports Direct Reporting of QoS monitoring events to Local NEF or AF (see clause 5.33.5). |
| 11/2 | MBSN4 | N4 | O | UPF supports sending MBS multicast session data to associated PDU sessions using 5GC individual delivery. |
| 11/3 | PSUPRM | N4, N4mb | O | UP function supports Per Slice UP Resource Management (see clause 5.35). |
| 11/4 | EPPPI | N4 | O | UP function supports Enhanced Provisioning of Paging Policy Indicator feature as specified in clause 5.36.2. |
| 11/5 | RATP | Sxb, N4 | O | UP function supports Redirection Address Types set to "Port", "IPv4 address and Port", "IPv6 address and Port", or "IPv4 and IPv6 addresses and Port". |
| 11/6 | UPIDP | N4 | O | UP function supports User Plane Inactivity Detection and reporting per PDR feature as specified in clause 5.11.3. |
| 11/7 | AFSFC | N4 | O | UP function supports inserting metadata when Application Function influence on Service Function Chaining is used as specified in clause 5.4.8. |
| 11/8 | MPQUIC-UDP | N4 | O | UPF support of MPQUIC-UDP Proxy functionality (see clause 5.20). (NOTE 6) |
| 12/1 | REDSM | N4 | O | UP function supports the Redundant Steering Mode (see clause 5.32.8 of 3GPP TS 23.501 [28] and clause 5.2.7.1) |
| 12/2 | DBDM | Sxa, N4 | O | The buffering parameters 'DL Buffering Duration' and 'DL Buffering Suggested Packet Count' in PFCP Session Modification Request are supported by the UP function. |
| 12/3 | TSCTS | N4 | O | AF requested support of Time Synchronization and/or Time Sensitive Communication is supported by the UP function (see clause 5.26.1). |
| 12/4 | DRTSC | N4 | O | UP function supports Direct Reporting of TSC management information events to TSN AF or TSCTSF (see clause 5.26.3.2). |
| 12/5 | N6JEDB | N4 | O | UPF support of N6 Jitter, DL Periodicity and UL Periodicity Measurement and Reporting, and End of Data Burst marking as specified in clause 5.38.4. |
| 12/6 | QMCON | N4 | O | UPF support of QoS monitoring of Congestion information (see clause 5.39.3.3). |
| 12/7 | DETNET | N4 | O | Deterministic Networking is supported by the UP function (see clause 5.26.1). |
| 12/8 | EML4S | N4 | O | UP function supports ECN Marking for L4S (see clause 5.38.1). |
| 13/1 | PDUSM | N4 | O | UP function supports PDU Set Marking (see clause 5.38.3). |
| 13/2 | CN-TL | N4 | O | UPF support of the TSN Talker and Listener (TL) functionality, i.e. CN-TL. See clause 5.26a. |
| 13/3 | QMDRM | N4 | O | UPF support of QoS monitoring of Data Rate Measurement (see clause 5.39.3.4). |
| 13/4 | EDBNC | N4 | O | UP function supports Extended DL Buffering Notification Control (see clause 5.2.4.1). |
| 13/5 | MT-SDT | N4 | O | UPF support of reporting the size of DL Data Packets per QoS flow (see clause 5.2.4.1). |
| 13/6 | UPSBIES | N4 | O | UPF support of event subscription via Service Based Interface. (See clause 5.40). |
| 13/7 | UMN6IP | N4 | O | UPF supports to allocate a mapped N6 IP address and perform IP replacement as described in clause 5.33.7. |
| 13/8 | UN6TU | N4 | O | UPF supports N6 tunnelling using N6 routing information as described in clause 5.33.7. |
| 14/1 | MBSCH | N4mb | O | MB-UPF support of MBS charging (see clause 5.34.2.5) |
| 14/2 | UN6DM | N4 | O | UPF support of N6 Delay Measurement and Reporting |
| 14/3 | NATPUB | N4 | O | UPF supports the NAT information exposure of the public IP address and port number retrieval for a particular PDU Session, based on the private IP allocated by 5GC (see clause 5.8.2.17 and clause 7.2.29 of 3GPP TS 23.501 [28]). |
| 14/4 | USHPH | N4 | O | UPF support of Handling of Payload Headers as described in clause 5.41 |
| 14/5 | MPQUIC-IP | N4 | O | UPF support of MPQUIC-IP Proxy functionality (see clause 5.20). |
| 14/6 | MPQUIC-E | N4 | O | UPF support of MPQUIC-E Proxy functionality (see clause 5.20). |
| 14/7 | DYNTR | N4 | O | UPF support of dynamically changing traffic characteristics via the User Plane (see clause 5.38.5), including: - DL Data Burst Size Marking - DL Time to Next Burst Marking - Expedited Data Transfer with reflective QoS |
| 14/8 | MUXMF | N4 | O | UPF support of Traffic identification and differentiated QoS for multiplexed media flows (see clause 5.38.6). |
| 15/1 | CONUDP | N4 | O | UPF support of transferring media related information over N6 using connect-UDP (see clause 5.38.7.3). |
| 15/2 | MOQ | N4 | O | UP function supports MoQ relay functionality (see clause 5.38.7.2). |
| 15/3 | ULM | N4 | O | UE Level measurement (see clause 5.15A) |
| 15/4 | PSITLM | N4 | O | UP function supports the PDU Set Importance based transport level packet marking. See also clause 5.4.13. |
| 15/5 | UDPOPT | N4 | O | UPF support of transferring media related information over N6 using UDP Option (see clause 5.38.7.4). |
| 15/6 | QMABR | N4 | O | UPF support of QoS monitoring of Available Bitrate measurement (see clause 5.39.3.5). |
| 15/7 | PAPFD | N4, Sxb, Sxc | O | UP function supports of partial acceptance of provisioning of the Application ID's PFDs. See also clause 5.42. |
| 15/x | RPDMI | N4 | O | UPF support of reporting the Packet Delay Measurement Information (Ssee clause 5.24.4.x). |
| NOTE 1: Features are defined as follows: - Feature Octet / Bit: The octet and bit number within the Supported-Features IE, e.g. "5 / 1". - Feature: A short name that can be used to refer to the octet / bit and to the feature. - Interface: A list of applicable interfaces to the feature. - M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O") for a UP function complying with this release of the specification. A feature defined as mandatory in this release of the specification may not necessarily be defined or mandatory to support in earlier releases (see the respective specifications), therefore a CP function complying with this release of the specification shall be prepared to receive a UP Function Features IE with a mandatory feature bit set to "0". - Description: A clear textual description of the feature. NOTE 2: Downlink data should be buffered preferably in the UP function. Downlink data may be buffered in the CP function when desired, e.g. for UEs using power saving methods. NOTE 3: If the traffic that the UP function can support may be subject to traffic redirection, traffic redirection enforcement in the UP function shall be supported by the UP function. NOTE 4: CP and UP functions complying with this release of the specification should support this feature. NOTE 5: A UPF that supports the SSET or MPAS feature shall support this feature. NOTE 6: The MPQUIC-UDP feature is referred to as "MPQUIC" feature in previous releases of this specification (that do not support the MPQUIC-IP functionality and the MPQUIC-E functionality). |  |  |  |  |

# *** Next Change ***

### 8.2.231	Measurement Indication

The Measurement Indication IE shall be encoded as shown in Figure 8.2.231-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = 337 (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  |  |  | RPDMI | DQFI |  |
|  | m to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.231-1: Measurement Indication

The following flags are coded within Octet 5:

-	Bit 1 – DQFI (Default QoS Flow Indication): If this bit is set to "1", it indicates that the UPF is requested to perform QoS Monitoring measurements on the QoS flow associated with the default QoS rule and the UPF shall include the same indication in the corresponding QoS Monitoring Measurement report (see clause 4.15.4.5.1 of 3GPP TS 23.502 [29]).

-	Bit 2 – RPDMI (Report Packet Delay Measurement Information): if this bit is set to "1", it indicates that the UPF is requested to report the Packet Delay Measurement Information as specified in clause 5.24.4.x.

-	Bit 3 to 8 Spare, for future use and set to "0".

# *** Next Change ***

### 8.2.xxx	Packet Delay Measurement Information

The Packet Delay Measurement Information IE contains the additional information related to QoS Monitoring for packet delay, which is reported by the UPF, to enable the SMF to determine if the QoS monitoring for packet delay for a QoS flow of a PDU session should be performed using the timestamps based mechanism or GTP-U path monitoring based mechanism. See also clause 5.24.4.x.

It shall be encoded as shown in Figure 8.2.xxx-1.

|  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Octets | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
|  | 1 to 2 | Type = aaa (decimal) |  |  |  |  |  |  |  |  |
|  | 3 to 4 | Length = n |  |  |  |  |  |  |  |  |
|  | 5 | Spare |  |  |  |  | NTDR | DERR | TISR |  |
|  | v to (n+4) | These octet(s) is/are present only if explicitly specified |  |  |  |  |  |  |  |  |

Figure 8.2.xxx-1: Packet Delay Measurement Information

The following flags are coded within Octet 5:

-	Bit 1 – TISR (Timestamps Received): If this bit is set to "1", it indcates that the PSA UPF has received the TimeStamps, i.e. the DL Sending Time Stamp Repeated, the DL Received Time Stamp and the UL Sending Time Stamp, included in the PDU Session Container extension header of the UL GTP-U packets.

-	Bit 2 – DERR (Delay Results Received): If this bit is set to "1", it indicates that the PSA UPF has received the delay results, i.e. the DL Delay Result and the UL Delay Result, included in the PDU Session Container extension header of the UL GTP-U packets.

-	Bit 3 – NTDR (No Timestamp nor Delay results Received): If this bit is set to "1", it indicates that the PSA UPF does not receive any TimeStamps nor delay results in the PDU Session Container extension header of the UL GTP-U packets.

-	Bit 4 to 8: Spare, for future use and set to "0".

The NTDR bit shall not be set to "1" if any of TISR and DERR are set to "1".

# *** End of Changes ***