3GPP TSG-CT WG4 Meeting #131	C4-254212

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0192 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cancel procedure for subscriber specific IMS Events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In comparison to 5G Event Exposure procedures, the following situations are not currently notified by NEF towards AF after creating IMS Event Exposure Subscription: - UE subscription (e.g. IMS Public Identity) associated to the created Event Exposure Subscription is deleted. - IMS Event Exposure Authorization for a particular UE Subscription This CR introduces feature parity for IMS Event Exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the new IMS Event Exposure Monitoring Revocation Notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF and NEF will not be informed about the subscription was cancelled consuming resources waiting to be notified about events which will never occur for a particular UE subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2.2.2, 5.7.2.x (new), 6.6.5, 6.6.5.1 (new), 6.6.5.2 (new), 6.6.61, 6.6.6.2.x (new), 6.6.6.2.y (new), 6.6.6.3.x (new), 6.6.7.3, A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.228 CR 1655 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features with impact on the following APIs: - TS29562_Nhss_imsEE.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

##### 5.7.2.2.2	Subscription to a specific subscriber related IMS event

Figure 5.7.2.2.2-1 shows a scenario where the consumer sends a request to the HSS to subscribe a specific subscriber related IMS event. The request contains a UE's identity (/{imsUeId}) which shall be IMPU.

Figure 5.7.2.2.2-1: Subscription to a specific subscriber related IMS event

1.	The NF consumer sends a POST request to the collection resource "ee-subscriptions" representing the subscriptions on the UE's IMS events. The request body shall contain a callback URI where subsequent event notifications shall be sent by IMS AS and may contain another callback URI where revocation notifications may be sent by HSS.

2a.	Upon success, the HSS shall include a HTTP Location header to provide the location of the newly created resource (ee-subscriptions) together with the status code "201 Created" in the response message indicating that the requested resource is created.

2b.	On failure, one of the HTTP status code listed in table 6.6.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.6.3.2.3.1-3.

- If the operation is not authorized due to, e.g. the requesting node not being authorized to subscribe to a specific subscriber related IMS event, HTTP status code "403 Forbidden" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

- If the UE identity is not found in HSS, HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in "ProblemDetails" data structure).

2c.	In the case of redirection, the HSS shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another HSS (service) instance.

# * * * Next Change * * *

#### 5.7.2.x	Notification

##### 5.7.2.x.1	General

The following procedures using the Notification service operation are supported:

-	Monitoring Revocation Notification;

##### 5.7.2.x.2	IMS Monitoring Revocation Notification

Figure 5.7.2.x.2-1 shows a scenario where the HSS notifies the NF service consumer (that has subscribed to receive such notification) about revocation of the Event Exposure Subscription. The request contains the notification URI as previously received in the ImsEeSubscription.

Figure 5.7.2.x.2-1: IMS Event Exposure Monitoring Revocation

1.	The HSS sends a POST request to the revocation notificationURI as provided by the NF service consumer during the subscription.

2.	The NF service consumer responds with "204 No Content".

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

In the case of redirection, the NF Service Consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to an alternative notification endpoint.

# * * * Next Change * * *

### 6.6.5	Notifications

#### 6.6.5.1	General

This clause specifies the use of notifications and corresponding protocol details.

#### 6.6.5.2	Monitoring Revocation Notification

The POST method shall be used for Monitoring Revocation Notifications and the URI shall be as provided during the subscription procedure.

Resource URI: {revocationNotifURI}

Support of URI query parameters is specified in table 6.6.5.2-1.

Table 6.6.5.2-1: URI query parameters supported by the POST method

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.6.5.2-2 and of response data structures and response codes is specified in table 6.6.5.2-3.

Table 6.6.5.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ImsEeMonitoringRevoked | M | 1 |  |

Table 6.6.5.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the notification endpoint of the subscribing NF Service Consumer. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - CONTEXT_NOT_FOUND See table 6.6.7.3-1 for the description of this error. |

Table 6.6.5.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

Table 6.6.5.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the notification endpoint of the subscribing NF. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected. |

# * * * Next Change * * *

#### 6.6.6.1	General

This clause specifies the application data model supported by the API.

Table 6.6.6.1-1 specifies the data types defined for the Nhss_ImsEE service based interface protocol.

Table 6.6.6.1-1: Nhss_ImsEE specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ImsEeSubscription | 6.6.6.2.2 | Represents an Individual IMS Event Subscription. |
| CreatedImsEeSubscription | 6.6.6.2.3 | Represents the created Individual IMS Event Subscription. |
| ImsEeMonitoringRevoked | 6.6.6.2.x | Represents the IMS Revoked Monitoring events information |
| ImsEeMonitoringEvent | 6.6.6.2.y | Represents the IMS Monitoring event |

Table 6.6.6.1-2 specifies data types re-used by the Nhss_ImsEE service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nhss_ImsEE service based interface.

Table 6.6.6.1-2: Nhss_ImsEE re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DateTime | 3GPP TS 29.571 [16] |  |
| ImsEvent | 3GPP TS 29.571 [16] | The IMS events. |
| ImsEventConfiguration | 3GPP TS 29.571 [16] | The IMS event configuration. |
| ImsEventReport | 3GPP TS 29.571 [16] | The IMS event report. |
| ImsReportingOptions | 3GPP TS 29.571 [16] | The IMS reporting options. |
| ImsPublicId | 3GPP TS 29.562 [15] | IMS Public Identity. |
| Uri | 3GPP TS 29.571 [16] |  |

# * * * Next Change * * *

##### 6.6.6.2.2	Type: ImsEeSubscription

Table 6.6.6.2.2-1: Definition of type ImsEeSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notifId | string | O | 0..1 | Notification Correlation ID provided by the NF service consumer. |
| notificationURI | Uri | M | 1 | Identifies the recipient of Notifications sent by the NF consumer. This parameter shall be supplied by the NF service consumer in the HTTP POST requests that create the subscriptions for event notifications. |
| subscrImsEvents | map(ImsEventConfiguration) | M | 1..N | A map (list of key-value pairs where referenceId converted from integer to string serves as key. Describes the IMS events to be subscribed in a subscription request or the events successfully subscribed for this subscription in a subscription response. |
| afId | string | O | 0..1 | The identifier of the originating AF, may be used by the HSS to perform necessary authorization prior to handling the request. |
| imsReportOptions | ImsReportingOptions | O | 0..1 | This IE may be included if the NF service consumer wants to describe how the reports of the event to be generated, If this attribute is absent, the default report mode shall be "ON_EVENT_DETECTION". |
| revocationNotifURI | Uri | O | 1 | Identifies the recipient of Monitoring Revocation Notifications sent by the NF consumer. |

# * * * Next Change * * *

##### 6.6.6.2.x	Type: ImsEeMonitoringRevoked

Table 6.6.6.2.x-1: Definition of type ImsEeMonitoringRevoked

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| revokedMonitoringEventList | map(ImsMonitoringEvent) | O | 0..1 | Shall contain a map (list of key-value pairs where referenceId converted from integer to string serves as key) of the ImsMonitoringEvents (see clause 6.6.6.2.7) that shall be revoked. The Reference ID which was provided as the key of the subscribed IMS Events in subscription request. The consumer can use this IE to uniquely associate the revoked monitoring event with the corresponding event that was requested to be monitored. |
| revokedImsUeId | ImsPublicId | O | 1 | Represents the IMS Subscription Identifier, either the IMS Public Identity (i.e. IMPU or Public PSI). See 3GPP TS 23.228 [6] clause 4.3). Pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}-.+)$" |

# * * * Next Change * * *

##### 6.6.6.2.y	Type: ImsEeMonitoringEvent

Table 6.6.6.2.y-1: Definition of type MonitoringEvent

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventType | ImsEvent | M | 1 | IMS Event; see clause 5.11.3.6 of 3GPP TS 29.571 [16] |
| revokedCause | ImsRevokedCause | O | 0..1 | When present, it contains the reason for revocation of the monitoring event. |

# * * * Next Change * * *

##### 6.6.6.3.x	Enumeration: ImsRevokedCause

Table 6.4.6.3.x-1: Enumeration RevokedCause

| Enumeration value | Description |
| --- | --- |
| "MONITORING_NOT_ALLOWED" | IMS Event Exposure Monitoring is not allowed |
| "IMS_UE_ID_REMOVED" | The IMS Subscription Identifier, either the IMS Public Identity (i.e. IMPU or Public PSI) or the IMS Private Identity (i.e. IMPI or PSI), used in the Event Exposure subscription is not longer valid for this individual subscription |

# * * * Next Change * * *

#### 6.6.7.3	Application Errors

The common application errors defined in the Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may also be used for the Nhss_imsEE service. The following application errors listed in Table 6.6.7.3-1 are specific for the Nhss_imsEE service.

Table 6.6.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The received IMS UE identity is unknown. |
| CONTEXT_NOT_FOUND | 404 Not Found | It is used during the modification of an existing subscription when no corresponding context exists. |
| OPERATION_NOT_ALLOWED | 403 Forbidden | Indicates the IMS Event subscription is not allowed to be created. |

# * * * Next Change * * *

# A.7	Nhss_imsEE API

openapi: 3.0.0

info:

version: '1.0.0-alpha.1'

title: 'Nhss_imsEE'

description: |

Nhss Event Exposure Service for IMS.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 V19.2.0; Home Subscriber Server (HSS) Services

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.562/

servers:

- url: '{apiRoot}/nhss-ims-ee/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

paths:

/{imsUeId}/ee-subscriptions:

post:

summary: Subscribe to IMS Event

operationId: ImsEventSubscribe

tags:

- IMS Event Subscription Creation

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

- oAuth2ClientCredentials:

- nhss-ims-ee

- nhss-ims-ee:ee-subscriptions:create

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: 'TS29562_Nhss_imsUECM.yaml#/components/schemas/ImsUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEeSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CreatedImsEeSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/nhss-ims-ee/v1/{imsUeId}/ee-subscriptions/{subscriptionId}

required: true

schema:

type: string

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

imsMonitoringRevocationNotification:

'{$request.body#/revocationNotifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEeMonitoringRevoked'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/ee-subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from Individual IMS Event

operationId: ImsEventUnsubscribe

tags:

- Individual IMS Event Subscription Deletion

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

- oAuth2ClientCredentials:

- nhss-ims-ee

- nhss-ims-ee:ee-subscriptions:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: 'TS29562_Nhss_imsUECM.yaml#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: modify the subscription

operationId: ImsEventSubsModify

tags:

- Individual IMS Event Subscription Modification

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ee

- oAuth2ClientCredentials:

- nhss-ims-ee

- nhss-ims-ee:subscription:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: 'TS29562_Nhss_imsUECM.yaml#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CreatedImsEeSubscription'

'204':

description: Successful response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-ee: Access to the Nhss IMS Event Exposure API

nhss-ims-ee:ee-subscriptions:create: >-

Access to create IMS Event Subscriptions resource

nhss-ims-ee:ee-subscriptions:modify: >-

Access to update/delete the Individual IMS Event Subscription resource

schemas:

#

# COMPLEX TYPES:

#

ImsEeSubscription:

description: An Individual IMS Event Subscription

type: object

required:

- notificationURI

- subscrImsEvents

properties:

notifId:

type: string

afId:

type: string

notificationURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

subscrImsEvents:

description: >

the IMS events to be subscribed or successfully subscribed in a

subscription reponse within the referenceId converted from integer

to string is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventConfiguration'

minProperties: 1

imsReportingOptions:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsReportingOptions'

revocationNotifURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

CreatedImsEeSubscription:

description: The created Individual IMS Event Subscription

type: object

required:

- imsEeSubscription

properties:

imsEeSubscription:

$ref: '#/components/schemas/ImsEeSubscription'

statusEvents:

type: array

minItems: 1

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventReport'

ImsEeMonitoringRevoked:

type: object

required:

- revokedMonitoringEventList

properties:

revokedMonitoringEventList:

description: A map (list of key-value pairs where ReferenceId serves as key) of ImsEeMonitoringEvents

type: object

additionalProperties:

$ref: '#/components/schemas/ImsEeMonitoringEvent'

minProperties: 1

revokedImsUeId:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId

ImsEeMonitoringEvent:

type: object

required:

- eventType

properties:

eventType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEvent'

revokedCause:

$ref: '#/components/schemas/ImsRevokedCause'

#

# SIMPLE TYPES

#

#

# ENUMS

#

ImsRevokedCause:

description: Revocation Cause for the IMS monitoring event.

anyOf:

- type: string

enum:

- MONITORING_NOT_ALLOWED

- IMS_UE_ID_REMOVED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

# * * * End of Changes * * * *