3GPP TSG-CT WG4 Meeting #131	C4-254270

Sophia Antipolis, France, 13th – 17th October 2025	revision of C4-254024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 1502 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deregistration Reason |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to clause 6.1.6.2.4 the UDM shall send a deregistration notification to the AMF when the corresponding access becomes restricted. However an appropriate value for DeregistrationReason is not defined. Existing DeregistrationReason values are not suitable for the use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new values for DeregistrationReason |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing values for DeregistrationReason |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.3, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to TS29503_Nudm_UECM.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Reason for change indicates that existing values of DeregistrationReason are not suitable for the identified use case. Note added to tabe 6.2.6.3.3-1 |  |  |  |  |  |  |  |  |

* * * For Information * * * *

##### 6.1.6.2.4	Type: AccessAndMobilitySubscriptionData

Table 6.1.6.2.4-1: Definition of type AccessAndMobilitySubscriptionData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| *************part of table not shown for clarity************** |  |  |  |  |  |
| accessTypeRestrictions | array(AccessType) | O | 0..2 | List of Access Types that are restricted. If non-3GPP access is restricted, then the UDM shall reject/deregister the AMF non-3GPP registration. If 3GPP access is restricted, then the UDM shall reject/deregister the AMF 3GPP registration. Otherwise, the UDM shall pass the IE to the AMF. |  |
| *************part of table not shown for clarity************** |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.6.3.3	Enumeration: DeregistrationReason

The enumeration DeregistrationReason represents the reason for the Deregistration Notification. It shall comply with the provisions defined in table 6.2.6.3.3-1.

Table 6.2.6.3.3-1: Enumeration DeregistrationReason

| Enumeration value | Description |
| --- | --- |
| "UE_INITIAL_REGISTRATION" | When sent by the HSS; indicates that the deregistration towards the UDM is due to an initial attach in EPS. When sent by the UDM; indicates that the deregistration in the old AMF is due to a new AMF serving the UE during an initial registration See 3GPP TS 23.502 [3] and 3GPP TS 23.632 [32]. |
| "UE_REGISTRATION_AREA_CHANGE" | see 3GPP TS 23.502 [3] |
| "SUBSCRIPTION_WITHDRAWN" | see 3GPP TS 23.502 [3] |
| "5GS_TO_EPS_MOBILITY" | see 3GPP TS 23.502 [3] and 3GPP TS 23.632 [32]. |
| "5GS_TO_EPS_MOBILITY_UE_INITIAL_REGISTRATION" | This value shall only be sent by the UDM. It indicates that the deregistration in AMF is due to an initial attach in EPS, See 3GPP TS 23.502 [3] and 3GPP TS 23.632 [32]. |
| "REREGISTRATION_REQUIRED" | see 3GPP TS 23.502 [3] |
| "SMF_CONTEXT_TRANSFERRED" | see 3GPP TS 23.502 [3] |
| "DUPLICATE_PDU_SESSION" | This value shall only be sent by the UDM to an SMF. It indicates that the deregistration in the SMF is due to a new PDU session with the same PDU session ID has been established in another SMF. |
| "PDU_SESSION_REACTIVATION_REQUIRED" | This value shall only be sent by the UDM to an SMF. It indicates that the PDU session being released is requested to be re-activated. |
| "DISASTER_CONDITION_TERMINATED" | This value shall be used by the UDM when the disaster condition ceases. |
| "OPERATOR_DETERMINED_BARRING" | This value indicates that the deregistration is due to ODB (see 3GPP TS 23.015 [71]). |
| "DUPLICATE_PDU_SESSION_EPDG" | This value shall only be sent by the UDM to an SMF. It indicates that the deregistration in the SMF is due to a new PDU session with the same PDU session ID has been established in another SMF via an ePDG. |
| 3GPP_ACCESS_NOT_ALLOWED | This value is used by the UDM in AMF Deregistration Notifications when 3GPP access becomes restricted. (NOTE) |
| NON_3GPP_ACCESS_NOT_ALLOWED | This value is used by the UDM in AMF Deregistration Notifications when Non-3GPP access becomes restricted. (NOTE) |
| NOTE: After sending the Deregistration Notification with this DeregistrationReason, the UDM still needs to send a Data Change Notification indicating the modification of accessTypeRestrictions in AccessAndMobilitySubscriptionData (see table 6.1.6.2.4-1) to NFs ( e.g. AMFs) that have subscribed to such notifications and the subscription is not implicitly unsubscribed by the Deregistration Notification. |  |

* * * Next Change * * * *

# A.3	Nudm_UECM API

openapi: 3.0.0

****************text not shown for clarity**************

DeregistrationReason:

description: >

It represents the reason of Deregistration Notification.

anyOf:

- type: string

enum:

- UE_INITIAL_REGISTRATION

- UE_REGISTRATION_AREA_CHANGE

- SUBSCRIPTION_WITHDRAWN

- 5GS_TO_EPS_MOBILITY

- 5GS_TO_EPS_MOBILITY_UE_INITIAL_REGISTRATION

- REREGISTRATION_REQUIRED

- SMF_CONTEXT_TRANSFERRED

- DUPLICATE_PDU_SESSION

- PDU_SESSION_REACTIVATION_REQUIRED

- DISASTER_CONDITION_TERMINATED

- OPERATOR_DETERMINED_BARRING

- DUPLICATE_PDU_SESSION_EPDG

- 3GPP_ACCESS_NOT_ALLOWED

- NON_3GPP_ACCESS_NOT_ALLOWED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

****************text not shown for clarity**************

* * * End of Change * * * *