3GPP TSG-CT WG4 Meeting #131	C4-254abc

Sophia Antipolis, France; 13th – 17th October 2025	was C4-254194

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0916 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Populating Service-level-AA container to (h-)SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in the CT4 LS out to SA5, C4-253415, CT4 answered SA5 question: "The UE supporting UAS services provides CAA-level UAV ID included in the Service-level-AA container IE to the AMF/SMF during the registration procedure/PDU session establishment procedure as described in clause 4.22 of 3GPP TS 24.501, Authentication and authorization of UAV. Accordingly, the AMF/SMF can identify the UE is an UAV UE or requesting UAV services." And also according to the requirements in clause 5.2.3.2 of TS 23,256: The procedure assumes that the UE/UAV has already registered on the AMF. 0. Steps 1 - 5 as in TS 23.502 [3] figure 4.3.2.2.1-1. The UAV includes the Service Level Device Identity (e.g. the CAA-Level UAV ID of the UVA) and may include the Authentication Server Address (i.e. the USS address or the USS address for the corresponding geographical area based on the UAV current location) and optionally Authentication Data (i.e. the UUAA Aviation Payload) in the PDU Session Establishment request. The SMF determines that it needs to invoke UAS NF/NEF service operation for UUAA Authentication/Authorization of the PDU session establishment request based on that the provided DNN/S-NSSAI combination is dedicated for aerial services (have aerial service indicator set) and that the Service Level Device Identity (CAA-Level-UAV ID) is included in the request. If the provided APN/DNN is dedicated for aerial services but Service Level Device Identity (CAA-Level UAV ID) is not provided, the SMF shall reject the establishment of the PDU Session and steps 1 - 9 are not performed. The SMF identifies the UAS NF/NEF based on local configuration or by NF discovery procedure using DNN/S-NSSAI and/or UE provided identity e.g. USS address. The UUAA-SM procedure may be performed during the UE-requested PDU session establishment procedure, or a PDU session modification for the re-authentication purpose as well as a PDU session establishment/modification for C2 authorization/communication, therefore the Service-level-AA container is included in PDU Session Establishment Request/accept/reject messages, PDU session Modification Command/Request message, PDU session Release command, and Service-Level Authentication Command/Complete message. (See 24.501) The Service-level-AA container should be transferred transparently between the (H-)SMF and I/V-SMF via n1SmInfoFromUe and n1SmInfoToUe IEs, for a Home-Routed PDU session or a PDU session with I-SMF. However, there is no IEI for Service-level-AA container in theService-Level Authentication Command/Complete message as shown below, this requires that the Service-level-AA container need to be transferred separately between (H-)SMF and V/I-SMF. Table 8.3.17.1.1: SERVICE-LEVEL AUTHENTICATION COMMAND message content IEI; Information Element; Type/Reference; Presence; Format; Length. Extended protocol discriminator; Extended protocol discriminator 9.2; M; V; 1. PDU session ID; PDU session identity 9.4; M; V; 1. PTI; Procedure transaction identity 9.6; M; V; 1. SERVICE-LEVEL AUTHENTICATION COMMAND message identity; Message type 9.7; M; V; 1. Service-level-AA container; Service-level-AA container 9.11.2.10; M; LV-E; 5-n. Table 8.3.17.1.1: SERVICE-LEVEL AUTHENTICATION COMMAND message content IEI; Information Element; Type/Reference; Presence; Format; Length. Extended protocol discriminator; Extended protocol discriminator 9.2; M; V; 1. PDU session ID; PDU session identity 9.4; M; V; 1. PTI; Procedure transaction identity 9.6; M; V; 1. SERVICE-LEVEL AUTHENTICATION COMMAND message identity; Message type 9.7; M; V; 1. Service-level-AA container; Service-level-AA container 9.11.2.10; M; LV-E; 5-n. Therefore, it is proposed to introduce a new simple data type for Service-level AA Container. This Service-level AA Container should be included in VsmfUpdateData and VsmfUpdatedData. (See also clause 6.3.1A of 3GPP TS 24.501 which describes Service-level authentication and authorization procedure, it is always (H-)SMF who initiates the procedure) |
| Summary of change: |  | Add ServiceLevelAaContainer as a simple data type VsmfUpdateData and VsmfUpdatedData to convey the Service-Level AA Container included in the Service-Level Authentication Command/Complete respectively. Add Service-level-AA container IE into the n1SmInfoFromUe and n1SmInfoToUe for other NAS messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UUAA-SM procedure can not be properly performed and UAS service can not be offered for HR PDU session and a PDU session with I-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.3.2, 6.1.6.2.15, 6.1.6.2.16, 6.1.6.4.4, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible correction the following openAPI file: TS29502_Nsmf_PDUSession.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Add ServiceLevelAaContainer as a simple data type VsmfUpdateData and VsmfUpdatedData to convey the Service-Level AA Container included in the Service-Level Authentication Command/Complete respectively. And then remove the new Note added in clause 6.1.6.4.4. |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service-based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Data within Create SM Context Request |
| SmContextCreatedData | 6.1.6.2.3 | Data within Create SM Context Response |
| SmContextUpdateData | 6.1.6.2.4 | Data within Update SM Context Request |
| SmContextUpdatedData | 6.1.6.2.5 | Data within Update SM Context Response |
| SmContextReleaseData | 6.1.6.2.6 | Data within Release SM Context Request |
| SmContextRetrieveData | 6.1.6.2.7 | Data within Retrieve SM Context Request |
| SmContextStatusNotification | 6.1.6.2.8 | Data within Notify SM Context Status Request |
| PduSessionCreateData | 6.1.6.2.9 | Data within Create Request |
| PduSessionCreatedData | 6.1.6.2.10 | Data within Create Response |
| HsmfUpdateData | 6.1.6.2.11 | Data within Update Request towards H-SMF, or from I-SMF to SMF |
| HsmfUpdatedData | 6.1.6.2.12 | Data within Update Response from H-SMF to V-SMF, or from SMF to I-SMF |
| ReleaseData | 6.1.6.2.13 | Data within Release Request |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF to V-SMF, or from SMF to I-SMF |
| VsmfUpdateData | 6.1.6.2.15 | Data within Update Request towards V-SMF, or from SMF to I-SMF |
| VsmfUpdatedData | 6.1.6.2.16 | Data within Update Response from V-SMF, or from I-SMF to SMF |
| StatusNotification | 6.1.6.2.17 | Data within Notify Status Request |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |
| SmContextRetrievedData | 6.1.6.2.27 | Data within Retrieve SM Context Response |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF, or from I-SMF to SMF |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF, or from SMF to I-SMF |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF, or from SMF to I-SMF |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |
| SmContext | 6.1.6.2.39 | Complete SM Context |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |
| PsaInformation | 6.1.6.2.41 | PSA Information |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |
| N4Information | 6.1.6.2.43 | N4 Information |
| IndirectDataForwardingTunnelInfo | 6.1.6.2.44 | Indirect Data Forwarding Tunnel Information |
| SmContextReleasedData | 6.1.6.2.45 | Data within Release SM Context Response |
| ReleasedData | 6.1.6.2.46 | Data within Release Response |
| SendMoDataReqData | 6.1.6.2.47 | Data within Send MO Data Request |
| CnAssistedRanPara | 6.1.6.2.48 | SMF derived CN assisted RAN parameters tuning |
| UlclBpInformation | 6.1.6.2.49 | UL CL or BP Information |
| TransferMoDataReqData | 6.1.6.2.50 | Data within Transfer MO Data Request |
| TransferMtDataReqData | 6.1.6.2.51 | Data within Transfer MT Data Request |
| TransferMtDataError | 6.1.6.2.52 | Transfer MT Data Error Response |
| TransferMtDataAddInfo | 6.1.6.2.53 | Transfer MT Data Error Response Additional Information |
| VplmnQos | 6.1.6.2.54 | VPLMN QoS |
| DdnFailureSubs | 6.1.6.2.55 | DDN Failure Subscription |
| RetrieveData | 6.1.6.2.56 | Data within Retrieve Request |
| RetrievedData | 6.1.6.2.57 | Data within Retrieve Response |
| SecurityResult | 6.1.6.2.58 | Security Result |
| UpSecurityInfo | 6.1.6.2.59 | User Plane Security Information |
| DdnFailureSubInfo | 6.1.6.2.60 | DDN Failure Subscription Information |
| AlternativeQosProfile | 6.1.6.2.61 | Alternative QoS Profile |
| ProblemDetailsAddInfo | 6.1.6.2.62 | Problem Details Additional Information |
| ExtProblemDetails | 6.1.6.2.63 | Extended Problem Details |
| QosMonitoringInfo | 6.1.6.2.64 | QoS Monitoring Information |
| IpAddress | 6.1.6.2.65 | IP Address |
| RedundantPduSessionInformation | 6.1.6.2.66 | Redundant PDU Session Information |
| QosFlowTunnel | 6.1.6.2.67 | Tunnel Information per QoS Flow |
| TargetDnaiInfo | 6.1.6.2.68 | Target DNAI Information |
| AfCoordinationInfo | 6.1.6.2.69 | AF Coordination Information |
| NotificationInfo | 6.1.6.2.70 | Notification Correlation ID and Notification URI provided by the NF service consumer |
| AnchorSmfFeatures | 6.1.6.2.71 | Anchor SMF supported features |
| HrsboInfoFromVplmn | 6.1.6.2.72 | HR-SBO Information signaled from VPLMN |
| HrsboInfoFromHplmn | 6.1.6.2.73 | HR-SBO Information signaled from HPLMN |
| EasInfoToRefresh | 6.1.6.2.74 | EAS information to be refreshed for EAS re-discovery |
| EcnMarkingCongestionInfoReq | 6.1.6.2.75 | ECN Marking or Congestion Information Request |
| EcnMarkingCongestionInfoStatus | 6.1.6.2.76 | ECN Marking or Congestion Information Status |
| TscAssistanceInformation | 6.1.6.2.77 | TSC Assistance Information |
| N6JitterInformation | 6.1.6.2.78 | N6 Jitter Information |
| TrafficInfluenceInfo | 6.1.6.2.79 | Traffic influence information applicable at the VPLMN for an HR-SBO PDU session. |
| TrafficInfluenceData | 6.1.6.2.80 | Traffic influence data comprising the Service Data Flow description and the Application Function influence on traffic routing Enforcement Control parameters of a PCC rule. |
| LocalOffloadingMgtInfoFromIsmf | 6.1.6.2.81 | Information signaled by the I-SMF to the SMF for I-SMF based Local Offloading Management |
| LocalOffloadingMgtInfoToIsmf | 6.1.6.2.82 | Information signaled by the SMF to the I-SMF for I-SMF based Local Offloading Management |
| AvailableBitrateMonitoringRequest | 6.1.6.2.83 | Available Bitrate Monitoring Request information |
| Teid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |
| EpsBearerContextStatus | 6.1.6.3.2 | EPS Bearer context status |
| DrbId | 6.1.6.3.2 | Data Radio Bearer Identifier |
| AdditionalTnlNb | 6.1.6.3.2 | Indication of additional indirect data forwarding tunnel for multi-connectivity |
| ForwardingBearerContainer | 6.1.6.3.2 | Forwarding bearer container |
| SecondaryRatUsageDataReportContainer | 6.1.6.3.2 | Secondary RAT usage data report container |
| DnsServerSecurityInformation | 6.1.6.3.2 | DNS server security information |
| PduSessionPriority | 6.1.6.3.2 | PDU Session Priority |
| ServiceLevelAaContainer | 6.1.6.3.2 | Service Level AA Container |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |
| HoState | 6.1.6.3.4 | Handover State |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |
| Cause | 6.1.6.3.8 | Cause information |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |
| QosFlowAccessType | 6.1.6.3.18 | Access type associated with the QoS Flow |
| UnavailableAccessIndication | 6.1.6.3.19 | Indicates the access type of a MA PDU session that is unavailable |
| ProtectionResult | 6.1.6.3.20 | Protection Result of the security policy indicated as "preferred" |
| QosMonitoringReq | 6.1.6.3.21 | Indicates to measure UL, or DL, or both UL/DL delays, or to stop on-going measurements. |
| Rsn | 6.1.6.3.22 | Redundancy Sequence Number |
| SmfSelectionType | 6.1.6.3.23 | SMF Selection Type |
| PduSessionContextType | 6.1.6.3.24 | PDU Session Context Type |
| PendingUpdateInfo | 6.1.6.3.25 | Pending Update Information |
| EstablishmentRejectionCause | 6.1.6.3.26 | PDU Session Establishment Rejection Cause |
| EcnMarkingReq | 6.1.6.3.27 | ECN Marking Request |
| CongestionInfoReq | 6.1.6.3.28 | Congestion Information Request |
| ActivationStatus | 6.1.6.3.29 | Activation Status |
| QosMonitoringPdSupported | 6.1.6.3.30 | QoS Monitoring for packet delay supported information |
| QosMonitoringCongestionSupported | 6.1.6.3.31 | QoS Monitoring for congestion supported information |
| AvailableBitrateRequest | 6.1.6.3.32 | Available Bitrate Request |
| AvailBitRateMonSupported | 6.1.6.3.33 | Available Bitrate Monitoring Support Indication |
| UliGranularity | 6.1.6.3.34 | ULI Granularity |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |
| PduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |
| PduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |
| UserLocation | 3GPP TS 29.571 [13] | User Location |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |
| RatType | 3GPP TS 29.571 [13] | RAT Type |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |
| ApnRateStatus | 3GPP TS 29.571 [13] | APN Rate Control Status |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |
| NfSetId | 3GPP TS 29.571 [13] | NF Set Identifier |
| MoExpDataCounter | 3GPP TS 29.571 [13] | MO Exception Data Counter |
| DddTrafficDescriptor | 3GPP TS 29.571 [13] | Traffic Descriptor |
| NfServiceSetId | 3GPP TS 29.571 [13] | NF Service Set ID |
| RedirectResponse | 3GPP TS 29.571 [13] | Response body of the redirect response message |
| ServerAddressingInfo | 3GPP TS 29.571 [13] | Information of a Provisioning Server |
| PcfUeCallbackInfo | 3GPP TS 29.571 [13] | The callback information of the PCF for the UE to allow the PCF for the PDU session to send SM Policy Association Establishment and Termination events notification |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [13] | Satellite backhaul category |
| Uint16 | 3GPP TS 29.571 [13] | Unsigned 16-bit integer |
| GeoSatelliteId | 3GPP TS 29.571 [13] | GEO satellite ID (string) |
| Uinteger | 3GPP TS 29.571 [13] | Unsigned Integer common data type |
| VplmnOffloadingInfo | 3GPP TS 29.571 [13] | VPLMN Specific Offloading Information |
| GroupId | 3GPP TS 29.571 [13] | Internal Group Id |
| OffloadIdentifier | 3GPP TS 29.571 [13] | Offload identifier uniquely identifying a VPLMN offloading policy information instance from the HPLMN (for HR-SBO) or a Local Offloading Management information instance of the PLMN (for I-SMF based Local Offloading Management). |
| Ipv4AddressRange | 3GPP TS 29.571 [13] | IPv4 address range |
| Ipv6AddressRange | 3GPP TS 29.571 [13] | IPv6 address range |
| Fqdn | 3GPP TS 29.571 [13] | FQDN |
| PduSetQosPara | 3GPP TS 29.571 [13] | PDU Set QoS Parameters |
| SmfChargingId | 3GPP TS 29.571 [13] | SMF Charging Identifier |
| Int32 | 3GPP TS 29.571 [13] | Signed 32-bit integer |
| VplmnDlAmbr | 3GPP TS 29.571 [13] | Authorized DL Session AMBR for Offloading for the V-PLMN |
| UeLevelMeasurementsConfiguration | 3GPP TS 29.571 [13] | 5GC UE level measurements configuration |
| LocalOffloadingManagementInfo | 3GPP TS 29.571 [13] | Local Offloading Management Information |
| SatelliteId | 3GPP TS 29.571 [13] | Unique identifier of a satellite |
| Bitrate | 3GPP TS 29.571 [13] | Bit rate |
| EcsAddrConfigInfo | 3GPP TS 29.503 [46] | ECS Address Configuration Information |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |
| WAgfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the W-AGF |
| TngfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TNGF |
| TwifInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TWIF |
| ChargingInformation | 3GPP TS 29.512 [30] | CHF addresses |
| TrafficControlData | 3GPP TS 29.512 [30] | Contains parameters influencing the routing of traffic for service data flows. |
| FlowInformation | 3GPP TS 29.512 [30] | Ethernet or IP flow packet filter information |
| MultiModalId | 3GPP TS 29.514 [49] | Multi-Modal Service Identifier |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |
| SbiBindingLevel | 3GPP TS 29.518 [20] | SBI Binding Level |
| IpIndex | 3GPP TS 29.519 [38] | Information that identifies which IP pool or external server is used to allocate the IP address. |
| EventType | 3GPP TS 29.564 [48] | Event type supported by the UPF Event Exposure service |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |

# *** Next Change ***

##### 6.1.6.2.15	Type: VsmfUpdateData

Table 6.1.6.2.15-1: Definition of type VsmfUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestIndication | RequestIndication | M | 1 | This IE shall indicate the request type. |  |
| sessionAmbr | Ambr | C | 1 | This IE shall be present if the Session AMBR authorized for the PDU session is modified. When present, it shall contain the new Session AMBR authorized for the PDU session. |  |
| qosFlowsAddModRequestList | array(QosFlowAddModifyRequestItem) | C | 1..N | This IE shall be present if QoS flows are requested to be established or modified. |  |
| qosFlowsRelRequestList | array(QosFlowReleaseRequestItem) | C | 1..N | This IE shall be present if QoS flows are requested to be released. |  |
| epsBearerInfo | array(EpsBearerInfo) | C | 1..N | This IE shall be present if the PDU session may be moved to EPS during its lifetime and the ePSBearerInfo has changed. When present, it shall only include epsBearerInfo IE(s) for new EBI or for EBIs for which the epsBearerInfo has changed. The complete epsBearerInfo shall be provided for an EBI that is included (i.e. the epsBearerInfo newly received for a given EBI replaces any epsBearerInfo previously received for this EBI). |  |
| assignEbiList | array(Arp) | C | 1..N | This IE shall be present if the H-SMF requests EBIs to be assigned. |  |
| revokeEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present if the H-SMF/SMF requests the V-SMF/I-SMF to revoke some EBI(s). When present, it shall contain the EBIs to revoke. |  |
| modifiedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present if a PDU session modification procedure resulted in the change of ARP for a QoS flow that was already allocated an EBI. |  |
| pti | ProcedureTransactionId | C | 0..1 | This IE shall be present if the request is sent in response to a UE requested PDU session modification or release. When present, it shall contain the PTI value received in the corresponding request. |  |
| n1SmInfoToUe | RefToBinaryData | C | 0..1 | This IE shall be present if the H-SMF/SMF needs to send N1 SM information to the UE that does not need to be interpreted by the V-SMF/I-SMF. When present, this IE shall reference the n1SmInfoToUe binary data (see clause 6.1.6.4.4). |  |
| alwaysOnGranted | boolean | C | 0..1 | This IE shall be present if: - an alwaysOnRequested IE was received in an earlier V-SMF/I-SMF initiated Update request to change the PDU session to an always-on PDU session; or - the H-SMF/SMF determines, based on local policy, that the PDU session needs to be established as an always-on PDU session. When present, it shall be set as follows: - true: always-on PDU session granted. - false (default): always-on PDU session not granted. |  |
| hsmfPduSessionUri | Uri | C | 0..1 | This IE shall be included if: - an Update Request is sent to the V-SMF/I-SMF before the Create Response (e.g. for EPS bearer ID allocation as specified in clause 4.11.1.4.1 of 3GPP TS 23.502 [3], or for Secondary authorization/authentication as specified in clause 4.3.2.3 of 3GPP TS 23.502 [3]), and - the H-SMF PDU Session Resource URI has not been previously provided to the V-SMF/I-SMF. This IE shall not be included otherwise. When present, this IE shall include the URI representing the PDU session resource in the H-SMF. |  |
| newSmfId | NfInstanceId | C | 0..1 | This IE may be present if the anchor SMF has changed and the SMF Instance ID of the new anchor SMF has not been already signalled to the I-SMF or V-SMF. When present, it shall carry the NF instance identifier of the new anchor SMF handling the PDU session. |  |
| newSmfPduSessionUri | Uri | C | 0..1 | This IE shall be present if the newSmfId is present. When present, it shall carry the URI representing the updated PDU session resource in the new anchor SMF. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if "hsmfPduSessionUri" IE is present and at least one feature defined in clause 6.1.8 is supported. |  |
| cause | Cause | O | 0..1 | When present, this IE shall indicate the cause for the requested modification. |  |
| n1smCause | string | O | 0..1 | When present, this IE shall contain the 5GSM cause the H-SMF proposes the V-SMF/I-SMF to send to the UE. It shall be encoded as two characters in hexadecimal representation with each character taking a value of "0" to "9" or "A" to "F", and represent the cause value of the 5GSM cause IE specified in clause 9.11.4.2 of 3GPP TS 24.501 [7]. Example: the cause "Invalid mandatory information" shall be encoded as "60". See NOTE. |  |
| backOffTimer | DurationSec | O | 0..1 | When present, this IE shall indicate a Back-off timer value, in seconds, that the V-SMF/I-SMF may use when sending the NAS message (PDU Session Release Command) towards the UE. |  |
| maReleaseInd | MaReleaseIndication | C | 0..1 | This IE shall be present if one access of a MA PDU session is to be released, when H-SMF or SMF initiates MA PDU session release over one access. When present, it shall indicate the access requested to be released. | MAPDU |
| maAcceptedInd | boolean | C | 0..1 | This IE shall be present if a request to modify a single access PDU session into a MA PDU session was accepted (see clause 4.22.6.3 of 3GPP TS 23.502 [3]). When present, it shall be set as follows: - true: MA PDU session - false (default): single access PDU session | MAPDU |
| additionalCnTunnelInfo | TunnelInfo | C | 0..1 | This IE shall be present for a MA-PDU session if the UE requested to establish resources for a MA PDU session over the other access. When present, it shall contain additional N9 tunnel information of the UPF controlled by the H-SMF or SMF. | MAPDU |
| dnaiList | array(Dnai) | C | 0..N | This IE shall be present if received from PCF during I-SMF Related Procedures with PCF (see clause 4.23.6.2 in 3GPP TS 23.502 [3]). When present, the IE shall include a list of DNAI(s) the SMF deems relevant for the PDU Session. If the I-SMF and the SMF support the DTSSA-Ext1 feature, when present, this IE should include the full list of DNAIs of interest for PDU session, including DNAIs that may not be supported by the I-SMF and excluding the ones supported by the Anchor SMF. If this IE is not present, the I-SMF shall consider that the dnaiList has not changed. If there is no more DNAI of interest for the PDU session, the dnaiList attribute shall be present and be encoded as an empty array. | DTSSA DTSSA-Ext1 |
| n4Info | N4Information | O | 0..1 | This IE may be present if the SMF needs to send N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during a change of PSA). | DTSSA |
| n4InfoExt3 | N4Information | O | 0..1 | This IE may be present if the SMF needs to send additional N4 information to the I-SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during simultaneous change of BP/ULCL and PSA). | SCPBU |
| smallDataRateControlEnabled | boolean | C | 0..1 | This IE shall be present if the applicability of small data rate control on the PDU session changes. When present, it shall be set as follows: - true: small data rate control is applicable. - false: small data rate control is not applicable. | CIOT |
| qosMonitoringInfo | QosMonitoringInfo | C | 0..1 | This IE may be present if QoS monitoring has been activated for at least one QoS flow of the PDU session (see the qosMonitoringReq attribute in clause 6.1.6.2.22). | DTSSA |
| epsPdnCnxInfo | EpsPdnCnxInfo | C | 0..1 | This IE shall be present if the PDU session may be moved to EPS during its lifetime and the EpsInterworkingIndication is changed to "WITH_N26". The IE shall also be present when the EPS PDN Connection Context Information of the PDU session is changed, e.g. due to change of anchor SMF. |  |
| n9DataForwardingInd | boolean | C | 0..1 | This IE shall be present and set as specified in clauses 4.23.9.4 and 4.23.9.5 of 3GPP TS 23.502 [3] during simultaneous change of Branching Points or UL CLs controlled by I-SMF or controlled by different I-SMFs. When present, it shall be set as follows: - true: setup N9 forwarding tunnels between Branching Points or UL CLs; - false (default): N9 forwarding tunnels between Branching Points or UL CLs are not required to be setup (see clauses 5.2.2.8.3.6 and 5.2.2.8.3.10). | N9FSC |
| n9InactivityTimer | DurationSec | O | 0..1 | When present, this IE shall indicate an inactivity detection timer, in seconds, that the I-SMF may use to set the N9 forwarding tunnel inactive traffic detection timer in Branching Point or UL CL as specified in clauses 4.23.9.4 and 4.23.9.5 of 3GPP TS 23.502 [3]. | N9FSC |
| hrsboInfo | HrsboInfoFromHplmn | O | 0..1 | This IE may be present if the H-SMF and the V-SMF supports the HR-SBO feature and if the H-SMF needs to update the HR-SBO information towards the V-SMF, e.g. due to a change of the user subscription data or policy information. When present, this IE shall include the complete information for HR-SBO as defined in clause 6.1.6.2.73 and the V-SMF shall replace any earlier received HR-SBO information by the new HR-SBO information. | HR-SBO |
| localOffloadMgtInfo | LocalOffloadingMgtInfoToIsmf | O | 0..1 | This IE may be present if I-SMF based Local Offloading Management applies and any information defined in clause 6.1.6.2.82 needs to be signaled to the I-SMF. When present, this IE shall include the complete localOffloadMgtInfo information as defined in clause 6.1.6.2.82 and the I-SMF shall replace any earlier received localOffloadMgtInfo information by the new localOffloadMgtInfo information. | ISMF-LOM |
| altHplmnSnssai | Snssai | C | 0..1 | This IE shall be present for a HR PDU session if the H-SMF determines to retain the PDU session for the alternative HPLMN S-NSSAI, during network slice replacement. When present, this IE shall indicate the alternative HPLMN S-NSSAI to be used by the HR PDU session. See clause 5.15.19 of 3GPP TS 23.501 [2]. | NSRP |
| pduSessionRetainInd | boolean | C | 0..1 | This IE shall be present and set to true, if the H-SMF determines to retain the PDU session for the alternative HPLMN S-NSSAI during network slice replacement. When present with true value, this IE indicates the PDU session is retained for the alternative HPLMN S-NSSAI. Present with false value shall be prohibited. See clause 4.3.3.3 of 3GPP TS 23.502 [3]. | NSRP |
| pendingUpdateInfoList | array(PendingUpdateInfo) | O | 0..N | When present, this IE shall indicate the updated list of information that are not required to be updated in real-time to the (H-)SMF, i.e. the change of the listed information (e.g. UE location or Timezone) may be piggybacked in a subsequent essential update (e.g. to exchange the N1 message from the UE) to the (H-)SMF. The NF service consumer (i.e. I-SMF/V-SMF) should not trigger an Update to the (H-)SMF including only the change(s) of the listed information. When present, the NF service consumer (i.e. I-SMF/V-SMF) shall replace any pendingUpdateInfoList received earlier by the new information received in this IE. |  |
| netLocInfoReqInd | boolean | C | 0..1 | This IE shall be present with the value true when the (H-)SMF requires the V-SMF or I-SMF to provide the current NetLoc information (UE Location and Time Zone). The presence of this IE with the value "false" shall be prohibited. | NLIRN16 |
| uliReportingGranularity | UliGranularity | O | 0..1 | This IE may be present to indicate the needed granularity for UE Location Information (ULI) reporting, when the V/I-SMF sends an Update Request to the anchor SMF for the purpose of reporting immediately a ULI change (i.e. when the anchor SMF did not provide the pendingUpdateInfoList with UE_LOCATION). When present, the V/I-SMF shall report immediate ULI change to the (H-)SMF when a change at the indicated granularity level (e.g. gNB, TAI) occurs. See clause 5.51.2.2.2 of 3GPP TS 23.501 [2]. |  |
| serviceLevelAaContainer | ServiceLevelAaContainer | O | 0..1 | The IE may be present to convey the Service-level-AA container included in the SERVICE-LEVEL AUTHENTICATION COMMAND message. |  |
| NOTE: This IE contains information that the V-SMF shall transfer to the UE without interpretation. It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the 5GSM cause IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Release Command message. |  |  |  |  |  |

# *** Next Change ***

##### 6.1.6.2.16	Type: VsmfUpdatedData

Table 6.1.6.2.16-1: Definition of type VsmfUpdatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qosFlowsAddModList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows have been successfully established or modified. For each GBR QoS flow, the V-SMF/I-SMF may also indicate an alternative QoS profile which the NG-RAN currently fulfils in the currentQosProfileIndex IE or indicate that the NG-RAN cannot even fulfil the lowest alternative QoS profile. |  |
| qosFlowsRelList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows have been successfully released. |  |
| qosFlowsFailedtoAddModList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows failed to be established or modified. |  |
| qosFlowsVsmfRejectedAddModList | array(Qfi) | O | 1..N | This IE may be present from V-SMF to H-SMF when the qosFlowsFailedtoAddModList IE is present. When present, this IE shall include the QFIs of the QoS flows failed to be established or modified due to rejection by the V-SMF. When the H-SMF subsequently send N1 Message to the UE to revert the update of QoS rules and QoS Flow level QoS parameters associated with the failed QoS Flow(s) indicated in the qosFlowsFailedtoAddModList IE, the H-SMF should exclude the QoS flow(s) that were failed to be established or modified due to rejection by the V-SMF. If all the failed QoS flow(s) were due to rejection by the V-SMF, the H-SMF should skip the subsequent N1 message update to the UE. |  |
| qosFlowsFailedtoRelList | array(QosFlowItem) | C | 1..N | This IE shall be present if QoS flows failed to be released. |  |
| n1SmInfoFromUe | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF/I-SMF has received known N1 SM information from the UE that does not need to be interpreted by the V-SMF/I-SMF. When present, this IE shall reference the n1SmInfoFromUe binary data (see clause 6.1.6.4.4). |  |
| unknownN1SmInfo | RefToBinaryData | C | 0..1 | This IE shall be present if the V-SMF/I-SMF has received unknown N1 SM information from the UE. When present, this IE shall reference the unknownN1SmInfo binary data (see clause 6.1.6.4.4). |  |
| ueLocation | UserLocation | C | 0..1 | This IE shall be present if it is available and QoS flows have been successfully established, modified or released. This IE shall also be present if the "netLocInfoReqInd" IE is present in the request and if the UE location is available. When present, this IE shall contain the UE location information (see clause 5.2.3.4). |  |
| ueTimeZone | TimeZone | C | 0..1 | This IE shall be present if it is available and QoS flows have been successfully established, modified or released. This IE shall also be present if the "netLocInfoReqInd" IE is present in the request and if the Time Zone of the UE is available. When present, this IE shall contain the new UE Time Zone. |  |
| addUeLocation | UserLocation | O | 0..1 | Additional UE location. This IE may be present, if anType previously reported is a non-3GPP access and a valid 3GPP access user location information is available. When present, it shall contain: - the last known 3GPP access user location (see clause 5.2.3.4); and - the timestamp, if available, indicating the UTC time when the addUeLocation information was acquired. |  |
| assignedEbiList | array(EbiArpMapping) | C | 1..N | This IE shall be present if the AMF assigned the requested EBI(s). When present, it shall contain the EBIs that were successfully assigned. |  |
| failedToAssignEbiList | array(Arp) | C | 1..N | This IE shall be present if the AMF failed to assign EBIs for a set of ARPs. |  |
| releasedEbiList | array(EpsBearerId) | C | 1..N | This IE shall be present if the NF Service Consumer requested the revoke EBI(s) or if the AMF revoked already assigned EBI(s) for this PDU session towards the V-SMF. This IE shall contain the list of EBI(s) released for this PDU session at the AMF. |  |
| secondaryRatUsageReport | array(SecondaryRatUsageReport) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows. (NOTE) |  |
| secondaryRatUsageInfo | array(SecondaryRatUsageInfo) | O | 1..N | This IE may be present to report usage data for a secondary RAT for QoS flows and/or the whole PDU session. |  |
| n4Info | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt1 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF. | DTSSA |
| n4InfoExt2 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during a change of PSA). | DTSSA |
| n4InfoExt3 | N4Information | O | 0..1 | This IE may be present if the I-SMF needs to send additional N4 response information to the SMF for the control of traffic offloaded at a PSA/BP/ULCL controlled by an I-SMF (e.g. during simultaneous change of BP/ULCL and PSA). | SCPBU |
| modifiedEbiListNotDelivered | boolean | C | 0..1 | This IE should be present with the value true when the modifiedEbiList IE was received in VsmfUpdateData in request but cannot be updated to the AMF, e.g., during (H-)SMF triggered PDU session modification procedure and the AMF doesn’t support the EAEA feature. When present, the IE shall be set as following: - true: the received modifiedEbiList was not delivered to the AMF. - false: the received modifiedEbiList was delivered to the AMF. Absence of this IE means whether modifiedEbiList was delivered or not is unknown. |  |
| ecnMarkingCongestionInfoStatus | array(EcnMarkingCongestionInfoStatus) | C | 1..N | This IE shall be present when the V/I-SMF receives the ECN Marking or Congestion Monitoring Reporting Status, e.g. during an PDU Session Modification procedure. When present, this IE shall contain a list of QoS flows with status for QoS monitoring for congestion information or for ECN marking for L4S. | EMECI |
| pduSetSupportInd | boolean | C | 0..1 | This IE shall be included when the V/I-SMF receives the PDU Set based Handling Indicator from 5G-AN, e.g. during an PDU Session Modification procedure. When present, this IE shall indicate whether the PDU Set based handling is supported by the 5G-AN: - true: the PDU Set based handling is supported. - false: the PDU Set based handling is not supported. | PDUSH |
| qosMonitoringPdSupported | QosMonitoringPdSupported | C | 0..1 | The IE shall be present if the QME feature is supported by the I-SMF/V-SMF and SMF and the value of the qosMonitoringPdSupported attribute has changed since last update to the H-SMF (for HR PDU session) or SMF (for PDU sessions with an I-SMF). When present, this IE shall indicate whether the NG-RAN supports the QoS monitoring for packet delay feature. See clause 5.45.1 of 3GPP TS 23.501 [2]. | QME |
| qosMonitoringCongestionSupported | QosMonitoringCongestionSupported | C | 0..1 | This IE shall be present if the QME feature and the EMECI feature are is supported by the I-SMF/V-SMF and the SMF and the value of the qosMonitoringCongestionSupported attribute has changed since last update to the H-SMF (for HR PDU session) or SMF (for PDU sessions with an I-SMF). When present, this IE shall indicate whether the NG-RAN supports the QoS monitoring for congestion feature. See clause 5.45.1 of 3GPP TS 23.501 [2]. | QME |
| availBitRateMonSupported | AvailBitRateMonSupported | C | 0..1 | The IE shall be present if the AVABIT feature is supported by I-SMF/V-SMF and the SMF and the value of the availBitRateMonSupported attribute has changed since last update to the H-SMF (for HR PDU session) or SMF (for PDU sessions with an I-SMF). When present, this IE shall indicate whether the NG-RAN supports the available bitrate monitoring feature. See clause 5.45.1 of 3GPP TS 23.501 [2]. | AVABIT |
| serviceLevelAaContainer | ServiceLevelAaContainer | O | 0..1 | The IE may be present to convey the Service-level-AA container included in the SERVICE-LEVEL AUTHENTICATION COMPLETE message. |  |
| NOTE: An SMF complying with this version of the specification should report secondary RAT usage using the secondaryRatUsageInfo attribute that replaces the secondaryRatUsageReport attribute. |  |  |  |  |  |

# *** Next Change ***

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| ProcedureTransactionId | integer | Unsigned integer representing a Procedure Transaction Identity, within the range 0 to 255, as specified in 3GPP TS 24.007 [8]. |
| EpsBearerId | integer | Integer identifying an EPS bearer, within the range 0 to 15, as specified in clause 11.2.3.1.5, bits 5 to 8, of 3GPP TS 24.007 [8]. |
| EpsPdnCnxContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the UeEpsPdnConnection IE specified in Table 7.3.1-2 or Table 7.3.6-2 of 3GPP TS 29.274 [16] for the N26 interface. |
| EpsBearerContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the Bearer Context IE specified in Table 7.3.2-2 of 3GPP TS 29.274 [16]. |
| Teid | string | 4-octet GTP tunnel endpoint identifier, as defined in 3GPP TS 29.274 [16], in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the TEID shall appear first in the string, and the character representing the 4 least significant bit of the TEID shall appear last in the string. Pattern: "^[A-Fa-f0-9]{8}" Example: A GTP TEID 0x5BD60076 shall be encoded as "5BD60076". |
| EpsBearerContextStatus | string | EPS bearer context status, as defined in octets 3 and 4 of the EPS bearer context status IE in clause 9.9.2.1 of 3GPP TS 24.301 [27], in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "a" to "f" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the EPS bearer context status shall appear first in the string, and the character representing the 4 least significant bit of the EPS bearer context status shall appear last in the string. Pattern: '^[A-Fa-f0-9]{4}$' Example: An EPS bearer context status IE where only the EBIs 2, 5, 6 and 9 are active shall be encoded as "6402". |
| DrbId | integer | Unsigned integer representing a Data Radio Bearer Identity, within the range 1 to 32, as specified in clause 9.3.1.53 of 3GPP TS 38.413 [9]. |
| AdditionalTnlNb | integer | Unsigned integer, within the range 1 to 3, indicating whether this is the first, second or third additional indirect data forwarding tunnel for multi-connectivity. |
| ForwardingBearerContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the Bearer Context IE within Context Acknowledge specified in Table 7.3.7-2 of 3GPP TS 29.274 [16]. |
| SecondaryRatUsageDataReportContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the Secondary RAT Usage Data Report IE within Forward Relocation Complete Acknowledge specified in Table 7.3.4-1 of 3GPP TS 29.274 [16] (starting from octet 1). |
| DnsServerSecurityInformation | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the DNS server security information with length of two octets container specified in clause 10.5.6.3.1 of 3GPP TS 24.008 [47]. |
| PduSessionPriority | integer | Unsigned integer representing the priority of the PDU session, within the range 0 to 31 (where 0 indicates the highest priority and 31 indicates the lowest priority), that is used to determine the SBI Message Priority (SMP) to set in subsequent signaling related to this PDU session as specified in clauses 5.22.2 and 5.22.3 of 3GPP TS 23.501 [2]. |
| ServiceLevelAaContainer | string | String with format "byte" as defined in OpenAPI Specification [15], i.e. base64-encoded characters, encoding the Service-level-AA Container IE specified in clause 9.11.2.10 of 3GPP TS 24.501 [7], starting from octet 2, i.e. including length of two octets and Service-level-AA container contents, but not the IEI field. |

# *** Next Change ***

##### 6.1.6.4.4	n1SmInfoFromUe, n1SmInfoToUe, unknownN1SmInfo

n1SmInfoFromUe, n1SmInfoToUe and unknownN1SmInfo shall encode one or more NAS SM IEs, including the Type and Length fields, as specified in 3GPP TS 24.501 [7], using the vnd.3gpp.5gnas content-type.

Clause 5.2.3.1 specifies the information that shall be included in these payloads.

n1SmInfoFromUe and n1SmInfoToUe may encode the 5GS NAS IEs listed in tables 6.1.6.4.4-1 and 6.1.6.4.4-2.

Table 6.1.6.4.4-1: n1SmInfoFromUE content

| 5GS NAS IE | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Message type | 9.7 | All NAS SM messages |
| PDU session type | 9.11.4.11 | PDU Session Establishment Request |
| SSC mode | 9.11.4.16 | PDU Session Establishment Request |
| Maximum number of supported packet filters | 9.11.4.9 | PDU Session Establishment Request PDU Session Modification Request |
| Integrity protection maximum data rate | 9.11.4.7 | PDU Session Modification Request (NOTE 3) |
| SM PDU DN request container | 9.11.4.15 | PDU Session Establishment Request |
| Extended protocol configuration options | 9.11.4.6 | PDU Session Establishment Request PDU Session Authentication Complete PDU Session Modification Request PDU Session Modification Complete PDU Session Modification Command Reject PDU Session Release Request PDU Session Release Complete |
| EAP message | 9.11.2.2 | PDU Session Authentication Complete |
| Requested QoS rules | 9.11.4.13 | PDU Session Modification Request |
| Requested QoS flow descriptions | 9.11.4.12 | PDU Session Modification Request |
| 5GSM cause | 9.11.4.2 | PDU Session Modification Request PDU Session Release Request PDU Session Release Complete (NOTE 2) |
| 5GSM capability | 9.11.4.1 | PDU Session Establishment Request PDU Session Modification Request (NOTE 1) |
| Mapped EPS bearer contexts | 9.11.4.8 | PDU Session Modification Request |
| Remote UE context connected | 9.11.4.29 | Remote UE Report |
| Remote UE context disconnected | 9.11.4.29 | Remote UE Report |
| Non-3GPP delay budget | 9.11.4.37 | PDU Session Modification Request |
| URSP rule enforcement reports | 9.11.4.38 | PDU Session Establishment Request PDU Session Modification Request |
| Non-3GPP device identifier connection information | 9.11.4.X | PDU Session Modification Request |
| Service-level-AA container | 9.11.2.10 | PDU Session Establishment Request PDU Session Modification Request |
| NOTE 1: The 5GSM capability IE shall be encoded as received from the UE. It may contain UE capabilities that the V-SMF (or I-SMF) only needs to transfer to the H-SMF (or SMF), e.g. support of reflective QoS, or support of multi-homed IPv6 PDU session, and/or capabilities to be interpreted and used by the V-SMF (or I-SMF). NOTE 2: The 5GSM cause IE shall be encoded as received from the UE. This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Modification Command Reject message, in which case it shall be sent as a separate n1SmCause IE over N16/N16a and not within the n1SmInfoToUE binary data. NOTE 3: This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Establishment Request, in which case it shall be sent as separate maximum integrity protected data rate IEs over N16/N16a and not within the n1SmInfoFromUE binary data. |  |  |

Table 6.1.6.4.4-2: n1SmInfoToUE parameters

| 5GS NAS IE | Reference (3GPP TS 24.501 [7]) | Related NAS SM message |
| --- | --- | --- |
| Message type | 9.7 | All NAS SM messages |
| RQ timer value | 9.11.2.3 | PDU Session Establishment Accept PDU Session Modification Command |
| EAP message | 9.11.2.2 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Authentication Command PDU Session Authentication Result PDU Session Release Command |
| Allowed SSC mode | 9.11.4.5 | PDU Session Establishment Reject |
| Extended protocol configuration options | 9.11.4.6 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Authentication Command PDU Session Authentication Result PDU Session Modification Reject PDU Session Modification Command PDU Session Release Reject PDU Session Release Command |
| 5GSM cause | 9.11.4.2 | PDU Session Establishment Accept PDU Session Modification Command (NOTE 1) |
| Mapped EPS bearer contexts | 9.11.4.8 | PDU Session Establishment Accept PDU Session Modification Command |
| ATSSS container | 9.11.4.22 | PDU Session Establishment Accept PDU Session Modification Command |
| N3QAI | 9.11.4.36 | PDU Session Establishment Accept PDU Session Modification Command (NOTE 2) |
| Service-level-AA container | 9.11.2.10 | PDU Session Establishment Accept PDU Session Establishment Reject PDU Session Modification Command PDU Session Release Command |
| NOTE 1: This IE indicates the 5GSM cause the H-SMF (or SMF) requires the V-SMF (or I-SMF) to send to the UE. The V-SMF (or I-SMF) shall transfer the received value to the UE without interpretation. This information is defined as a "V" IE (i.e. without a Type field) in other NAS messages, e.g. PDU Session Establishment Reject message, in which case it shall be sent as a separate n1SmCause IE over N16/N16a and not within the n1SmInfoToUE binary data. NOTE 2: N3QAI is configured in SMF based on the S-NSSAI and DNN for PIN service (see clause 5.44.3.3 of 3GPP TS 23.501 [2]). |  |  |

The Message Type shall be present and encoded as the first 5GS NAS IE in any n1SmInfoFromUe, n1SmInfoToUe and unknownN1SmInfo binary data, to enable the receiver to decode the 5GS NAS IEs.

NOTE:	The Information Element Identifier (see clause 11.2.1.1.3 of 3GPP TS 24.007 [8]) of a 5GS NAS IE uniquely identifies an IE in a given message.

# *** Next Change ***

# A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.4.0-alpha.5'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.502 V19.4.0; 5G System; Session Management Services; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.502/

********Skipped for clarity*******

VsmfUpdateData:

description: Data within Update Request towards V-SMF, or from SMF to I-SMF

type: object

properties:

requestIndication:

$ref: '#/components/schemas/RequestIndication'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsAddModRequestList:

type: array

items:

$ref: '#/components/schemas/QosFlowAddModifyRequestItem'

minItems: 1

qosFlowsRelRequestList:

type: array

items:

$ref: '#/components/schemas/QosFlowReleaseRequestItem'

minItems: 1

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

assignEbiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

minItems: 1

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

modifiedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

alwaysOnGranted:

type: boolean

default: false

hsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

newSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

newSmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

cause:

$ref: '#/components/schemas/Cause'

n1smCause:

type: string

backOffTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

maAcceptedInd:

type: boolean

default: false

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

n4InfoExt3:

$ref: '#/components/schemas/N4Information'

smallDataRateControlEnabled:

type: boolean

qosMonitoringInfo:

$ref: '#/components/schemas/QosMonitoringInfo'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

n9DataForwardingInd:

type: boolean

default: false

n9InactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

hrsboInfo:

$ref: '#/components/schemas/HrsboInfoFromHplmn'

localOffloadMgtInfo:

$ref: '#/components/schemas/LocalOffloadingMgtInfoToIsmf'

altHplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

pduSessionRetainInd:

type: boolean

enum:

- true

pendingUpdateInfoList:

type: array

items:

$ref: '#/components/schemas/PendingUpdateInfo'

netLocInfoReqInd:

type: boolean

enum:

- true

uliReportingGranularity:

$ref: '#/components/schemas/UliGranularity'

serviceLevelAaContainer:

$ref: '#/components/schemas/ServiceLevelAaContainer'

required:

- requestIndication

********Skipped for clarity*******

VsmfUpdatedData:

description: Data within Update Response from V-SMF, or from I-SMF to SMF

type: object

properties:

qosFlowsAddModList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsRelList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsFailedtoAddModList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsVsmfRejectedAddModList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

minItems: 1

qosFlowsFailedtoRelList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

assignedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

failedToAssignEbiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

minItems: 1

releasedEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

secondaryRatUsageReport:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageReport'

minItems: 1

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

n4InfoExt3:

$ref: '#/components/schemas/N4Information'

modifiedEbiListNotDelivered:

type: boolean

ecnMarkingCongestionInfoStatus:

type: array

items:

$ref: '#/components/schemas/EcnMarkingCongestionInfoStatus'

minItems: 1

pduSetSupportInd:

type: boolean

qosMonitoringPdSupported:

$ref: '#/components/schemas/QosMonitoringPdSupported'

qosMonitoringCongestionSupported:

$ref: '#/components/schemas/QosMonitoringCongestionSupported'

availBitRateMonSupported:

$ref: '#/components/schemas/AvailBitRateMonSupported'

serviceLevelAaContainer:

$ref: '#/components/schemas/ServiceLevelAaContainer'

********Skipped for clarity*******

# SIMPLE DATA TYPES

#

ProcedureTransactionId:

description: Procedure Transaction Identifier

type: integer

minimum: 0

maximum: 255

EpsBearerId:

description: EPS Bearer Identifier

type: integer

minimum: 0

maximum: 15

EpsPdnCnxContainer:

description: UE EPS PDN Connection container from SMF to AMF

type: string

format: byte

EpsBearerContainer:

description: EPS Bearer container from SMF to AMF

type: string

format: byte

Teid:

type: string

description: GTP Tunnel Endpoint Identifier

pattern: '^[A-Fa-f0-9]{8}$'

EpsBearerContextStatus:

description: EPS bearer context status

type: string

pattern: '^[A-Fa-f0-9]{4}$'

DrbId:

description: Data Radio Bearer Identity

type: integer

minimum: 1

maximum: 32

AdditionalTnlNb:

description: indicates first, second or third additional indirect data forwarding tunnel

type: integer

minimum: 1

maximum: 3

ForwardingBearerContainer:

description: Forwarding Bearer Container

type: string

format: byte

SecondaryRatUsageDataReportContainer:

description: Secondary Rat Usage Data Report Container

type: string

format: byte

DnsServerSecurityInformation:

description: DNS server security information

type: string

format: byte

PduSessionPriority:

description: The priority of the PDU session

type: integer

ServiceLevelAaContainer:

description: Service Level AA Container

type: string

format: byte

#

# ENUMERATIONS

********Skipped for clarity*******

# *** End of Changes ***