3GPP TSG-CT WG4 Meeting #133	C4-260325

Goa, India; 09th – 13th February 2026	 	                                                          (revision of C4-260088)

# Source:	Nokia, Huawei

# Title:	New SID on Control Plane Protocols in Core Network of the 6G System

# Document for:	Discussion

# Agenda Item:	20.2.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Control Plane Protocols in Core Network of the 6G System

Acronym:	FS_6G_CPCN_CT

Unique identifier:	TBD

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1080057 | Study on Architecture for 6G System | SA2 6G study; TR 23.801-01 |
| 1050110 | Study on 6G Use Cases and Service Requirements | SA1 6G study; TR 22.870 |
| 1060079 | Study on 6G Scenarios and Requirements | RAN 6G study; TR 38.914 |
| 1080072 | Study on 6G Radio | RAN 6G study; TR 38.760-3 |
| 1090044 | Study on Security for the 6G System | SA3 6G study; TR 33.801-01 |
| 800045 | Study of IETF QUIC Transport for 5GC Service Based Interfaces | CT4 study of HTTP/3 for 5GS in Rel-18; TR 29.893 |

# 3	Justification

SA1 is currently studying the 6G use cases and service/operational requirements (FS_6G_REQ study item), SA2 the system architecture of the 6G system (FS_6G_ARC study item), and SA3 the security aspects of 6GS (FS_6G_SEC study item). In addition, TSG RAN and RAN WGs are also studying the requirements and architecture for 6G Radio.

CT4 is responsible for the overall design of protocols within the Core Network. This study item focuses on the control plane protocol for the core network of the 6G system. It aims to study the technical realization of the control plane in the core network, analyse protocol candidates for core network, and provide technical inputs for subsequent standardization of the core network protocols of the 6GS. The study will align with the service requirements defined by SA1 and the architectural requirements specified by SA2.

CT4 also studied in Rel-18 (FS_QUIC study item) HTTP/3 for 5GC Service-Based Interfaces and concluded in TR 29.893 not to adopt HTTP/3 as a possible alternative protocol option to HTTP/2, considering among others the lack of maturity of specifications and implementations. Since then, IETF has finalized the QUIC and HTTP/3 RFCs and HTTP/3 implementations are expected to have gained in maturity, enabling further evaluation of HTTP/3 as one of the potential candidate protocols for the Control Plane of the 6GS.

# 4	Objective

The objective of this study item is to study stage 3 protocol aspects for the control plane within the core network of the 6G System, based on the service requirements developed by SA1, architecture requirements developed by SA2, security requirements developed by SA3, and new radio access technologies developed by RAN.

More specifically, the CT4 study should be based on SA2 agreements documented in TR 23.801-1. In its early phase, prior to SA2 interim agreements and conclusions being available, the CT4 work shall rely on the Architectural Assumptions and Requirements, Key Issues and Solutions specified in TR 23.801-1. The study shall be aligned with the latest status of the SA2 study. The study shall not conclude on items under discussion by stage 2 working groups for which conclusions have not been agreed yet.

NOTE:	How to enable the use of AI (e.g. AI agent) will be described in TR 23.801-1.

The 6G System shall support multi-vendor interoperable Control Plane interfaces within the Core Network.

The expected work includes the following tasks:

- Study candidate protocols for the Control Plane within the Core Network of the 6G System, based on architectural and functional requirements (including enabling the use of AI) defined by SA2.

- Study the protocol framework for the technical realization of the Control Plane within the Core Network for 6GS, including:

  - Review potential limitations of the Core Network protocol framework defined in 5GS and study possible improvements or simplifications to overcome the identified limitations.

  - Study possible protocol framework impacts required to support 6G services and requirements (e.g. to enable the use of AI) defined by SA2.

  - Study possible protocol impacts required to support 6G security requirements defined by SA3.

- Study the technical realization of the registration and discovery framework for 6GS, including:

  - Review potential limitations of the registration and discovery framework defined in 5GS and study possible improvements to overcome the identified limitations, in coordination with SA2 for aspects that are dependent on stage 2 requirements.

  - Study the impacts required to support 6G services and requirements (e.g. to enable the use of AI) defined by SA2.

- Study protocol aspects for the event exposure framework within the Core Network for 6GS.

  - Review potential limitations of the event exposure framework mechanisms within the Core Network defined in 5GS and study protocol solutions for the event exposure framework within the Core Network in 6GS, based on architectural and functional requirements defined by SA2.

- Study protocol impacts for supporting interworking between 5GS and 6GS, based on SA2 requirements.

- Study the design of subscriber data models and procedures for 6GS, following SA2 requirements.

- Study protocol aspects for supporting essential and regulatory services (such as Voice Services, Emergency Services, Location Services, Messaging Services, MPS/MCX Service, PWS) in accordance with requirements from SA2 and CT1.

- Study the Model, Principles and Guidelines for API definitions in 6GS and Interface Definition Languages.

- Study the formats of UE and Network Identifications for 6GS, following SA2 requirements.

The expected work will be updated based on the progress of the stage 2 studies in SA2 and SA3.

# 5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 29.8xx | Study on Control Plane Protocols in Core Network of the 6G System | TSG#115 (March 2027) | TSG#116 (June 2027) | {<FamilyName>, <GivenName>, <Company>, <email address>. SA2 or SA3 status will be added as a note when unstable topics in SA2 or SA3 are described in the TR. |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

# 7	Work item leadership

CT4

# 8	Aspects that involve other WGs

SA1 for the service aspects, SA2 for the architectural aspects, and SA3 for the security aspects.

CT4 will coordinate with RAN3 on WT#4.

CT4 will coordinate with CT3 on WT#5.

# 9	Supporting Individual Members

Supporting IM name

AT&T ?

CATT

China Mobile

China Unicom

Cisco ?

Deutsche Telekom ?

HPE ?

Huawei

Nokia

Oracle ?

Samsung ?

SIA

T-Mobile USA ?

Turkcell

Verizon ?

vivo

Vodafone ?

Xiaomi

ZTE ?