3GPP TSG-CT6 Meeting #123	C6-250463

# Stor-Göteborg, Sweden, 26th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0822 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TC 27.22.4.7.3; PROACTIVE COMMAND: REFRESH 3.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R18 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 27.22.4.7.3 needs correction as in the expected sequence 3.7 on ”(REFRESH, Steering of roaming with SOR-CMCI criterions 'DNN' and 'S-NSSAI SST and SD' and 'match all')” the coding of the length of the SOR-CMCI rule contents is incorrect. Quote of 27.22.4.7.3 Expected Sequence 3.7: As seen in the picture above, the Length of the SOR CMCI rule contents should be “00 0C” and not ”00 0B”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The coding of the coding of the length of the SOR-CMCI rule contents which belong to the the expected sequence 3.7 of TC 27.22.4.7.3 is corrected so it shows “00 0C”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the test case 27.22.4.7.3 the coding of the PROACTIVE COMMAND: REFRESH 3.7.2 is not correct. Hence, a conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 27.22.4.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

##### 27.22.4.7.3	REFRESH (Steering of roaming)

*****skip unchanged part change *****

Expected Sequence 3.7 (REFRESH, Steering of roaming with SOR-CMCI criterions 'DNN' and 'S-NSSAI SST and SD' and 'match all')

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | NG-SS | The NG-SS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - TAI (MCC/MNC/TAC): 254/001/000001. - Access control: unrestricted. |  |
| 2 | ME NG-SS | The ME registers to the NG-SS. | No SOR transparent container Information Element in REGISTRATION ACCEPT message |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.1.1 | [Setting up LOCATION STATUS Event] |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1 |  |
| 7 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.5.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.7.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: REFRESH 3.7.2 |  |
| 11 | ME | Update of ME's internal memory on SOR-CMCI rules | [Not explicitly verified] |
| 12 | ME UICC | TERMINAL RESPONSE: REFRESH 3.7.2 | [normal ending] |
| 13 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 14 | ME NG-SS | First PDU Session is established successfully. | DNN = 'internet', S-NSSAI = ‘01010101’ |
| 15 | ME NG-SS | Second PDU Session is established successfully. | DNN is not 'internet', Without S-NSSAI |
| 16 | ME NG-SS | First PDU session is released | First PDU session is released after SOR-CMCI Tsor-cm expiration (2 minutes) since step 14 completion |
| 17 | ME NG-SS | Second PDU session is released | Second PDU session is released after SOR-CMCI Tsor-cm expiration (3 minutes) since step 15 completion |
| 18 | USER ME | SWITCH OFF ME |  |

PROACTIVE COMMAND: REFRESH 3.7.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwAcT List

empty

SOR-CMCI rules

Rule 1:

Tsor-cm timer value:	'00100001' (1 minute)

Criterion type:	DNN

Criterion value:	internet

Rule 2:

Tsor-cm timer value:	'00100010' (2 minutes)

Criterion type:	S-NSSAI SST and SD

Criterion value:	'01 01 01 01' (SST: eMBB, SD: '010101')

Rule 3:

Tsor-cm timer value:	'00100011' (3 minutes)

Criterion type:	match all

Coding:

| BER-TLV: | D0 | 27 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 73 | 1A | 00 | 0C | 21 | 01 | 09 | 08 | 69 | 6E | 74 |
|  | 65 | 72 | 6E | 65 | 74 | 00 | 06 | 22 | 03 | 01 | 01 | 01 |
|  | 01 | 00 | 02 | 23 | FF |  |  |  |  |  |  |  |

TERMINAL RESPONSE: REFRESH 3.7.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.7.3.5	Test requirement

The ME shall operate in the manner defined in expected sequences 3.1 to 3.7.

For sequences 3.5 to 3.7, an extra guard time of 10% is allowed.

# *** End of Changes ***