3GPP TSG RAN Meeting #109		 RP-251939

Beijing, China, September 15 - 18, 2025

Agenda Item: 	8.2.1.3

Source:	Boost Mobile Network

Title:	Proposing Dual Stack for 6G RAN Study

Document for:	Discussion

# 1. Introduction

As mobile networks evolve toward 6G, ensuring interworking with existing generations is essential for seamless commercial connectivity services during the transition that usually takes considerable time due to the long lifecycle of legacy UEs. It is also important that the transition must have no impact on the existing stable production networks. At the time when 6G is standardized, it is anticipated that 4G and 5G mobile networks are still in operation in markets.

According to GSMA’s report “The Mobile Economy 2025”, 4G continues to dominate with 58% of global connections in 2024, compared to 25% on 5G. By 2030, when 6G deployment is anticipated, over 50% of connections in certain regions (e.g. Eurasia) will still rely on 4G. Therefore, interworking with 4G should be considered alongside 5G in 6G standardization to enable service continuity across different generations.

While 5G-6G interworking was approved as a 6G study item (RP-251881) at RAN Plenary #108, the decision on 4G-6G interworking was deferred. This contribution proposes to study Dual Stack as part of the 6G study item:

- To support 4G-6G and 5G-6G interworking without introducing additional impact on the RAN. It is noted that SA1 has tentatively agreed on a 6G-4G interworking requirement as part of 6G SI activities.

- To enable a gradual migration from 4G/5G to 6G without disrupting stable legacy production networks.

Conventional interworking between 6G and legacy systems (5G and 4G) would introduce significant complexity in 6G standardization and deployment. A 6G NodeB would need to support Xn, X2, and new intra-6G RAN interfaces (6G X). Alternatively, UEs could hand over from 6G to 5G and then to 4G using existing 4G-5G mobility mechanisms, but this approach does not address scenarios where operators wish to transition directly from 4G to 6G.

Dual Stack capable UEs can register with multiple generation networks simultaneously and maintain parallel connections, enabling seamless connection switching without service interruption during mobility. This approach can remove the need to upgrade legacy infrastructure for 6G interworking as it avoids direct cross-generation interfaces at RAN level.

Although Dual Stack introduces additional complexity at the UE side, somewhat similar techniques are already widely adopted in commercial multi-SIM devices. Overall, this approach is expected to simplify standardization by avoiding interface-heavy designs and to support a more robust and simpler 6G design.

# 2. Discussion

## 2.1 Study Item Objectives

The following topics are proposed for study. As Dual Stack is related to UE, RAN, and Core, collaboration between RAN and SA WGs is required.

1) Use Cases

The study will identify and analyze the following use cases utilizing Dual Stack.

- Network migration: How Dual Stack can facilitate smooth transition during 4G/5G to 6G network evolution, supporting scenarios with overlapping or disjoint coverage between different generations.

- Handover mechanisms: How Dual Stack can be used for UEs to switch between networks seamlessly as they move across coverage areas or when the serving network’s signal weakens.

- Dual connectivity utilization: How simultaneous dual connections can be leveraged to enhance throughput, improve reliability, or select the most suitable connection based on application requirements. ATSS(Access Traffic Steering, Switching, and Splitting) features may also be considered.

2) Network Architecture for Dual Stack

The study will target defining a reference network architecture to support Dual Stack operation. It will identify the required functionalities for RAN, Core, and UE as well as analyze the impact on relevant interfaces.

3) Features and Procedures

The study will identify features and define necessary procedures to support the identified use cases in 6G. For example, including:

- Seamless service continuity through network-level or application-level handover

- Uplink and downlink traffic steering, switching, splitting, duplicating functionality

4) Evaluation and Recommendation

The study will evaluate the feasibility and performance of Dual Stack approach as an alternative solution for interworking between 4G and 6G. Additionally, the study will assess whether this approach can be extended to other scenarios, such as 5G-6G interworking, NTN-6G integration, or non-3GPP-6G interworking, and provide recommendations accordingly.

## 2.2 Example Architecture Enabling Dual Stack Operation

Two representative network deployment scenarios are illustrated in Figure 1. The figure shows combinations of  4G&6G, and 5G&6G, which are intended to capture practical deployment conditions expected during the migration from legacy networks to 6G. The concept can be extended to any combination of two or multiple networks.

In both scenarios (Figure 1), a dual-stack capable UE may separately register with each network (e.g., 4G or 5G, and 6G). This allows the UE to operate in either dual-registration mode or single-registration mode at any given time, depending on operator policy, subscription configuration, or the availability of each network.

Figure 1.

(a) Dual-stack UEs with two independent networks            (b) Dual-stack UEs with two associated networks

In Figure 1(a), a dual-stack capable UE is independently served by two separate networks (e.g., 4G and 6G), each providing its distinct IP address. This model represents scenarios where networks are deployed and operated independently with distinct core network anchors. Dual stack operation allows the UE to maintain concurrent registrations, enabling seamless service continuity at the application layer.

In Figure 1(b), the two networks share a common IP anchor, e.g., UPF, acting as the user plan anchor. In this model, the dual stack capable UE is assigned a single IP address that is valid across both networks while maintaining independent control plane connections. This approach does not change IP addresses while switching networks and prevents session disruption for applications.

Depending on operator policy, subscription configuration, and network availability or status, the UE should be able to dynamically select the most suitable connection for data transmission at any given moment. The selection may also be influenced by application requirements. For example, a network throughput testing application may mandate the use of a specific network, while normal traffic can be balanced or duplicated across both connections. To optimize power consumption, UEs may also be configured to activate dual connections only during mobility events or when switching between networks is required.

## 2.3 Collaboration with SA

As Dual Stack involves Core Network functions, as discussed in clause 2.2, it is recommended that RAN initiate a liaison request to SA to study 4G-6G interworking at the Core level.

# 3. Conclusion

- RAN shall initiate a study on Dual Stack as part of the 6G Study to address 4G-6G and 5G-6G interworking without impacting RAN.

- RAN shall initiate a study on Dual Stack as part of the 6G Study to address the migration from existing generation networks (4G or 5G) to 6G without having impact on the previous generation RANs.

- RAN shall initiate a liaison request to SA to study 4G-6G interworking at the Core level.

# Reference

- GSMA, “The Mobile Economy 2025”, March 2025

- RP-251881, “Study on 6G Radio,” 3GPP TSG RAN Meeting #108, Prague, CZ, June 2025