3GPP TSG RAN meeting #109                                                RP-252037

Beijing, China, Sep 15-18, 2025

Title: 	Views on Rel-20 demodulation topics for 5G-A

Source: 	ZTE Corporation,Sanechips

Agenda item:	9.1.4.5

Document for:	Discussion

Introduction

In the RAN#108 meeting, there were lots of discussions on Rel-20 RAN4 package and the discussion outcomes are endorsed in the way forward on RAN4 R20 package [1]. In this contribution, we would like to share some further views on Rel-20 Demod topics for 5G-A.

Discussion

- 2.1. Demod enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ proposals on Rel-20 demod enhancement so far are summarized in the following table.

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - (e)RedCap UE enhancement - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI - This also include example bands, UE RF scope including potential complexity reduction, UE demod - Fast SCell activation - Further discussion on the possible down scoping in RAN#109 based on the following two objectives - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation - No RAN1/2 scope is expected - Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

Proposal 1: for Rel-20 demod requirement, we propose to include the following detailed objectives into Rel-20 demod enhancement;

- Proposal 2: for SCM for MU-MIMO, propose to consider the applicability for both 5G-Adv and 6G.

- 2.1.2 WID objective

- Normative work on SCM based requirements for SU-MIMO in FR1

- Core part for SCM based SU-MIMO:

  - To specify SCM channel model

    - Applicable for both FR1 FDD and TDD bands

    - Applicable scenarios:

      - SU-MIMO PDSCH

        - 4 layers for 4Rx and 8layers for 8Rx

      - SU-MIMO PMI

        - 4 layers for 4Rx

    - Take Rel-19 SCM study outcome in TR 38.753 into account

- Perf part for SCM based SU-MIMO:

    - To define the UE demod requirement based on SCM channel model

- Study on SCM for MU-MIMO

  - 4Rx and 8Rx as baseline;

  - MMSE-IRC receiver as baseline;

- Performance requirements for 6Rx UE with interference

  - Up to 4 MIMO layers for the serving cell is considered

  - Use MMSE-IRC receiver as baseline;

  - With both inter-cell interference and intra-cell inter-user interference considered;

  - Take the legacy approach to derive the interference profile.

- No RAN1/2 impact is expected

- Target 0.5 TU allocation from Q1’26 and 1 TU from Q2’26.

Conclusions

In this contribution, we shared some further views on Rel-20 RRM enhancement based on RAN#108 moderator’s way forward.

References

[1] RP-251879,Way forward on RAN4 R20 package, RAN4 Chair (Apple), Approved.