3GPP TSG RAN Meeting #109                                 RP-252086

Beijing, China, September 15-18, 2025

Source: 	CMCC

Title:	Discussion on IMT-2030 TPRs

Agenda item:	8.2.1.2

Document for:	Discussion & Decision

Background

In response to the Liaison Statement (LS) from ITU-R WP5D (Document 5D/TEMP/167) on minimum requirements related to technical performance (TPR) for IMT-2030 radio interface(s) [1], 3GPP TSG RAN#106 plenary initiated a study on 6G scenarios/use cases and requirements [2] and focused on investigating a candidate set of items for minimum TPRs for IMT-2030 radio interfaces based on the Recommendation ITU-R M.2160 [3]. In RAN#107, an additional part on general 6G RAN study was added to the RAN 6G SI [4].

During the previous RAN plenary meetings, the following set of items was endorsed for IMT-2030 TPRs from RAN’s perspective. Furthermore, the last RAN meeting made agreements on the requirement values of some TPR items and provided further inputs to ITU-R in the reply LS [7].

| Proposed TPR items | Proposed requirement values |
| --- | --- |
| - Peak Data Rate | TBD |
| - Peak Spectral Efficiency | TBD on the values |
| - User Experienced Data Rate | TBD on the values 3GPP RAN agreed to use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. Due to the different evaluation methodology, a new title can be considered to avoid the confusion with previous IMT-2020 values. |
| - 5th percentile user Spectral Efficiency | Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor and other operational considerations may limit the number of receiver antennas of UE |
| - Average Spectral Efficiency | Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor and other operational considerations may limit the number of receiver antennas of UE |
| - Sustainability / Energy Efficiency | TBD |
| - Area Traffic Capacity | TBD on values |
| - User Plane Latency | IC: 4ms HRLLC: 1ms |
| - Control Plane Latency | 20ms |
| - Connection Density | 10⁶ (devices/km2) for massive communication |
| - Reliability | 1-10-5 with same IMT-2020 assumptions |
| 12. Mobility | Indoor Hotspot-IC: 2.25 (bit/s/Hz) @10 km/h Dense Urban- IC: 1.68 (bit/s/Hz) @30 km/h Rural-IC: [0.8~1.2] (bit/s/Hz)@120 km/h [0.45~0.675] (bit/s/Hz) @500 km/h NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. NOTE2: For Rural-IC, TBD on whether to include other velocities, e.g. 30km/h, 60km/h |
| - Mobility Interruption Time | TBD |
| - Bandwidth | 400MHz (same definition as IMT-2020) |
| - Positioning | Quantitative TPR, TBD on the values |
| - Sensing-related capabilities | TBD |
| - AI-related capabilities | Qualitative TPR |
| - Joint/composite requirement | TPR related to data rate, latency, reliability, capacity etc. Applicable usage scenario: immersive communication |

Based on the above progress, this contribution continues to discuss definitions, associated target values and evaluation methods for the proposed TPR items.

TPRs for IMT-2030

In this section, we discuss definitions, associated target values and evaluation methods for the proposed TPR items.

For the following thirteen proposed TPRs inherited from IMT-2020 TPRs shown in Table 1, the definitions captured in the latest ITU working document of 49th WP5D meeting can be reused. It is worthy to notice that a new title “5th percentile user data rate” is considered in ITU to substitute the user experienced data rate. According to the agreement made in the last RAN plenary, the new title should also be adopted in 3GPP to avoid the confusion with previous IMT-2020 values. Therefore, the new title “5th percentile user data rate” instead of user experienced data rate will be used for IMT-2030 TPR in the following text of this contribution.

Proposal 1: For the proposed IMT-2030 TPRs inherited from IMT-2020 (except for energy efficiency), the definitions captured in the latest ITU working document of 49th WP5D meeting can be reused, as shown in the following Table 1.

Table 1: Definitions of TPRs inherited from IMT-2020 TPRs except for energy efficiency

|  | TPR Items | Definition |
| --- | --- | --- |
| 1 | Peak data rate | Peak data rate is the theoretical maximum [achievable] data rate under ideal conditions, which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times). |
| 2 | Peak spectral efficiency | Peak Spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times). |
| 3 | 5th percentile user data rate | The 5th percentile user data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time. |
| 4 | 5th percentile user spectral efficiency | The 5th percentile user Spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz. The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio. |
| 5 | Average spectral efficiency | Average Spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio. |
| 6 | Area traffic capacity | Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time. |
| 7 | User plane latency | User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active [or battery efficient (e.g. inactive)] state. |
| 8 | Control plane latency | Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). |
| 9 | Connection density | Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2). |
| 10 | Reliability | Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability. Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. |
| 11 | Mobility | Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h). |
| 12 | Mobility interruption time | Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions. |
| 13 | Bandwidth | Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation. |

Then, regarding target values associated with the above candidate IMT-2030 TPRs, we have the following aspects of consideration:

- In contrast to those of IMT-2020 TPRs, the values of IMT-2030 TPRs should reflect the advanced capabilities of a new generation of mobile communication technology.

- For the peak data rate, the following two alternatives were provided during the CC#1 [8]:

  - Alternative 1: Not to define a requirement, e.g. leave the value for peak data rate for the technology proponent to provide

  - Alternative 2: Define a requirement with reasonable value

Based on the principle reflected in the first bullet, Alternative 2 is preferred and 3-5 x IMT-2020 can be considered as a requirement with reasonable value.

- For the spectral efficiency of IMT-2030, this contribution provides simulation results of massive MIMO systems (64 TXRUs with 128/256/512 antenna elements and 128 TXRUs with 512/1024 antenna elements) based on 7GHz carrier frequency. The detailed evaluation assumptions are given in Table A-1 and the detailed simulation results for spectral efficiency are shown in Table A-2 to Table A-5 under the scenarios of 10 users per TRP and 15 users per TRP, respectively (See Appendix). To be more intuitive, the multiple relationship between the simulated values of spectral efficiency and the corresponding target values in IMT-2020 is depicted in Figure 1 (DL, 64 TXRUs), Figure 2 (DL, 128 TXRUs) and Figure 3 (UL, 128 TXRUs). It was agreed in the last RAN plenary that the target values of 5th percentile user spectral efficiency and average spectral efficiency is 3 x IMT-2020 for downlink. According to the simulation results, 3 x IMT-2020 is also recommended for the target values of 5th percentile user spectral efficiency and average spectral efficiency for uplink assuming 7GHz with UE 4Tx.

- For the 5th percentile user data rate, according to the NR self-evaluation in 3GPP TR 37.910 [10], it can be observed that to satisfy the requirement on DL user experienced data rate defined for IMT-2020 [9], i.e., 100Mbit/s, the assumed DL system bandwidth is at least 160MHz. So, when determining the values of 5th percentile user data rate for IMT-2030, the operators’ spectrum bandwidth should also be taken into account. A similar principle also applies to area traffic capacity. Based on these observations, 3 x IMT-2020 is recommended for the target values of DL 5th percentile user data rate and area traffic capacity. For the evaluation of UL 5th percentile user data rate, it was agreed to use system level simulation with no bandwidth scaling taking UE maximum output power into account.

Proposal 2: For the peak data rate, support to define a requirement with reasonable value, e.g., 3-5 x IMT-2020.

Figure 1: The multiple relationship between the simulated values of spectral efficiency and the corresponding target values in IMT-2020 (DL, 64 TXRUs)

Figure 2: The multiple relationship between the simulated values of spectral efficiency and the corresponding target values in IMT-2020 (DL, 128 TXRUs)

Figure 3: The multiple relationship between the simulated values of spectral efficiency and the corresponding target values in IMT-2020 (UL, 128 TXRUs)

Energy efficiency

In IMT-2020 [9], Energy efficiency is defined as an inspection TPR while current ITU-R WP5D is discussing whether it can be a quantitative TPR in IMT-2030. Simultaneously, compare with 5G, 6G:

- Introduces higher frequency range (e.g. 6.425-7.125 GHz), which results in higher requirement on accuracy and power consumption for RF component.

- Introduces larger bandwidth (e.g. up to 200/400 MHz), which results in higher requirement (e.g. FFT size) and power consumption for BB component.

- Introduces larger TRX number (e.g. up to 128/256 TRX), which results more complexity (e.g. beamforming operation in analog/digital domain) and higher power consumption for BB/IF/RF component.

Based on the above analysis, 6G has a higher demand for energy and energy efficiency should be given special attention. Therefore, it is recommended that sustainability/energy efficiency should be considered as a quantitative TPR (i.e. the evaluation method is simulation).

Proposal 3: Support to define energy efficiency as a quantitative TPR (i.e. the evaluation method is simulation).

Furthermore, ITU-WP5D is discussing the evaluation method for energy efficiency and is considering to define the relative power saving ratio under different load cases. From our point of view, such mechanism is a good starting point and can be further considered with the following aspects:

- The first point is how to calculate the power consumption for the network and UE. Thanks to the hard work during Rel-18/19 NES and Rel-15/16/17/18/19 UE power saving, the comprehensive power models for BS and UE are provided in TR 38.864 [13], TR 38.840 [14], TR 38.869 [15] and TR 38.865 [16], respectively. Such mechanism can be considered as baseline, or other power model should be clarified/reported by the companies, if any.

- The second point is how to ensure the experience while enhancing the energy efficiency. Regarding such issue, we prefer to introduce the constraint on certain metric while evaluate the network EE or Device EE. To be specific:

  - The relative target of certain metric (e.g. threshold value for the metric, or certain performance loss compared with the benchmark) can be set. Constant like certain percent (e.g. 90%) of the UE fulfils the target should be achieved while evaluating the relative power saving for each load case.

  - In addition, since different service may sensitive to different type of metric, it is recommended that the certain metric can be selected according to the service type, for instance:

    - For Vo6G (i.e. VoNR-like) service, the certain metric is latency

    - For burst-buffer service, the certain metric is QoS or UPT

    - For XR service, the certain metric is latency

- The third point is the definition for reference case of relative power saving under UE EE. From our understanding, when NW is fully loaded, a certain UE may not be fully loaded due to there is more than one UE under each BS. Moreover, the load of UE under NW fully loaded case is strongly relevant to the number of UEs under a certain BS, and simply reuse the definition of reference cases under NW EE will cause ambiguity. Therefore, it is better to clarify that the reference case of UE EE is when UE is fully-loaded/has full-buffer service, and it is also necessary to report the number of UE per BS and the traffic model.

Proposal 4: For energy efficiency, support the following definition to be captured in TR 38.914 and the LS reply:

5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. The requirement is defined as the relative energy savings (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. The minimum requirements for Energy efficiency are summarized in Tables below. Table 5.1.15-1: The minimum requirements for Network Energy efficiency Test environment(s)**; Selected load case(s)*; The reference case; Relative energy saving (%). N. A; empty load; Fully loaded case*; [X0%]. Dense urban-IC**; low load, light load and/or medium load *; [X1, X2, X3%]. Rural-IC**; low load, light load and/or medium load *; [Y1, Y2, Y3%]. Indoor Hotspot-IC**; low load, light load and/or medium load *; [Z1, Z2, Z3%]. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Table 5.1.15-2: The minimum requirements for Device Energy efficiency Test environment(s)**; Selected load case(s)*; The reference case; Relative energy saving (%). N. A; empty load; Device is fully-loaded/has full-buffer service; [A0%]. Dense urban-IC**; low load, light load and/or medium load *; [A1, A2, A3%]. Rural-IC**; low load, light load and/or medium load *; [B1, B2, B3%]. Indoor Hotspot-IC**; low load, light load and/or medium load *; [C1, C2, C3%]. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

Composite requirement

To evaluate the performance of immersive communication services, e.g., XR, a new TPR considering joint requirements of latency, reliability, data rate, and capacity was discussed in ITU-R. The previous RAN plenary meeting agreed to define joint/composite requirement related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR for immersive communication [6].

In the most recent ITU-R 49th meeting of Working Party 5D, the following definition of composite requirement is provided in the working document [12]:

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per [km2 or TRxP]. This requirement is defined as the number of users per [km2 or TRxP] of which at least [90%/95%] are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface. This requirement is defined for the purpose of evaluation in the Immersive Communication [and Hyper-Reliable Low Latency Communication] usage scenario. # The minimum performance requirement of composite requirement is summarized in Table X. TABLE X Composite requirement Test environment; Data rate (Mbits/s); Latency (ms); Packet success probability; Number of users ([/km2 or /TRxP]). Dense Urban - Immersive Communication; [DL = 30 Mbps/UL=10 Mbps]; [DL=10 ms / UL=30 ms / In total 40 ms]; [99%]; TBD. [Indoor Factory – HRLLC]. Others if any, are FFS.

By reviewing the 3GPP study item (SI) on XR evaluations for NR in Rel-17 (TR 38.838 [11]), it can be observed that the KPI identified for XR capacity study is system capacity, which is defined as the maximum number of users per cell with at least Y% of UEs being satisfied (Y=90 is baseline). A UE is declared as a satisfied UE if all the considered streams meet their own packet error rate (PER) and packet delay budget (PDB) requirements, i.e., more than X% of packets are successfully transmitted within a given air interface PDB. More specifically, PDB indicates the latency requirement of XR traffic in RAN side (i.e., air interface), and for a given packet, it is counted as failure if its delivery delay exceeds the given PDB. The packet success rate requirement for DL single stream is 99% as a baseline.

Considering that XR is a typical service for Immersive Communication usage scenario, the existing XR capacity evaluation can be reused for IMT-2030. Actually, the system capacity identified for Rel-17 XR SI also reveals a joint requirement on latency (PDB), data rate, capacity (number of users), and packet success rate. It should be noted that the packet success rate here is different from the reliability defined in IMT-2020, where the definition of reliability is the success probability of transmitting a packet within a required maximum delivery time, but the packet success rate is the percentage of packets which are successfully transmitted within a required PDB. That is to say, the KPI identified for XR capacity in Rel-17 is not concerned with the success probability of transmitting one packet. For a given packet, no matter whether re-transmission is needed, it is counted as a successfully transmitted packet as long as its delivery time satisfies the PDB requirement.

Considering that the definition captured in the latest ITU working document can well reflect the above observations, we think it can be reused in TR 38.914.

Proposal 5: For the composite requirement, suggest the following definition to be captured in TR 38.914 based on that captured in the latest ITU working document of 49th WP5D meeting:

5.1.16 Composite requirement The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP. This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface. This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1. Table 5.1.16-1: Composite requirement Test environment; Data rate (Mbits/s); Latency (ms); Packet success probability; Number of users ( /TRxP). Dense Urban - Immersive Communication; DL = 30 Mbps/UL=10 Mbps; DL=10 ms / UL=30 ms / In total 40 ms; 99%; 10. Others if any, are FFS.

Sensing-related capabilities

Integrated Sensing and Communication (ISAC) is defined as one of the six usage scenarios of IMT-2030 and will be the enabler for future networks to provide novel sensing services to customers. Representative use cases include unmanned aerial vehicle (UAV) detection, smart transportation, intelligent manufacturing, etc.

In the most recent ITU-R 49th meeting of Working Party 5D, the following definitions of sensing metrics are provided in the working document [12]:

4.13 Sensing-related capabilities Sensing-related capabilities are measured in terms of the following technical performance requirements: - Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. - Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. - Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. - [Sensing Resolution: TBD]

According to the context, detection probability and false alarm probability, horizontal/vertical localization accuracy and velocity accuracy are selected as the metrics for TPR. However, the definition of detection probability and false alarm probability can only be applied to binary sensing results with a positive state and a negative state.

For most of the use cases, there are usually more than one target. Since the number and the position of the targets are unknown, sensing algorithms cannot output a binary sensing result for each target. The sensing algorithms can only report a set of detected targets with optional attributes including position and velocity.

To evaluate the detection probability false alarm probability, association between the set of true targets and the set of detected targets is required to decide whether a true target is “correctly detected” or not. After association, those true targets, which are not associated with any detected target, are defined as missed targets. And those detected targets, which are not associated with any true target, are defined as false alarmed targets.

In the case with multiple targets, association is essentially synonymous with “correctly detecting”. It emphasizes that there are multiple possible criterions for ‘correctly detecting’. And association can also be used to define false alarm. Thus, we recommend adding a sentence to the existing definition, which writes “For cases with multiple objects, association between the detected sensing objects and true sensing objects is required.”.

Note that the diversity of association schemes or the diversity of definitions of “correctly detecting” does not make the definition useless. One option is to further define a rule for association, which is hard to reach an agreement according to R20 ISAC discussions. Another option is to retain the freedom of the rules for association. A rule, which leads to a better apparent performance of detection and false alarm, leads to a worse performance of localization accuracy. Since localization accuracy is also defined as a metric and all requirements shall be met, this diversity has little impact on sensing TPRs.

Regarding the accuracy, we think the word “accuracy” refers to a statistic. And the difference between the estimated value and the actual value is just the error.

Proposal 6: For the sensing TPRs, the definition captured in the latest ITU working document of 49th WP5D meeting shall be modified as follows.

| Metric | Definition | Proposal on IMT-2030 KPIs |
| --- | --- | --- |
| Detection Probability and False alarm probability | Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. For cases with multiple objects, association between the detected sensing objects and true sensing objects is required. | Detection Probability: ≥90% False Alarm Probability: ≤10% |
| Horizontal/Vertical Localization Accuracy | Horizontal/vertical localization accuracy is defined as the 90th percentile point of the cumulative distribution function (CDF) of the horizontal/vertical localization estimation error. Localization estimation error is the norm of estimated horizontal/vertical position and the corresponding true position of a sensing target. | Horizontal Localization Accuracy: 3-5m Vertical Localization Accuracy:3-5m |
| Velocity Accuracy | Velocity accuracy is defined as the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. Velocity estimation error is the norm of the estimated velocity and the corresponding true velocity of a sensing target. | Velocity Localization Accuracy: 3-5m/s |

AI-related capabilities

Considering the difficulty and complexity of defining AI-related capabilities as a quantitative TPR, it was agreed to define AI-related capabilities as qualitative TPR evaluated by inspection in RAN#107. In this meeting, the definition for AI-related capabilities should be further discussed.

- Manageable AI Data Collection

In 5G/5G-A, the data collection procedure is triggered by OAM, and cannot be initiated by gNB autonomously which is not flexible especially for the RAN-centric use cases.

For collected data transfer, there are some limitations using the legacy CP/UP solution as follows:

- The existing SRB cannot transmit the large volumes of data for model training (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes).

- The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the large volumes of data collected by the UE cannot be transferred to the gNB directly via DRB.

On the other hand, the data for model training is collected per use case, which can result in duplicated data collection and reporting, especially for basic measurement results. For example, L1 beam measurement results can be used for AI/ML based beam management and AI/ML for mobility use cases.

AI data collection should be one of the most important AI-related capabilities supported by the RIT. To ensure user privacy and AI data reliability, the collected data should be fully manageable by MNO which prevents the leakage and unauthorized use of sensitive information from user and network.

- Efficient AI Computing processing

Efficient computing processing is a crucial AI-related capability to support low latency and high complexity scenarios in 6G. For model inference and real-time online training, AI models need to process and produce accurate results quickly, as well as update and optimize model parameters rapidly, especially for time-sensitive use cases for better user experience and network efficiency.

To support the extensive data processing, complicated AI/ML algorithms operation and real-time decision-making procedures for various use cases, such as CSI compression, beam management, mobility management and network energy saving, powerful and efficient computing are required in RAN side.

- Low total cost of ownership (TCO)

TCO is one of the important considerations for operators in network deployment for AI. Centralized deployment of computing power can reduce costs compared to distributed deployment in each RAN node. To achieve low TCO, the AI-related capabilities should have following characteristics:

- Decoupling Computing Module with Legacy Communication Module

By separating AI computing module from traditional communication module, network operators gain unprecedented flexibility and scalability in deployment. Computing nodes can be deployed wherever processing demand is highest—whether at the network edge, in regional data centers, without impacting the underlying radio access infrastructure. This decoupling also allows for independent upgrades: communication module can evolve on its own cadence while computing module can be updated frequently to keep pace with the latest machine-learning innovations to enable a flexible deployment for AI computing module.

- Openness of Hardware Infrastructure

Openness of Hardware Infrastructure provides the flexibility and compatibility of AI models from different AI solution providers. This separation empowers operators to match AI model with hardware infrastructure to realize low vendor lock-in. It also simplifies maintenance: a software patch or algorithm update can be rolled out across diverse hardware nodes without an entire upgrade, reducing both operational costs and time to market for new AI-driven network features.

- Flexible Scheduling of Computing Resources

From the perspective of operator, not all gNBs would be equipped with powerful AI capabilities due to deployment cost consideration, which is especially insufficient for resource-intensive Large Language Models (LLMs). In scenarios where multiple gNBs collaborate for model training, such as federated learning, efficient sharing of computing resource becomes critical. The RAN in the future needs to be equipped with advanced AI computing management and AI resource coordination capabilities, which provides the ability to dynamically allocate and orchestrate AI computing resources across distributed nodes for driving down costs in response to varying network loads and AI service requirements. AI computing resources can be balanced across multiple nodes to ensure overall network intelligence, paving the way for a smarter and more efficient 6G infrastructure.

Considering RIT/SRIT should support mechanisms and/or signaling related to AI functionalities over the radio interface (between UE and network), including AI data collection and other capabilities, the following definition for AI-related capabilities can be adopted.

Proposal 7: It is proposed to adopt the following definition for AI-related capabilities:

AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including. • RIT/SRIT facilitating AI applications i.e. “Network for AI” • AI to improve RIT/SRIT performance, i.e. “AI for network” The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities. This should include support for one or more of the following: – Manageable AI data collection – Distributed processing – Distributed learning – AI model training – Efficient AI computing processing – AI model execution – AI model inference – Low total cost of ownership (TCO) - Decoupling Computing Module with Legacy Communication Module - Openness of Hardware Infrastructure - Flexible Scheduling of Computing Resources – or other AI capabilities of the RITs/SRITs (to be reported by the proponent) Option A RIT/SRIT should have the capability to enable and disable AI functionalities within the network (AI for network scenario only) Option B The proponent should report about mechanisms, e.g, related to enabling or disabling the supported AI functionalities, where applicable.

Proposed values for IMT-2030 TPR

Based on the considerations in Section 2, this section summarizes our views on target values for IMT-2030 TPRs.

Proposal 8: For the proposed IMT-2030 TPRs, adopt the associated target values in the following Table 2.

Table 2: Views on target values for IMT-2030 TPRs

| TPRs |  | IMT-2020 KPIs (Report ITU-R M.2410) | NR self-evaluation (3GPP TR 37.910 [10]) | IMT-2030 Framework (Rec. ITU-R M.2160) | Proposal on IMT-2030 KPIs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate (Gbps) | DL: 20Gbit/s UL: 10Gbit/s | 171.2Gbit/s @ FR2, DL, TDD(DSUUD), 748MHz BW | e.g. 50, 100, 200Gbit/s | DL: 60-100Gbit/s UL: 30-50Gbit/s |  | 3-5 x IMT-2020 |
| 2 | Peak spectral efficiency (bps/Hz) | DL: 30bit/s/Hz UL: 15bit/s/Hz | 46.8bit/s/Hz @FR1, DL, TDD(DSUUD), 100MHz BW | SE: e.g. 1.5x, 3x IMT-2020 | DL: 60bit/s/Hz UL: 30bit/s/Hz | 2 x IMT-2020 Agreement in CC#1: [1.5-2.5x] of IMT-2020 |  |
| 3 | 5th percentile user data rate (Mbps) | User experienced data rate: DL: 100Mbit/s UL: 50Mbit/s | 104.66 Mbps @ Dense Urban, ISD 200m, 4GHz, DL, 160MHz BW, 32x4 MU-MIMO Type II Codebook; gNB Config = (8,16,2,1,1;1,16) | e.g. 300, 500Mbit/s | DL: 300Mbit/s UL: 50 Mbit/s |  | DL: 3 x IMT-2020 UL: 1 x IMT-2020 |
| 4 | 5th percentile user spectral efficiency (bit/s/Hz) | Dense Urban - eMBB: DL: 0.225bit/s/Hz UL: 0.15bit/s/Hz | 0.68bit/s/Hz @ Dense Urban, 4 GHz, TDD, DL 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | SE: e.g. 1.5x, 3x IMT-2020 | Dense Urban - IC: UL: 0.45bit/s/Hz | UL: 3 x IMT-2020, assuming 7GHz with UE 4Tx Agreement in RAN#108: Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx |  |
| 5 | Average spectral efficiency (bit/s/Hz/TRxP) | Dense Urban - eMBB: DL: 7.8bit/s/Hz/TRxP UL: 5.4bit/s/Hz/TRxP | 19.91bit/s/Hz/TRxP @ Dense Urban, 4 GHz, TDD, DL 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | SE: e.g. 1.5x, 3x IMT-2020 | Dense Urban - IC: UL: 16.2bit/s/Hz/TRxP | UL: 3 x IMT-2020, assuming 7GHz with UE 4Tx Agreement in RAN#108: Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx |  |
| 6 | Area traffic capacity (Mbit/s/m2) | Indoor Hotspot 10 Mbit/s/m2 | 10.66 Mbit/s/m2 @Indoor Hotspot, ISD 20m, 4 GHz, DL, 120MHz BW, 36TRxP | e.g. 30, 50 Mbit/s/m2 | Indoor Hotspot - IC: 30 Mbit/s/m2 |  | 3 x IMT-2020 |
| 7 | Latency (ms) - user plane | 4ms for eMBB 1ms for URLLC | 0.99ms @DL, TDD(DSUUD), SCS=30kHz, TypeBResource Mapping, UEcapability2 | e.g. 0.1-1ms | Agreement in RAN#108: - IC: 4ms - HRLLC: 1ms |  |  |
| 8 | Latency (ms) - control plane | 20ms | 12.9ms @DL, TDD(DSUUD), SCS=30kHz, TypeBResource Mapping, UEcapability2 | / | Agreement in RAN#108: 20ms |  |  |
| 9 | Connection density (devices/km2) | 106 devices per km2 | 36,323,844 devices/km2 @180kHz Bandwidth, Evaluation configuration A (ISD=500 m) | e.g. 106-108 devices per km2 | Agreement in RAN#108: 10⁶ (devices/km2) for massive communication |  |  |
| 10 | Reliability | 1-10-5 | > 99.9999% | e.g. 1-10-5 -1-10-7 | Agreement in RAN#108: 1-10-5 with same IMT-2020 assumptions |  |  |
| 11 | Mobility | classes of mobility: Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicular: 10 km/h to 120 km/h High speed vehicular: 120 km/h to 500 km/h Indoor Hotspot-eMBB: 1.5bit/s/Hz@10km/h Dense Urban-eMBB: 1.12@30km/h Rural-eMBB: 0.8@120km/h, 0.45@500km/h |  | e.g., 500 - 1000 km/h | Rural-IC: 1.5 x IMT-2020 - 1.2 (bit/s/Hz) @120 km/h - 0.675 (bit/s/Hz) @500 km/h Agreement in RAN#108: Indoor Hotspot-IC: 2.25 (bit/s/Hz) @10 km/h Dense Urban- IC: 1.68 (bit/s/Hz) @30 km/h Rural-IC: [0.8~1.2] (bit/s/Hz) @120 km/h [0.45~0.675] (bit/s/Hz) @500 km/h NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. NOTE2: For Rural-IC, TBD on whether to include other velocities, e.g. 30km/h, 60km/h |  |  |
| 12 | Mobility interruption time | 0ms | Beam mobility: 0ms CA Mobility: 0ms | / | 0ms |  | 1 x IMT-2020 |
| 13 | Bandwidth | 100MHz |  | / | RAN#107 agreement: 400MHz (same definition as IMT-2020) |  |  |
| 14 | Energy Efficiency | / | / |  | TBD |  |  |
| 15 | Positioning | / | / | 1 – 10 cm | Quantitative TPR Horizontal accuracy: 20-100cm@90% for indoor, 3-5m@90% for outdoor FFS the necessity of vertical accuracy |  |  |
| 16 | Sensing-related capabilities | / | / | / | Detection Probability: ≥90% Probability and False Probability: ≤10% |  |  |
|  |  |  |  |  | Horizontal Localization Accuracy: ≤3-5m Vertical Localization Accuracy: ≤3-5m |  |  |
|  |  |  |  |  | Velocity Localization Accuracy: ≤3-5m/s |  |  |
| 17 | AI-related capabilities | / | / | / | Qualitative TPR |  |  |
| 18 | Joint/composite requirement | / | / | / | 10 user/TRxP |  |  |

Conclusions

In this contribution, we discuss the definition, evaluation methods, and the associated target values for the identified IMT-2030 TPRs, and the following proposals are provided.

Proposal 1: For the proposed IMT-2030 TPRs inherited from IMT-2020 (except for energy efficiency), the definitions captured in the latest ITU working document of 49th WP5D meeting can be reused, as shown in Table 1.

Proposal 2: For the peak data rate, support to define a requirement with reasonable value, e.g., 3-5 x IMT-2020.

Proposal 3: Support to define energy efficiency as a quantitative TPR (i.e. the evaluation method is simulation).

Proposal 4: For energy efficiency, support the following definition to be captured in TR 38.914 and the LS reply:

5.1.15 Energy efficiency Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving in relation to the traffic characteristics. The requirement is defined as the relative energy savings (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. For network energy efficiency, the reference case is when network is fully loaded. For UE energy efficiency, the reference case is when UE is fully-loaded/has full-buffer service. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. The minimum requirements for Energy efficiency are summarized in Tables below. Table 5.1.15-1: The minimum requirements for Network Energy efficiency Test environment(s)**; Selected load case(s)*; The reference case; Relative energy saving (%). N. A; empty load; Fully loaded case*; [X0%]. Dense urban-IC**; low load, light load and/or medium load *; [X1, X2, X3%]. Rural-IC**; low load, light load and/or medium load *; [Y1, Y2, Y3%]. Indoor Hotspot-IC**; low load, light load and/or medium load *; [Z1, Z2, Z3%]. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Table 5.1.15-2: The minimum requirements for Device Energy efficiency Test environment(s)**; Selected load case(s)*; The reference case; Relative energy saving (%). N. A; empty load; Device is fully-loaded/has full-buffer service; [A0%]. Dense urban-IC**; low load, light load and/or medium load *; [A1, A2, A3%]. Rural-IC**; low load, light load and/or medium load *; [B1, B2, B3%]. Indoor Hotspot-IC**; low load, light load and/or medium load *; [C1, C2, C3%]. * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation. Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

Proposal 5: For the composite requirement, suggest the following definition to be captured in TR 38.914 based on that captured in the latest ITU working document of 49th WP5D meeting:

5.1.16 Composite requirement The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP. This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface. This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario. The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1. Table 5.1.16-1: Composite requirement Test environment; Data rate (Mbits/s); Latency (ms); Packet success probability; Number of users ( /TRxP). Dense Urban - Immersive Communication; DL = 30 Mbps/UL=10 Mbps; DL=10 ms / UL=30 ms / In total 40 ms; 99%; 10. Others if any, are FFS.

Proposal 6: For the sensing TPRs, the definition captured in the latest ITU working document of 49th WP5D meeting shall be modified as follows.

| Metric | Definition | Proposal on IMT-2030 KPIs |
| --- | --- | --- |
| Detection Probability and False alarm probability | Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. For cases with multiple objects, association between the detected sensing objects and true sensing objects is required. | Detection Probability: ≥90% False Alarm Probability: ≤10% |
| Horizontal/Vertical Localization Accuracy | Horizontal/vertical localization accuracy is defined as the 90th percentile point of the cumulative distribution function (CDF) of the horizontal/vertical localization estimation error. Localization estimation error is the norm of estimated horizontal/vertical position and the corresponding true position of a sensing target. | Horizontal Localization Accuracy: 3-5m Vertical Localization Accuracy:3-5m |
| Velocity Accuracy | Velocity accuracy is defined as the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. Velocity estimation error is the norm of the estimated velocity and the corresponding true velocity of a sensing target. | Velocity Localization Accuracy: 3-5m/s |

Proposal 7: It is proposed to adopt the following definition for AI-related capabilities:

AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including. • RIT/SRIT facilitating AI applications i.e. “Network for AI” • AI to improve RIT/SRIT performance, i.e. “AI for network” The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signalling/ related to AI functionalities. This should include support for one or more of the following: – Manageable AI data collection – Distributed processing – Distributed learning – AI model training – Efficient AI computing processing – AI model execution – AI model inference – Low total cost of ownership (TCO) - Decoupling Computing Module with Legacy Communication Module - Openness of Hardware Infrastructure - Flexible Scheduling of Computing Resources – or other AI capabilities of the RITs/SRITs (to be reported by the proponent) Option A RIT/SRIT should have the capability to enable and disable AI functionalities within the network (AI for network scenario only) Option B The proponent should report about mechanisms, e.g, related to enabling or disabling the supported AI functionalities, where applicable.

Proposal 8: For the proposed IMT-2030 TPRs, adopt the associated target values in the following Table 2.

Table 2: Views on target values for IMT-2030 TPRs

| TPRs |  | IMT-2020 KPIs (Report ITU-R M.2410) | NR self-evaluation (3GPP TR 37.910 [10]) | IMT-2030 Framework (Rec. ITU-R M.2160) | Proposal on IMT-2030 KPIs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Peak data rate (Gbps) | DL: 20Gbit/s UL: 10Gbit/s | 171.2Gbit/s @ FR2, DL, TDD(DSUUD), 748MHz BW | e.g. 50, 100, 200Gbit/s | DL: 60-100Gbit/s UL: 30-50Gbit/s |  | 3-5 x IMT-2020 |
| 2 | Peak spectral efficiency (bps/Hz) | DL: 30bit/s/Hz UL: 15bit/s/Hz | 46.8bit/s/Hz @FR1, DL, TDD(DSUUD), 100MHz BW | SE: e.g. 1.5x, 3x IMT-2020 | DL: 60bit/s/Hz UL: 30bit/s/Hz | 2 x IMT-2020 Agreement in CC#1: [1.5-2.5x] of IMT-2020 |  |
| 3 | 5th percentile user data rate (Mbps) | User experienced data rate: DL: 100Mbit/s UL: 50Mbit/s | 104.66 Mbps @ Dense Urban, ISD 200m, 4GHz, DL, 160MHz BW, 32x4 MU-MIMO Type II Codebook; gNB Config = (8,16,2,1,1;1,16) | e.g. 300, 500Mbit/s | DL: 300Mbit/s UL: 50 Mbit/s |  | DL: 3 x IMT-2020 UL: 1 x IMT-2020 |
| 4 | 5th percentile user spectral efficiency (bit/s/Hz) | Dense Urban - eMBB: DL: 0.225bit/s/Hz UL: 0.15bit/s/Hz | 0.68bit/s/Hz @ Dense Urban, 4 GHz, TDD, DL 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | SE: e.g. 1.5x, 3x IMT-2020 | Dense Urban - IC: UL: 0.45bit/s/Hz | UL: 3 x IMT-2020, assuming 7GHz with UE 4Tx Agreement in RAN#108: Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx |  |
| 5 | Average spectral efficiency (bit/s/Hz/TRxP) | Dense Urban - eMBB: DL: 7.8bit/s/Hz/TRxP UL: 5.4bit/s/Hz/TRxP | 19.91bit/s/Hz/TRxP @ Dense Urban, 4 GHz, TDD, DL 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | SE: e.g. 1.5x, 3x IMT-2020 | Dense Urban - IC: UL: 16.2bit/s/Hz/TRxP | UL: 3 x IMT-2020, assuming 7GHz with UE 4Tx Agreement in RAN#108: Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx |  |
| 6 | Area traffic capacity (Mbit/s/m2) | Indoor Hotspot 10 Mbit/s/m2 | 10.66 Mbit/s/m2 @Indoor Hotspot, ISD 20m, 4 GHz, DL, 120MHz BW, 36TRxP | e.g. 30, 50 Mbit/s/m2 | Indoor Hotspot - IC: 30 Mbit/s/m2 |  | 3 x IMT-2020 |
| 7 | Latency (ms) - user plane | 4ms for eMBB 1ms for URLLC | 0.99ms @DL, TDD(DSUUD), SCS=30kHz, TypeBResource Mapping, UEcapability2 | e.g. 0.1-1ms | Agreement in RAN#108: - IC: 4ms - HRLLC: 1ms |  |  |
| 8 | Latency (ms) - control plane | 20ms | 12.9ms @DL, TDD(DSUUD), SCS=30kHz, TypeBResource Mapping, UEcapability2 | / | Agreement in RAN#108: 20ms |  |  |
| 9 | Connection density (devices/km2) | 106 devices per km2 | 36,323,844 devices/km2 @180kHz Bandwidth, Evaluation configuration A (ISD=500 m) | e.g. 106-108 devices per km2 | Agreement in RAN#108: 10⁶ (devices/km2) for massive communication |  |  |
| 10 | Reliability | 1-10-5 | > 99.9999% | e.g. 1-10-5 -1-10-7 | Agreement in RAN#108: 1-10-5 with same IMT-2020 assumptions |  |  |
| 11 | Mobility | classes of mobility: Stationary: 0 km/h Pedestrian: 0 km/h to 10 km/h Vehicular: 10 km/h to 120 km/h High speed vehicular: 120 km/h to 500 km/h Indoor Hotspot-eMBB: 1.5bit/s/Hz@10km/h Dense Urban-eMBB: 1.12@30km/h Rural-eMBB: 0.8@120km/h, 0.45@500km/h |  | e.g., 500 - 1000 km/h | Rural-IC: 1.5 x IMT-2020 - 1.2 (bit/s/Hz) @120 km/h - 0.675 (bit/s/Hz) @500 km/h Agreement in RAN#108: Indoor Hotspot-IC: 2.25 (bit/s/Hz) @10 km/h Dense Urban- IC: 1.68 (bit/s/Hz) @30 km/h Rural-IC: [0.8~1.2] (bit/s/Hz) @120 km/h [0.45~0.675] (bit/s/Hz) @500 km/h NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. NOTE2: For Rural-IC, TBD on whether to include other velocities, e.g. 30km/h, 60km/h |  |  |
| 12 | Mobility interruption time | 0ms | Beam mobility: 0ms CA Mobility: 0ms | / | 0ms |  | 1 x IMT-2020 |
| 13 | Bandwidth | 100MHz |  | / | RAN#107 agreement: 400MHz (same definition as IMT-2020) |  |  |
| 14 | Energy Efficiency | / | / |  | TBD |  |  |
| 15 | Positioning | / | / | 1 – 10 cm | Quantitative TPR Horizontal accuracy: 20-100cm@90% for indoor, 3-5m@90% for outdoor FFS the necessity of vertical accuracy |  |  |
| 16 | Sensing-related capabilities | / | / | / | Detection Probability: ≥90% Probability and False Probability: ≤10% |  |  |
|  |  |  |  |  | Horizontal Localization Accuracy: ≤3-5m Vertical Localization Accuracy: ≤3-5m |  |  |
|  |  |  |  |  | Velocity Localization Accuracy: ≤3-5m/s |  |  |
| 17 | AI-related capabilities | / | / | / | Qualitative TPR |  |  |
| 18 | Joint/composite requirement | / | / | / | 10 user/TRxP |  |  |

References

- RP-243202, LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#106, Madrid, Spain, December 9-12, 2024.

- RP-243327, New SID: Study on 6G scenarios/use cases and requirements, RAN#106, Madrid, Spain, December 9-12, 2024.

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, RAN#107, Incheon, Korea, March 12-14, 2025.

- RP-243325, Draft Reply to ITUR_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#106, Madrid, Spain, December 9-12, 2024.

- RP-250819, Moderator’s summary for IMT-2030 TPRs, RAN#107, Incheon, Korea, March 12-14, 2025.

- RP-251825, Reply LS to ITU-R WP5D = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#108, Prague, Czech Republic, June 9-13, 2025.

- RT-250024, Moderator’s summary for IMT-2030 TPRs in CC#1, CC#1 for IMT-2030 TPR discussion, April 29, 2025, 14:00-16:00 CEST.

- Report ITU-R M.2410, Minimum requirements related to technical performance for IMT-2020 radio interface(s).

- TR 37.910, Study on self evaluation towards IMT-2020 submission.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

- “Working Document towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]: Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, Annex 5.14 to Document 5D/TEMP/249(Rev.2) (Report on the 49th meeting of Working Party 5D), Kobe, Japan, 24 June-3 July 2025.

- 3GPP TR 38.864, Study on network energy savings for NR.

- 3GPP TR 38.840, Study on User Equipment (UE) power saving in NR.

- 3GPP TR 38.869, Study on low-power wake up signal and receiver for NR.

- 3GPP TR 38.865, Study on further NR RedCap UE complexity reduction.

Appendix

Table A-1: Evaluation assumptions for spectral efficiency

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 7GHz |
| Bandwidth | 20MHz |
| Subcarrier Spacing | 30kHz |
| TDD | DDDDDDDSUU; S(6DL:4GP:4UL) |
| Channel Model | UMa in TR 38.901 |
| Scheduling algorithm | Proportional fair scheduling |
| ISD | 200m |
| BS Tx power | 53dBm |
| UE Tx power | 23dBm |
| BS Antenna Configuration | (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 8, 8, 2, 4, 8) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 8, 16, 2, 4, 8) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 16, 16, 2, 4, 8) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 16, 16, 2, 4, 16) (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 16, 32, 2, 4, 16) (dH, dV) = (0.7, 0.5)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree; electrical downtilt: 9 degree |
| UE Configuration | (Mg, Ng, M, N, P, Mp, Np) = (1, 1, 2, 2, 2, 2, 1); (dV, dH) = (0.5, 0.5)λ. |
| BS antenna pattern | θ3dB=65°, φ3dB=90° with 6 dBi element gain |
| UE antenna pattern | Omni-directional with 0 dBi element gain |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| Noise figure for BS | 4dB |
| UE receiver noise figure | 7dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP; 15 users per TRP |
| Traffic Model | Full buffer |

Table A-2: Evaluation results for spectral efficiency under the scenario of 10 users per TRP, downlink

| Number of TXRUs | [Mg, Ng, M, N, P] | 5th percentile user spectral efficiency (bit/s/Hz) | Average spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- |
| 64 TXRU (Mp, Np) = [4, 8] | [1, 1, 8, 8, 2](128) | 0.28 | 12.71 |
|  | [1, 1, 8, 16, 2](256) | 0.43 | 14.68 |
|  | [1, 1, 16, 16, 2](512) | 0.51 | 16.02 |
| 128 TXRU (Mp, Np) = [4, 16] | [1, 1, 16, 16, 2](512) | 0.54 | 18.22 |
|  | [1, 1, 16, 32, 2](1024) | 0.65 | 19.44 |

Table A-3: Evaluation results for spectral efficiency under the scenario of 15 users per TRP, downlink

| Number of TXRUs | [Mg, Ng, M, N, P] | 5th percentile user spectral efficiency (bit/s/Hz) | Average spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- |
| 64 TXRU (Mp, Np) = [4, 8] | [1, 1, 8, 8, 2](128) | 0.16 | 14.48 |
|  | [1, 1, 8, 16, 2](256) | 0.30 | 17.29 |
|  | [1, 1, 16, 16, 2](512) | 0.31 | 18.88 |
| 128 TXRU (Mp, Np) = [4, 16] | [1, 1, 16, 16, 2](512) | 0.53 | 23.39 |
|  | [1, 1, 16, 32, 2](1024) | 0.51 | 24.47 |

Table A-4: Evaluation results for spectral efficiency under the scenario of 10 users per TRP, uplink

| Number of TXRUs | [Mg, Ng, M, N, P] | 5th percentile user spectral efficiency (bit/s/Hz) | Average spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- |
| 128 TXRU (Mp, Np) = [4, 16] | [1, 1, 16, 16, 2](512) | 0.33 | 11.14 |
|  | [1, 1, 16, 32, 2](1024) | 0.39 | 11.49 |

Table A-5: Evaluation results for spectral efficiency under the scenario of 15 users per TRP, uplink

| Number of TXRUs | [Mg, Ng, M, N, P] | 5th percentile user spectral efficiency (bit/s/Hz) | Average spectral efficiency (bit/s/Hz) |
| --- | --- | --- | --- |
| 128 TXRU (Mp, Np) = [4, 16] | [1, 1, 16, 16, 2](512) | 0.26 | 15.38 |
|  | [1, 1, 16, 32, 2](1024) | 0.27 | 16.10 |

Table B-1: Evaluation method of sensing capacity for detection and localization purpose

|  | Detail |
| --- | --- |
| Step 1 | Perform system-level simulation and initialize evaluation parameters with N targets dropped. |
| Step 2 | Generate sensing transmitted-signal, obtain sensing received-signal, and perform sensing signal processing to get localization estimation error Δr / detection probability for each sensing target. |
| Step 3 | Update N, which is the number of dropped targets, until at least [95%] of the N’ total targets fulfil localization accuracy / detectability requirement. |
| Step 4 | Calculate the sensing capacity as C = N’/A, where A is the TRxP area |

Table B-2: Evaluation method of sensing capacity for tracking purpose

|  | Detail |  |
| --- | --- | --- |
| Step 1 | Perform system-level simulation and initialize evaluation parameters with N targets dropped. |  |
| Step 2 | Generate sensing transmitted-signal, obtain sensing received-signal, and perform sensing signal processing and tracking processing to get localization estimation error Δr at sampling time t for each sensing target. |  |
| Step 3 | Remove those estimations which do not fulfil localization accuracy requirement. Then obtain maximum latency, which is the time interval between two consecutive estimations, of each sensing target. |  |
| Step 4 | Option-A | Update N, which is the number of dropped targets, until at least [95%] of the cases with N' targets dropped satisfy that all N' targets fulfil maximum latency requirement. |
|  | Option-B | Update N, which is the number of dropped targets, until at least [95%] of the N’ total targets fulfil maximum latency requirement. |
| Step 5 | Calculate the sensing capacity as C = N’/A, where A is the TRxP area. |  |