3GPP TSG RAN Meeting #109						RP-252323

Beijing, China, September 15-18, 2025

Agenda Item:		8.2.1.1

Source:		AT&T

Title:				Deployment Scenarios for 6G Evaluations

Document for:		Discussion/Decision

# 1	Introduction

In this document, views are provided on the deployment scenarios considered for 6G Evaluations, included in 3GPP TR38.914, “Study on 6G Scenarios and Requirements” [1].

# 2	Discussion

Deployment scenarios for 6G were addressed during RAN#109, with the conclusions summarized in TR38.914 [1]. The group agreed to include several scenarios for consideration for 6G deployments—such as indoor hotspot, dense urban, rural, urban macro, sub-urban macro, high speed, extreme long distance, and urban coverage for massive connection—while further discussion is required to finalize their detailed attributes. In this contribution, we present our views on some of the remaining attributes.

For the dense urban, urban and suburban macro scenarios, the following frequency aggregation should be confirmed to be captured in the attributes.

Proposal 1: Capture the following in the attributes for carrier frequency for Dense Urban, Urban and Sub-Urban scenarios

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz Around 7GHz + Around 4GHz + Around 2GHz + Around 700MHz |

For the Sub-Urban Macro scenario, the description of the scenario remains to be finalized. We believe the following definition is suitable to capture the Sub-Urban Macro scenario, as it captures its main attributes, especially following the work in RAN1 Rel. 19 on channel modelling [2].

## Proposal 2: Capture the following as the description of the Suburban Macro Scenario in section 4.5

## “The suburban deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous wide area coverage supporting moderate speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.”

## Similarly for the users’ distribution and UE speed for the sub-urban macro scenario, following the agreement from RAN1 Rel. 19 channel modelling [2], the following should be captured in the table of attributes.

## Proposal 3: Capture the following as the user distribution and UE speed for the suburban macro scenario in Table 4.5: Attributes for Sub-urban Macro Scenario

| ## User distribution and UE speed | ## 80% of user indoors, 20% outdoors 90% of indoor users in residential building, 10% in commercial buildings 100% of outdoor users in car |
| --- | --- |

# 4	Conclusion

In this contribution we discussed the deployment scenarios for 6G in 38.914. The following proposals were made to be captured in Section 4 of 38.914.

Proposal 1: Capture the following in the attributes for carrier frequency for Dense Urban, Urban and Sub-Urban scenarios

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz Around 7GHz + Around 4GHz + Around 2GHz + Around 700MHz |

## Proposal 2: Capture the following as the description of the Suburban Macro Scenario in section 4.5

## “The suburban deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous wide area coverage supporting moderate speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.”

## Proposal 3: Capture the following as the user distribution and UE speed for the suburban macro scenario in Table 4.5: Attributes for Sub-urban Macro Scenario

| ## User distribution and UE speed | ## 80% of user indoors, 20% outdoors 90% of indoor users in residential building, 10% in commercial buildings 100% of outdoor users in car |
| --- | --- |

# 5	References

- 3GPP TR 38.914, Study on 6G Scenarios and requirements, Release 20 (draft)

- 3GPP TR38.901 v19.0.0 (2025-06), Study on channel model for frequencies from 0.5 to 100GHz (Release 19)