3GPP TSG RAN Meeting #109	RP-252466

# Beijing, China, September 15-18, 2025

Agenda item:	8.2.2

Source:	Samsung

Title:	Discussion on 6G Spectrum

Document for:	Discussion and Decision

1	Introduction

Spectrum is a fundamental enabler of each cellular generation, and 6G is expected to utilize a wider range of frequencies than ever before. In 5G NR, spectrum usage has been structured into two ranges: Frequency Range 1 (FR1, below 7.125 GHz) and Frequency Range 2 (FR2, 24.25–71 GHz). This categorization was mainly driven by propagation characteristics, device implementation constraints, and regulatory allocation at the time of standardization.

As 6G evolves, there is growing consensus that a new Frequency Range, covering the 7–24 GHz mid-band spectrum, will play a critical role in bridging coverage and capacity between FR1 and FR2. This range offers more favorable propagation compared to FR2 while enabling significantly wider bandwidth than FR1. In addition, discussions have emerged on whether further refinements (e.g., sub-ranges within FR3 or extensions toward sub-THz spectrum) are necessary to address diverse deployment needs.

A key consideration in defining frequency ranges for 6G is the appropriate choice of subcarrier spacing (SCS) and numerology. The propagation delay spread, phase noise, and hardware limitations vary significantly across frequency ranges, making it impractical to adopt a single SCS configuration across all bands. For example, smaller SCS values are desirable in lower bands to improve coverage and robustness, while larger SCS values are necessary in higher bands to manage phase noise and support wideband channels.

In RAN1#122 the following was agreed.

Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary.

Therefore, it is important for 3GPP/RAN to establish clear guidelines to WGs for frequency range classification in 6G, including the mapping of spectrum ranges to suitable SCS options. Such a framework will ensure consistency across deployments, enable efficient device design, and provide flexibility for future extensions toward higher frequency bands.

2	Discussion

Frequency Range Categorization for 6G

In 5G NR, spectrum was divided into two ranges: FR1 (below 7.125 GHz) and FR2 (24.25–71 GHz). With the evolution toward 6G, broader spectrum utilization is expected, and one possibility is to extend FR1 to 8.4 GHz. This ensures that emerging 7–8 GHz bands can be treated within the FR1 framework, benefiting from similar propagation and deployment characteristics as the legacy sub-7 GHz spectrum.

For new spectrum for 6GR, three options can be considered:

- Option A: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology.

- Option B: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family.

- Option C: define separate FR for 7.125 – 24.25 GHz.

In either case, spectrum at or above 7.125 GHz should adopt a single SCS, suitable for wider bandwidths and the phase noise environment of higher frequencies.

Proposal: Among the three options, we propose to extend FR1 up to 8.4 GHz (option A or option B) considering the channel propagation characteristics and the objective to have similar coverage as for 3.5 GHz.

Single-SCS Principle

To simplify implementation and improve spectral efficiency, we propose a single-SCS per range principle:

- FR1-FDD (if FR1 is extended ≤ 8.4 GHz): 15 kHz

- FR1-TDD (if FR1 is extended ≤ 8.4 GHz): 30 kHz

- New FR ≥ 8.4 GHz: 60 (if separate FR) or 120 kHz (if merged in FR2-1)

This approach avoids multi-numerology coexistence within a band, thereby reducing guard overhead and complexity. In particular, FR1-TDD = 30 kHz offers clear benefits in spectrum-sharing scenarios such as MRSS, where alignment across operators and networks is essential.

Maximum Channel Bandwidth (CBW) and FFT Size Requirements

We propose a baseline maximum channel bandwidth (CBW) of 200 MHz across ranges, with a possibility to allow up to 400 MHz in specific bands where contiguous spectrum is available. The corresponding FFT sizes are selected to be powers of two, aligned with sampling rates (up to 491.52 Msps).

- FR1-FDD (15 kHz): Optimized for wide coverage and legacy continuity; suitable for IoT and control traffic with stringent link budget requirements.

- FR1-TDD and FR1-extended (30 kHz): Balances coverage and efficiency, minimizes guard overhead in shared spectrum (MRSS), and enables wideband deployments with practical FFT sizes.

- New FR above 8.4 GHz (60 or 120 kHz): Appropriate for ≥ 8.4 GHz, providing resilience against phase noise and Doppler while keeping FFT sizes moderate for 200–400 MHz CBW.

In summary,

- FR1-FDD with 15 kHz SCS

| Max CBW | SCS | FFT size | Sampling rate |
| --- | --- | --- | --- |
| 100 MHz | 15 kHz | 8K | 122.88 Msps |
| 200 MHz | 15 kHz | 16K | 245.76 Msps |

- FR1- TDD and FR1-extended with 30 kHz SCS

| Max CBW | SCS | FFT size | Sampling rate |
| --- | --- | --- | --- |
| 200 MHz | 30 kHz | 8K | 245.76 Msps |
| 400 MHz | 30 kHz | 16K | 491.52 Msps |

- New FR above 8.4 GHz with 60 kHz SCS (if defined)

| Max CBW | SCS | FFT size | Sampling rate |
| --- | --- | --- | --- |
| 200 MHz | 60 kHz | 4K | 245.76 Msps |
| 400 MHz | 60 kHz | 8K | 491.52 Msps |

- FR2-1 with 120 kHz SCS

| Max CBW | SCS | FFT size | Sampling rate |
| --- | --- | --- | --- |
| 200 MHz | 120 kHz | 2K | 245.76 Msps |
| 400 MHz | 120 kHz | 4K | 491.52 Msps |

Minimum Channel bandwidth

To accommodate diverse deployment scenarios, the minimum channel bandwidth should be defined as 5 MHz (FR1-FDD), 10/20 MHz (FR1-TDD and FR1-extended), and 20 MHz (if separate FR) or 50 MHz (FR2-1), while the maximum channel bandwidth is set to 200 MHz as a baseline with an optional extension to 400 MHz for bands where sufficient contiguous spectrum is available.

3	Proposal

We propose the following for further detailed discussions in WG meetings:

| FR | Range | SCS (kHz) | Required FFT size | Max CBW (MHz) | Min CBW (MHz) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 400 MHz – 6.425 GHz | 15 (FDD) 30 (TDD) | 8192 | 100 (FDD) 200 (TDD) | 5 (FDD) 10 (TDD) |
|  | 6.425 – 7.125 GHz | 30 | 8192, [16384] | 200, [400] | 20 |
| FR1-extended | 7.125 – 8.4 GHz | 30 (if extended) | 8192, [16384] | 200, [400] | 20 |
| [new FR] | 8.4 – 24.25 GHz | 60 (if separate FR) or 120 (if merged in FR2-1) | 8192 | 400, 800 | 20 |
| FR2-1 | 24.25 – 52.6 GHz | 120 | 4096, 8192 | 400, 800 | 50 |