3GPP TSG RAN #109	RP-252518

Beijing, China, 15th – 18th September, 2025

Agenda item:		8.2.2

Source:	MediaTek Inc.

Title:	Scalability for Diverse device types

Document for:			Discussion and Decision

# Introduction

This contribution raises some issues discussed at RAN1#122 regarding diverse device types and we provide our thoughts and recommendations for moving forward even though this topic was not scheduled for discussion at RAN#109.

# Status in 3GPP

The following is included in both RAN plenary and RAN WG 6G Studies [1, 2]:

- Target scalable and forward compatible design for diverse device types.

At RAN1#122 / Aug 2025 there was some discussion about enabling scalable device types from PHY perspective, and the following was agreed:

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

As further background, during the discussion at RAN1#122 on the above, there was discussion about:

-	How to avoid excessive UE capabilities when enabling diverse device types

-	Mandating certain features for specific “device types”

Somewhat separately to the above, but at the same meeting, there was discussion on the lowest expected device bandwidth envelope, and the following agreement was reached:

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Proposal 1: As “scalable and forward compatible design for diverse device types” impacts RAN1/2/4, RAN plenary should first discuss such aspects and then provide guidance to WGs, to avoid duplicated efforts.

Note that the reference to device types in the 6G SIDs does not imply the introduction of device types as such, but only relates to the diversity of the device market in 6G that the 6G design need to be scalable for.

The above aspects are addressed in the following.

# 3	Scalability for diverse device types: rationale and implications

There are a few major drivers behind the “scalability to support diverse device types” objective from [1, 2]:

-	To enable ANY 6G device to be capable of “basic” access to a 6G radio network across the lifecycle of the 6G system, without requiring network upgrades first. Lesson learnt from RedCap and earlier NB-IoT/cat-M.

-	To enable a vibrant device ecosystem, minimizing product development overheads for network and device vendors to support diverse devices (e.g. low-end IoT, wearable, handheld, CPE) without sacrificing performance, using scaled functional designs.

-	To avoid “siloed” functionality e.g. for lower end devices, which are most sensitive to product development cost.

The above points lead to the need for two fundamental aspects to be addressed in the 6GR study:

-	Identifying what the “common minimum functional capability” i.e. 6G Nucleus of ALL 6G devices should be, and the impacts thereof on fundamental radio interface design for basic connectivity provisioning.

-	Understanding the “likely” kinds of device characteristics that need to be catered for, to ensure the fundamental radio interface design can be designed with the required flexibility and scalability from the start.

Observation 1: Two fundamental aspects to address in the 6GR study are:

-	Identifying what the “common minimum functional capability” i.e. 6G Nucleus of ALL 6G devices should be, and the impacts thereof on fundamental radio interface design for basic connectivity provisioning.

-	Understanding the “likely” kinds of device characteristics that need to be catered for, to ensure the fundamental radio interface can be designed with the required flexibility and scalability from the start.

# 4	6G Nucleus: common minimum functionality for all devices

The definition of the 6G Nucleus, common minimum functionality baseline for ALL devices, should take the following into account:

-	FR1, FR2, FR3: a “common minimum functionality” for each of FR1, FR2 and FR3 should be identified that enables operation in each of FR1, FR2 and FR3 respectively. However, since the lowest-end capabilities would typically be related to massive IoT and requiring relatively ubiquitous coverage, it is FR1 that is expected to put most constraints on the PHY design.

-	Enabling “basic” connectivity means the minimum functionality for initial access, control and data transmission and reception must be the same for all devices. Furthermore, given that wide area coverage, Network & UE power saving, MRSS designs should be “natively” supported in 6G, the functions for those needed for basic connectivity must be available in all devices.

-	To facilitate that, the “common minimum functionality” would need to be natively supported (and hence mandatorily available) in both devices and networks along with crucial IODT availability for that functionality.

Proposal 2: Common minimum functionality should ensure all 6G devices (initial and future) are able to obtain basic connectivity from any 6G network, from a functional compatibility perspective. This should include support for 6G native features for wide area coverage, energy saving and MRSS.

Our view on what the common minimum functionality should consist of is provided in Table 1 below for FR1. For FR2 and FR3 further discussion would be needed.

Table 1: Suggested 6G Nucleus for FR1

| Parameters | Proposals |
| --- | --- |
| Device bandwidth envelope | 20MHz |
| Spectrum allocations supported | Can operate in 3MHz upwards |
| Duplex mode | Half Duplex operation |
| #Tx/#Rx antenna; #MIMO Layer | 1, 1 |
| Modulation order | 64QAM (and lower modulation orders) |
| UE processing time | [4] ms |
| BS/UE power saving | Yes - exact features depend on studies |
| Wide area coverage | Yes – up to [10] dB improved MCL vs NR. Exact features depend on studies |
| Maximum output power? | [23] dBm |
| MRSS | Yes |

Further rationale for some of the parameters in Table 1 is provided below:

-	Both economies of scale and component cost should be considered for the “lowest-end” device.

-	Better economies of scale will be generated for a device with broader capabilities as it will also cater for a broader set of service needs than a device that only caters for a subset of those needs – which is beneficial for per-unit device cost and cause less device ecosystem fragmentation.

-	“Overall” build-cost is also of course relevant, but:

-	We do not expect that reducing the Device bandwidth envelope below 20MHz will have any noticeable impact on the overall device cost by the time such devices are required.

-	Whatever the device bandwidth envelope is, the device will need to be able to operate in spectrum allocations as low as 3MHz in FR1 bands to cover global deployment needs.

-	We expect instead that relaxing the required UE processing time to e.g. ~4ms will provide noticeable benefits to the overall device cost.

-	We anticipate that legacy RAT support would also be needed for such a device to be globally viable.

Observation 2: There are different factors impacting overall device “cost” beyond 6G “component cost”, such as economies of scale, and these will need to be carefully considered in the 6GR study.

Another question is whether it is essential to define all of the parameter values now. We would urge NOT to rush a decision on this fundamental point. Furthermore, for the basic PHY structure it would only seem necessary to make an early decision where the parameter value does constrain the design.

For “device bandwidth envelope” it seems that the “minimum spectrum allocation” will be the most constraining factor. Therefore, taking more time to align on aspects such as “device bandwidth envelope” will not impact progress.

Observation 3: No need to rush decisions on common minimum capability parameters/values, especially those parameters that do not themselves drive the PHY structural design.

Proposal 3: Carefully consider the common minimum capability values and do NOT rush agreements on this. A common understanding should however be reached by RAN#112 / June 2026.

# 5	Catering for the range of device characteristics

Distinguishing characteristics of devices

Beyond the 6G Nucleus in section 4, additional capabilities will of course be supported depending on the different characteristics and constraints of a device. We provide below the key parameters, mainly considering: Complexity, Form-factor, Operational Constraints (such as battery lifetime, etc).

-	Number of Tx and Rx antennas

-	MIMO layers, Device bandwidth envelope, Modulation order

-	Duplex operation

-	Maximum output power

-	UE processing latency

-	Spectrum aggregation capabilities

Proposal 4: Consider the following key parameters to determine device characteristics: #Tx/#Rx antennas, #MIMO layers, device bandwidth envelope, modulation order, max output power, device processing latency, spectrum aggregation capabilities.

Initial thoughts on some of the types of device characteristics

We consider in the Table 2 below the need for at least the following kinds of device characteristics to be considered as representative examples to progress the radio interface design. This should NOT mean however that the radio interface design is optimised purely to cover ONLY these types of device i.e. other types of devices with characteristics corresponding to a combination of capabilities “in between” each should be possible.

Table 2: Examples of possible typical capabilities of different kinds of devices

| Example Device | Level of constraints | Example Capabilities Envelope |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Duplex | MIMO layers / Antennas | Tx power (dBm) | Modulation (QAM) | CBW envelope (MHz) | Processing Latency (ms) | Spectrum Agg. |
| High (e.g. CPE) | Low | FD-FDD, TDD | 4T/8R | 29 | 1024 DL 256 UL | 200 | [1] | Yes (BC dependent) |
| Handheld Compute | Medium | FD-FDD, TDD | 2T/4R mid-band | 26 | 256 DL/UL | 200 | [1] | Yes (BC dependent) |
| High-end Wearable | High: FF, Operational | FD-FDD, TDD | 1T/1R | 23 | 256 DL 64 UL | 200 | [1] | Yes (BC dependent) |
| Nucleus (e.g. IoT) | Highest; FF, Cost, Operational | Half Duplex (may depend on band) | 1T/1R (as Table 1) | 23 (as Table 1) | 64 DL/UL (as Table 1) | 20 (as Table 1) | [4] (as Table 1) | No |

Proposal 5: Consider the following example device characteristics in table 2 for the 6GR study. Ensure the radio interface design does not preclude other characteristics for devices with capabilities higher than the Nucleus capabilities.

# 6	Whether to define “Device types” in 6G specs

Our key driver for defining device types would be the following:

Ensuring a basic set of capabilities that we expect a certain kind of device to support, assuming network support and IODT availability, for:

- System performance for a certain service/use case for a certain device.

- Ease of alignment across the ecosystem of applicable functionality for a certain device.

However, it is important for TSG-RAN to ensure good quality decisions on this topic and take the necessary time.

Observation 4: It is important for TSG-RAN to ensure quality decision-making on “whether to define device types in 3GPP specs” and take the necessary time.

# 7	Conclusion

Proposal 1: As “scalable and forward compatible design for diverse device types” impacts RAN1/2/4, RAN plenary should first discuss such aspects and then provide guidance to WGs, to avoid duplicated efforts.

Observation 1: Two fundamental aspects to address in the 6GR study are:

-	Identifying what the “common minimum functional capability” i.e. 6G Nucleus of ALL 6G devices should be, and the impacts thereof on fundamental radio interface design for basic connectivity provisioning.

-	Understanding the “likely” kinds of device characteristics that need to be catered for, to ensure the fundamental radio interface can be designed with the required flexibility and scalability from the start.

Proposal 2: Common minimum functionality should ensure all 6G devices (initial and future) are able to obtain basic connectivity from any 6G network, from a functional compatibility perspective. This should include support for 6G native features for wide area coverage, energy saving and MRSS.

Observation 2: There are different factors impacting overall device “cost” beyond 6G “component cost”, such as economies of scale, and these will need to be carefully considered in the 6GR study.

Observation 3: No need to rush decisions on common minimum capability parameters/values, especially those parameters that do not themselves drive the PHY structural design.

Proposal 3: Carefully consider the common minimum capability values and do NOT rush agreements on this. A common understanding should however be reached by RAN#112 / June 2026.

Proposal 4: Consider the following key parameters to determine device characteristics: #Tx/#Rx antennas, #MIMO layers, device bandwidth envelope, modulation order, max output power, device processing latency, spectrum aggregation capabilities.

Proposal 5: Consider the following example device characteristics in table 2 for the 6GR study. Ensure the radio interface design does not preclude other characteristics for devices with capabilities higher than the Nucleus capabilities.

Observation 4: It is important for TSG-RAN to ensure quality decision-making on “whether to define device types in 3GPP specs” and take the necessary time.

# References

[1]	RP-250810, “Revised SID: Study on 6G Scenarios and requirements”; CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

[2]	RP-251881, 3GPP TSG RAN Study Item Description “Study on 6G Radio”; NTT DOCOMO (Moderator)