3GPP TSG RAN Meeting #109								   RP-252885

Beijing, China, September 15-18, 2025

Agenda Item:	          10.6.4

Source:			Astrum Mobile

Title:			Summary for WI New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite

WI code(s):			LTE_band_5G_bcast_GSO

leading WG:		RAN4

Release:			Rel-19

1	Introduction

A new SDO (Standalone Downlink Only) band was specified for LTE-based 5G Broadcast operation in 1467 to 1492 MHz in Region 3 for GSO (Geosynchronous Orbit) communication, based on E-UTRA operation over NTN (Non-Terrestrial Network).

2	Description

A new SDO (Standalone Downlink Only) band 246 was specified for LTE-based 5G Broadcast operation in 1467 to 1492 MHz in Region 3 for GSO (Geosynchronous Orbit) communication.

UE RF requirements were defined for 10 MHz PMCH (Physical Multicast Channel) in TS 36.102 (E-UTRA UE radio transmission and reception for satellite access). At the network side, related SAN (Satellite Access Node) RF requirements for 10 MHz PMCH were defined in TS 36.108 (E-UTRA Satellite Access Node radio transmission and reception), considering applicable regulatory requirements based on WRC-19 Resolution 761.

3	References

- RP-250788, “New WID: New band for 5G Broadcast Utilizing Geosynchronous Satellite”, Astrum Mobile, Qualcomm, RAN#107, March 2025.

- R4-2504725, “Way Forward for [114bis][318] 5G_Broadcast_GSO_NTN_band”, Astrum Mobile, Qualcomm, ZTE, Huawei, RAN4#114bis, April 2025.

- R4-2505517, “[LTE_band_5G_bcast_GSO] Big CR to TS 36.102: New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite, Rel-19”, Astrum Mobile, RAN4#115, May 2025

- R4-2508648, “Way Forward for [115][318] 5G_Broadcast_GSO_NTN_band”, Astrum Mobile, Qualcomm, ZTE, Huawei, RAN4#115, May 2025.

- RP-251419(Revision of RP-250788), “Revised WID: New LTE band for 5G broadcast for region 3 utilizing geosynchronous satellite”, Astrum Mobile, Qualcomm, RAN#108, Jun 2025.

- R4-2512553, Big CR on TS 36.102: New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite.

- R4-2512555, Big CR on TS 36.108: New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite.

- R4-2512556, CR to TS 36.307: release independence aspects for the LTE-based 5G broadcast operation over geosynchronous satellite.