3GPP TSG RAN Meeting #110 								                   RP-253044

Baltimore, Maryland, USA, December 8-11, 2025

Agenda Item:	8.2.1.1

Source:	OPPO

Title:	Discussion on 6G Key performance indicators

Document for:	Discussion

- Introduction

The study item on 6G scenarios and requirements was approved in RAN#107 (SID in [1]). In RAN#109, most of KPIs for 6G communication were discussed and agreed [2], with corresponding LS reply sent to ITU-R [3]. But some aspects are still left TBD for further study. In this paper, our views on the remaining KPI values are presented.

- Discussions on remaining KPIs

2.1	Positioning

Positioning is a vertical service supported by 5G NR specifications, but without large-scale deployments. The first release of 6G standards should focus on the evolvement of the service with large-scale market demands, e.g., eMBB and IoT. Rel-21 6G standards can support positioning services with 5G NR technical solutions. The further enhancements can be studied in later release by taking market into account the feedback of 5G positioning commercial deployments.

For indoor factory, for example, industrial IoT use cases, the positioning target can be horizontal accuracy < 0.2 for 90% UEs while vertical accuracy < 1m for 90% UEs. For general commercial use cases, the positioning target can be horizontal accuracy < 1m and vertical accuracy < 3m for 90% UEs. Hence, the following KPI values are proposed:

Table 1: Positioning accuracy

| Test Environment | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- |
| Indoor Factory | 0.2m | 1m |
| Urban Macro | 1m | 3m |

Proposal 1: Take the KPI values in Table 1 for 6G requirement on positioning accuracy.

2.2	Energy efficiency

The evaluation methodology for energy efficiency in ITU-R IMT-2030.EVAL [4] can be used as a reference. With the preliminary study on the service requirements and potential energy/power saving schemes 6GR may adopt, the minimum requirements for network and device energy efficiency can be proposed as in Table 2 and 3, respectively.

From our perspective, the relative energy efficiency consumption is used for metric in [4], rather than energy saving gain. So, the “saving” in the last column can be deleted. And, the relative energy consumption is mainly related to the system load level, rather than the test environment. Dense urban can be assumed for the most typical deployment scenario for BS energy saving. “Rural” scenario is not needed for defining this KPI. In Table 2, TDD DL:UL=9:1 is assumed.

In Table 2, the relative energy consumption in 15% and 30% loaded case would be higher than in 5% loaded case. The values can be further studied, which is also related to the DL/UL ratio of the frame structure.

In Table 3, the “empty load” implies the UE power saving performance can stay in Idle mode, when UE resource utilization is zero. “30% loaded” implies the UE resource utilization ratio is 30%, when the UE needs to stay in Connected state and keep monitoring PDCCH.

Table 2: The minimum requirements for Network Energy efficiency

| Test Environment | Selected load case | The reference case | Relative energy consumption (%) |
| --- | --- | --- | --- |
| N.A | Empty load | Fully loaded case* | [16%] |
| Dense urban-IC | [5%] |  | [26%] |
| Dense urban-IC | [15%] |  | [TBD] |
| Dense urban-IC | [30%] |  | [TBD] |

Table 3: The minimum requirements for Device Energy efficiency

| Test Environment | Selected load case | The reference case | Relative energy consumption (%) |
| --- | --- | --- | --- |
| N.A | Empty load | Fully loaded case* | [25%] |
| Dense urban-IC | [30%] |  | [90%] |

Proposal 2: Take the KPI values in Table 2 and 3 for 6G requirement on Network and Device energy efficiency.

2.3	Coverage

In 5G NR requirement TR 38.913, the target for coverage is represented by metrics of MCL. Specifically, the target under data rate of 160bps is 164dB, and the maximum coupling loss is 140dB/143dB in case of MBB service characterized by a downlink data rate of 2Mbps/1Mbps and an uplink data rate of 60kbps/30kbps. The coupling loss is defined as the total long-term channel loss over the link between the UE antenna ports and the BS antenna ports, and includes in practice antenna gains, path loss, shadowing, body loss, etc.

During the R17 SI on 5G NR Coverage Enhancement (CE), MPL (Maximum Path Loss) and MIL (maximum isotropic loss) were introduced. Companies evaluated the coverage performance of different channels and identified potential bottleneck channels as follows.

Table 3: Potential bottleneck channels identified during Rel-17 Coverage Enhancement SI for FR1

| Scenario |  | Channels (and Frame format) | Representative MCL value | Representative MIL value | Representative MPL value |
| --- | --- | --- | --- | --- | --- |
| Urban 4GHz TDD |  | PUSCH for eMBB DDDSU | 130.76 | 139.01 | 109.84 |
|  |  | PUSCH for eMBB DDDSUDDSUU | 132.37 | 140.33 | 110.87 |
|  |  | Broadcast PDCCH (PDCCH of Msg.2) (24dBm/MHz DL Tx power) | 141.19 | 149.54 | 115.80 |
| Urban 2.6GHz TDD |  | PUSCH for eMBB DDDDDDDSUU | 131.05 | 139.66 | 109.10 |
| Rural 4GHz TDD NLOS O2I |  | PUSCH for eMBB DDDSU | 137.58 | 142.25 | 124.53 |
|  |  | PUSCH for eMBB DDDSUDDSUU | 138.13 | 144.22 | 126.18 |
|  |  | PRACH Format B4 | 145.11 | 146.48 | 125.85 |
|  |  | PUCCH Format 3 22bit | 142.29 | 147.71 | 126.09 |
|  |  | PUSCH for VoIP DDDSUDDSUU | 139.46 | 147.87 | 129.78 |
|  |  | PUSCH for VoIP DDDSU | 143.79 | 148.04 | 129.69 |
|  |  | PUSCH of Msg.3 | 143.86 | 148.11 | 129.68 |
|  |  | PUCCH Format 3 11bit | 144.91 | 150.63 | 129.04 |
|  |  | PUCCH Format 1 | 146.57 | 152.06 | 130.34 |
| Rural 2.6GHz TDD NLOS O2I |  | PUSCH for eMBB DDDDDDDSUU | 137.71 | 142.46 | 123.97 |
|  |  | PUCCH Format 3 11bit | 146.40 | 149.30 | 127.33 |
| Rural 2GHz FDD NLOS O2I |  | PRACH Format B4 | 132.26 | 142.35 | 122.45 |
| Rural 700MHz FDD NLOS O2I | Target MCL=139.2d B | PUSCH for VoIP UUUUU | 134.19 | 146.94 | 129.26 |
|  |  | PUSCH of Msg.3 | 134.71 | 147.50 | 129.88 |
|  |  | PUCCH Format 3 11bit | 135.85 | 148.87 | 127.90 |
|  |  | PUCCH Format 1 | 137.52 | 150.54 | 129.47 |
|  | Target ISD=4000m | PUCCH Format 3 22bit | 130.99 | 144.54 | 123.37 |
|  |  | PUSCH for eMBB UUUUU | 132.45 | 144.76 | 127.43 |
|  |  | PUSCH for VoIP UUUUU | 134.19 | 146.94 | 129.26 |
|  |  | PUSCH of Msg.3 | 134.71 | 147.50 | 129.88 |
|  |  | PUCCH Format 3 11bit | 135.85 | 148.87 | 127.90 |
|  |  | PUCCH Format 1 | 137.52 | 150.54 | 129.47 |
| Rural 4GHz with long distance TDD LOS O2O |  | PUSCH for eMBB DDDSU | 132.14 | 140.28 | 129.28 |
|  |  | PUSCH for VoIP DDDSU | 140.57 | 149.06 | 137.59 |

Table 4: Potential bottleneck channels identified during Rel-17 Coverage Enhancement SI for FR2

| Scenario | Channels (and Frame format) | Representative MCL value | Representative MIL value | Representative MPL value |
| --- | --- | --- | --- | --- |
| Urban 28GHz TDD NLOS O2I | PUSCH eMBB DDDSU | 94.05 | 125.31 | 91.38 |
|  | PUSCH eMBB DDSU | 90.38 | 123.94 | 92.76 |
|  | PUCCH Format 3 11bits | 109.88 | 142.27 | 105.62 |
|  | PUCCH Format 3 22bits | 106.45 | 139.18 | 102.78 |
|  | PRACH Format B4 | 116.28 | 141.22 | 102.80 |
|  | PUSCH of Msg3 | 110.81 | 139.72 | 103.53 |
| Urban 28GHz TDD NLOS O2O | PUSCH eMBB DDDSU | 92.49 | 123.60 | 110.94 |
|  | PUSCH eMBB DDSU | 90.63 | 123.99 | 110.62 |
|  | PRACH Format B4 | 111.12 | 136.13 | 121.36 |

Different channels and scenarios have different MCL/MIL/MPL values ​​for baseline performance under given assumptions.

Observation 1: Some scenarios and channels do not have corresponding target for coverage performance.

Based on the evaluation of coverage performance in Rel-17, some solutions for coverage enhancement had been introduced in Rel-17 and later versions as following.

- PUSCH

- [Rel-17] For PUSCH repetition type A

  - Increasing the maximum number of repetitions up to 32.

  - Support the number of repetitions counted on the basis of available UL slots.

- [Rel-17] Support TB processing over multi-slot PUSCH.

- [Rel-17] Support DMRS bunding to enable joint channel estimation.

- [Rel-20] Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI

- PUCCH

- [Rel-17] Support dynamic PUCCH repetition factor indication.

- [Rel-17] Support DMRS bunding to enable joint channel estimation.

- Msg3

- [Rel-17] Support Type A PUSCH repetitions for Msg3

- PRACH

- [Rel-18] Support multiple PRACH transmissions with same beams for 4-step RACH procedure.

- [Rel-20] Specify enhancements to support multiple PRACH transmissions with different Tx beams for 4-step RACH procedure.

- Others

- [Rel-18] Support dynamic switching between DFT-S-OFDM and CP-OFDM.

- [Rel-20] Specify enhancements to improve PUSCH coverage for higher uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables

Observation 2: Not all solutions for improving coverage performance can be used to every channel.

For the determination of the coverage target, baseline performance in NR and the schemes introduced to enhance the coverage performance can be considered.

Table 5: NR coverage enhancement performance

| Scenario & Channels (and Frame format) | Configuration | MCL | MIL | MPL |
| --- | --- | --- | --- | --- |
| Urban 4GHz TDD PUSCH for eMBB DDDSU | Baseline | 130.76 | 139.01 | 109.84 |
|  | Configured with schemes specified in Rel-17 and Rel-18, (e.g. PUSCH transmission with 32 repetitions, counting based on available slots, and DMRS bundling) | ~146dB | ~155dB | ~126dB |

For IoT service, if it could support larger number of repetitions than eMBB, e.g., 4 times than eMBB, it’s possible to achieve 6dB improvement over eMBB.

Observation 3: If considering the schemes for improving the coverage performance specified in NR, eMBB under some conditions (e.g., transmission with 32 repetitions, counting based on available slots, and DMRS bundling) could achieve MCL=~146dB / MIL=~ 155dB, MPL=~126dB.

Observation 4: If considering larger number of repetitions and the schemes for improving the coverage performance specified in NR, IoT service under some conditions (e.g. transmission with 128 repetitions, counting based on available slots, and DMRS bundling) could achieve MCL=~152dB / MIL=~ 161dB, MPL=~132dB.

With the development of materials science, antenna technology and semiconductor technology, the hardware capabilities of 6G terminals and network equipment will be further improved. These technological enhancements will collectively improve the link budget, allowing 6G networks to achieve coverage capabilities beyond those of 5G. Furthermore, 6G system design should consider differentiated coverage targets based on the requirements of different service, use cases, and device types. For example, for enhanced mobile broadband (eMBB) services, the primary focus is ensuring continuous coverage with high throughput and low latency. In contrast, for massive Internet of Things (IoT) applications, devices generally have lower data rate requirements and can achieve greater coverage than eMBB.

Since in 5G NR requirement TR 38.913 the target for coverage is represented by metrics of MCL. The same metric can be used for comparing 6GR coverage with 5G NR baseline. Based on the above analysis, MCL=146dB can be assumed for coverage target for 6G eMBB and MCL=153dB can be assumed for coverage target for 6G IoT. The target DL and UL data rates can be further studied.

Proposal 3: The targets for coverage for 6GR are:

- MCL=146dB for 6G eMBB for DL data rate of X1 Mbps and UL data rate of Y1 Mbps

- MCL=153dB for 6G IoT for DL data rate of X2 Mbps and UL data rate of Y2 Mbps.

- FFS the exactly value of X1, X2, Y1, Y2.

3	Conclusion

In this contribution, we discussed the remaining KPI values for 6GR. The following proposals are suggested. And the TP for TR38.914 is proposed in Section 5.

Proposal 1: Take the KPI values in Table 1 for 6G requirement on positioning accuracy.

Proposal 2: Take the KPI values in Table 2 and 3 for 6G requirement on Network and Device energy efficiency.

Observation 1: Some scenarios and channels do not have corresponding target for coverage performance.

Observation 2: Not all solutions for improving coverage performance can be used to every channel.

Observation 3: If considering the schemes for improving the coverage performance specified in NR, eMBB under some conditions (e.g., transmission with 32 repetitions, counting based on available slots, and DMRS bundling) could achieve MCL=~146dB / MIL=~ 155dB, MPL=~126dB.

Observation 4: If considering larger number of repetitions and the schemes for improving the coverage performance specified in NR, IoT service under some conditions (e.g. transmission with 128 repetitions, counting based on available slots, and DMRS bundling) could achieve MCL=~152dB / MIL=~ 161dB, MPL=~132dB.

Proposal 3: The targets for coverage for 6GR are:

- MCL=146dB for 6G eMBB for DL data rate of X1 Mbps and UL data rate of Y1 Mbps

- MCL=153dB for 6G IoT for DL data rate of X2 Mbps and UL data rate of Y2 Mbps.

- FFS the exactly value of X1, X2, Y1, Y2.

4	References

- RP-251881	New SID: Study on 6G Radio, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- RP-252882	TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, CMCC, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- RP-252946	Draft Reply LS to ITU-R_WP5D_TEMP on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

- ITU-R Document 5D/989-E, Annex 5.2 to Working Party 5D Chair’s Report, WORKING DOCUMENT TOWARDS PRELIMINARY DRAFT NEW REPORT ITU-R M.[IMT-2030.EVAL] Guidelines for evaluation of radio interface technologies for IMT-2030.

5	Text Proposal to TR38.914 v0.1.0

TP Start

5	Requirements and key technical principles

5.1	Key performance indicators

### 5.1.14	Positioning

Positioning is one important application and it imposes more and more stringent requirements on accuracy. Accurate positioning can usually be achieved through a combination of various technologies, including, e.g., GNSS, Inertial Measurement Unit, and radio technologies.

The positioning accuracy is the [90%] point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s].

Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)]. For indoor factory, for example, industrial IoT use cases, the positioning target can be horizontal accuracy < 0.2 for 90% UEs while vertical accuracy < 1m for 90% UEs. For general commercial use cases, the positioning target can be horizontal accuracy < 1m and vertical accuracy < 3m for 90% UEs.

The minimum requirements for the device positioning accuracy for various [test environments] are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| [Test Environment] | Horizontal Accuracy | Vertical Accuracy |
| --- | --- | --- |
| Indoor Factory | 0.2m | 1m |
| Urban Macro | 1m | 3m |

### 5.1.15	Energy efficiency

Editor note: The text captured in RAN#109 is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.Energy efficiency is an important metric of sustainability. This requirement of the candidate RIT/SRIT is characterized by evaluating network and device energy efficiency. Network energy efficiency is the capability of a RIT/SRIT to support radio access network energy saving [in relation to the traffic capacity provided]. Device energy efficiency is the capability of the RIT/SRIT to support device modem power saving [in relation to the traffic characteristics].

The requirement is defined as the relative energy [savings/consumption] (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

Proponents should report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies.

The minimum requirements for Energy efficiency are summarized in Tables below. In Table 2, TDD DL:UL=9:1 is assumed. In Table 3, the “empty load” implies the UE power saving performance can stay in Idle mode, when UE resource utilization is zero. “30% loaded” implies the UE resource utilization ratio is 30%, when the UE needs to stay in Connected state and keep monitoring PDCCH.

Table 5.1.15-1: The minimum requirements for Network Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy consumption (%) |
| --- | --- | --- | --- |
| N. A | empty load | Fully loaded case* | [16%] |
| [Dense urban-IC]** | [5%] |  | [26%] |
| [Dense urban-IC]** | [15%] |  | [TBD] |
| [Dense urban-IC]** | [30%] |  | [TBD] |
| * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation For Table 3, the typical eMBB UE is assumed. “Empty load” implies the UE power saving performance can stay in Idle mode, when UE resource utilization is nearly zero. “30% loaded” implies the UE resource utilization ratio is 30%, when the UE needs to stay in Connected mode and to keep monitoring PDCCH. |  |  |  |

Table 5.1.15-2: The minimum requirements for Device Energy efficiency

| [Test environment(s)]** | Selected load case(s)* | The reference case | Relative energy consumption (%) |
| --- | --- | --- | --- |
| N. A | empty load | Fully loaded case | [25%] |
| [Dense urban-IC]** | [30%] |  | [90%] |
| * empty load: L=0, low load: 0 < L≤15, light load: 15 < L≤30, medium load: 30 < L≤50, fully loaded case: L=100 ** needed only in case of simulation |  |  |  |

Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

### 5.1.x	Coverage

The coupling loss is defined as the total long-term channel loss over the link between the UE antenna ports and the eNode B antenna ports, and includes in practice antenna gains, path loss, shadowing, body loss, etc. The MaxCL is the limit value of the coupling loss at which the service can be delivered, and therefore defines the coverage of the service. The MaxCL is independent of the carrier frequency. It is defined in the UL and DL as:

-	UL MaxCL = UL Max Tx power - eNB Sensitivity

-	DL MaxCL = DL Max Tx power - UE Sensitivity

The MaxCL is evaluated via link budget analysis (supported by link level simulations). The proposed MaxCL calculation template is given in following table:

Table 5.1.x-1: MaxCL calculation template

| Item | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) |  |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) |  |
| (3) Receiver noise figure (dB) |  |
| (4) Interference margin (dB) |  |
| (5) Occupied channel bandwidth (Hz) |  |
| (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm) |  |
| (7) Required SINR (dB) |  |
| (8) Receiver sensitivity = (6) + (7) (dBm) |  |
| (9) MaxCL = (1) - (8) (dB) |  |

For a basic eMBB service characterized by a downlink datarate of [TBD]bps and an uplink datarate of [TBD]bps for stationary users, the target on maximum coupling loss is 146dB.

For a low-end IoT service characterized by a downlink datarate of [TBD]bps and an uplink datarate of [TBD]bps for stationary users, the target on maximum coupling loss is 153dB.

Link budget and/or link level analysis are used as the evaluation methodology.

End of TP