3GPP TSG RAN #110	 	                                                                    RP-253171

Baltimore, USA, Dec 8-11, 2025

Source:	Intel Corporation

Title:	Requirements on diverse device types

Agenda item:	8.2.1.3.2

Document for:	Discussion and Decision

# Introduction

In RAN #109, the following proposals were discussed regarding diverse device types [1]. In this contribution, we exams the purpose of the device type discussion and a potential way forward.

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

# Discussion

The diverse mobile communication services naturally require device types of devices, ranging from the low-end IoT devices to mid-range wearable devices to high-end MBB and FWA devices. The question in discussion is whether we should specifically define those types of devices.

LTE defined device categories to differentiate device and radio technologies capabilities. NR instead used capability signaling for more granular and flexible differentiation of device capabilities. The LTE approach has simpler reporting mechanism but lacks flexibility due to the static definition of capabilities. The NR approach has great flexibility but is cumbersome on capability reporting and testing.

In 6G, from our understanding, there are two main purposes for the device type discussion:

- Dimension the range of device capabilities and performance requirements for a unified 6GR design. For this motivation, defining the lowest type and the highest type devices is sufficient to draw the system dimension and provide guidance for the unified 6GR design. In particular, the lowest-type device is more important as its capabilities will regulate the unified initial access channel design.

- Design a device capability signaling framework with balanced complexity and flexibility. For this motivation, the focus is on the signaling framework design rather than the device type definition. Recognizing the need for device types is sufficient for starting the discussion on signaling framework. The signaling framework shall be extendable such that device types can be added when necessary.

Furthermore, in our view, the first motivation shall be treated with urgency since many ongoing L1 design discussions depend on inputs on system dimensioning, while the second motivation can be for a later discussion. Since both motivations do not require decision on the full list of device types of 6GR, the device type discussion can be held at a later stage when RAN has a better clarity of the 6GR design and when the detailed device capability signaling design needs to start.

From the above discussions, we can draw the following observations and proposals:

Observation 1: Main purposes for the device type discussion are:

- Dimension the range of device capabilities and define performance requirements for a unified 6GR design

- Design a device capability signaling framework with balanced complexity and flexibility

Proposal 1: Prioritize the decision on the range of device capabilities and performance requirements for unified 6GR design.

Proposal 2: Acknowledge the need to have more than 2 device types, while defer detailed device type definitions until capability signalling design.

# Conclusion

In this contribution, we have the following observations and proposals regarding device types:

Observation 1: Main purposes for the device type discussion are:

- Dimension the range of device capabilities and performance requirements for a unified 6GR design

- Design a device capability signaling framework with balanced complexity and flexibility

Proposal 1: Prioritize the decision on the range of device capabilities for unified 6GR design.

Proposal 2: Acknowledge the need to have more than 2 device types, while defer detailed device type definitions until capability signalling design.

# References

- RP-252873 Moderator's summary on 6G diverse device types (incl. minimum UE bandwidth)