3GPP TSG-RAN Meeting #110			RP-253183

Baltimore, United States, December 8-11, 2025

Agenda item:	8.2.1.2

Title:	Views on 6G key performance indicators

Source:	NTT DOCOMO, INC.

Document for:	Discussion

# 1	Introduction

In RAN#109, the target values of minimum technical performance requirements (TPRs) for IMT-2030 radio interface(s) were discussed and some of them were agreed for reply LS to ITU-R WP 5D [1]. From RAN#110, RAN will focus on 3GPP internal key performance indicators (KPIs) for 6G. Based on the agreements in RAN#109 and the latest ITU-R WP 5D meeting, this contribution provides our views on KPIs for 3GPP internal evaluation of 6G.

# 2	Discussions

## 2.1	Target values for legacy KPIs from 5G NR

In RAN#109, most of target values for IMT-2030 minimum TPRs inherited from IMT-2020 were agreed, while the target values of peak data rate and mobility for Rural-IC have not been decided yet. RAN should resolve the remaining issues on peak data rate and mobility for Rural-IC, and discuss target values of legacy KPIs for 3GPP internal evaluation of 6G based on the agreements for the TPRs of IMT-2030 as a starting point. Table 1 summarizes the agreements in RAN#109 (highlighted in green) and our proposals and views (highlighted in cyan) on target values for legacy KPIs from 5G NR for 3GPP internal evaluation of 6G.

Table 1: Target values for KPIs inherited from 5G NR

| KPI | Agreed target value in RAN#109 for IMT-2030 | Proposed target value for 3GPP internal evaluation | Comments |
| --- | --- | --- | --- |
| Peak data rate |  | 1.5x of IMT-2020 DL: 30 Gbps UL: 15 Gbps | DCM view: It is preferable to define a requirement with reasonable target value. 1.5x is achieved, assuming 1024QAM / 256QAM, 8 layers for DL / UL and bandwidth of 500 MHz. |
| Peak spectral efficiency | 2x of IMT-2020 | DL: 60 bps/Hz UL: 30 bps/Hz |  |
| 5th percentile user data rate | DL: 3x of IMT-2020 UL: 50 Mbps 3GPP RAN agreed to use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. | DL: 300 Mbps UL: 50Mbps |  |
| 5th percentile user spectral efficiency | Indoor Hotspot-IC/Dense Urban-IC/Rural-IC: DL: 3x of IMT-2020 UL: 3x of IMT-2020 Assuming 7GHz with: - BS: up to 1024 elements - UE: - 4Rx or 6Rx or 8Rx - 20 MHz UE transmission bandwidth in simulation Form factor (e.g. number of UE antennas) and other operational considerations may limit the performance. | Indoor Hotspot-IC: DL: 0.9 bps/Hz UL: 0.63 bps/Hz Dense Urban-IC: DL: 0.675 bps/Hz UL: 0.45 bps/Hz Rural-IC: DL: 0.36 bps/Hz UL: 0.135 bps/Hz |  |
| Average spectral efficiency | Indoor Hotspot-IC/Dense Urban-IC/Rural-IC: DL: 3x of IMT-2020 UL: 3x of IMT-2020 Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor (e.g. number of UE antennas) and other operational considerations may limit the performance. | Indoor Hotspot-IC: DL: 27 bps/Hz UL: 20.25 bps/Hz Dense Urban-IC: DL: 23.4 bps/Hz UL: 16.2 bps/Hz Rural-IC: DL: 9.9 bps/Hz UL: 4.8 bps/Hz |  |
| Area traffic capacity | 4x of IMT-2020 for Indoor Hotspot – IC | 40 Mbps/m2 for Indoor Hotspot-IC |  |
| Latency | User plane: IC: 4 ms HRLLC: 1 ms Control plane: 20 ms | User plane: IC: 4 ms HRLLC: 1 ms Control plane: 20 ms |  |
| Connection density | 106 devices/km2 for massive communication | 106 devices/km2 for massive communication |  |
| Reliability | 1-10-5 with same IMT-2020 assumption | 1-10-5 with same IMT-2020 assumption |  |
| Mobility | Indoor Hotspot-IC: 2.25 bps/Hz @ 10 km/h Dense Urban-IC: 1.68 bps/Hz @ 30 km/h Rural-IC: 1.1-1.5x of IMT-2020 [0.88-1.2] bps/Hz @ 120 km/h [0.495-0.675] bps/Hz @ 500 km/h NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. | Indoor Hotspot-IC: 2.25 bps/Hz @ 10 km/h Dense Urban-IC: 1.68 bps/Hz @ 30 km/h Rural-IC: 1.5x of IMT-2020 1.2 bps/Hz @ 120 km/h 0.675 bps/Hz @ 500 km/h | DCM view: In RAN#109 agreement, target value of normalized traffic channel link data rate for Rural-IC is within the range of 1.1-1.5x of IMT-2020. According to self-evaluation results [2], we believe that 1.5x of IMT-2020 can be achieved. |
| Mobility interruption time | Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable. | Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable. |  |
| Bandwidth | 400 MHz (same definition as IMT-2020) | 400 MHz (same definition as IMT-2020) |  |

Proposal 1: To resolve the remaining issues on peak data rate and mobility for Rural-IC, and to discuss the target values of legacy KPIs for 3GPP internal evaluation of 6G based on the agreements for IMT-2030 TPRs in RAN#109 as a starting point, as described in Table 1.

## 2.2	New KPIs of 6G

### 2.2.1	Energy efficiency for sustainability

Energy efficiency is the important KPI which indicates the sustainability for 6G. RAN should define the energy efficiency as a quantitative KPI for both network and UE for the internal evaluation in 3GPP. Regarding energy efficiency for sustainability, following is agreed in RAN#109 [1].

Agreement in RAN#109 - Network and UE Energy efficiency in unloaded case compared to fully loaded case is evaluated analytically using power models. - Network and UE Energy efficiency in partial loaded case(s) are evaluated via simulation using power models - FFS on the load cases for evaluation, e.g. low load (0%-15%) , light load (15%-30%) and/or medium load (30%-50%) - 3GPP recommends to not define a targe value for energy efficiency, means that proponent has to report achievable energy efficiency.

Based on the above RAN agreement, ITU-R WP 5D discussed and agreed the definition and evaluation metrics for energy efficiency as follows [3]:

Definition for energy efficiency agreed in ITU-R WP 5D #50 ## 4.16 Energy efficiency for sustainability Energy efficiency is an important metric of sustainability. Energy efficiency of the candidate RIT/SRIT can be characterized by evaluating network and device energy efficiency. Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case. The RIT/SRIT shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio. Proponents should report the evaluated energy efficiency values and the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving technologies. Proponents are encouraged to describe mechanisms of the RIT/SRIT that improve the support of low energy consumption operation for both network and device. This requirement is defined for the purpose of evaluation in the Immersive communication usage scenario. Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

For 3GPP internal evaluation of energy efficiency, RAN should use the above definition and metrics as baseline to evaluate the relative energy consumption in both unloaded and partial loaded cases. As discussed in RAN#109, the partial loaded case can be considered as low load (0% - 15%), light load (15% - 30%) and/or medium load (30% - 50%). Conditions for evaluation including power model should also be discussed.

Proposal 2: Regarding energy efficiency for sustainability, the definition and metrics agreed in ITU-R WP 5D should be used as baseline for 3GPP internal evaluation

- Loaded case can be considered as low load (0% - 15%), light load (15% - 30%) and/or medium load (30% - 50%), but other case is not precluded

- Evaluation condition including power model should be discussed

### 2.2.2	Positioning Accuracy

As positioning-related KPI, positioning accuracy can be defined, and RAN can use the following definition agreed in ITU-R WP 5D [3]:

Definition of positioning accuracy agreed in ITU-R WP 5D #50 4.12 Positioning Accuracy Positioning is the ability to estimate the position of connected devices. The positioning accuracy is the 90% point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s]. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)]. This requirement is defined for the purpose of evaluation in the Indoor Factory and Urban Macro ISAC test environments and is applicable to HRLLC and IC usage scenarios.

Regarding positioning accuracy, it is preferable to study the target value considering reasonable level of enhancement on positioning techniques. Target values of the positioning accuracy can be considered based on the evaluation results of horizontal accuracy in [4]. The target values for positioning accuracy are proposed as shown in Table 2. It should be discussed whether the evaluation of vertical accuracy is necessary.

Table 2: Proposed target values for positioning accuracy

| Test environment | Horizontal accuracy | Vertical accuracy |
| --- | --- | --- |
| Indoor | < 2 m for 90% of UEs/devices | [meter-level] |
| Outdoor | < 6 m for 90% of UEs/devices | [meter-level] |

Proposal 3: To study the target values of positioning accuracy with reasonable level of enhancement as described in Table 2 and discuss whether the evaluation of vertical accuracy is necessary or not.

### 2.2.3	Sensing-related requirements

For sensing-related requirements, in the latest ITU-R WP 5D meeting, the definitions of performance metrics/requirements were agreed as follows [3]:

Definition of sensing-related requirements agreed in WP 5D #50 4.14 Sensing-related requirements Sensing-related capabilities are measured in terms of the following technical performance requirements: – Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target when it is present, and false alarm probability is the [associated] probability that a sensing target is detected when no sensing target is present. – Horizontal/Vertical Localization Accuracy: It is defined as the absolute difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. Both the horizontal and vertical localization accuracy are in Cartesian coordinates. – Velocity Accuracy: Velocity accuracy is defined as the absolute difference between the estimated velocity and the actual velocity of the sensing target. The value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error. In the above technical performance requirements, the sensing target is a passive object, which is not connected to the radio network for communication. This requirement is defined for the purpose of evaluation in the Integrated Sensing and Communication usage scenario.

In RAN1#122bis, the following metrics same as the definition in ITU-R WP 5D and tentative target values were agreed for evaluation purpose of NR ISAC [5].

Agreement for NR ISAC in RAN1#122bis The following performance objectives are adopted for evaluation purpose of NR ISAC. Metric; Value. Missed detection Probability; [5]%. False Alarm Probability Type 1; [5]%. False Alarm Probability Type 2; [5]%. Horizontal Positioning Accuracy; [10] m with confidence level 90%. Vertical Positioning Accuracy; [10] m with confidence level 90%. Velocity Accuracy; [5] m/s with confidence level 90%.

For consideration of sensing-related requirements for 6G, target values can be discussed based on the above objectives for NR ISAC as a starting point.

Proposal 4: For the evaluation of 6G sensing in 3GPP, target values should be discussed based on the performance objectives agreed for evaluation purpose of NR ISAC in RAN1#122bis as a starting point.

### 2.2.4	AI-related requirement

For internal evaluation of 6G in 3GPP, RAN should define AI-related requirement as a qualitative KPI. In the latest ITU-R WP 5D meeting, the following definition of AI-related requirement were agreed [3].

Definition of AI-related requirement agreed in ITU-R WP 5D #50 4.15 AI-related requirement AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications, including. – RIT/SRIT facilitating AI applications i.e. “Network for AI”. – AI to improve RIT/SRIT performance, i.e. “AI for network”. The RIT should support AI-related capabilities, which refer to the ability for the RIT/SRIT to support mechanisms and/or signaling/ related to AI functionalities. This should include support for one or more of the following: – Data collection – Distributed processing – Distributed learning – AI model training – AI computing – AI model inference – or other AI capabilities of the RITs/SRITs (to be reported by the proponent). If applicable, proponents are encouraged to report about mechanisms related to enabling or disabling the supported AI functionalities.

For 3GPP internal evaluation, the above definition should be used as baseline, and it should be discussed whether to introduce other qualitative KPIs.

Proposal 5: For AI-related requirement, the definition agreed in ITU-R WP 5D should be used as baseline and it should be discussed whether to introduce other qualitative KPIs for 3GPP internal evaluation.

### 2.2.5	Composite requirement

Regarding the composite requirement, which refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users, ITU-R WP 5D agreed the following definition [3]:

Definition of composite requirement agreed in ITU-R WP 5D #50 4.10 Composite requirement The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP. This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface. This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

In RAN#109, the following target values for composite requirement were agreed [1] and should be used as baseline for 3GPP internal KPIs.

Agreement in RAN#109 - Applicable usage scenario: immersive communication - The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP. - This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. - Define the composite requirement as following Test environment; Data rate (Mbits/s); Latency (ms); Packet success probability; Number of users (TRxP). Dense Urban - Immersive Communication; DL = 30 Mbps/UL=10 Mbps; DL=10 ms / UL=30 ms (or DL+UL total 40 ms); 99%; 6.

Proposal 6: To adopt the target values of composite requirement agreed in RAN#109 as baseline for 3GPP internal KPIs.

### 2.2.6	Resilience and Extended Connectivity

ITU-R WP 5D agreed including resilience and extended connectivity as a qualitative TPR, and its definition was agreed as follows [3]:

Definition of resilience and extended connectivity agreed in ITU-R WP 5D #50 4.17 Resilience and Extended Connectivity This requirement includes the following capabilities: Resilience: It refers to capabilities of the networks and systems to continue operating correctly during and after a service disruption, such as the loss of primary source of power, etc. The RIT/SRIT should support functionalities over the radio interface that enable continuous operation or rapid temporary restoration during and after disturbance to radio infrastructure. Extended Connectivity: It refers to the capability to provide connectivity in areas that are currently uncovered or scarcely covered due to challenges arising from geographical conditions, such as the lack of ground infrastructure (e.g., fiber cables and power supply), where users desire access to communication services. The RIT/SRIT should have functionalities that support providing geographically extended connectivity in these areas. Functionalities related to resilience and extended connectivity may include, but are not limited to, support for connecting to one or more of the following advanced network infrastructures within the terrestrial component of IMT: - HIBS - Relays and repeaters - Shared network architectures to provide redundancy and backup - Base station onboard a vehicle, airship and/or vessel - or other mechanisms for these capabilities using network infrastructures within the terrestrial component of IMT This requirement is defined for the purpose of evaluation in the Ubiquitous Connectivity usage scenario and may also be applicable to other usage scenarios.

RAN should use the above definition as baseline for 3GPP internal evaluation of resilience and extended connectivity.

Proposal 7: For resilience and extended connectivity, the definition agreed in ITU-R WP 5D should be used as baseline for 3GPP internal evaluation.

### 2.2.7	Coverage

Regarding coverage, RAN has recognized its importance for 6GR standard already. Accordingly, 6GR SID [6] has captured some relevant objectives, as follows:

- Single technology framework based on […]: [RAN1], [RAN2], [RAN3], [RAN4] […] - Enhanced overall coverage, focus on cell-edge performance and UL coverage. […] - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. […]

The above texts triggered some discussions in both RAN and RAN1, that resulting in the following outcomes (listed in chronological order):

Agreement (RAN1#122) - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types Outcomes from RAN#109: Proposal 4: For 3GPP internal study, link budget is used as the evaluation methodology for coverage when applicable Proposal 5: For 3GPP internal study, the target for coverage is to be determined by RAN. - FFS: Exact coverage target value(s). - FFS: Additional details considering control/data channel Agreement (RAN1#122bis, under AI11.1) - RAN1 provides methodology and corresponding initial analysis of potentially achievable coverage to RAN#110 to determine the coverage target(s) Agreement (RAN1#122bis, under AI11.2) For link budget template, consider the following candidates: - Candidate 1: Reusing the link budget template from TR38.830, i.e., the following table with notes as follows: - The values of the parameters are TBD. - MCL in row (22bis) is TBD. - FFS: whether/how/why to update [template omitted] - Candidate 2: Template as Table 7.10.1-1 from TR38.913. - FFS: whether/how/why to update. [template omitted] Agreement (RAN1#123) - For the RAN1 study of “Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band”, - The link budget template candidates 1 and 2 are used to calculate the metric(s) as starting point to compare existing 5G mid-band and 6G deployments in at least around 7 GHz, with potential future update. - During initial access/random access - Coverage target is referring the bottleneck channel (i.e. Rel-15 NR Msg3) during initial access/random access for existing 5G mid-band - FFS target value(s) of data rate for data channels relative to 5G mid-band - Following deployment scenarios are considered - Urban macro (both O2I and outdoor) - Sub-urban macro (both O2I and outdoor) - Following carrier frequencies are considered to calculate the metric(s) - [4 GHz] as the existing 5G mid-band - 7 GHz as 6G deployment Template in R1-2509615 is to be used for collecting inputs on the values from companies.

Procedure-wise, we understand the latest status as below:

  - It is clear that SID captures sub-objective d) and f) under objective (1) related to coverage performance of 6GR. For both, in our understanding, RAN TR will capture some requirements on coverage, subject to the discussion during SI.

    - For sub-objective d), there are two implications. One is that, since it describes cell-edge performance, 6GR may try to improve performance of physical signals/channels (which may not be a bottleneck in that scenario) at cell-edge. The other is rather to focus on bottleneck channel(s) in particular scenarios.

  - According to RAN#109 outcomes, two decisions were made: one is to use link budget for coverage (which is very straightforward), and the other is RAN to decide coverage target(s). Note that this was intended for 3GPP internal study.

  - An agreement in RAN1#122bis was reached to help the upcoming RAN task of target decision, i.e., in order to help RAN discussion, RAN1 agreed to provide methodology and initial analysis of potential achievable coverage. Note that until this, neither RAN nor RAN1 do not have any clear text in the agreements that may differentiate target d) and f). Also, there was another agreement made in AI 11.2 regarding candidates for link budget, i.e., candidate 1 and 2.

  - At RAN1#123, the discussions under AI 11.1 were proceeded by focusing on target f), which resulted in the bottom agreement. Note that this itself does not have clear implication toward corresponding discussions in RAN since it describes “for the RAN1 study” clearly.

We assume that the discussion on coverage for 6GR KPI will be proceeded based on the understanding above. In other words, RAN1 needs to proceed with the discussion on target(s) for coverage. Meanwhile, according to RAN1 discussion, there are multiple candidates even for link budget. And SID captures multiple sub-objectives for coverage. Hence it may be necessary to clarify which candidate for link budget will be used for each of the objectives. For this purpose, we believe one of the proposed observations in moderator summary would be helpful, shown below:

#### [H]Proposed observation 5.4: - For the potential metrics for coverage target(s), it is RAN1 understanding that: - MaxCL in Candidate 2 agreed in RAN1#122bis - Considers Total Tx power and Rx sensitivity (including Occupied CBW, Required SINR) - If this metric is used for coverage target(s), the Tx/Rx antenna gain components and large-scale channel characteristics are up to operator’s choice - MCL in Candidate 1 agreed in RAN1#122bis - Considers MaxCL, [system configuration,] Tx/Rx antenna gain component 2, and HARQ gain - If this metric is used for coverage target(s), the Tx/Rx antenna gain components 3/4 and large-scale channel characteristics are up to operator’s choice - MIL in Candidate 1 agreed in RAN1#122bis - Considers MCL, Tx/Rx antenna gain components 3 and 4 - If this metric is used for coverage target(s), large-scale channel characteristics are up to operator’s choice - MPL in Candidate 1 agreed in RAN1#122bis - Considers MIL and large-scale channel characteristics

For sub-objective d), irrespective of implications described above, we believe MCL would fit the best. First of all, MIL/MPL (or corresponding details) are not needed since this sub-objective only needs to see one scenario for signal/channel coverage comparison. One could argue that MaxCL can still be used. We do not strongly oppose, while our view is that at least HARQ gain should be taken into account to make the evaluation more realistic in reasonable manner. One potential target is 10 dB gap between PDSCH and PUSCH, based on the result of Rel-17 CovEnh WI SI [7]. Also, another important aspect is signal/channel to be considered, especially for bottleneck channel. We believe the bottleneck channel should be identified from the one used during initial access procedure, e.g., Msg3 PUSCH. It is because in reality, channels in initial access procedure is typically considered as a factor to determine cell deployment planning. To further clarify the described direction, we quoted a table from [7] below, which summarizes MCL results of several PHY signals/channels for Urban 4GHz TDD scenario.

Representative values of MCL for Urban 4GHz TDD scenario with BS Tx power of 33 dBm/MHz (Table 5.1.1.1-1 in [7])

| Channels (and Frame format) | Number of Samples | Representative value | Standard Deviation (w/o outlier) |
| --- | --- | --- | --- |
| PUSCH for eMBB DDDSU | 14 | 130.76 | 0.99 |
| PUSCH for eMBB DDDSUDDSUU | 11 | 132.37 | 0.86 |
| PUSCH for VoIP DDDSU | 11 | 142.78 | 2.40 |
| PUSCH for VoIP DDDSUDDSUU | 12 | 141.08 | 5.06 |
| PUSCH with SIP invite DDDSU | 1 | 144.80 | 0.00 |
| PUSCH with SIP invite DDDSUDDSUU | 0 |  |  |
| PUSCH for CSI 11bit DDDSU | 2 | 146.00 | 2.00 |
| PUSCH for CSI 22bit DDDSU | 2 | 144.80 | 2.00 |
| PUCCH Format 1 | 12 | 146.90 | 2.20 |
| PUCCH Format 3 11bits | 10 | 146.10 | 1.36 |
| PUCCH Format 3 22bit | 12 | 143.08 | 1.53 |
| SSB | 9 | 153.71 | 3.65 |
| PRACH Format 0 | 2 | 144.85 | 0.02 |
| PRACH Format B4 | 8 | 145.27 | 3.09 |
| PRACH Format C2 | 0 |  |  |
| Broadcast PDCCH(PDCCH of Msg.2) | 7 | 150.19 | 1.66 |
| PDSCH for Msg.2 | 8 | 151.33 | 3.72 |
| PUSCH of Msg.3 | 10 | 143.93 | 1.11 |
| PDSCH of Msg.4 | 7 | 150.42 | 4.23 |
| PUCCH w/ HARQ-ACK for Msg.4 | 3 | 148.59 | 2.23 |
| Unicast PDCCH | 13 | 156.83 | 3.91 |
| PDSCH for eMBB DDDSU | 12 | 153.24 | 4.59 |
| PDSCH for eMBB DDDSUDDSUU | 9 | 151.97 | 5.52 |

For example, among signals/channels with bold font (which are all related to initial access procedure), PUSCH of Msg.3 reached the smallest MCL value, around 144 dB. Also, all the other UL signals/channels do not achieve 150 dB. Compared with that, DL signals/channels achieved around 150 dB or larger. This result indicates that there is a gap between DL and UL which 6GR has to address. In our understanding, this trend is generally the same for many other scenarios. Given this situation, one potential KPI for 6GR coverage could be to set MCL target as “Msg.3 PUSCH MCL + X” dB, where X is around 7 to 10.

For sub-objective f), RAN1#123 agreement (while it targets RAN1 study) captures what is needed clearly and hence it should be considered as baseline. On top of that, our preference is to use MPL from candidate 1, i.e., taking large-scale channel characteristics into account. One may argue that by setting a specific value for pathloss, etc, the corresponding evaluation can still rely on simpler metric, such as MaxCL. However, since this sub-objective is to tackle potential problems in reality, we believe it would be great if we can try to analyse deeper.

Proposal 8: For coverage evaluated in 3GPP internal evaluation, support:

  - For sub-objective 1-d), taking MCL in candidate 1 as methodology for evaluation

    - Potential target is {Msg.3 PUSCH MCL + X} dB

    - For bottleneck coverage evaluation, focus on initial access signal/channel, such as Msg3

  - For sub-objective 1-f), taking MPL in candidate 1 as methodology for evaluation

    - With the realistic assumption of BS/US antenna setting, targeting equivalent MPL performance between 5G mid-band and 6G 7GHz deployment

# 3	Conclusion

In this contribution, the following proposals are provided to discuss 3GPP internal KPIs for 6G.

Proposal 1: To resolve the remaining issues on peak data rate and mobility for Rural-IC, and to discuss the target values of legacy KPIs for 3GPP internal evaluation of 6G based on the agreements for IMT-2030 TPRs in RAN#109 as a starting point, as described in Table 1.

Proposal 2: Regarding energy efficiency for sustainability, the definition and metrics agreed in ITU-R WP 5D should be used as baseline for 3GPP internal evaluation

- Loaded case can be considered as low load (0% - 15%), light load (15% - 30%) and/or medium load (30% - 50%), but other case is not precluded

- Evaluation condition including power model should be discussed

Proposal 3: To study the target values of positioning accuracy with reasonable level of enhancement as described in Table 2 and discuss whether the evaluation of vertical accuracy is necessary or not.

Proposal 4: For the evaluation of 6G sensing in 3GPP, target values should be discussed based on the performance objectives agreed for evaluation purpose of NR ISAC in RAN1#122bis as a starting point.

Proposal 5: For AI-related requirement, the definition agreed in ITU-R WP 5D should be used as baseline and it should be discussed whether to introduce other qualitative KPIs for 3GPP internal evaluation.

Proposal 6: To adopt the target values of composite requirement agreed in RAN#109 as baseline for 3GPP internal KPIs.

Proposal 7: For resilience and extended connectivity, the definition agreed in ITU-R WP 5D should be used as baseline for 3GPP internal evaluation.

Proposal 8: For coverage evaluated in 3GPP internal evaluation, support:

  - For sub-objective 1-d), taking MCL in candidate 1 as methodology for evaluation

    - Potential target is {Msg.3 PUSCH MCL + X} dB

    - For bottleneck coverage evaluation, focus on initial access signal/channel, such as Msg3

  - For sub-objective 1-f), taking MPL in candidate 1 as methodology for evaluation

    - With the realistic assumption of BS/US antenna setting, targeting equivalent MPL performance between 5G mid-band and 6G 7GHz deployment

# 4	References

[1] RP-252946, Draft Reply LS to ITU-R_WP5D_TEMP_167  = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN

[2] TR 37.910

[3] Annex 5.6 to Doc. 5D/989, Preliminary draft new Report ITU-R M.[IMT-2030.TECH PERF REQ] – Minimum requirements related to technical performance for IMT-2030 radio interface(s), ITU-R WP 5D

[4] TR 38.855

[5] R1-250727, Summary #5 on evaluations for NR ISAC, Moderator (Xiaomi)

[6] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, 3GPP TSG RAN Meeting #109

[7] TR 38.830