3GPP TSG-RAN Meeting #110	RP-253205

# Baltimore, USA, 8th Dec 2025 – 11th Dec 2025

Agenda item:	8.2.1.2

Source: 	Viasat, Novamint, Airbus, Sateliot, Gatehouse Satcom, ESA, ST Engineering iDirect, EchoStar, Thales, Skylo, Terrestar

Title: 	NTN specific proposals related to updates to architecture and migration clause

Document Type:	Discussion

Document for:	Approval

## 1.	Introduction

This paper provides our views on the architectural requirements related to NTN as part of 6G multi-RAT spectrum sharing feature.

## 2.	Evolution of NTN in 3GPP

NTN support is going to be an integral and native component of the first release of 6G specification.

NTN has been specified in 3GPP since Release 17. The following access technologies are supported within 3GPP:

- NR NTN for FR1 and FR2 NTN specific bands

- LTE for eMTC based IoT NTN for sub-3 GHz FR1 NTN bands

- NB-IoT NTN for sub-3 GHz FR1 NTN bands

With respect to NB-IoT NTN, both standalone operation as well as in-band operation in NR NTN via the same satellite access node (SAN) have been specified. Guard band operation is left to implementation based on channel size and availability of adequate guard band.

NB-IoT NTN is currently deployed by satellite operators via GSO and NGSO satellites. Given the time frame for 6G, development of NR NTN infrastructure is likely to be deployed before matured 6G standards are specified.

It is critical that the NTN operators can share their authorized spectrum across NB-IoT NTN, NR NTN and 6GR NTN radio accesses. Many of the NTN operators like their TN counterparts, may not have additional unused spectrum available to deploy NR NTN and 6GR NTN channels side by side, especially in sub-3 GHz FR1-NTN bands. Dynamic multi-RAT spectrum sharing is critical to enable 6GR NTN.

## 3. Proposals

Proposal 1: Unless specified otherwise, the 6G RAN requirements in clause 5.2 shall apply to TN and NTN systems.

Proposal 2: The 6G RAN shall support Multi-RAT Spectrum Sharing (MRSS) between 6GR NTN and NR NTN.

Proposal 3: Spectrum sharing between NR NTN and 6GR NTN accesses shall be dynamic across frequency and time domains.

Proposal 4: The 6G RAT shall support intra-SAN and inter-SAN spectrum aggregation (e.g., carrier aggregation, dual connectivity) for both uplink and downlink.

A potential incorporation of above proposals in Clause 5.2 is depicted in the Annex of this document.

## 4.		References

[1]	TR 38.914, v0.2.0, Study on 6G Scenarios and Requirements

=====================================

## Annex	Potential Updates to TR 38.914 [1]

Based on the above discussion we propose the following text proposal to clause 5.2.

====================Start of the changes==================

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

Unless specified otherwise, the 6G RAN requirements below shall apply to TN and NTN systems.

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing (MRSS) between 6GR and NR.

- 	6GR TN – NR TN

-   6GR NTN – NR NTN

-	Spectrum Sharing between 6GR and NR shall be dynamic across both frequency and time domains.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR across TN and NTN deployments.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	The 6G RAT shall support intra-SAN and inter-SAN Spectrum Aggregation (e.g. carrier aggregation, dual connectivity) for both uplink and downlink.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

==================End of the changes========================