3GPP TSG RAN Meeting #110			                                        RP-253298

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.3.2

Source:	CSCN, CBN, China Broadnet

Title:	Views on device type for 6G

Document for:	Discussion/Decision

Introduction

In RAN#109, several agreements and proposal about possible device types and defining device types for 6G were achieved as following [1].

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type Proposal 2: 6GR aims to specify a set of mandatory baseline functionalities for all UEs, with details to be further studied, by taking into account diverse device types for scalable and forward compatible design, and the requirements, as necessary, as in the 6G SID. Further study whether there is a need to mandate base stations for certain requirements/functionalities.

In this contribution, we provide our views about the device types for further discussion, especially from the perspective of supporting 6G NTN.

Discussion

Compared to 5G and earlier generations, ubiquitous connectivity is one of the core demands for 6G. While, NTN can achieve ubiquitous connectivity and service availability on a global scale. Further, 6G NTN is expected to support various services and use cases, such as messaging, voice, video, eMBB, IoT, and even positioning, sensing, timing and other functionalities, to meet comprehensive requirements from different types of UEs and diverse applications. Undoubtedly, NTN is the core component of 6G network, instead of a complement or optional configuration.

As is well known, NTN can overcome TN coverage limitations especially in low-density populated areas and maritime areas, and improve overall 6G network resiliency by supporting in priority critical communications and emergency communications. It is essential in situations where disasters disrupt terrestrial networks, and therefore provides significant social value.

Nowadays, all kinds of 5G NTN related products, services and applications have been a matter of common occurrence in our daily lives. Even commercial wearables and XR products that support NTN have appeared in the consumer market recently. We could expect that along with the tangible deployments of 6G NTNs in the future and deep reuse between TN and NTN industry chain, the services and applications through 6G NTN will become more available, widespread and affordable, and can be rooted in almost any device as a device independent functionality.

6GR is intended to support a diverse set of device types in the first release, namely 6G network including NTN and/or TN should be accessible by all 6G terminals. So, many capabilities, parameters and factors are being considered for defining the device types. As mentioned above, since the ubiquitous connectivity by NTN is a mandatory baseline functionality in 6G, the capability of access to NTN should be a crucial requirement for all device types. In other words, for most 6G devices, defining a separate of NTN-specific or NTN type device is unreasonable and unnecessary, which is contrary to the basic assumptions of 6G.

In addition, based on the fundamental principle of harmonized 6G radio interface design for TN and NTN, it is beneficial and feasible to realize the seamless service across TN and NTN under a unified framework and consistent solution for all 6G devices. In terms of UE implementations, RAN also could define a single device type covers all implementation requirements.

Observation 1: In 5G, NTN related capability is already a common requirement for various UE device types. NTN-based products, services and applications demonstrate huge commercial potential.

Proposal 1: The capability of access to 6G NTN should be one of baseline functionalities mandatory for most 6G devices, rather than for NTN-specific device type defined separately.

Conclusion

In this contribution, we discuss the issue of device type from the perspective of supporting 6G NTN, and have conclusions as following.

Observation 1: In 5G, NTN related capability is already a common requirement for various UE device types. NTN-based products, services and applications demonstrate huge commercial potential.

Proposal 1: The capability of access to 6G NTN should be one of baseline functionalities mandatory for most 6G devices, rather than for NTN-specific device type defined separately.

Reference

[1] RP-252873, Summary of 6G Diverse Device Types (including Minimum UE Bandwidth)