3GPP TSG RAN Meeting #110					RP-253308

Baltimore, U.S.A., December 8th -11th, 2025

Agenda Item:	8.2.1.2

Source:	Ericsson

Title:	6G RAN Architecture Requirements

Document for:	Approval

# 1	Introduction

The 6G RAN architecture should support UE handling in an efficient way, in both intra-and inter-RAN scenarios, guaranteeing the support for key multi-vendor interfaces, and the support of legacy and new use cases.

In this paper we discuss the requirements on 6G RAN architecture stemming from these principles.

# 2	Discussion

## 2.1	Assumptions

A first important assumption, needed to derive requirements on standardisation of the 6G RAN, is that the core of 6GS is an evolution of the 5G Core. This is motivated by the relatively recent uptake by many operators of 5G Core solutions. In the light of that, it seems plausible to assume that a new “rewrite” of the core of 6GS, not as an evolution, but as a clean sheet “revolutionary” design, would require investments and changes in an operator´s network that are well ahead of their times.

Therefore, we have assumed that at least the 5G Core legacy functionalities will be supported according to legacy 5G design in the core of 6GS. In line with that, we assume that the “AMF” in the core of 6GS will support at least the functionality of the AMF in the 5GC, with the functional split between core and RAN in 6GS following the same principles as for 5GS.

With the above we do not exclude enhancements to legacy functionalities and procedures, however the general legacy design framework should be respected.

With respect to new functionalities supported in 6G, we assume that an open and legacy-independent design approach can be followed.

## 2.2	Design Principles

One of the key principles to follow for the standardization of the 6G RAN is the high-cohesion principle, that is that the functionalities related to configuration and state handling of the UE shall remain under the control of a single logical entity at the RAN. Splitting the decisions of how to configure features of a UE (e.g., carrier assignment, MIMO, CSI-RS, measurements, DRBs, SRS, ...) or managing a UE context across different logical RAN entities suffers from a number of shortfalls.

Firstly, it requires signalling among the involved entities. This signalling is required to exchange relevant information about the UE, to update and maintain consistency among such entities when UE information changes, to exchange information about decisions taken locally at one entity, to update UE context information, etc. Such signalling is associated with processing inefficiencies, due to computations needed in multiple logical entities, and ultimately it is associated with increased latency for configuring a UE.

Secondly, it introduces functional cross-dependences among involved entities. In fact, certain decisions taken by one entity could impact the decisions to be taken by another entity. If such suboptimal decisions are not addressed, the impact is a drop in performance due to sub-optimal UE configuration, e.g., missing to achieve the best cell-group configuration made of decisions by different logical entities, or failing to achieve an optimal measurement configuration if different entities configure different aspects of the measurement.

A further side-effect of splitting the UE configuration process can be seen when considering the case of a UE being simultaneously active over multiple RAN nodes, e.g., in functionalities like dual-connectivity. In these cases, UE's resources like serving Cells, TX/RX layers, TX power, CSI-RS resources, etc. are shared across RAN nodes (potentially of different RATs), where such RAN nodes must jointly create a UE configuration that matches their own configurations and UE's capabilities. This brings to a design where a secondary entity adapts its configuration based on what previously decided by the primary entity, with a likely sub-optimal UE configuration as a result.

In line with the above principle of high-cohesion, we propose a 6G RAN design where RAN functions and procedures are modularised, but where the 6G RAN architecture consists of a single entity as a single logical standardised entity supporting the full protocol stack and including all the modular functions defined for the RAN. This design guarantees an architecture which avoids complex inter-entity negotiations, enables a faster innovation pace and ultimately gives more opportunities for obtaining optimal UE configuration.

A number of interfaces will have to be defined, with termination at the 6G RAN.

With respect to the interface between 6G RAN nodes, we foresee it as a multi-vendor interface for essential functionalities, for example, mobility.

We believe that resource aggregation shall be enabled by means of carrier aggregation, used to enhance capacity and data throughput. Considering that dual connectivity is practically neither able to cope with bursty traffic in a sufficiently dynamic way, and that it cannot deliver throughput aggregation promised at its introduction in 5G, carrier aggregation enables fast dynamic resource arbitration between different transmission points due to the above discussed principle of high-cohesion applied to scheduling at a single arbitration point.

Considering RAN-CN interfaces, previous generations have been designed considering one single direct termination in the core network for UE-related signalling (registration, mobility, connection, etc.). From a system point of view, this design guarantees that all functionalities (e.g., reachability) which are needed for all UE-core network communication functionalities are handled in a single entity (AMF in 5GC), and it also guarantees a clear separation of concerns between RAN and CN for UE-related signalling towards the CN. Considering this as a design to be followed in 6G, we propose that the 6G-RAN communicates directly with the equivalent functions of an AMF in the 6GC over a point-to-point 6G RAN-AMF interface.

Likewise, and in line with the assumption outlined in Section 2.1, we assume that the core for 6GS will follow the same design for legacy features supported in the 5GC. For that reason, we extend our proposal above to reflect that legacy features already supported over the 5G RAN to 5GC interface, should be supported by a point-to-point interface following the same design principles established for 5G. With that we do not preclude possible enhancements to the NGAP, that can be considered for the 6G RAN-CN interface.

With respect to new features not supported over the 5G RAN-CN interface, we are open to study how such features shall be supported over an interface between the 6G RAN and CN. As an example of such enhancements, we believe that 6G RAN terminated interfaces should ensure cloud-friendliness. To achieve this, the control plane interface should support more cloud-friendly protocols, e.g., QUIC, in order to benefit from its support and routability in cloud environments and its large community support. The application protocol of the control plane interface and the underlying transport layer should be decoupled, this following the design logic of cloud environments. Following this principle avoids that changes in transport layer become visible and should be explicitly handled by the application protocol, and it guarantees that the evolution of the interface application protocol is not tightly coupled to the transport protocol. Routability of application protocol messages within the cloud environment should be enabled by means of new information within RAN-CN AP messages that enable routing of AP messages to the instance of the function that needs to receive the AP message content. The interface application protocol should be designed for resilience to message losses and duplication should be supported.

As an instance of new functionalities, which would not necessarily need to follow the legacy RAN-CN interface design, we take Sensing. We support the introduction of a new direct control plane interface between the 6G RAN and a Sensing Core Controller function in the CN. The termination point of such interface at the 6G RAN would be the same logical node identified above as the logical 6G RAN node.

Related to User Plane interfaces, between 6G-RAN and UPF, as well as between the 6G-RAN nodes, GTP/UDP has proved to be sufficient and efficient, so we support to maintain GTP/UDP for 6G.

Connected mode mobility between NG-RAN and 6G-RAN (inter-RAT HO) and idle mode mobility between 6G-RAN and E-UTRAN should be supported. The 6G-RAN to 6G-RAN interface should support better scalability, for example, to enable more efficient neighbouring cell information exchange.

Enhancements are also needed in the area of data collection. Data collection needs a framework that enables configuration of data collection campaigns and collection of potentially very large amounts of data. Such data should be reused as much as possible, namely any consumer needing such data should be able to get them without triggering a new data collection campaign, but by reusing previously stored data. For this reason, we advocate for a management centric data collection framework, where management level data repository functions can store data generated by data producers and provide such data to data consumers across the whole 6G network.

To meet the requirements described here, involvement of the 6G Radio and CN is necessary. Thus, coordination and cooperation between RAN WGs and SA WGs is needed.

# 3 Text proposal for TR 38.914

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	RAN interfaces should be based on a design where the application protocol and the transport protocol layers are decoupled and independent

-	RAN interfaces shall be “cloud friendly” namely interface application protocol messages shall be routable within a cloud environment.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The RAN shall support a management centric data collection framework enabling data collection and distribution to/from different parts of the 6G system. The framework shall enable local consumption of data at the RAN and data exposure from the RAN to data collection entities.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The RAN architecture shall enable fast and dynamic resource arbitration between different transmission points serving the same UE .

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 4	Conclusion

In this contribution, we make a text proposal to Chapter 5.2 in TR 38.914.