3GPP TSG RAN Meeting #110  			 	            RP-253327

Baltimore, USA, December 8-11, 2025              	 (update of  RP-252229)

Agenda Item:	8.2.1.2 Requirements for architecture and migration

Source:	Vodafone, NTT DOCOMO, SK Telecom, CHTTL

Title:	        	Proposed Requirements and SID update: Interworking between
              6GR and 4G

Document for:	Agreement
___________________________________________________________________

## 1	Introduction

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]
NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

In RAN #108 in June 2025, the RAN Working Group SID for 6G RAN was agreed in RP-251881. On Migration and Interwoking it contains the following objective:

This document addresses the above Note on interworking between 6GR and 4G. It follows on from the discussions on RP-252229 in RAN #109, and it proposes the agreement of requirements in TR 38.914 and an update of the RAN Working Group SID (latest version is in RP-252912).

## 2	General

Operators in different regions experience different market and regulatory conditions and the 3GPP standards should endeavour to accomodate these.

The 3GPP standards should also accomodate different operator schedules for the shut down of different technologies.

Note that in Europe, several operators have shut down 3G before 2G, and that at least Vodafone has contracts that require LTE service to be maintained in UK until 2040.

Voice call failure rates and particularly emergency voice call failure rates are key performance indicators for operators (and regulators) and at least emergcy calls are likely to remain improtant in the 6G era.

3GPP work should also address real world scenarios, e.g.

- where an operator saves energy by powering down lightly loaded transmitters on a multi-frequency site until only one carrier remains,

- where the operator uses a satellite system that is accessible by as many devices as possible.

## 3	Idle mode mobility between 6GR and 4G

5G was developed to support interworking with 4G and to not support interworking between 5G and 2G/3G.

The accidental consequence of this was that Release 15 mandated that 4G to 5G handovers failed if the mobile had visited 2G/3G and then moved to 4G. Vodafone led work in a later release to specify a partial but pragmatic solution for this.

To avoid a repeat of this release 15 situation (i.e. to avoid 5G to 6GR handovers failing after the UE visted 4G), the SA2 study item will work on this topic. From Work Task 2 in the agreed SA2 SID on Architecture for 6G System in SP-250806:

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

Solutions for this issue are likely to provide the core network components for idle mode mobility between 6G and 4G, and, provide the foundations for core network support of handover.

## 4	Use cases for 6G handover with 4G

### 4.1	Network Energy Saving

Network Energy Saving is a key operator objective but the main gains in energy saving have not come from 3GPP’s recent features, but instead from improvements in the RAN vendors’ implementations when coupled with operators turning off lightly loaded frequencies. This trend is likely to continue and, when considered in more detail (see steps below), results in Network Energy Saving needing 6G to 4G handover.

- While some operators foresee the shutdown of their 4G network, other operators foresee that 4G IoT will be very long lived.

- Operators foreseeing shutdown of 4G may well launch 6G before 4G shutdown.

- The commencement of 4G eCall in only 2026 (for new car models) and 2027 (for existing car models) also indicates that 4G will be maintained in (at least) Europe for a long while.

- Future voice-only devices are likely to support 4G but might well not support 5G or 6G. These devices need to be able to make emergency calls (and with consistently good indoor coverage -> i.e. satellite coverage is not sufficient)

- In a multi-frequency base station site, the most complete way to save energy is to power down cells with light load (after moving the traffic to another, typically lower, frequency).

- Currently, it does not look like 4G-6G DSS will be specified/supported.

- 4G-5G DSS consumes more base station power than 4G-only operation, and, significantly reduces cell capacity.

- Thus, when the traffic load on a 4G/5G/6G site decreases, carriers can get switched off until just a 4G-only carrier remains.

- Adjacent to this lightly loaded site may be a more heavily loaded site that is still operating 6G.

- As a mobile in a voice call moves from the 6G site into the energy saving site, a 6G (emergency) voice call will drop without handover from 6G to 4G.

## Observation 1: operators that maintain 4G after the launch of ”voice over 6G” and who also use strong network energy saving regimes will need 6G to 4G voice call handover to avoid (emergency) voice call interuption.

Note: 	handover from a 4G-only Network Energy Saving site to 6G does not seem to be essential as the 6G site can be expected to also have 4G operational. Then the mobile can return to 6G after the call ends, or, via 4G to 5G handover follwed by 5G to 6G handovers.

### 4.2	Integration of 6GR and Satellite Access

The integration of 6G with satellite systems is a key objective of some governments and many companies. Objectives include ensuring complete national (and super-national) coverage, and ensuring access to everyone and everything everywhere.

By carefully considering these aspects (in the steps below) and the trajectory of our industry on the use of low frequency band 6G and 6G IoT, we can see the need for 6G to 4G handover.

  - Currently deploying commercial satellite systems support the use of unmodified 3GPP mobiles and use unmodified interfaces to a 3GPP core network.

  - “Digital Inclusivity” and the support for IoT means full coverage is needed for 4G.

  - Satellite systems are limited in capacity, hence the use of 4G-5G DSS on a satellite system is undesirable.

  - The good ambition of 3GPP to avoid 5G-6G Dual Connectivity implies that 6G must be able to be rolled out easily on a low frequency band.

  - With the use of a low band for 6G, it might be expected that 6G, 5G and 4G terrestrial coverage runs out at same point….

  - however, the ambition of building an IoT “kernel” into the day-1 6G specifications implies that 6G will support network based “coverage recovery” mechanisms for “6G-RedCap” devices with a single antenna. This would mean that 6G TN coverage would exceed that of 4G TN and 5G TN….

  - thus, at the edge of TN coverage, a 6G mobile would be on 6G and not on 4G or 5G.

  - Hence, at least for (emergency) voice calls, there is a need for 6G TN to 4G handover (with the 4G satellite system appearing to be a TN to the 6G system)

## Observation 2: integration of 6GR with commercially deploying satellite systems, coupled with the 6G IoT coverage recovery concepts, mean that 6G to 4G handover is needed to avoid (emergency) voice call interuption.

Note: 	handover from 4G-satellite to 6G TN is not essential. Instead 4G-satellite to 4G/5G terrestrial could be used and then followed by 5G to 6G handover.

### 4.3	Other use cases

Other companies might wish to elaborate on:

- LTE (rather than NR) as primary voice layer leading to the need for handover from 6G to a 4G-VoLTE coverage layer;

- at the early stage of 6G commercialization, 5G coverage might not be complete and lead to the need for direct handover from 6G to 4G; and,

- direct migration from 5G-NSA to 6G.

## 5	Co-existence of LTE/NB-IoT and 6G spectrum

This was discussed at RAN#109 and the following text was added in v0.2.0 of the RAN plenary TR 38.914:

### 5.4.3	Massive Communication (IoT)

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT):

- 6G Massive Communication (IoT) shall be supported for FR1.

  - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB

    - Prioritize 6GR design for eMBB

  - The above usage scenario should not overlap with Ambient IoT and NB-IoT

- [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device.

Editor note:	“6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

However, the Working Group WID has not yet been updated, nor the text moved to section 5.2.

## 6	Proposals

Proposal 1: the work on 6G  study should study how to support 6G TN to 4G TN handover. Support for 4G to 6G handover need not be considered in the first phase of 6G.

Proposal 2: Update the RAN plenary TR 38.914 clause 5.2 ” Requirements for architecture and migration” to include requirements for mobility and co-existence with 4G as below:

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support idle mode inter-RAT mobility between the 6GR and E-UTRA and connected mode mobility from 6GR to E-UTRA.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

- /.... rest of section not copied .../

Proposal 3: the 6G RAN WG study (latest version in RP-252912) is updated as follows:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

- 6GR coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration [RAN1, RAN4]

- Handover from 6GR to E-UTRA, and idle mode mobility between 6GR and E-UTRA [RAN2, RAN3, RAN4] Note: coordination with SA WGs will be done, as necessary

*** end ***