3GPP TSG RAN #110			RP-253443

Baltimore, USA, December 8th – 11th, 2025

Source:	Sharp, Thales, Novamint, Eutelsat, Tejas Networks Fraunhofer HHI, Fraunhofer IIS, Amazon, Gatehouse Satcom, ST Engineering iDirect, Sony, Skylo

Title:	Views on study of GNSS-less/resilient NTN operation

Agenda Item:	8.2.2

Document for:	Discussion and Decision

# Introduction

At RAN#108, the Study on 6G Radio (aka 6GR SID) was approved. The study identifies RAN1 as primary, and RAN2/3/4 secondary. The study includes the following objective on NTN ‎[1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] […] - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. […]

In RAN1 it is clear from the following agreement found in the RAN1 chair end of meeting notes [3] that "GNSS-less/resilient based operation" is included in the scope of the 6GR SID.

Agreement made at RAN1#122bis [3]

Agreement made at RAN1#122bis - The aspects to consider for supporting NTN include, but not limited to - Initial access, including cell search and SSB periodicity - Coverage - Duplexing - Capacity - Signalling overhead - GNSS-less/resilient/based operation - Large/varying doppler and propagation delay - Beamforming / beam management / beam hopping

However, the 6GR SID does not specify whether GNSS-less/resilient design is to be included in the scope, which has led to some confusion in RAN2. As such, different views have emerged in RAN2 WG discussions regarding whether GNSS-less/resilient NTN operation should be studied for 6G Day-1. To facilitate alignment and provide guidance to the WGs, the RAN2 chair encouraged companies to share their perspectives at the RAN Plenary, as found in the RAN2 chair end of meeting notes ‎[2]:

Agreements made at RAN2#132 [2]

Agreements 1 The basic assumption with 5G NTN deployment scenarios (LEO, MEO, GSO) and payload type can be adopted in 6G NTN. FFS whether we will study with and/or without GNSS, depending on RANP and other WG discussion first.

This paper provides sourcing companies’ view on studying GNSS-less/resilient NTN operation for 6G.

# Discussion

GNSS-less operation refers to devices that do not possess GNSS capability, and GNSS-resilient operation refers to scenarios where GNSS receiver is available, but the GNSS signal may be subject to degradation, for example due to signal obstruction, intermittency, etc.

Studying GNSS-less/resilient operation from the first release of 6G is strongly motivated by the following technical and architectural considerations:

Support for new device types/capabilities: 6G introduces UE/device categories. Low-capability UEs such as IoT devices may not integrate GNSS due to cost, power, or form factor constraints. It is essential to ensure robust NTN support for such devices to achieve broad 6G applicability across verticals.

Clean design: Incorporating GNSS-less/resilient design principles from Day 1 allows the system architecture, radio protocols, and procedures to be designed holistically without later retrofitting. This avoids complexity that may arise from adding GNSS-resilient enhancements in later releases.

NTN deployment scenarios: Some NTN operating environments challenge GNSS availability, for example devices in obstructed environments.

Alignment with 6G objectives: 6G is expected to improve service towards ubiquitous service availability, including in remote, degraded, or infrastructure-limited environments, which motivate GNSS-less/resilient design.

Reduced cost and power consumption: Eliminating the requirement for GNSS can reduce hardware cost and power consumption for UEs, for example for low-capability IoT devices.

As outlined above, there is strong motivation for the 6GR SID to specify GNSS-less/resilient operation explicitly for integrated TN-NTN for 6G Day-1 in pursuit of a coherent, future-proof NTN framework.

In NR Rel-20, the Study on GNSS resilient NR-NTN operation [4] (aka GNSS resilient NR-NTN SI) had further progress in RAN1#123, which included an agreed TR skeleton. The study will continue in the next RAN1 WG meetings, and upon conclusion, a work item on the subject is expected to be discussed for approval in Rel-20.

Numerous candidate solutions have been proposed in the current GNSS resilient NR-NTN SI , which are expected to be captured in the TR. The solutions are then subject to down-selection in the normative phase. Two major aspects of the GNSS-resilient operation are as follows:

- For initial access, how to perform preamble transmission and detection when the timing/Doppler error exceeds the tolerance level, especially for short preamble formats.

- In RRC connected mode, how to reduce the overhead for timing/Doppler update and indication.

For 6GR SID, GNSS-less/resilient operation can be readily considered based on the progress made thus far in the GNSS resilient NR-NTN SI . Furthermore, as the normative work of the follow-on GNSS resilient NR-NTN work item starts and progresses, the outcome of that work item can be considered for further refinement in the 6GR study. For example, the 6GR study may place more emphasis on scenarios and solutions that will be decided not to be supported in NR Rel-20.

As an example, the current GNSS resilient NR-NTN SI assumes that GNSS is available, but it is temporarily unusable, or GNSS measurements are not updated for an extended period. Several proposals make use of a reference position to determine the initial time and Doppler compensation. In contrast, in 6GR study, NTN without any GNSS should be discussed as well. If GNSS is not available at the UE, then there is no UE location information available to be used for determining timing advance and Doppler. Therefore, alternatives should be considered for distance estimation, Doppler compensation, and so on.

Observation: NTN GNSS-less/resilient can be readily considered for the 6GR SID based on the progress of the GNSS resilient NR-NTN SI .

Proposal: Update the Study on 6G Radio to specify explicitly that GNSS-less/resilient is included in the study of NTN.

# Conclusion

We discussed GNSS-less/resilient NTN operation in 6G and made the following observation and proposal:

Observation: NTN GNSS-less/resilient can be readily considered for 6GR SID based on the progress of the NR NTN GNSS-resilient SI.

Proposal: Update the Study on 6G Radio to specify explicitly that GNSS-less/resilient is included in the study of NTN.

# References

- RP-252912, “Revised SID: Study on 6G Radio”, RAN#109, Beijing, China, September 2025.

- R2_132_ChairNotes_11-21_final_clean.zip

- Chair notes RAN1#122b eom4.docx

- RP-251944, “Revised SID: Study on GNSS resilient NR-NTN operation”, RAN#109, Beijing, Sept 2025