3GPP TSG RAN WG1 #122bis		R1-2507124

Prague, Czech Republic, Oct 13th - 17th, 2025

Source:	CATT

Title:	Maintenance for Rel-19 IoT NTN

Agenda Item:	8.7.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will provide a text proposal for some remaining issues of Rel-19 IoT NTN.

- Text Proposals

2.1 Text Proposal #1 for multi-TB scheduling for IoT NTN Phase3

Reason for change: Current multi-TB scheduling cannot clarify DCI format indicating OCC enabled and there is inconsistency in the use of the term “is OCC enabled” and “indicates OCC enabled”.

Current multi-TB scheduling in draft CR 36.213 are as the following:

-	For ,

-	if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and  where  for ,  otherwise.

-	NB-IoT UL slots  with l=0,1,…g-1,  c=0,1,…NRep/(CMOCC)-1, g=CMOCCNRUNslotsUL are associated with TBr+1 , .  MOCC=2 if NscRU=1 and the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled in the corresponding DCI, MOCC=1 otherwise.

-	otherwise,

-	NB-IoT UL slots  with  are associated with TBr+1 ,

As we all know, NPUSCH format 1 is scheduled by DCI Format N0, the meaning of “corresponding DCI” is not clear. Additionally, it is necessary to align the term of “is OCC enabled” and “indicates OCC enabled”. Consequently, it should be clarified that DCI format and indicating “OCC enabled/disabled indicates OCC enabled”.

Summary of change: In the section 16.5.1 of TS 36.213, it is necessary to clarify the DCI format and align the term of “is OCC enabled” and “indicates OCC enabled”.

Consequences if not approved: The unclear indication issue will cause ambiguous understanding between UE and gNB for “OCC enabled/disabled indicates OCC enabled”indicated by DCI Format N0.

Proposal 1: Adopt the following TP for draft CR 36.213 Rel-19 IoT_NTN Ph3.

TS 36.213 V19.0.0 - 16.5.1 UE procedure for transmitting format 1 narrowband physical uplink shared channel *** Unchanged parts are omitted *** - For , - if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and where for , otherwise. - NB-IoT UL slots with l=0,1,…g-1, c=0,1,…NRep/(CMOCC)-1, g=CMOCCNRUNslotsUL are associated with TBr+1 , . MOCC=2 if NscRU=1 and the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled/disabled indicates OCC enabledin the DCI Format N0, MOCC=1 otherwise. - otherwise, - NB-IoT UL slots with are associated with TBr+1 , *** Unchanged parts are omitted ***

2.2 Text Proposal #2 for NPDSCH/NPUSCH related procedures for IoT NTN TDD

Reason for change: In current specification, whether a downlink/uplink subframe configured for NB-IoT DL/UL transmission is a NB-IoT DL/UL subframe only specified for anchor carrier, not for non-anchor carrier.

Current NPDSCH/NPUSCH related procedures regarding IoT NTN TDD are as following:

- 16.4 Narrowband physical downlink shared channel related procedures A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows - If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe. - Else if the UE is in a NTN TDD serving cell and the UE determines the subframe is not one of the D consecutive downlink subframes according to the TDD pattern and the value of D defined in [3], then the subframe is not assumed as a NB-IoT DL subframe. - 16.5 Narrowband physical uplink shared channel related procedures A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows - In all other cases, - for TN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers - for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe - for NTN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if it is one of the U consecutive uplink subframes according to the TDD pattern and the value of U defined in [3].

In RAN1#122meeting, the agreement has been reached as follows:

Agreement The set of D/U-subframes in a non-anchor carrier is the same as the set of D/U-subframes in the anchor carrier, and are time-aligned. - FFS: specification impact, if any.

Summary of change: In the section 16.4 and section 16.5 of TS 36.213, it is necessary to specify how to determine NB-IoT DL/UL subframes for IoT NTN TDD for anchor carrier and non-anchor carrier.

Consequences if not approved: The issue will cause that a IoT NTN TDD UE in a non-anchor carrier determines IoT TDD DL/UL subframes as per legacy FDD procedures.

Proposal 2: Adopt the following TP for TS 36.213 Rel-19 IoT_NTN TDD.

TS 36.213 V19.0.0 - 16.4 Narrowband physical downlink shared channel related procedures A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows - If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe. - Else if the UE is in a NTN TDD serving cell and the UE determines the subframe is not one of the D consecutive downlink subframes according to the TDD pattern and the value of D defined in [3], then the subframe is not assumed as a NB-IoT DL subframe. When multiple carriers are configured for one UE, the D consecutive downlink subframes in non-anchor carrier are same as the D consecutive downlink subframes in an anchor carrier, and are time aligned. *** Unchanged parts are omitted *** - 16.5 Narrowband physical uplink shared channel related procedures *** Unchanged parts are omitted *** A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows *** Unchanged parts are omitted *** - In all other cases, - for TN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers - for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe - for NTN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if it is one of the U consecutive uplink subframes according to the TDD pattern and the value of U defined in [3]. When multiple carriers are configured for one UE, the U consecutive uplink subframes in non-anchor carrier are same as the U consecutive uplink subframes in an anchor carrier, and are time aligned. *** Unchanged parts are omitted ***

Conclusion

In this contribution, we presented a text proposal for remaining issues of Rel-19 IoT NTN:

Proposal 1: Adopt the following TP for draft CR 36.213 Rel-19 IoT_NTN Ph3.

Reason for change: Current multi-TB scheduling cannot clarify DCI format indicating OCC enabled and there is inconsistency in the use of the term “is OCC enabled” and “indicates OCC enabled”. Current multi-TB scheduling in draft CR 36.213 are as the following: - For , - if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and where for , otherwise. - NB-IoT UL slots with l=0,1,…g-1, c=0,1,…NRep/(CMOCC)-1, g=CMOCCNRUNslotsUL are associated with TBr+1 , . MOCC=2 if NscRU=1 and the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled in the corresponding DCI, MOCC=1 otherwise. - otherwise, - NB-IoT UL slots with are associated with TBr+1 , As we all know, NPUSCH format 1 is scheduled by DCI Format N0, the meaning of “corresponding DCI” is not clear. Additionally, it is necessary to align the term of “is OCC enabled” and “indicates OCC enabled”. Consequently, it should be clarified that DCI format and indicating “OCC enabled/disabled indicates OCC enabled”. Summary of change: In the section 16.5.1 of TS 36.213, it is necessary to clarify the DCI format and align the term of “is OCC enabled” and “indicates OCC enabled”. Consequences if not approved: The unclear indication issue will cause ambiguous understanding between UE and gNB for “OCC enabled/disabled indicates OCC enabled”indicated by DCI Format N0.

TS 36.213 V19.0.0 - 16.5.1 UE procedure for transmitting format 1 narrowband physical uplink shared channel *** Unchanged parts are omitted *** - For , - if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and where for , otherwise. - NB-IoT UL slots with l=0,1,…g-1, c=0,1,…NRep/(CMOCC)-1, g=CMOCCNRUNslotsUL are associated with TBr+1 , . MOCC=2 if NscRU=1 and the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled/disabled indicates OCC enabledin the DCI Format N0, MOCC=1 otherwise. - otherwise, - NB-IoT UL slots with are associated with TBr+1 , *** Unchanged parts are omitted ***

Proposal 2: Adopt the following TP for TS 36.213 Rel-19 IoT_NTN TDD.

Reason for change: In current specification, whether a downlink/uplink subframe configured for NB-IoT DL/UL transmission is a NB-IoT DL/UL subframe only specified for anchor carrier, not for non-anchor carrier. Current NPDSCH/NPUSCH related procedures regarding IoT NTN TDD are as following: - 16.4 Narrowband physical downlink shared channel related procedures A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows - If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe. - Else if the UE is in a NTN TDD serving cell and the UE determines the subframe is not one of the D consecutive downlink subframes according to the TDD pattern and the value of D defined in [3], then the subframe is not assumed as a NB-IoT DL subframe. - 16.5 Narrowband physical uplink shared channel related procedures A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows - In all other cases, - for TN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers - for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe - for NTN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if it is one of the U consecutive uplink subframes according to the TDD pattern and the value of U defined in [3].. In RAN1#122meeting, the agreement has been reached as follows: Agreement The set of D/U-subframes in a non-anchor carrier is the same as the set of D/U-subframes in the anchor carrier, and are time-aligned. - FFS: specification impact, if any.. Summary of change: In the section 16.4 and section 16.5 of TS 36.213, it is necessary to specify how to determine NB-IoT DL/UL subframes for IoT NTN TDD for anchor carrier and non-anchor carrier. Consequences if not approved: The issue will cause that a IoT NTN TDD UE in a non-anchor carrier determines IoT TDD DL/UL subframes as per legacy FDD procedures.

TS 36.213 V19.0.0 - 16.4 Narrowband physical downlink shared channel related procedures A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows - If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe. - Else if the UE is in a NTN TDD serving cell and the UE determines the subframe is not one of the D consecutive downlink subframes according to the TDD pattern and the value of D defined in [3], then the subframe is not assumed as a NB-IoT DL subframe. When multiple carriers are configured for one UE, the D consecutive downlink subframes in non-anchor carrier are same as the D consecutive downlink subframes in an anchor carrier, and are time aligned. *** Unchanged parts are omitted *** - 16.5 Narrowband physical uplink shared channel related procedures *** Unchanged parts are omitted *** A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows *** Unchanged parts are omitted *** - In all other cases, - for TN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers - for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe - for NTN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if it is one of the U consecutive uplink subframes according to the TDD pattern and the value of U defined in [3]. When multiple carriers are configured for one UE, the U consecutive uplink subframes in non-anchor carrier are same as the U consecutive uplink subframes in an anchor carrier, and are time aligned. *** Unchanged parts are omitted ***

Reference

[1]R1-2506563	Session notes for 8.11 (Maintenance on Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode)	Ad-Hoc Chair (Huawei)

[2] 36.213 V19.0.0