3GPP TSG RAN WG1 #122-bis		   	 R1-2507253

Prague, Czech, 13th – 17th October 2025

Agenda item:		11.2

Source: 		Samsung

Title:		 	Evaluation assumptions for 6GR

Document for:	Discussion and Decision

Introduction

6GR promises advances in wireless communication, including ultra-high data rates, ultra-low latency, high energy efficiency, massive connectivity, seamless mobility, AI-enhanced performance, and precise sensing. Designing the 6GR air interface requires a thorough evaluation of the 6GR to ensure reliability, efficiency, and adaptability to future needs. Evaluating 6GR is complex due to its various use cases and technological advances. It spans innovations from low-band communications to AI-driven networks, requiring a comprehensive evaluation. We will examine key assumptions shaping evaluation methods, such as design areas, deployment scenarios, antenna layout, and environmental conditions. The performance of 6GR should be compared with NR approaches as a benchmark.

From RAN1#122, the following agreements were made for 6GR evaluation:

Agreement - The deployment scenarios in TR38.914 should be considered for evaluation assumption - The common evaluation assumptions including the antenna modelling, general system-level simulation assumptions (including the carrier frequency, bandwidth and subcarrier spacing used for link-level simulation) for the deployment scenarios in TR38.914, link budget and traffic models will be discussed in AI 11.2 - Other assumptions including for link-level simulation specific to each technical topic will be separately discussed under each individual agenda. - Note: Subcarrier spacing decision is up to AI 11.3.2. Conclusion - Template in R1-2506582 is to be used for collecting inputs from companies. - Additional NTN or TN assumptions, if any, or any necessary change of the parameters, are to be incorporated into the updated one of R1-2506582. Agreement - Study which of the following traffic models are to be used for 6G evaluations, e.g., - Full buffer - FTP Model 1 (in TR 36.814) - FTP Model 2 (in TR 36.814) - FTP Model 3 (in TR 36.872) - XR Traffic models (in TR 38.838) - VoIP model (as in TR 36.814) - Instant message (as in TR 38.840) - Study whether to introduce the following traffic models for 6G evaluations considering, e.g., - FTP-3 variant with packet delay budget requirement - Details FFS - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget) - Details FFS - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc. - Details FFS - Study whether to introduce new/additional approaches that can reflect the impact of bidirectional traffic flows on performance metrics (e.g., impact of UL TCP ACK latency on DL throughput/latency) Note: Whether/how to consider the combination of traffic model and loading level will be studied under individual agendas..

This contribution discussed the aspects of general assumptions and models for 6GR evaluation.

General considerations

Although RAN1 agreed to consider all deployment scenarios in TR 38.914, it is unaffordable to evaluate all possible combination cases as per carrier frequency, antenna configuration bandwidth and so on. It is desirable to limit the simulation to some cases with representative assumptions, considering typical deployment, practical implementation and specific evaluation purpose. On the other hand, more comprehensive evaluation on new candidate frequency bands for 6GR may be expected since 5G NR may not provide enough reference. To better utilize the simulation effort, some baseline assumptions can be decided as higher priority and minimal requirement, while it is up to companies to report other options for diversity and reasonable balance. For instance, typical 1 TX antenna at UE for existing FR1 can be the baseline assumption considering the popularity in reality.

Common evaluation assumptions to be discussed under this agenda item at least include deployment scenarios, antenna modelling, traffic models and quite many system-level simulation assumptions. It can be envisioned that many diversified options are to be proposed for most ones in the list of “common evaluation assumptions” due to various reasonings, making simulation campaign very difficult.

Proposal #1:

- If there are too diversified options in the list of common evaluation assumptions, RAN1 strives for deciding on a minimal set of baseline assumptions, while companies report other options

Other than common evaluation assumptions, specific assumptions related to those technical topics with assigned agenda item should be discussed separately under each individual item, including waveform, channel coding, initial access and energy efficiency. For those technical topics without individual agenda items yet, some early discussion may be expected and then views on evaluation assumptions can be provided herein.

Some realistic models should be considered to evaluate the performance accurately in 6GR evaluation. RF impairment in reality is one of typically limiting factors to link performance, modelling of which can help draw the meaningful observations for schemes targeting to provide potential benefits in such conditions. Assumption of perfect interference estimation in MIMO system level simulation is also concerned as another example, which may result in too optimistic performance given the increase of antenna ports in 6GR. Having said that, RAN1 would study the need of realistic models per evaluation purpose, e.g., to determine the practically achievable performance of different schemes.

Proposal #2:

- The need of realistic models in RAN1 evaluation should be discussed per evaluation purpose, and if applicable, should be prioritized to determine the performance

Evaluation assumptions

  - 3.1 Assumptions related to MIMO

In 6GR, new frequency band e.g., around 7GHz may be introduced as additional spectrum. And FR1 and FR2 can be supported through spectrum sharing. Therefore, to evaluate MIMO schemes designed for specific frequency range around 7GHz, an appropriate channel model should be considered respectively.

For new spectrum around 7GHz, up to 256 antenna ports can be supported to compensate for reduced coverage by increasing and exploiting beamforming gain. However, a higher number of antenna ports incurs RS overheads for CSI measurement. Therefore, the CSI-RS design should consider RS overhead as well as the performance metric such as channel estimation accuracy and throughput. To reduce RS overheads when supporting up to 256 antenna ports, sparser CSI-RS resource mapping could be considered. In this case, evaluation should be performed to compare various mapping patterns that reduce RS overhead while maintaining similar or slightly degraded channel estimation accuracy compared to legacy scheme.

To support high frequency range, i.e., FR2, analog beamforming scheme can be employed. For CSI acquisition based on analog beamforming, precoded CSI-RS can be considered. Additionally, to promote more use cases for FR2, various device types like CPE/FWA should be considered. Compared to conventional handheld mobile devices, CPE/FWA like devices typically have larger form-factor allowing integration of more antenna elements and arrays. Therefore, for 6G evaluation, the precoded CSI-RS scheme and beam management procedures may require various simulation assumptions covering different types of devices supporting various antenna configurations and transmission power.

In FR1, the EVM can be extended based on the existing NR EVMs for sTRP and mTRP (CJT, coherent joint transmission). Since it is expected that antenna port configurations with 64 ports or more become more popular in real field deployments, such antenna configurations can be a baseline and should be included in the 6G EVM. In addition, mTRP-based MIMO schemes are still important in 6G and can be further enhanced and evaluated using SLS analysis. Especially, mTRP CJT schemes should be prioritized as it is expected to have a clear and significant performance gain with the scheme compared to sTRP in FR1 including low carrier frequency bands such as 700 MHz. Therefore, it is desirable to include EVM for evaluation of 6G mTRP CJT schemes, with Rel-18 NR EVM as a baseline.

For handheld device evaluation, baseline simulation assumptions should align with practical handheld device capabilities. Recent smartphones can support up to 4 layers using 4 RX antennas which can be positioned in possible candidate locations specified in TR 38.901. For practical UL implementation in a handheld device, a high-power PA with single transmit (i.e. 1TX) configuration targeting 23, and 26 dBm should be adopted as the baseline assumption. Consequently, up to 4 layers per UE for DL and 1 layer per UE for UL can be supported as the baseline for maximum SU-MIMO layers or MU-MIMO layers. While multi-port UE configurations may be explored for new spectrum investigations around 7GHz, this 1TX baseline will serve as the primary benchmark for performance assessment.

To consider realistic assumption, both UE side and BS side impairments can be modeled. If the certain impairment exists, it can degrade performance, for example, UE Tx impairment might affect PUSCH throughput gain or channel estimation accuracy via SRS based DL CSI. Furthermore, the impairment can affect DL-UL reciprocity assumption. Therefore, depending on BS or UE assumption, DL-UL reciprocity model can be considered to evaluate reciprocity-based schemes. In addition, non-ideal channel and interference estimation should be considered especially for evaluation of the UL performance with large number of antenna port.

Proposal #3:

- For BS and UE antenna configurations

  - In around 7GHz, assume 128 and 256 antenna ports at the BS.

  - In FR2, assume analog beamforming including precoded CSI-RS.

  - In FR1, evaluate both sTRP and mTRP CJT

- To evaluate the tradeoff between RS overhead and performance, sparser RS mapping should be assumed.

- For maximum SU-MIMO layers or MU-MIMO layers,

  - For handheld, supporting up to 4 layers per UE for DL and 1 layer per UE for UL

  - FFS for other device types

- For the evaluation of mTRP CJT scenario, NR Rel-18 EVM for mTRP CJT scheme can be baseline

- To reflect realistic assumption for both UE and BS,

  - Consider channel and interference estimation error, especially for UL evaluation with large number of antenna ports

- Simulation assumptions in Table 3.1-1 and Table 3.1-2 for MIMO features in around 7GHz

- Simulation assumptions in Table 3.1-3 for MIMO features in around 30GHz

- Simulation assumptions in Table 3.1-4 and Table 3.1-5 for MIMO features in FR1

Table 3.1-1: System-level assumptions related to around 7GHz evaluation

|  | UMa | InH | RMa |
| --- | --- | --- | --- |
| Deployment | 21 cells (7 sites w/ 3 sectors) | 12 cells (12 sites w/ 1 sector) | 21 cells (7 sites w/ 3 sectors) |
| ISD | 200 | 20 | 1732 |
| # of UEs per sector | 10, [30] | 10, [30] | 10 |
| Carrier freq. [GHz] | 7 | 7 | 7 |
| BW [MHz] | [20, 100, 200] | [20, 100, 200] | [20, 100, 200] |
| SCS [kHz] | 30 kHz | 30 kHz | 30 kHz |
| Indoor ratio | 80 | 100 | 50 |
| UE speed | Indoor: 3 km/h Outdoor (in car): 30 km/h | Indoor: 3 km/h | Indoor: 3 km/h Outdoor (in car): 120 km/h |
| Probability of Low loss model | 20% high loss, 80% low loss | - | 100% low loss |
| BS height [m] | 25 | 3 | 35 |
| TRP (dBm) | 44 dBm for 20 MHz 51 dBm for 100 MHz 54 dBm for 200 MHz | 24 dBm for 20 MHz 31 dBm for 100 MHz 34 dBm for 200 MHz | 49 dBm for 20 MHz 56 dBm for 100 MHz 59 dBm for 200 MHz |
| UE power class | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm |
| BS Element gain [dBi] | 8 | 5 | 8 |
| BS noise figure [dB] | 5 | 5 | 5 |
| UE Element gain [dBi] | 0 (Isotropic antenna) |  |  |
| UE noise figure [dB] | 7 | 7 | 7 |
| CSI acquisition | Codebook and SRS-based. Details should be reported by company. Note: NZP CSIRS and SRS overhead should be accounted in evaluations. |  |  |
| # of layers per UE | DL: up to 4 layers, UL: up to 1 layer for handheld FFS for CPE |  |  |
| BS Antenna Configuration | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (8,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.5)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ |
| UE Antenna Configuration | 1T4R: (M,N,P,Mp,Np) = (1,2,2,1,2); 4 ports, (dH,dV) = (0.5,0.5)λ |  |  |
| Mechanical tilt | 12 deg | 90 deg | 1.5 deg |
| Scheduling and precoding | The scheduling and precoding algorithms including frequency domain granularity of precoder should be reported by company |  |  |
| Max. MU layer @BS | The maximum number of MU-MIMO layers should be reported by company |  |  |
| Traffic Model | 1) Full buffer 2) Non-full buffer (FTP traffic model 3, S=0.5Mbytes) |  |  |
| Open loop UL power control parameters | Open-loop with fractional path loss compensation. The parameters should be reported by company. |  |  |
| DMRS / CSIRS channel estimation error modelling | Realistic model, e.g., Hl=βHl+∆Hl, ∆Hl∈Nc0,α∙σni2 The parameter α should be reported by company. |  |  |
| Inter-cell interference estimation modelling | Realistic model, e.g., Wishart distribution-based model. The parameters of the model should be reported by company. |  |  |
| UL overhead | The actual overhead can be also reported by company. |  |  |
| DL overhead | The actual overhead can be also reported by company. |  |  |

Table 3.1-2: Link-level assumptions related to DMRS and CSIRS evaluations for around 7GHz

| Parameter | Value |
| --- | --- |
| Subcarrier spacing | 30 kHz as baseline |
| Carrier freq. [GHz] | 7 |
| Bandwidth | 200 MHz as baseline |
| FFT size | 8K as baseline |
| TDD configuration | 4:1 (DDDFU); 6 DL symbols and 4 UL symbols in F slots |
| MCS | Up to 256QAM (MCS index table 2 based) Fixed and the same for all UEs; MCS index = 4, 10, 19, 27 (Mandatory), other MCS indices (Optional) |
| DL precoding | (Default) SRS-based precoding (Default) Codebook based precoding (Optional) Ideal CSI-based precoding Zero-forcing or SLNR as baseline for MU-MIMO |
| Channel model | CDL in TR 38.901 |
| Mean angle shift for MU | Option 1: Equally and uniformly distributed AODs/ZODs among UEs with range of AODs = [-60(Optional)line metric for CSI feedback overhead, an Option 2: Randomly distributed with range of AODs = [-60º, 60º] and Range of ZODs = [90º, 135º] Option 3: DFT-based distributed; LOS rays in Azimuth or Zenith of each UE are orthogonally shifted based on DFT directions - AOD shift: ϕ=sin-1λΔHsinθ1NHn+l for ∀n∈ 0, 1, …,NH-1 - ZOD shift: θ=cos-1λΔV1NVm+k,for ∀m∈0, 1, …,NV-1,∀k∈Z - ΔH= Spacing between ports in horizontal direction, NH= # of ports in horizontal direction, ΔV= Spacing between ports in vertical direction, NV= # of ports in vertical direction |
| UE speed | 3 km/h (Mandatory), 30 km/h (Optional) |
| Delay spread | 10, 30, 100, 300 ns |
| BS port configuration | (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ |
| UE port configuration | 1T4R: (M,N,P,Mp,Np) = (1,2,2,1,2); 4 ports, (dH,dV) = (0.5,0.5)λ |
| FDRA for PDSCH | 10 PRBs, 546 RBs (Wideband) |
| SRS | - Full-band SRS over 546 RBs as baseline - (Default) 1 OFDM symbol, 13th symbol in F slots - (Optional) 2 OFDM symbols, 11th and 13th symbols in F slots - Periodicity = 20 slots (Mandatory), 10 and 40 slots (Optional) - Transmission Comb (kTc) = 2 or 4 - Note: SRS resources for different UEs are not overlapped by using different comb offset values or different slots |
| DMRS | - (Default) DMRS configuration enhanced type 1, 2-symbol, Additional DMRS (optional), - (Default) DMRS configuration enhanced type 2, 2-symbol (front-loaded), Additional DMRS (optional), - (Optional) New DMRS configurations having increased # of orthogonal DMRS ports |
| MIMO receiver for PDSCH | MMSE-IRC |
| Channel estimation for DM-RS | 2D-MMSE |
| Channel estimation for SRS | MMSE |
| Channel estimation for CSIRS | OCC de-spreading with optional MMSE compensation |
| Link adaptation / HARQ | No link adaptation and no HARQ |

Table 3.1-3: System-level assumptions related to FR2 evaluation

|  | UMi (Mobile) | UMi (FWA) |
| --- | --- | --- |
| Deployment | 21 cells (7 sites w/ 3 sectors) | 21 cells (7 sites w/ 3 sectors) |
| ISD | 200 | 200 |
| # of UEs per sector | 10 | 10 |
| Carrier freq. [GHz] | 28 | 28 |
| BW [MHz] | 800 | 800 |
| SCS [kHz] | 120 kHz for 800 MHz | 120 kHz for 800 MHz |
| Indoor ratio | 80 | 0 |
| UE speed | Indoor: 3 km/h Outdoor (in car): 30 km/h | 1km/h (for moving environment modelling) |
| Probability of Low loss model | 20% high loss, 80% low loss | 20% high loss, 80% low loss |
| BS height [m] | 15 | 15 |
| TRP (dBm) | 32.6 dBm for 800 or 400 MHz BW 29.6 dBm for 200 MHz BW 26.6 dBm for 100 MHz BW | 32.6 dBm for 800 or 400 MHz BW 29.6 dBm for 200 MHz BW 26.6 dBm for 100 MHz BW |
| UE power class | 23 dBm | 1) 35 dBm (55 dBm Max EIRP) 2) 23 dBm (43 dBm Max EIRP) |
| BS Element gain [dBi] | 5.6 | 5.6 |
| BS noise figure [dB] | 7 | 7 |
| UE Element gain [dBi] | 4.5 | 4.5 |
| UE noise figure [dB] | 13 | 10 |
| CSI acquisition | Precoded CSIRS based | Precoded CSIRS based |
| # of layers per UE | DL: up to 2 layers, UL: up to 2 layers | DL: up to 2 layers, UL: up to 2 layers |
| BS Antenna Configuration | (M,N,P,Mg,Ng; Mp, Np) = (24,16,2,1,1; 1, 1); (dH,dV) =(0.5, 0.8)λ | (M,N,P,Mg,Ng; Mp, Np) = (24,16,2,1,1; 1, 1); (dH,dV) = (0.5, 0.8)λ |
| UE Antenna Configuration | (M,N,P,Mg,Ng; Mp, Np) = (4,1,2,1,1; 1, 1); (dH,dV) =(0.5, 0.5)λ | 1) (M,N,P,Mg,Ng; Mp, Np) = (8,8,2,1,1; 1, 1) ; or (M,N,P,Mg,Ng; Mp, Np) = (4,4,2,1,1; 1, 1) ; 2) (M,N,P,Mg,Ng; Mp, Np) = (4,1,2,1,1; 1, 1); (dH,dV) =(0.5, 0.5)λ |
| Mechanical tilt | 12 deg |  |
| Scheduling and precoding | The scheduling and precoding algorithms including frequency domain granularity of precoder should be reported by company |  |
| Max. MU layer @BS | DL: up to 2 layers, UL: up to 2 layers |  |
| Traffic Model | 1) Full buffer 2) Non-full buffer (FTP traffic model 1 or 3, S=0.5Mbytes) |  |
| Open loop UL power control parameters | Open-loop with fractional path loss compensation. The parameters should be reported by company. |  |
| DMRS / CSIRS channel estimation error modelling | Realistic model, e.g., Hl=βHl+∆Hl, ∆Hl∈Nc0,α∙σni2 The parameter α should be reported by company. |  |
| Inter-cell interference estimation modelling | Realistic model, e.g., Wishart distribution-based model. The parameters of the model should be reported by company. |  |
| UL overhead | The actual overhead can be also reported by company. |  |
| DL overhead | The actual overhead can be also reported by company. |  |

Table 3.1-4: System-level assumptions related to FR1 non-CJT evaluation

|  | UMa / UMi | InH | RMa |
| --- | --- | --- | --- |
| Deployment | 21 cells (7 sites w/ 3 sectors), 57 cells (19 sites w/ 3 sectors) | 12 cells (12 sites w/ 1 sector) | 21 cells (7 sites w/ 3 sectors) 57 cells (19 sites w/ 3 sectors) |
| ISD | 200, 500 | 20 | 1732 |
| # of UEs per sector | 10, 30 | 10, [30] | 10, 30 |
| Carrier freq. [GHz] | 2, 4 | 4 | 0.7, 2, 4 |
| BW [MHz] | 10, 20, 100 | [20, 100] | 10, 20, 100 |
| SCS [kHz] | 15 / 30 kHz | 30 kHz | 15 / 30 kHz |
| Indoor ratio | 80 | 100 | 50 |
| UE speed | Indoor: 3 km/h Outdoor (in car): 30 km/h | Indoor: 3 km/h | Indoor: 3 km/h Outdoor (in car): 120 km/h |
| Probability of Low loss model | 20% high loss, 80% low loss | - | 100% low loss |
| BS height [m] | 25 | 3 | 35 |
| TRP (dBm) | 41 dBm for 10 MHz 44 dBm for 20 MHz 51 dBm for 100 MHz | 24 dBm for 20 MHz 31 dBm for 100 MHz | 46 dBm for 10 MHz 49 dBm for 20 MHz 56 dBm for 100 MHz |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| BS Element gain [dBi] | 8 | 5 | 8 |
| BS noise figure [dB] | 5 | 5 | 5 |
| UE Element gain [dBi] | 0 (Isotropic antenna) |  |  |
| UE noise figure [dB] | 7 | 7 | 7 |
| CSI acquisition | Codebook and SRS-based. Details should be reported by company. Note: NZP CSIRS and SRS overhead should be accounted in evaluations. |  |  |
| # of layers per UE | DL: up to 4 layers, UL: up to 1 layer for handheld |  |  |
| BS Antenna Configuration | Around 2 GHz and 4 GHz: (M,N,P,Mp,Np) = (12,8,2,4,8); (M,N,P,Mp,Np) = (4,16,2,2,16); (M,N,P,Mp,Np) = (8,8,2,8,4); (M,N,P,Mp,Np) = (16,4,2,8,4); 64 ports; (dH,dV) = (0.5,0.8)λ Other port configs are not precluded | (M,N,P,Mp,Np) = (4,8,2,4,8); 64 ports; (dH,dV) = (0.5,0.5)λ | Around 700 MHz: (M,N,P,Mp,Np) = (2,2,2,2,2); 8 ports; (dH,dV) = (0.5,0.8)λ Around 2 GHz: (M,N,P,Mp,Np) = (4,4,2,2,4); 16 ports; (dH,dV) = (0.5,0.8)λ Around 4 GHz: (M,N,P,Mp,Np) = (12,8,2,4,8); (M,N,P,Mp,Np) = (4,16,2,2,16); (M,N,P,Mp,Np) = (8,8,2,8,4); (M,N,P,Mp,Np) = (16,4,2,8,4); 64 ports; (dH,dV) = (0.5,0.8)λ |
| UE Antenna Configuration | 1T4R: (M,N,P,Mp,Np) = (1,2,2,1,2); 4 ports, (dH,dV) = (0.5,0.5)λ for 4 GHz 1T2R: (M,N,P,Mp,Np) = (1,1,2,1,1); 2 ports, (dH,dV) = (0.5,0.5)λ for 0.7 GHz and 2 GHz |  |  |
| Mechanical tilt | 12 deg | 90 deg | 1.5 deg |
| Scheduling and precoding | The scheduling and precoding algorithms including frequency domain granularity of precoder should be reported by company |  |  |
| Max. MU layer @BS | The maximum number of MU-MIMO layers should be reported by company |  |  |
| Traffic Model | 1) Full buffer 2) Non-full buffer (FTP traffic model 1 or 3, S=0.5Mbytes) |  |  |
| Open loop UL power control parameters | Open-loop with fractional path loss compensation. The parameters should be reported by company. |  |  |
| DMRS / CSIRS channel estimation modelling | Realistic model, e.g., Hl=βHl+∆Hl, ∆Hl∈Nc0,α∙σni2 The parameter α should be reported by company. |  |  |
| Inter-cell interference estimation modelling | Realistic model, e.g., Wishart distribution-based model. The parameters of the model should be reported by company. |  |  |
| UL overhead | The actual overhead can be also reported by company. |  |  |
| DL overhead | The actual overhead can be also reported by company. |  |  |

Table 3.1-5: System-level assumptions related to FR1 CJT (mTRP) evaluation (based on Rel-18 MIMO CJT EVM)

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Scenario |  | Layout1: - intra-cell (TRPs within each cell) - RMa - N_TRP (#TRPs): 2, 3, 4 Layout2: - intra-site inter-cell (3 TRPs per cell/site) - Dense urban Layout3: - inter-site inter-cell (3 TRPs per cell/site) - Dense urban. |
| Frequency Range |  | FR1 only, 700 MHz intra-cell, 2 GHz inter-cell |
| Inter-BS distance |  | 1.7km (700 MHz); 200m (2 GHz); |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 700MHz - 4 ports: (M,N,P,Mg,Ng, Mp,Np) = (1,2,2,1,1,1,2), (dH, dV) = (0.5, 0.8)λ - 8 ports: (M,N,P,Mg,Ng, Mp,Np) = (2,2,2,1,1,2,2), (dH, dV) = (0.5, 0.8)λ Total #ports = N_TRP x {4, 8} 2GHz: - 8 ports: (M,N,P,Mg,Ng, Mp,Np) = (4,4,2,1,1,1,4), (dH, dV) = (0.5, 0.8)λ - 16 ports: (M,N,P,Mg,Ng, Mp,Np) = (4,4,2,1,1,2,4), (dH, dV) = (0.5, 0.8)λ - 32 ports: (M,N,P,Mg,Ng, Mp,Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ Total #ports = N_TRP(=3) x {16} |
| Antenna setup and port layouts at UE |  | 2RX: (M,N,P,Mg,Ng, Mp,Np) = (1,1,2,1,1,1,1), (dH, dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | RMa (intra-cell scenario, 700 MHz): - 46 dBm for 10 MHz, 49 dBm for 20 MHz - (normalized by N TRP, i.e., distributed across TRPs) Dense Urban (inter-cell scenario, 2 GHz): - 41 dBm for 10 MHz, 44 dBm for 20 MHz, 47 dBm for 40 MHz - (not normalized) |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | Follow TR 36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256-QAM |
| Coding on PDSCH |  | LDPC Max code-block size = 8448 bits |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15 kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 20 MHz for 15 kHz as a baseline (optional for 10 MHz with 15 kHz) |
| Frame structure |  | Slot format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g., 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme . CSI feedback periodicity (full CSI feedback) : 5 ms, . Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 or 3 |
| Traffic load (Resource utilization) |  | 50/70% for SU/MU-MIMO with rank adaptation 20% for SU-MIMO with rank adaptation Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback) for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Intra-cell scenario - Single-TRP (N=1) with Rel-16 eTypeII CB Inter-cell scenario - Single-TRP with Rel-16 eType-II CB (no collaboration across the sectors in each site) |

  - 3.2 Assumptions related to ISAC

ISAC enables a new sensing capability over cellular network, which requires new performance metrics for evaluation. The metrics at least include, sensing accuracy, detection probability and sensing resolution. The first two metrics can be evaluated via link-level simulation, and the last one, which is related to the ability of identifying multiple objects as separate objects, can be theoretically analyzed if necessary. For a realistic evaluation, potential interference should also be considered, e.g., self-interference from Tx to sensing Rx in case of simultaneous communication and sensing Rx, co-site interfere-section interference and inter-site interference. In addition, the evaluation should be conducted per sensing modes and targeting use cases, while there are many sensing modes and uses cases identified in previous study in TR 22.837. Therefore, it is desirable to select most interested use cases and sensing modes considering the time budget and efforts for ISAC evaluations in Rel-20 6GR SI.

Proposal #4:

- Sensing modes: prioritize TRP monostatic and TRP-TRP bistatic.

- OFDM-based sensing waveform is considered as baseline. Other waveforms, if considered, should be fairly compared with the baseline.

- Multiplexing scheme of sensing and communication resources including the details of sensing signal and overhead should be reported.

- Both FR1 and FR2 should be evaluated, and up to max. channel bandwidth can be assumed.

  - At least the following performance metrics should be evaluated and provided for comparison among candidate schemes. And the receiver methods related to sensing performance improvement should be reported.

    - Range/Velocity/Angle accuracy

    - Detection probability including miss detection rate and false alarm rate

    - Sensing coverage (e.g., achievable ranging in meter with -95% confidence)

- Rel-19 ISAC channel model can be considered as the baseline evaluation model.

- Interference modelling

  - In case of simultaneous communication TX and sensing RX, modeling of self-interference from TX to sensing RX is up to company report.

  - Inter-cell and inter-site interference are to be considered

- Sensing types: UAV, AGV, human and automotive

  - Note: UAV sensing target in outdoor scenario and AGV sensing target in indoor scenario can be considered as representative use cases for evaluations and simulation assumptions are provided as follows as examples.

  - Simulation assumptions in Table 3.2-1 for UAV sensing targets (outdoor scenario)

  - Simulation assumptions in Table 3.2-2 for AGV sensing targets (indoor scenario)

Table 3.2-1 UAV sensing targets (outdoor)

| Assumptions | Value |
| --- | --- |
| Target type | 0.3m x 0.4m x 0.2m (UAV of small size) |
| RCS for each scattering point | Component A: -12.81 dB Component B1: 0 dB Component B2: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS |
| Absolute delay | The model of UMa scenario defined in Table 7.6.9-1 (TR 38.901) is reused for UMa-AV for all sensing modes. |
| Fast fading model | TR 36.777 Annex B.1.3 |
| (u, std) for XPR of target | Mean 13.75 dB, deviation 7.07 dB |
| BS Rx saturation power | [-30 dBm] |
| Antenna isolation (in terms of TRP) | FR1: [65-80] dB FR2: [90-100] dB |
| BS Tx power | FR1: 49 dBm TRP FR2: 30 dBm TRP |
| BS noise figure | FR1: 5 dB FR2: 7 dB |
| BS antenna model | For around 4 GHz, (M, N, P, Mg, Ng, Mp, Np) = (12, 8, 2, 1, 1, 4, 8) For around 30 GHz, (M, N, P, Mg, Ng, Mp, Np) = (16, 16, 2, 1, 1, 1, 1) |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |

Table 3.2-2 AGV sensing targets

| Assumptions | Value |
| --- | --- |
| Scenario | InF: 18 TRPs per the big hall: L = 300 m x W = 150 m - X-axis is pointing down to the floor - The antenna array is mounted in the Y-Z plane with boresight along the X-axis - The X-axis/Y-axis/Z-axis refer to LCS - 8 m for high BS scenario |
| Target type | 0.5m x 1.0m x 0.5m (Option 1) |
| RCS for each scattering point | The values/pattern of component A*B1 are generated by Table 7.9.2.1-6 (TR 38.901) The same values are used for monostatic RCS and bistatic RCS |
| Absolute delay | The model of InF scenario defined in Table 7.6.9-1 (TR 38.901) is reused for all sensing modes. |
| Fast fading model | For BS to target link: Follow the procedure defined in TR38.901 |
| (u, std) for XPR of target | Mean 9.60 dB, deviation 6.85 dB |
| BS Rx saturation power | -30 dBm |
| Antenna isolation (in term of TRP) | FR1: [65-80] dB FR2: [90-100] dB |
| BS Tx power | FR1: 24 dBm TRP FR2: 26 dBm TRP |
| BS antenna model | For around 4 GHz, (M, N, P, Mg, Ng, Mp, Np) = (12, 8, 2, 1, 1, 4, 8) For around 30 GHz, (M, N, P, Mg, Ng, Mp, Np) = (8, 8, 2, 1, 1, 1, 1) |
| Sensing target distribution | 100% indoor, 1 target uniformly distributed (across multiple drops) over the convex hull of the TRP deployment |

Evaluation models

  - 4.1 Traffic models

The existing 3GPP traffic models – such as full-buffer for sustained maximum-load testing, FTP3 for burst-plus-idle transfer patterns, FTP3-IM for burst transfers with intermittent pauses, and VoIP for periodic low-bitrate transmissions with silence suppression – already provide a sufficiently realistic representation of real-world traffic from the perspective of the physical interface. These models inherently define the two key PHY-relevant aspects: the temporal activity profile (when transmissions start, stop, or repeat periodically) and the corresponding transport block sizes per transmission opportunity. For example, the ON/OFF structure of FTP3 closely mimics typical file download or web browsing behavior, where data arrives in bursts followed by reading or idle time; FTP3-IM reflects short-form video or gaming traffic where brief pauses occur even within an active burst; and the VoIP model’s 20ms packet periodicity with silence suppression mirrors the scheduling and discontinuous transmission (DTX) behavior of real voice calls. By accurately reproducing the time-varying load patterns and burstiness that the PHY actually experiences, these models enable reproducible and practically representative PHY performance evaluations across different simulation setups.

Proposal #5:

- Use conventional traffic models (FTP1, FTP3, FTP3-IM, and VOIP) as baseline for 6GR evaluations

It should be noted that the existing traffic models used in RAN1 for SLS evaluations, such as FTP Models 1 and 3, assume fixed packet sizes, typically relatively large, e.g., 0.5MBytes. While this simplifies analysis, it does not accurately reflect real-world traffic characteristics. In practice, packet sizes vary, and the current approach may therefore misrepresent performance of the system in the scenarios dominated by small and medium packet services.

Figure 1 illustrates an example of packet size distribution obtained from a commercial network traffic logs, where a large fraction of small and medium packets is observed, while larger packet transmissions occur less frequently.

Figure 1: Example of packet size distribution from real-life NW traffic logs

Observation:

- Packets with variable sizes are observed from real traffic logs, with small and medium packets occurring much more frequently than larger ones

Based on this observation it seems natural to consider new bursty traffic models for 6GR evaluation that incorporates the variable packet sizes. More specifically, to better reflect realistic traffic patterns while maintaining manageable simulation complexity, the FTP-based traffic modelling framework could be extended to support variable packet sizes with different packet arrival rates reflecting different probability of the packet sizes.

Proposal #6:

- In addition to conventional traffic models, consider optional 6GR evaluation with non-full buffer traffic models supporting variable packet sizes using FTP modeling framework

For that purpose, multiple reference packet sizes, e.g., corresponding to small, medium and large packet sizes with size of SS,SM,SL, can be introduced each with an associated packet arrival rate λS,λM,λL reflecting different packet loading scenario. The traffic profile for simulation should be properly selected to capture the observation from Figure 1 that small and medium packets occur more frequently then packets of larger sizes, i.e., λS>λM>λL. The actual values of the packet arrival profile can be scaled up or down in SLS simulation to reflect variable traffic load, but their relative ratio should be remaining constant, i.e., λS,λM,λL=∝rS,rM,rL, where rS,rM,rL is reference packet arrival rate ratio, which can be set to 4,8,1 according to distribution observed in Figure 1.

Proposal #7:

- For traffic model supporting variable packet sizes, define three reference packet sizes SS,SM,SL corresponding to small, medium and large packet sizes along with associated packet arrival rates λS,λM,λL

  - The proportion of the packet arrival rates λS,λM,λL should be maintained to ensure that smaller packets are occurring much more frequently than larger ones

The reference packet sizes along with their arrival rate profile can be obtained from the real traffic logs, e.g., can be set to 0.3,20,150kB according to Figure 1. However, due to limited time alignment of reference packet sizes among companies may be a complex process within RAN1. As an alternative approach, reference packet sizes could be defined based on scheduling considerations. In particular, from a scheduling perspective, packet sizes SS,SM,SL can be broadly classified into three categories:

- Small packets: Transmittable within a single RBG.

- Medium packets: Requiring multiple RBGs but fitting within a single slot.

- Large packets: Requiring multiple RBGs across multiple slots.

This classification can serve as a practical basis for defining three reference packet sizes to be used in SLS simulations.

Proposal #8:

- For traffic model supporting variable packet sizes, consider scheduling aspects to determine reference packet sizes SS,SM,SL

The next aspect of non-full buffer traffic modelling is how to assign packets to different UEs during simulations. The simplest approach is to conduct multiple independent SLS evaluations, each with a single packet size, similar to current FTP traffic modelling. While this approach minimizes simulation complexity, it does not capture scenarios with mixed packet sizes. A more realistic alternative would be to perform joint SLS evaluations using different packet sizes SS,SM,SL within a single simulation drop. In this approach two sub-options can be considered:

- Per-UE fixed packet size: Each UE is assigned packets of a single size, but different UEs may have different packet sizes.

- Per-UE variable packet size: Each UE may receive packets of varying sizes over time.

Proposal #9:

- For traffic model supporting variable packet sizes, consider the following options of packet assignment to the UE for joint SLS evaluation with different packet sizes:

- Each UE can receive the packet of only one size from the reference packet sizes

- Each UE can receive packet of different sizes from reference packet sizes

Both approaches have a similar impact on scheduling operations at the BS, with the main difference lying in the post-processing of the SLS results. Specifically, in the first approach, UE throughput should be reported separately for different UE groups corresponding to each reference packet size. In the second approach, throughput is reported jointly for all UEs. These aspects, however, can be discussed in more detail later, once the basic principles of non-full-buffer traffic modeling with variable packet sizes are established.

  - 4.2 Antenna models

In 6GR, it will be important to specify evaluation assumptions to verify the performance of new 6G schemes in FR1 and FR2. This is because FR1 can be used for coverage or high spectral efficiency via mTRP CJT and FR2 can be used to increase capacity further as in NR. In 6GR, newly proposed schemes to overcome pain points in NR can be introduced after verifying the performance of proposed schemes. For evaluation in FR1 and FR2, the antenna configurations which were used in NR and commercialized BS and UE in real field should be adopted as baseline.

The antenna technology has developed significantly so far. In particular, BSs that can achieve further beamforming gain by increasing the number of AEs (antenna elements) have been commercialized. However, for practical handheld devices, there are constraints on antenna placement due to limited size of form-factor which is caused by portability and weight. Nevertheless, antenna placement technology has advanced, and commercial handheld devices which enable to receive up to 4 layers for DL are widely used. On the other hand, while high-end handheld devices now support up to 2 TX, 1Tx handhelds which are equipped with one high-power PA remain practical. Therefore, 1TX should be a baseline assumption for handheld devices.

Accurate performance evaluation of 6GR necessitates clearly defined antenna assumptions to ensure meaningful comparisons across frequency bands, particularly when assessing coverage equivalence between the mid-band (e.g., 3.5 GHz) and FR3 spectrum (as mentioned in SID). Baseline evaluations should consider a BS architecture that selects both the number of horizontal and vertical antenna element, alongside sub-array structures defining antenna port configurations – representative examples include 192 elements for 4 GHz and 768 elements for 7 GHz scenarios for the total number of antenna elements. With the increased number of antenna elements for 7 GHz scenarios, more antenna ports, i.e., 256 enabling more precoding gain as well as the same number of antenna ports as NR Rel-19, i.e. 128 can be supported. Therefore, the antenna configurations for BS and UE in Table 4.2-1 can be supported as a baseline for antenna models in 7 GHz.

Proposal #10:

- For evaluation in FR1 and FR2, the antenna configurations which were used in NR and commercialized BS and UE in real field should be adopted as baseline.

- For handheld antenna configuration, 1Tx should be a baseline assumption.

- To assess coverage equivalence between the mid-band (e.g., 3.5 GHz) and FR3 spectrum (as mentioned in SID), the antenna configurations for BS and UE in Table 4.2-1 can be supported as baseline assumptions for antenna models in 7 GHz

Table 4.2-1 Parameters for antenna models in 7 GHz

|  | UMa | InH | RMa |
| --- | --- | --- | --- |
| SCS [kHz] | 30 kHz for 20, 100 MHz, 200 MHz | 30 kHz for 20, 100 MHz, 200 MHz | 30 kHz for 20, 100 MHz, 200 MHz |
| Probability of Low loss model | 20% high loss, 80% low loss | - | 100% low loss |
| BS height [m] | 25 | 3 | 35 |
| TRP (dBm) | 44 dBm for 20 MHz 51 dBm for 100 MHz 54 dBm for 200 MHz | 24 dBm for 20 MHz 31 dBm for 100 MHz 34 dBm for 200 MHz | 49 dBm for 20 MHz 56 dBm for 100 MHz 59 dBm for 200 MHz |
| UE power class | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm | 1) 23 dBm; 2) 26 dBm |
| BS Element gain [dBi] | 8 | 5 | 8 |
| BS noise figure [dB] | 5 | 5 | 5 |
| UE Element gain [dBi] | 0 (Isotropic antenna) |  |  |
| UE noise figure [dB] | 7 | 7 | 7 |
| BS Antenna Configuration | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (8,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.5)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ | 1) (M,N,P,Mp,Np) = (24,16,2,8,16); 256 ports; (dH,dV) = (0.5,0.8)λ 2) (M,N,P,Mp,Np) = (24,16,2,4,16); 128 ports; (dH,dV) = (0.5,0.8)λ |
| UE Antenna Configuration | 1T4R: (M,N,P,Mp,Np) = (1,2,2,1,2); 4 ports, (dH,dV) = (0.5,0.5)λ FFS for other structures |  |  |
| Mechanical tilt | 12 deg | 90 deg | 1.5 deg |

  - 4.3 RF model

Nonlinearity, in particular from transmitter’s power amplifier (PA), is a long-held problem in reality, which is expected to better addressed in 6GR to further improve the link performance and/or energy efficiency. To evaluate the effectiveness of proposed techniques related to nonlinearity, e.g., AI based nonlinearity compensation / equalization, PA models in TR 38.803 can be considered in link-level simulations, e.g., generalized memory-based polynomial (GMP) model, possibly in conjunction with modelling of other non-ideal factors including I/Q imbalance, fractional timing error and so on. The need of such models is subject to the proposed techniques for handling RF impairment in RAN1.

In OFDM-based systems, I/Q imbalance caused by amplitude mismatches and phase offsets between the in-phase and quadrature channels in transceivers distorts the received signal, degrades performance metrics like EVM and BLER. This issue grows significantly more severe with higher-order modulation schemes (e.g., 256 QAM and higher order). As high order modulation scheme is sensitive to I/Q imbalance, it is suggested to consider an I/Q imbalance model for 6GR evaluation.

The following I/Q imbalance model [1] can be considered.

Let st=sIt+jsQt and sLO,It=cos2πf0t denote perfect baseband signal and perfect local oscillator in-phase output, respectively.

Then perfect I/Q modulation can be written as follows

rIQ,idealt=sItcos2πf0t+sQtcos2πf0t+π2=sItcos2πf0t-sQtsin2πf0t,

Let gI and gQ denote the amplitude impairment of in-phase reference signal and quadrature reference signal, respectively. Furthermore, define phase impairment of in-phase reference signal and quadrature reference signal as θI and θQ. With such distorted reference signals, I/Q modulation output is given by:

rIQt=gI∙sItcos2πf0t+θI-gQ∙sQtsin2πf0t+θQ,

Corresponding baseband equivalent signal of the in-phase part and quadrature part of rIQt can be expressed as follows:

rI,bbt=LPFrIQtcos2πf0t=gI∙sItcosθI-gQ∙sQtsinθQ,

rQ,bbt=LPF-rIQtsin2πf0t=gI∙sItsinθI+gQ∙sQtcosθQ,

Then the baseband equivalent signal of rIQt is given by:

rIQ,bbt=rI,bbt+jrQ,bbt

=gI∙sItcosθI-gQ∙sQtsinθQ+jgI∙sItsinθI+gQ∙sQtcosθQ

=β∙st+γ∙s*t,

where β=12gIejθI+gQejθQ and γ=12gIejθI-gQejθQ  represent the coefficients of st and its mirrored component s*t, respectively.

Imbalance between the in-phase and quadrature components of signal output from RF receivers can be cost-effectively compensated rather than improving the analog front-end RF hardware. If gI≈gQ and θI≈θQ after compensation, β≈1 and will be significantly greater than γ. In such case, mirrored component s*t can be ignored and I/Q imbalance can be approximated as a slight amplitude scaling and a slight phase rotation.

In general, it is enough to model I/Q imbalance with the above frequency-independent model. If imperfect RF frequency response in in-phase and quadrature brunches are further considered, the two multiplication operations in rIQ,bbt=β∙st+γ∙s*t should be replaced with convolutions:

rIQ,bbt=βt*st+γt*s*t,

where βt and γt are the inverse Fourier transform of the imperfect frequency response in in-phase and quadrature brunches, respectively.

βt=IFT12HIf∙gIejθI+HQf∙gQejθQ,

γt=IFT12HIf∙gIejθI-HQf∙gQejθQ

Given the RF distortion in practice can be caused by PA nonlinearity and I/Q imbalance typically, we suggest to jointly consider both factors when RF modelling is desired for evaluating 6G new technologies carefully.

Proposal #11:

- Consider a PA model (e.g., GMP) in TR 38.803 and an I/Q imbalance model for the evaluation of technologies highly related to RF impairment aspects.

  - 4.4 Interference estimation model

The MMSE-IRC receiver is widely used for performance evaluation in NR systems and is also implemented in commercial modems. This receiver requires knowledge of both the channel and the interference-plus-noise covariance matrix. While channel estimation modeling is typically included in SLS evaluations, interference estimation is often assumed to be perfect, which can lead to overly optimistic results.

According to 3GPP TR 36.829, even with a small number of receive antennas (e.g., 2), estimating a 2×2 interference-plus-noise covariance matrix has a notable impact on cell-edge performance. In the context of 6GR especially uplink evaluations, where the BS receiver may have up to 256 antenna ports, the impact becomes even more significant due to the increased dimensionality of the covariance matrix (e.g., up to 256×256). Consequently, 6GR performance estimates (especially uplink) based on ideal interference assumptions for demodulation purposes are unrealistic and may lead to misleading conclusions.

More specifically, the interference samples ym are typically obtained from DMRS REs by subtracting the re-constructed signal hmsmDMRS and the received DMRS rm according to

ym=rm-hmsmDMRS,

where hm is estimated from DMRS channel of dimension N×1 and smDMRS is DMRS signal. The interference covariance matrix R is then computed by averaging the outer products of the interference samples ym over one or several resource blocks:

R=1Mm=1MymymH.

Due to the limited number M of DMRS REs available for R estimation compared to matrix dimension (i.e., N>M), the resulting estimate of R can be highly inaccurate. For example, it may not be positive definite, necessitating the use of diagonal loading schemes to enable R matrix inversion in MMSE-IRC processing. Such regularization further reduces the interference rejection capability compared to the case of ideal interference covariance knowledge. This effect should be reflected in simulations to produce more realistic 6GR performance evaluations.

Therefore, it is recommended to apply a more realistic interference-plus-noise estimation model, as described in 3GPP TR 36.829, for 6GR analysis in SLS:

A spatial interference-plus-noise covariance matrix can be written as follows

R=1Mm=1MymymH.

where M is the number of data samples used in order to estimate the covariance matrix

It can be shown that the sample correlation matrix can be approximated using the complex Wishart distribution with M degrees of freedom

m=1MymymH≈WpR,M,

where R is the ideal covariance matrix. Correspondingly, it can be modelled in the system simulator as

R≈QAAHQH,

where Q=chol{R} is created by the Cholesky decomposition of the ideal spatial correlation matrix R.

The lower-triangular matrix A is generated according to the complex Wishart distribution as

A=c1/200n21c2/20n31n32c3/2⋯⋯⋯000⋮            ⋮            ⋮⋱⋮ nN1      nN2      nN3⋯cN/2,

where the coefficients ci follow a Chi-square distribution, i.e. ci ~ χ2 (2*(M – i +1)), and nij ~ CN(0,1).

Proposal #12:

- Use realistic model, e.g., Wishart distribution-based model according to TR 36.829 (Note: the parameters of the model should be reported by company)

Conclusion

Based on discussion, we suggest to consider the following proposals for 6GR evaluation assumptions.

Proposal #1:

- If there are too diversified options in the list of common evaluation assumptions, RAN1 strives for deciding on a minimal set of baseline assumptions, while companies report other options

Proposal #2:

- The need of realistic models in RAN1 evaluation should be discussed per evaluation purpose, and if applicable, should be prioritized to determine the performance

For MIMO,

Proposal #3:

- For BS and UE antenna configurations

  - In around 7GHz, assume 128 and 256 antenna ports at the BS.

  - In FR2, assume analog beamforming including precoded CSI-RS.

  - In FR1, evaluate both sTRP and mTRP CJT

- To evaluate the tradeoff between RS overhead and performance, sparser RS mapping should be assumed.

- For maximum SU-MIMO layers or MU-MIMO layers,

  - For handheld, supporting up to 4 layers per UE for DL and 1 layer per UE for UL

  - FFS for other device types

- For the evaluation of mTRP CJT scenario, NR Rel-18 EVM for mTRP CJT scheme can be baseline

- To reflect realistic assumption for both UE and BS,

  - Consider channel and interference estimation error, especially for UL evaluation with large number of antenna ports

- Simulation assumptions in Table 3.1-1 and Table 3.1-2 for MIMO features in around 7GHz

- Simulation assumptions in Table 3.1-3 for MIMO features in around 30GHz

- Simulation assumptions in Table 3.1-4 and Table 3.1-5 for MIMO features in FR1

For ISAC,

Proposal #4:

- Sensing modes: prioritize TRP monostatic and TRP-TRP bistatic.

- OFDM-based sensing waveform is considered as baseline. Other waveforms, if considered, should be fairly compared with the baseline.

- Multiplexing scheme of sensing and communication resources including the details of sensing signal and overhead should be reported.

- Both FR1 and FR2 should be evaluated, and up to max. channel bandwidth can be assumed.

  - At least the following performance metrics should be evaluated and provided for comparison among candidate schemes. And the receiver methods related to sensing performance improvement should be reported.

    - Range/Velocity/Angle accuracy

    - Detection probability including miss detection rate and false alarm rate

    - Sensing coverage (e.g., achievable ranging in meter with -95% confidence)

- Rel-19 ISAC channel model can be considered as the baseline evaluation model.

- Interference modelling

  - In case of simultaneous communication TX and sensing RX, modeling of self-interference from TX to sensing RX is up to company report.

  - Inter-cell and inter-site interference are to be considered

- Sensing types: UAV, AGV, human and automotive

  - Note: UAV sensing target in outdoor scenario and AGV sensing target in indoor scenario can be considered as representative use cases for evaluations and simulation assumptions are provided as follows as examples.

  - Simulation assumptions in Table 3.2-1 for UAV sensing targets (outdoor scenario)

  - Simulation assumptions in Table 3.2-2 for AGV sensing targets (indoor scenario)

For evaluation models,

Proposal #5:

- Use conventional traffic models (FTP1, FTP3, FTP3-IM, and VOIP) as baseline for 6GR evaluations

Proposal #6:

- In addition to conventional traffic models, consider optional 6GR evaluation with non-full buffer traffic models supporting variable packet sizes using FTP modeling framework

Proposal #7:

- For traffic model supporting variable packet sizes, define three reference packet sizes SS,SM,SL corresponding to small, medium and large packet sizes along with associated packet arrival rates λS,λM,λL

  - The proportion of the packet arrival rates λS,λM,λL should be maintained to ensure that smaller packets are occurring much more frequently than larger ones

Proposal #8:

- For traffic model supporting variable packet sizes, consider scheduling aspects to determine reference packet sizes SS,SM,SL

Proposal #9:

- For traffic model supporting variable packet sizes, consider the following options of packet assignment to the UE for joint SLS evaluation with different packet sizes:

- Each UE can receive the packet of only one size from the reference packet sizes

- Each UE can receive packet of different sizes from reference packet sizes

Proposal #10:

- For evaluation in FR1 and FR2, the antenna configurations which were used in NR and commercialized BS and UE in real field should be adopted as baseline.

- For handheld antenna configuration, 1Tx should be a baseline assumption.

- To assess coverage equivalence between the mid-band (e.g., 3.5 GHz) and FR3 spectrum (as mentioned in SID), the antenna configurations for BS and UE in Table 4.2-1 can be supported as baseline assumptions for antenna models in 7 GHz

Proposal #11:

- Consider a PA model (e.g., GMP) in TR 38.803 and an I/Q imbalance model for the evaluation of technologies highly related to RF impairment aspects.

Proposal #12:

- Use realistic model, e.g., Wishart distribution-based model according to TR 36.829 (Note: the parameters of the model should be reported by company)

- Reference

[1] Anttila, L., M. Valkama, and M. Renfors. "Blind compensation of frequency-selective I/Q imbalances in quadrature radio receivers: Circularity-based approach", Proc. IEEE ICASSP, pp.III-245–248, 2007.