3GPP TSG-RAN WG1 Meeting #122bis	R1-2507347

Prague, Czechia, October 13th- 17th, 2025

Agenda Item:	11.3.2

Source:	Ericsson

Title:	On 6G frame structure

Document for:	Discussion

# Introduction

In [1], a new study item for 6G radio was endorsed, and as part of the detailed objectives, the following was stated:

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

In this paper, we will discuss the numerology, duplexing and the frame structure.

# Discussion

In RAN1#122, the following was agreed:

Agreement

CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink

- Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions

- Other OFDM based waveforms are not precluded.

Agreement

CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink

- Enhancements/modifications on CP-OFDM will be studied as potential additions

- DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

The agreement will be used in the following discussion.

## Numerology and frame structure

In RAN#109, the following agreement was made:

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

The agreement contains an FFS on additional SCSs for sub 6 GHz FDD. To support MRSS, NR and 6GR should use the same SCSs, and since all FDD bands in NR use 15 kHz SCS, initial 6GR FDD deployments will also use 15 kHz SCS. Commercial UE implementations can only be tested on what is deployed, so the only SCS that can be tested in for 6GR in FDD bands is 15 kHz:

The only SCS that can be fully IODT tested for 6GR in FDD bands is 15 kHz.

Therefore, there seems to be little point in specifying additional SCSs for FDD bands:

For FDD bands below sub 6 GHz, support only 15 kHz SCS.

In NR, it is possible to configure SCS 240 kHz for SSB in FR2, and the possibility to have different SCS for SSB and PDxCH/PUxCH could exist in general. However, the benefit of this possibility is not clear to us, and since it creates additional complexity, we see no reason to support this possibility in 6GR:

SSB uses the same SCS as PDxCH/PUxCH.

For the bands where NR is not deployed, there is some flexibility to choose SCS, and the relevant options were captured in the agreement above.

The most important case is for around 7 GHz, where the options are 30 or 60 kHz. To assess performance for the different SCSs, extensive link simulations have been performed. The simulation assumptions are provided in the appendix, and here we discuss a subset of the results.

Figure 1: Performance with SCS 30 and 60kHz for 3 km/h and 64QAM for around 7GHz.

For 7 GHz and 64QAM, the performance of 30 and 60 kHz SCS are similar, as shown in Figure 1 for UE speed 3 km/h. For 64QAM and UE speed 3 km/h, the performance for 30 and 60 kHz are virtually identical: for the larger delay spread (300 ns), 30 kHz SCS has a small advantage. The corresponding result for UE speed 30 km/h is provided in Figure 2. Here 60 kHz SCS has the small advantage.

Figure 2: Performance at 7GHz with SCS 30 and 60kHz for 30 km/h and 64QAM for around 7GHz.

The results for 256QAM are shown in Figure 3 and Figure 4. The properties of the results for 256QAM remain: the results are similar, and 30 kHz performs better for the larger delay spread, whereas the 60 kHz perform slightly better for moderate speed. For 256QAM however, 30 kHz performs relatively better: the performance benefit at 300 ns delay spread is now significant, whereas for 30 km/h UE speed, the performance is only slightly worse for the smaller delay spreads and better for the larger delay spread 300 ns.

Figure 3: Performance at 7GHz with SCS 30 kHz and 60kHz for 3 km/h and 256QAM for around 7GHz.

Figure 4: Performance at 7GHz with SCS 30 kHz and 60kHz for 30 km/h and 256QAM for around 7GHz.

Finally, we will look at 1024QAM, with results in Figure 5 and Figure 6. Here we see that 60 kHz does not work at all for delay spread 300 ns. Also, for delay spread 100 ns and UE speed 30km/h, 30 kHz performs better.

Figure 5: Performance at 7GHz with SCS 30 kHz and 60kHz for 3 km/h and 1024QAM for around 7GHz.

Figure 6: Performance at 7GHz with SCS 30 kHz and 60kHz for 30 km/h and 1024QAM for around 7GHz.

Looking at the results, 30 kHz SCS is clearly better for the highest modulation order investigated and has an advantage for larger delay spread. The only case where 60 kHz SCS has an advantage is for moderate UE speed and 64QAM. Overall, the simulation results suggest that 30 kHz is the better option. Furthermore, 30 kHz SCS has a distinct advantage that it is robust with respect to non-ideal time-domain synchronization, for example in D-MIMO deployments or in carrier aggregation scenarios. In addition, carrier aggregation is simpler if all component carriers use the same numerology, and the TDD bands below 6 GHz support only SCS 30 kHz. Based on this, we propose

For around 7GHz, support (only) 30kHz SCS.

In 6GR, larger carrier bandwidths will be desirable: it would be beneficial to support 400 MHz BW using 30 kHz SCS. Therefore, we propose

Support 400 MHz CBW using 30 kHz SCS.

The bandwidth requirement in Proposal 4 implies that the carrier contains at most 13333 subcarriers. In the discussion, this is often stated as corresponding to a certain “FFT size”. To handle 13333 subcarriers with one FFT, the closest FFT size that is a power of two is 16384, or 16k for short. Note that the FFT size is essentially an implementation size: it is also possible to support 13333 subcarriers using two or more smaller FFTs. For ease of presentation, we will still use the term “FFT size”, which simply provides a measure on how many subcarriers fit in the carrier.

As explained above, 400 MHz CBW corresponds to a 16k FFT. Using the same FFT size also for sub-6GHz frequencies is one option, but it is unlikely that such large carrier bandwidths are available for sub 6GHz. Instead, we can assume that an 8k FFT is used for the sub 6GHz frequencies, leading to a maximum CBW of 100 MHz for FDD and 200 MHz for TDD. Note that the RAN1 specifications will not distinguish between these cases, since the RAN1 specifications are typically agnostic to the carrier frequency. On the other hand, the RAN4 specifications will distinguish between the FDD and TDD bands.

In NR, the maximum CBW for FR2 was 400 MHz, but this was not implemented. Instead, carrier bandwidths of 100 MHz were the main design choice. Considering this, extending the maximum CBW to 800 MHz, i.e., using an 8k FFT, seems unnecessary, and staying with 400 MHz would be appropriate.

The minimum CBW is discussed in our companion contribution [2].

The SCS and FFT properties are summarized in Table 1.

Table 1: 6GR numerologies. The minimum CBW is discussed in our companion contribution [2].

|  | FR1 FDD | FR1 TDD | Around 7GHz | FR2 |
| --- | --- | --- | --- | --- |
| Subcarrier spacing | 15 kHz | 30 kHz | 30 kHz | 120 kHz |
| FFT size | 8k | 8k | 16k | 4k |
| Maximum carrier bandwidth | 100 MHz | 200 MHz | 400 MHz | 400 MHz |
| Minimum carrier bandwidth | 3 MHz | 10 MHz | 20 MHz | 50 MHz |

In RAN1#122, the following was agreed

Agreement

- 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

  - FFS potential need for other CP

The agreement includes an FFS for other CP lengths. The extended CP specified for 60 kHz SCS was never used in NR, and we see no reason to introduce it in 6GR, at least for communication purposes. The normal CP length has been designed as a compromise between overhead and robustness with respect to delay spread, and we do not see what would make it necessary to reopen that discussion. Furthermore, MRSS would be less efficient if the CP lengths were different. Based on this, we propose

Support only the normal CP for communication purposes in 6GR.

## 6GR duplexing study

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

In RAN1 #122, the following agreement was made regarding the duplex modes.

RAN1 has agreed to consider at least FDD (including half-duplex FDD on the UE side), semi-static and dynamic TDD, and gNB semi-static SBFD for the duplex types. This provides a strong foundation for further studies. Key lessons learned from these duplex modes can be summarized as follows.

- FDD and semi-static TDD are already widely deployed and well understood. Half-duplex FDD, in particular, is important for cost-efficient, low-end IoT devices.

- Dynamic TDD is already a part of 5G specification. The concept of “flexible symbols” forms the basis for SBFD as well as more advanced duplexing schemes. For 6G, dynamic TDD remains very relevant as it is a natural intermediate step toward even more aggressive duplexing schemes such as SBFD and same-frequency full duplex. Any new duplex scheme must outperform dynamic TDD in real-world conditions to justify the added complexity. However, the dynamic SFI in 5G has shown to be complex to implement in the specification and could be simplified.

- gNB semi-static SBFD has been extensively studied in the context of 5G Advanced. While its deployment scenarios are somewhat limited and its feasibility/performance potential is yet to be proven in real-world networks, it is expected to be part of 6G specifications.

Looking ahead, more advanced duplexing schemes listed in the agreement above (e.g., gNB dynamic SBFD, UE SBFD, gNB same-frequency full duplex) could certainly be explored. However, it should be noted that the primary motivation behind SBFD and the advanced duplexing schemes is to opportunistically improve UL coverage, latency, and performance. Consequently, any evaluation of these schemes must not be conducted in isolation. A fair comparison requires benchmarking against other UL enhancement technologies already standardized in NR and beyond. Therefore, it is critical to evaluate these technologies, comparing them with other alternative solutions to fully understand their real-world applicability, potential performance benefits, and implementation feasibility. Only through a holistic evaluation framework it can be determined whether the additional complexity introduced by these schemes is justified compared to alternative UL enhancement solutions offering similar or better gains.

- gNB dynamic SBFD was studied in the Rel-18 study item for NR duplex evolution, and the following observations regarding its impact on complexity and cross-link interference (CLI) aspects were agreed [3]:

  - Dynamic SBFD may increase gNB implementation complexity due to dynamic antenna/panels switching and filters/RF tuning, may incur loss of resources due to transition time, may increase inter-gNB CLI, may increase scheduling complexity, and can result in additional specification impact on top of semi-static SBFD

  - UE implementation complexity may be increased if the UE supports dynamic SBFD and dynamic SBFD may result in increased UE-to-UE CLI.

During the Rel-18 study item, we presented system level simulation results [4] wherein dynamic SBFD was evaluated as a duplexing scheme wherein transmission direction for semi-statically configured sub-bands is overridden to support DL transmission in an UL sub-band and/or UL transmission in a DL sub-band. Our results clearly demonstrated that dynamic TDD supported by NR specifications performs equally as well or better than dynamic SBFD and has significantly lower complexity. Considering these complexity and performance aspects, our proposal is to not consider gNB dynamic SBFD in the 6GR duplexing study.

- While UE SBFD is being proposed by some companies, existing simpler alternatives should be leveraged before studying UE SBFD. For instance, a UE should support simultaneous TX and RX across frequency bands for as many band combinations as possible so that it can transmit UL on one band while receiving DL on another band to potentially improve performance. Each carrier/band can have its own TDD configuration, and different carriers across frequency bands can be aggregated with different TDD patterns and/or time-shifted TDD patterns. This can provide simultaneous DL reception and UL transmission without requiring any UE self-interference cancellation, while still enabling benefits from full duplex operation. Therefore, it can be noted that UE SBFD should only be studied if it shows clear additional gains beyond this baseline.

Summing up, for any new duplexing scheme considered for 6GR, we propose

New duplexing schemes for 6G should be studied in a holistic manner and must demonstrate clear, measurable performance gains with reasonable complexity compared to other solutions/technologies before being adopted.

## 6GR frame configuration

Like NR, 6GR will utilize a common frame structure, composed of 10 ms frames. Each such frame is split into 10 subframes, each with a duration of 1 ms. Frames and subframes are used as timing references, independent of the numerology. Frames are associated with a frame counter, and subframes are associated with a subframe counter, and these counters can be used, e.g., to initialize sequence generation.

Keep 10 ms frames, and 1 ms subframes as timing references.

Frame and subframe counters are convenient to use to identify transmissions timings on several signals. For example, the SSB periodicity is given in subframes.

NR also defines 14-symbols slots, which are used for the following purposes:

- Slots are used as numerology dependent time reference. For example, the period of some periodic signals, such as CSI-RS and PUCCH, are given in slots. Since the symbol length is known, a time duration that is expressed in slots could just as well be expressed in subframes.

- Slots are used a basis for scheduling. In Rel-15, one PxSCH allocation could start at any symbol in the slot but could not stretch across a slot boundary. On top of this, repetition was used, where the same transport block is repeated in multiple slots, using the same allocation in every slot. In NR Rel-17, the PUSCH repetition was enhanced. Also, transport block over multiple slots (TBoMS) was introduced, where one transport block is mapped across multiple slots.

- Slots are used to define duplex patterns, using so-called slot formats, which specifies, for each symbol within a slot, whether it is allocated for downlink (DL) or uplink (UL) transmission or flexible (F).

If slots are to be defined in 6GR, it is important to clarify their purpose. Having a time unit that scales with symbol length is not obviously useful. Furthermore, restricting the scheduling to operate on a slot-basis is too restrictive: increased coverage by transmitting a transport block over more than 14 symbols should be natively supported, as should transmissions that span short duration, e.g., 3 symbols.

Before defining slots in 6GR, determine what they should be used for.

It is worth noting that the NR slot structure was primarily designed to support traditional duplexing methods: FDD using paired spectrum and TDD using unpaired spectrum. In both cases, the slots are configured semi-statically, with flexible symbols enabling more dynamic TDD operation. However, the original design did not account for other duplexing methods like SBFD (which was specified in NR in Rel-19). or the potential enhancements for duplexing in 6GR.

As mentioned above, slots are also used to define slot formats, which specifies, for each symbol within a slot, whether it is allocated for downlink (DL) or uplink (UL) transmission or flexible (F). Flexible symbols were particularly specified to support dynamic TDD scenarios, where the direction of transmission can change dynamically to meet varying traffic demands.

Rel-19 SBFD symbols can be indicated using the DL and/or flexible symbols. The configuration gets more complex because for an SBFD symbol, the frequency configuration of the UL sub-band is separately configured for the SBFD-aware UEs. The SBFD-aware UE needs to interpret whether a DL or a flexible symbol is actually being used as an SBFD symbol based on whether the frequency configuration is provided for the UL sub-band and DL sub-band(s). Therefore, there is a need to flexibly configure time and frequency resources in a systematic manner in 6GR.

For 6GR, study frame format configurations that natively support flexible time-frequency configurations for traditional and advanced duplexing modes such as SBFD.

Moreover, looking forward, 6GR radio access networks will need to support a wide range of services and use cases across diverse user devices. To ensure different devices coexist harmoniously within a cell, the frame structure must be carefully designed. In some cases, specific time-frequency resources should be reserved exclusively for a particular service. For example, for services provided by ISAC there is a need to allocate time-frequency resources to transmit and receive pulses for sensing and to configure guard symbols. For other services, it must be flexible to be made accessible only to the group of UEs that support it. For UEs that do not support that service, these resources should be marked as non-accessible. This approach not only simplifies UE design but also reduces power consumption. Therefore, we propose the following

6GR frame structure should natively support configuration of time-frequency resource reservation to user groups with specific requirements and prevent access of those resources by other user groups.

## Frequency domain structure

As in NR, one resource block constitutes of 12 consecutive subcarriers during one OFDM symbol. One resource block is the smallest resource that can be scheduled to one UE:

In 6GR, a resource block constitutes of 12 consecutive subcarriers.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The only SCS that can be fully IODT tested for 6GR in FDD bands is 15 kHz.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For FDD bands below sub 6 GHz, support only 15 kHz SCS.

Proposal 2	SSB uses the same SCS as PDxCH/PUxCH.

Proposal 3	For around 7GHz, support (only) 30kHz SCS.

Proposal 4	Support 400 MHz CBW using 30 kHz SCS.

Proposal 5	Support only the normal CP for communication purposes in 6GR.

Proposal 6	New duplexing schemes for 6G should be studied in a holistic manner and must demonstrate clear, measurable performance gains with reasonable complexity compared to other solutions/technologies before being adopted.

Proposal 7	Keep 10 ms frames, and 1 ms subframes as timing references.

Proposal 8	Before defining slots in 6GR, determine what they should be used for.

Proposal 9	For 6GR, study frame format configurations that natively support flexible time-frequency configurations for traditional and advanced duplexing modes such as SBFD.

Proposal 10	6GR frame structure should natively support configuration of time-frequency resource reservation to user groups with specific requirements and prevent access of those resources by other user groups.

Proposal 11	In 6GR, a resource block constitutes of 12 consecutive subcarriers.

# References

RP-251881, New SID: Study on 6G Radio, NTT Docomo, RAN#108, Prague, Czech Republic, June 2025

R1-2506848, Overview of 6GR air interface, Ericsson, RAN1#122, Bengaluru, India, Aug 2025

3GPP TR 38.858, Study on Evolution of NR Duplex Operation, RAN#106, V 18.6.0, December 2024

R1-2307325, Subband non-overlapping full duplex, Ericsson, RAN1#114, Toulouse, France, August 2023

# Appendix: Simulation assumptions

| Parameters |  |
| --- | --- |
| Channel model | TDL-A with medium correlation Delay spread: 30/100/300 ns |
| Carrier frequency | 7GHz |
| UE antenna setup | 4Rx |
| gNB antenna setup | 4Tx |
| Link Direction | DL |
| Simulation bandwidth | 66 PRBs |
| SCS | 30kHz, 60kHz |
| FFT size | 2048 |
| Cyclic prefix | normal |
| UE speed | 3/30 km/h |
| PTRS configuration | PTRS enabled Every 2 PRBs (frequency domain) Every symbol (except for DMRS symbols) Port 0 |
| DMRS configuration | Type A - [2, 11] |
| Phase noise | Example I TS 38.303 both at UE and TRP side |
| Link adaptation | Fixed rank 1, No OLLA MCS = 23, R = 719/1024, (NR_TABLE_1_64QAM) MCS = 21, R = 711/1024, (NR_TABLE_2_256QAM) MCS = 23, R = 805.5/1024 , (NR_TABLE_4_1024QAM) |
| Channel estimation | Channel estimation in the frequency domain based on DMRS |
| Receiver | LMMSE |
| LDPC decoder | Fixed point |