3GPP TSG RAN WG1 Meeting #122-bis	                                                                 R1-2507410

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:  	10.2.2

Source:             	Fraunhofer IIS, Fraunhofer HHI

Title:                  	DL CSI acquisition enhancements for Rel. 20 MIMO

Document for: 		Decision

# Introduction

In RAN1#108 [1], a new work item for NR MIMO phase 6 has been agreed.

# On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: ## Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) ### Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking ### For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively ## For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs ### Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

# Early SRS/CSI/CSI-RS triggering

When a UE transitions from IDLE to CONNECTED mode, the UE performs initial access to a serving cell. The contention based random access procedure consists of four steps, where in Message 1, a UE transmits a random-access preamble to which the network responds with a random-access response in Message 2 that contains a timing advance command and information of uplink resources to be used in Message 3. Using the uplink resources, the UE transmits its identity whereas in Message 4, the network responds by transmitting a contention-resolution message to resolve any contention. After Message 4, the UE is said to be fully connected to the network. For the transmission of PUSCH in Message 3 and PDCCH/PDSCH in Message 2 and 4, due to the unavailability of the CSI in the uplink and downlink, the transmissions are performed without closed-loop precoding with low MCS values resulting in low spectral efficiency. When a UE is in INACTIVE mode, the UE still has a valid RRC connection. As the UE transitions from INACTIVE to CONNECTED mode, the UE sends an RRC Resume request via dedicated resources to re-establish the RRC connection. Only in the case where the UL resources are not available, random-access procedure is used to establish the RRC connection.

However, according to the current standard, the network can trigger a CSI report only via RRC re-configuration after the UE is fully connected to the network which delays the CSI acquisition. To improve the spectral efficiency of the data transmission during the random-access procedure, it has been agreed in RAN#108 [1] to introduce early SRS/CSI/CSI-RS triggering so that the gNB/UE can derive CSI and perform link adaptation to improve the spectral efficiency.

The CSI-RS configuration and reporting framework is complex in NR, therefore RAN1 needs to agree on low complexity and low configuration overhead solutions especially when a UE transitions from IDLE/INACTIVE to CONNECTED mode. As it has been already agreed to support only Type-I codebooks with wideband CSI reporting (low resolution CSI), low resolution CSI-RS configurations can be supported. For example, CSI-RS configurations spanning only a small number of PRBs (i.e., small density values) in the DL bandwidth or CSI-RS configurations spanning small number of time and frequency domain resources within a PRB or a combination of both can be supported to provide low-resolution CSI during the pre-RRC connection phase or RRC resume phase.

Observation: As only Type-I wideband reporting is agreed, high-resolution CSI-RS is not required for acquiring DL CSI during the pre-RRC connection phase or RRC resume phase.

Proposal: Study low-resolution CSI-RS configurations occupying a low number of time and frequency domain resources and occupying a small number of PRBs within the DL bandwidth.

# Additional density values for CSI-RS resources

In Rel. 19, up to four CSI-RS resources are aggregated to realize a 48, 64 and 128 CSI-RS transmission. The CSI-RS resource generation and RE mapping for CDM group generation follows the legacy CSI-RS design specified in Rel.15. The density of a CSI-RS resource, ρ, defined as the number of REs per RB per port takes values of 1 and 12. Depending on the time behavior of the CSI-RS resources, the up to four CSI-RS resources are transmitted either in a single slot or two slots.  In case of single slot transmission as supported for periodic and semi-persistent CSI-RSs, the CSI-RS overhead increases as up to four CSI-RS resources are transmitted on every RB in the case of density 1 and every second RB in the case of density 1/2. In the case of two slot transmission as supported for aperiodic CSI-RS resources, the CSI-RS overhead reduces compared to the single slot transmission case although it is still higher compared to a single CSI-RS resource transmission.  To overcome the increased CSI-RS overhead, it has been agreed in RAN#108 to support additional density values of  ρ=13,14,16,18 in addition to 1 and 12 .

Although, additional density values can be supported without making any changes to the CSI-RS resource generation, some changes in the mapping of the CSI-RS sequence r(m) to the resource elements k,lp,μ are necessary.  According to TS 38.211, the CSI-RS sequence r(m) is mapped to resource elements k,lp,μ according to

αk,lp,μ=βCSIRSwfk'⋅wtl'⋅rl,ns,fm'

m'= nα+k'+ kρNscRB

k=nNscRB+ k+k'

l =l+l'

α = 2ρ for X>1

n=0,1,…

where the resource element k,lp,μ is within the RBs occupied by the CSI-RS resource. For density values other than 1 and  12, the mapping shown above cannot be used. Therefore, a new mapping needs to be agreed.

Observation: The CSI-RS sequence r(m) to resource elements k,lp,μ mapping provided in TS 38.211 does not work for density values ρ=13,14,16,18.

Disadvantages of supporting low density values:

For a CSI-RS resource with density 1/x, x∈3,4,6,8, the CSI-RS is transmitted on every x-th PRB. Ideally, the UE performs channel estimation on the PRBs carrying the CSI-RS and then interpolates the estimated channels to obtain the channels on the other PRBs on which the CSI-RS resource is not available. As the CSI-RS is sparsely allocated in the frequency domain, the CSI-RS overhead as well as the channel estimation accuracy reduces. Although, it may not be a problem for low or limited frequency selective channels, the channel estimation accuracy may worsen in case of medium to high frequency selective channels due to aliasing.

One way to overcome this issue is to transmit a CSI-RS resource with density 1/x over n consecutive slots with each transmission occasion associated with a different RB-level offset. Upon reception, the UE estimates the channel for each of the n transmission occasions of the single CSI-RS resource and combines them. After combining, the resulting channel may be equivalent to that of a single CSI-RS with density n/x. For example, a CSI-RS resource with density 1/8, can be transmitted over n=2 consecutive slots, with RB level offset 0 in slot 0 and RB level offset 4 in slot 1. After combining the estimates from the two transmission occasions, the resulting estimate may be equivalent to that of a single CSI-RS resource with density of ¼. This approach allows for finer estimates of the channel by transmitting the same CSI-RS resource across multiple consecutive or non-consecutive slots. The only prerequisite is to choose the RB level offsets of each transmission occasion carefully. For n transmission occasions with density 1/x, the RB level offsets of the CSI-RS resource in each transmission occasion need to be chosen uniformly from the possible offset values {0,1,2,…,x-1}. For example, for n=2, the offset values of the CSI-RS resource in each transmission occasion must differ by a value of x/n. Therefore, to obtain the trade-off between sparse frequency allocation of CSI-RS and CSI estimation accuracy, the method of transmitting a single CSI-RS resource with density 1/x over n consecutive or non-consecutive slots to realize a CSI-RS resource with density n/x needs to be studied.

Observation: Channel estimation accuracy may worsen for low density values due to aliasing especially for medium to high frequency selective channels.

Proposal: Study the trade-off between supporting CSI-RS with low density values and the CSI estimation accuracy.

Proposal: Study the method of transmitting a single CSI-RS resource with density 1/x over n consecutive or non-consecutive slots to realize a CSI-RS resource with an increased density of n/x.

# Conclusion

From the above discussion, the following conclusions are made.

Observation: As only Type-I wideband reporting is agreed, high-resolution CSI-RS is not required for acquiring DL CSI during the pre-RRC connection phase or RRC resume phase.

Proposal: Study low-resolution CSI-RS configurations occupying a low number of time and frequency domain resources and occupying a small number of PRBs within the DL bandwidth.

Observation: The CSI-RS sequence r(m) to resource elements k,lp,μ mapping provided in TS 38.211 does not work for density values ρ=13,14,16,18.

Observation: Channel estimation accuracy may worsen for low density values due to aliasing especially for medium to high frequency selective channels.

Proposal: Study the trade-off between supporting CSI-RS with low density values and the CSI estimation accuracy.

Proposal: Study the method of transmitting a single CSI-RS resource with density 1/x over n consecutive or non-consecutive slots to realize a CSI-RS resource with an increased density of n/x.

# References

- 3GPP RAN#108, New WID: NR MIMO Phase 6, RP-251856