3GPP TSG RAN WG1 #122		                               R1-2507685

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	5

Source: 	Qualcomm Incorporated

Title: 	Power ramping and RRC configuration for CB-Msg3-EDT

Document for:	Discussion

# Background

In R1-2506717, RAN1 received an LS from RAN2 with the following actions for RAN1:

RAN2 kindly invites RAN1 to take the above agreements into account and: - Check the specification impact in RAN1 to support power ramping for CB-Msg3 (re-)transmissions, and update the corresponding specification if needed. - Check whether RAN1 has any concern regarding the additional TBS size configured for CB-Msg3, i.e., 144 bits. - Check the specification impact in RAN1 to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding specification if needed. - Check the specification impact in RAN1 to support both multi-PRB allocation and sub-PRB allocation for CB-Msg3 in eMTC, as well as both single-tone and multi-tone transmissions for CB-Msg3 in NB-IoT, and update the corresponding specification if needed. 2. Actions: To RAN1: ACTION: RAN2 respectfully asks RAN1 to take the above information into account, provide feedback if any problem found, and update the specification if needed.

In this document, we discuss the LS above.

# CB-msg3 power ramping

RAN2 has reached the following agreement:

Agreements regarding CB-Msg3-EDT power ramping: - We inform RAN1 that RAN2 agreed to support power ramping for both NB-IoT and eMTC copying RAN2 agreements asking RAN1 to check the specification impact for RAN1 specs - RAN2 to define two new RRC parameters for CB-Msg3 power ramping (i.e., powerRampingStep and cb-Msg3-InitialReceivedTargetPower), with the same value ranges as those defined for legacy Msg3 power ramping. - The UE applies power ramping when the CB-Msg3ResponseTimer has expired and the UE proceeds to the next CB-msg3 transmission, by reusing the CB_MSG3_TRANSMISSION_COUNTER_CE as defined in the MAC running CR. - The CB-Msg3 received target power can be calculated as below: the CB-MSG3_RECEIVED_TARGET_POWER is set to cb-Msg3-InitialReceivedTargetPower + (CB_MSG3_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

We propose a TP to capture the behavior above in RAN1 specifications:

Proposal 1: Endorse TP1 capturing power ramping for CB-msg3-EDT, and include it in the LS reply to RAN2.

================================= <TP1, 36.213>================================= 16.2.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	PO_NPUSCH,cj is a parameter composed of the sum of a component  provided from higher layers and a component  provided by higher layers for j=1, 3 and for serving cell wherej∈{1,2,3,4}. For NPUSCH (re)transmissions corresponding to a dynamic scheduled grant or a semi-persistent grant then j=1, for NPUSCH (re)transmissions corresponding to the random access response grant then j=2,for NPUSCH transmission using preconfigured uplink resource then j=3, and for NPUSCH transmissions corresponding to CB-msg3-EDT, j=4. PO_UE_NPUSCH,c2=PO_UE_NPUSCH,c4=0. PO_NOMINAL_NPUSCH,c4=CB_MSG3_RECEIVED_TARGET_POWER.  If enhanced random access power control is not applied, PO_NOMINAL_NPUSCH,c(2)=PO_PRE+∆PREAMBLE_Msg3, where the parameter preambleInitialReceivedTargetPower [8] () and ∆PREAMBLE_Msg3 are signalled from higher layers for serving cell . If enhanced random access power control is applied,

PO_NOMINAL_NPUSCH,c2=MSG3_RECEIVED_TARGET_POWER +ΔPREAMBLE_Msg3

<Unchanged parts are omitted>

================================= </TP1>=================================

# Additional TBS for CB-msg3

RAN2 has agreed to support a TBS of 144 bits for CB-Msg3-EDT.

Agreements regarding TBS configuration for CB-Msg3: - We don’t support multiple TBSs per CE level for CB-Msg3 EDT. - We introduce one additional value smaller than the smallest possible TBS size at the moment (328bits), i.e. 144bits.

We do not see any issue with this agreement, and propose to positively reply to RAN2.

Proposal 2: Reply to RAN2 that, on the TBS of 144 bits, RAN1 has no concerns.

# Support of multiple MPDCCH narrowbands

RAN2 made the following agreement:

Agreements regarding MPDCCH narrowband configuration: - We configure 2 MPDCCH narrowbands as a set (inform RAN1)

This agreement, in our view, is incomplete and may end up being problematic from RAN1 perspective.

As background, current specifications support multiple MPDCCH narrowbands for the random access response, based on the following table from TS 36.213:

In short, the UE will only monitor one narrowband, and that narrowband is given by the preamble index. Since a UE will only transmit one preamble during a random access attempt, the specification protects the UE from having to simultaneously monitor multiple narrowbands.

In the case of CB-msg3-EDT, the obvious extension of the RAR mechanism would be to replace “Preamble index” by some “CB-msg3-index” that depends on the time-frequency location selected by the UE for the PUSCH transmission. However, this will be problematic for the UE, since CB-msg3-EDT allows the UE to transmit multiple PUSCH in one CB-msg3-EDT attempt. These multiple transmissions may result in different narrowbands to monitor, which the UE cannot do.

Observation 1: Extending the RAR narrowband selection to CB-msg3-EDT is not trivial, since the UE may transmit multiple CB-msg3 PUSCH resulting in different narrowbands to monitor.

In view of the above, we propose to reply to RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

Proposal 3: On the MPDCCH narrowbands, provide the following reply to RAN2:

- RAN1 has no concerns on the agreement to support two MPDCCH narrowbands, assuming that RAN2 designs a mechanism for a UE to determine a unique MPDCCH narrowband to monitor for the CB-msg3 response. From RAN1 perspective, an eMTC UE does not support simultaneously monitoring more than one MPDCCH narrowband.

# Summary of proposals

In this contribution we discussed our view on RAN2 LS R1-2506717, we made the following observations and proposals:

Proposal 1: Endorse TP1 capturing power ramping for CB-msg3-EDT, and include it in the LS reply to RAN2.

Proposal 2: Reply to RAN2 that, on the TBS of 144 bits, RAN1 has no concerns.

Observation 1: Extending the RAR narrowband selection to CB-msg3-EDT is not trivial, since the UE may transmit multiple CB-msg3 PUSCH resulting in different narrowbands to monitor.

Proposal 3: On the MPDCCH narrowbands, provide the following reply to RAN2:

- RAN1 has no concerns on the agreement to support two MPDCCH narrowbands, assuming that RAN2 designs a mechanism for a UE to determine a unique MPDCCH narrowband to monitor for the CB-msg3 response. From RAN1 perspective, an eMTC UE does not support simultaneously monitoring more than one MPDCCH narrowband.