3GPP TSG RAN WG1 Meeting #122bis                                                         R1-2507708

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	9.11

Source: 	Qualcomm

Title: 	UE features for MCE for NR Phase 3

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on UE features for the following Rel-19 MC enhancements.

Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

- Different SCS/carrier type among co-scheduled cells by the single DCI

For FG66-1/2, the components defined in the Rel-18 MCE for FGs 49-1/1b/2/2b are necessary.

Proposal 1:

- FGs 66-1 has the following components:

  - Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}

  - Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}

  - Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}

  - Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG xx-1

  - Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both}

  - Support Type-2 for ‘Antenna port(s)’ field

  - The number of unicast DL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3

    - One DCI format 1_3 for the set of cells and,

    - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3

    - where:

      - For low-to-high SCS1-to-SCS2, N = 1.

      - For high-to-low SCS1-to-SCS2, N is based on pair of (SCS1, SCS2): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15)

      - SCS1: SCS of the scheduling cell

      - SCS2: The lowest SCS of the scheduled cells in the set

  - Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases

    - 1) Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or;

    - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells

    - UE supporting FG 66-1 can additionally report whether the UE support following case

      - 3) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

Proposal 2:

- FGs 66-2 has the following components:

  - Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}

  - Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}

  - Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}

  - Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both}

  - Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields

  - The number of unicast UL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3

    - For FDD scheduling cell

      - One DCI format 0_3 for the set of cells and,

      - One unicast DL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells that are not scheduled by DCI 0_3

      - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell

    - For TDD scheduling cell

      - Up to two DCI format 0_3 for the set of cells and,

      - Up to two unicast DL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells that are not scheduled by DCI 0_3

      - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell

    - where:

      - For low-to-high SCS1-to-SCS2, N = 1.

      - For high-to-low SCS1-to-SCS2, N is based on pair of (SCS1, SCS2): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15)

      - SCS1: SCS of the scheduling cell

      - SCS2: The lowest SCS of the scheduled cells in the set

  - Monitoring SS set(s) for DCI format 0_3 for a set of cells for the following cases

    - 1) Search space set configuration for DCI format 0_3 for the set of cells is provided only on the scheduling cell, or;

    - 2) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells

    - UE supporting FG 66-2 can additionally report whether the UE support following case

      - 3) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

- Multiple PDSCHs/PUSCHs on a cell scheduled by a DCI format 1_3/0_3

Regarding FR/SCS constraint for multi-cell multi-PxSCH scheduling by DCI x_3, although we consider normative specs (211, 212, 213, 214) can specify multi-PDSCH/PUSCH scheduling by a DCI format 0_3/1_3 without explicit SCS/carrier type limitations, enabling the feature for scenarios that were not supported in the legacy DCI formats 0_1/1_1 in Rel-16, 17, and 18, is upscoping. Therefore, the Rel-19 features should be limited to the SCSs/carrier types that have already been supported by legacy DCI formats.

Regarding prerequisite, we consider 49-1/1b/2/2b are the prerequisite for the FG66-3/4, since we have not discussed/agreed to support multi-cell multi-PDSCH/PUSCH scheduling with different SCSs with a single DCI format.

Proposal 3:

- Clarify following for FG66-3:

  - The FG is applicable to the frequency band in FR2-1 or FR2-2

  - The FG is applicable to SCS 120kHz

  - Prerequisite is 49-1 or 49-1b

- Clarify following for FG66-4:

  - The FG is applicable to SCS 30kHz and SCS 120kHz

  - Prerequisite is 49-2 or 49-2b

- Proposed UE feature list for Rel-19 MCE

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66. NR_MC_enh2 | 66-1 | Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and/or different carrier type | 1. UE supports monitoring DCI format 1_3 for DL scheduling with different SCS and/or different carrier type for the cells in the set 2. Supported applicable combinations of the following from the band combination: - Two sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2, 60kHz), (FR2, 120kHz)} - A set of (carrier type, SCS) for scheduling cellin the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} 3. Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 4. Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 5. Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} 6. Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG xx-1 7. Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8. Support Type-2 for ‘Antenna port(s)’ field 9. The number of unicast DL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3 - One DCI format 1_3 for the set of cells and, - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3 where: - For low-to-high SCS1-to-SCS2, N = 1. - For high-to-low SCS1-to-SCS2, N is based on pair of (SCS1, SCS2): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15) - SCS1: SCS of the scheduling cell - SCS2: The lowest SCS of the scheduled cells in the set 10. Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases - 1) Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or; - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells - UE supporting FG 66-1 can additionally report whether the UE support following case - 3) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells FFS: Other component(s) |  | Yes | n/a | FFS | Per BC | n/a | n/a | n/a | FFS | Optional with capability signalling |
| 66. NR_MC_enh2 | 66-1 | Support of three sets of (carrier type, SCS) for the cells in the set for multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and/or different carrier type | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 66. NR_MC_enh2 | 66-2 | Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and/or different carrier type | 1. UE supports monitoring DCI format 0_3 for UL scheduling with different SCS and/or different carrier type for the cells in the set 2. Supported applicable combinations of the following from the band combination: - Two sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2, 60kHz), (FR2, 120kHz)} - A set of (carrier type, SCS) for scheduling cell: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} 3. Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 4. Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 5. Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} 6. Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 7. Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields 8. The number of unicast UL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3 For FDD scheduling cell - One DCI format 0_3 for the set of cells and, - One unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells that are not scheduled by DCI 0_3 - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell For TDD scheduling cell - Up to two DCI format 0_3 for the set of cells and, - Up to two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells that are not scheduled by DCI 0_3 - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell where: - For low-to-high SCS1-to-SCS2, N = 1. - For high-to-low SCS1-to-SCS2, N is based on pair of (SCS1, SCS2): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15) - SCS1: SCS of the scheduling cell - SCS2: The lowest SCS of the scheduled cells in the set 9. Monitoring SS set(s) for DCI format 0_3 for a set of cells for the following cases - 1) Search space set configuration for DCI format 0_3 for the set of cells is provided only on the scheduling cell, or; - 2) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells - UE supporting FG 66-2 can additionally report whether the UE support following case - 3) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells FFS: Other component(s) |  | Yes | n/a | FFS | Per BC | n/a | n/a | n/a | FFS | Optional with capability signalling |
| 66. NR_MC_enh2 | 66-2a | Support of three sets of (carrier type, SCS) for the cells in the set for multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and/or different carrier type | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 66. NR_MC_enh2 | 66-3 | Multi-cell multi-PDSCH scheduling by DCI format 1_3 | 1. UE supports DCI format 1_3 for DL scheduling of one or more PDSCH(s) per cell in the set of cells 2. Type 1 and Type 2 HARQ codebook for supporting multi-cell multi-PDSCH scheduling with single DCI 3. Maximum number of PDSCHs that can be scheduled by a DCI format 1_3 for multi-cell multi-PDSCH scheduling This FG is applicable to the frequency band in FR2-1 This FG is applicable to SCS 120kHz FFS: Whether to limit this FG to particular FR and/or SCS FFS: Whether to separate this FG per FR and/or SCS FFS: other component(s) | 49-1 or 49-1b | Yes | N/A | Multi-cell multi-PDSCH scheduling by DCI format 1_3 is not supported |  |  |  |  | Candidate values for component 3: {8, 12, 16, 32} | Optional with capability signalling |
| 66. NR_MC_enh2 | 66-4 | Multi-cell multi-PUSCH scheduling by DCI format 0_3 | 1. UE supports DCI format 0_3 for UL scheduling of one or more PUSCH(s) per cell in the set of cells 2. Maximum number of PUSCHs that can be scheduled by a DCI format 0_3 for multi-cell multi-PUSCH scheduling This FG is applicable to SCS 30kHz and 120kHz FFS: Whether to limit this FG to particular FR and/or SCS FFS: Whether to separate this FG per FR and/or SCS FFS: other component(s) | 49-2 or 49-2b | Yes | N/A | Multi-cell multi-PUSCH scheduling by DCI format 0_3 is not supported |  |  |  |  | Candidate values for component 2: {8, 12, 16, 32} | Optional with capability signalling |