3GPP TSG RAN WG1 #123		 R1-2508596

Dallas, USA, Nov 17th – 21st, 2025

Source:	CATT

Title:	Frame structure for 6GR

Agenda Item:	11.3.2

Document for:	Discussion and Decision

Introduction

The SI [1] on 6G Radio study was approved in RAN#108. After discussion in RAN1#122bis meeting, the following agreements were achieved.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Conclusion Numerologies for sensing is up to sensing agenda discussion. Agreement 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP. Agreement Study and identify the lessons learned from NR duplex modes. On 6GR duplexing study, RAN1 considers at least following duplex types: - FD-FDD - Semi-static TDD - gNB semi-static SBFD - HD-FDD on UE side - Dynamic TDD Study whether to consider following duplexing types - gNB dynamic SBFD - UE SBFD - gNB FD - Note: Other duplex modes are not precluded. Agreement For communication, 6GR considers NR frame structure used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12 - Radio Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design. - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings. Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session. Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

This contribution further provides our views on 6GR frame structure and numerology.

Discussion

Duplex

FDD and Semi-static TDD are foundational duplexing techniques for bidirectional communications and have been widely used in communication systems. In 6G, FDD and Semi-static TDD can still be supported by default.

To address low-capability UE requirements and non-terrestrial network demands, 6G should incorporate half-duplex FDD (HD-FDD). This implementation effectively reduces UE complexity while facilitating cost optimization through streamlined hardware design.

6G may natively support BS-side semi-static SBFD from initial deployment, leveraging its proven benefits in uplink latency reduction and coverage enhancement.

To adapt to dynamic changing of uplink/downlink traffic ratios, dynamic TDD is also proposed to be supported.

Proposal 1: FDD, Semi-static TDD, HD-FDD, BS-side semi-static SBFD and dynamic TDD should be supported in 6G Day-1.

For BS-side SBFD, further support for dynamic SBFD and PDCCH enhancement can be considered. For dynamic SBFD, focus on simpler implementation schemes, such as dynamically switching SBFD symbols to full downlink symbols, while avoiding complex dynamic adjustments to SBFD sub-band sizes or positions. PDCCH enhancement is to reduce CORESET configuration constraints. For example, CORESET can be configured to overlap with resources outside DL subband, but PDCCH candidates outside the DL subband will not be detected.

Proposal 2: Further study on dynamic SBFD and PDCCH enhancement can be considered for BS-side SBFD.

Frame structure

- Symbol type

In 5G, flexible symbol type is adopted in addition to uplink symbol type and downlink symbol type. One motivation is to address the guard period and another motivation is to achieve dynamic TDD to quickly match UL and DL traffic load.

Considering the needs of guard period/transition period in TDD operation and SBFD operation, and the different requirements in TDD/SBFD operation with different SCS, a flexible symbol type is preferred to be adopted in 6G. This allows the network to configure different guard period lengths and locations by configuring a different number of flexible symbol(s) and their locations. This approach can avoid predefining many sets of guard period lengths and locations based on practical scenarios.

Additionally, to adapt UL/DL traffic load timely, dynamic TDD via scheduling on flexible symbols can be considered while cross link interference can be managed via CLI handling schemes introduced in R19 SBFD. As for dynamic TDD via SFI, we prefer to exclude SFI to avoid additional signaling overhead, UE PDCCH blind detection and spec complexity.

In R19 SBFD, the SBFD symbol is configured via overriding the TDD cell common configuration. To natively support SBFD to address the uplink coverage issue in 6G and simplify signaling design, a duplex symbol type is proposed to be supported as a basic symbol type, as one example as depicted in Figure 1. Then a unified frame structure can be defined to support the configuration of TDD and SBFD.

Proposal 3: Support uplink, downlink, flexible and duplex symbol type in 6G frame format configuration in time domain.

symbol

Downlink

Uplink

SBFD

Flexible

symbol

Downlink

Uplink

SBFD

Flexible

slot

slot

Pattern

Pattern

Figure 1 Basic symbol types of 6G

Before the introduction of SBFD in Rel-19, the frame format configuration of 5G applied the same designation of UL symbols, DL symbols, and flexible symbols across the entire carrier bandwidth. With the introduction of SBFD in Rel-19, frequency-domain division can be applied within part or all of the cell’s commonly configured DL symbols and flexible symbols (i.e., SBFD symbols). For these SBFD symbols, the same UL/DL resource division is applied in the frequency domain.

For 6G SBFD frequency division, the SBFD frequency configuration defined in 5G can serve as a starting point. If more flexible configurations on SBFD symbols are to be considered, such as allowing different UL/DL resource divisions across SBFD symbols, clear justification is required. This should include the target scenario, expected benefits, implementation feasibility, interference management, and other relevant aspects.

Proposal 4: For 6G frequency division on SBFD symbols, 5G frequency configuration on SBFD can be discussed as a starting point.

- TDD Frame format

For 5G, TDD frame format is configured via TDD pattern which consists of continuous downlink symbols at the beginning of the TDD pattern, continuous uplink symbols at the ending of the TDD pattern, and continuous flexible symbol(s) between the downlink symbols and the uplink symbols. For 6G TDD frame format configuration, similar way as 5G could be reused by introducing duplex symbol type and the symbol order can be {DFXFU} in one TDD frame format pattern as depicted in Figure 2, where ‘X’ means duplex symbol type and could be omitted when SBFD is not used.

In 5G, two TDD patterns are supported in combination to provide two DL-UL switching points and longer or multiple TDD periods to better match DL/UL traffic load. In Rel-19, SBFD is configured per TDD pattern and can be applied to either or both of the two TDD patterns. As a result, the SBFD period can extend up to twice the length of one TDD period, i.e., 20 ms. This approach could be reused in 6G to support flexible TDD and SBFD configurations. Furthermore, to better match UL/DL traffic load or UL coverage requirement, and to avoid network frequently switching between TDD and SBFD, more than 2 TDD patterns combination can be considered, e.g., 4 TDD patterns combination. Network can configure SBFD on any one of the 4 TDD patterns combinations. As a result, SBFD period can be up to 4 times of TDD. Therefore, we prefer to support more than two patterns combination.

Proposal 5: For 6G, the TDD frame format could be configured via one pattern or multiple (including more than two) patterns combination with a duplex symbol type to enable more flexible TDD and SBFD configurations.

As for one TDD pattern, multiple periods are supported in 5G, including 0.5ms, 0.625ms, 1ms, 1.25ms, 2ms, 2.5ms, 5ms and 10ms. If the existing TDD period is used in NTN, the DL-UL switching period doesn’t match NTN RTT and it will cause unnecessary resource waste. Instead, a long guard period should be considered to address long propagation delay. Consequently, a longer TDD pattern period is preferred. Considering the schedule latency, NTN TDD is more suitable for LEO scenario. In typical case, the maximum RTT in LEO is less than 20ms, which should be taken into account in TDD pattern design.  For example, a 40ms TDD pattern is one potential candidate.

In addition, using a longer period within a single TDD pattern, rather than combining two patterns to extend the SBFD period, can reduce the number of SBFD switching points and thereby lower switching overhead and complexity. Therefore, to meet the requirements of NTN and SBFD with respect to pattern period, a longer TDD pattern period is proposed.

Proposal 6: Support a TDD pattern period longer than 10ms in 6G design to meet the requirements of both NTN and TN.

- Configuration/Indication

To support TDD operation in 5G, the following three types of frame structure are supported.

- Cell common TDD configuration: It configures the number and the locations of DL, UL and flexible symbols.

- UE specific semi-static configuration: It can only override the flexible symbols configured by cell common TDD configuration as DL or UL.

- UE group-common configuration via DCI format 2-0 (SFI): It can only override the flexible symbols configured by cell common TDD configuration and UE specific semi-static configuration (if configured) as DL or UL.

For 6G frame structure configuration, cell common and UE specific RRC configuration as 5G should be supported. As for SFI, it is not adopted in 5G deployment. Considering additional signaling overhead, UE PDCCH blind detection and spec complexity introduced by SFI, we prefer to exclude it.

Proposal 7: For 6G frame structure configuration, support cell common and UE-specific semi-static configurations as in 5G, while excluding dynamic SFI to avoid additional signaling overhead, UE blind detection burden, and specification complexity.

- Time unit for scheduling

Time unit for scheduling was discussed without consensus in last meeting. From our view, slot based and symbol based (mini-slot) should be supported as 5G to provide scheduling flexibility and satisfy different latency requirements. To improve coverage and reduce reference signal/control signaling overhead, multi-slot scheduling can be considered.

Proposal 8: Support mini-slot, slot and multi-slot scheduling.

- How to support sensing

Compared with the symbol/slot structure for communication, the symbol/slot structure for sensing may need to be the same or different, depending on the sensing requirements under discussion. For UEs without sensing capability, the sensing symbol/slot structure can remain transparent. These UEs only need to recognize that sensing resources are unavailable to them, which can be achieved by configuring such resources as reserved. In this way, UEs without sensing capability simply treat sensing resources as invalid, while the network can configure a sensing-specific symbol/slot structure on these resources, regardless of whether it is identical to or different from the communication structure. .

Proposal 9: In 6G, sensing resources should be configured as reserved for non-sensing UEs, enabling a transparent design while supporting sensing-specific symbol/slot structures.

Numerology

The issue of whether to allow additional subcarrier spacings (SCS) for SSB was discussed in the last meeting. Except FR2-1, it was agreed that 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. For same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1 is open. From our perspective, the motivation for this is not clear at the current stage, while several concerns are introduced by supporting additional SCS. If more than one SCS is configured for SSB on the same carrier/band/sub-range, UEs would need blind detection to determine the SCS, which increases implementation complexity and may impact SSB detection latency. If a larger SCS is adopted, it results in a larger minimum channel bandwidth. If a smaller SCS is adopted, it leads to a longer SSB duration. Therefore, introducing additional SCS for SSB for FR2-1 is not preferred. Additionally, to keep the design consistent with other frequency subrange, same SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1 is also preferred.

Proposal 10: Not support additional subcarrier spacing for SSB for FR2-1.

Following the agreement in RAN1#122bis that RAN1 assumes a 400 MHz maximum channel bandwidth at the network side with 30 kHz SCS around 7 GHz, further study is needed on whether and how to enable UE to support 400 MHz bandwidth.

This study aims to identify the feasibility, architectural implications, and possible design options for UE-side bandwidth extension in this frequency range. Key aspects to be considered include:

- RF and baseband capability impact: investigate feasible RF and baseband architectures for 6GR UE to support up to 400 MHz instantaneous bandwidth, including options such as segmented or subband-based processing.

- Channelization and CA options: evaluate if 400 MHz operation can be realized through multiple 100 MHz carriers or via a single wideband carrier configuration.

- Power consumption and complexity trade-off: analyze how wider bandwidth affects UE energy efficiency, sampling rate, and ADC/DAC requirements.

Proposal 11: RF and baseband capability impact, channelization and CA options, power consumption and complexity trade-off should be the main focus of the study on whether and how to enable UE to support 400 MHz bandwidth.

- Longer CP for NTN

Additionally, potential needs on CP from NTN should be taken into account. For NTN, the LOS path is dominant in DL and synchronization error is heavy in UL, which affects the CP length design. Thus different CP length can be considered for DL and UL respectively. And to address the issue of heavy synchronization error in UL and larger coverage requirements in NTN system, longer CP than normal CP should be considered at least for high SCS, e.g., 120 KHz. As we know, normal CP length is 0.59us for 120 kHz SCS, which should cover all synchronization errors including service link and feeder link. It imposes strict requirement for UE implementation. In real trial, the result showed the risk of UL synchronization performance.

Moreover, in 6G NTN, one specific requirement is to support UE access in GNSS information unavailable, which will cause big challenge of UL synchronization. When GNSS information is unavailable, the timing accuracy will be degraded unavoidably due to less accurate pre-compensation in UL. It will impact the design of PRACH and PUSCH. Hence, longer CP is very necessary to enable UL synchronization robustness. Then extended CP should be considered in PUSCH transmission. At least for 60 kHz and above SCS extended CP should be supported.

Proposal 12: For 6GR numerology design, support extended CP in NTN scenario.

MRSS

For 6G system, considering that both coverage and capacity are crucial aspects, re-farming the low band spectrum from 5G to 6G is inevitable. However, at the early stage of 6G deployment, most of the traffic will still be carried by 5G network. It won’t be feasible to re-farm the spectrum from 5G to 6G until 6G devices fully replace existing 5G devices. Multi-RAT spectrum sharing (MRSS) between 5G and 6G should be introduced to enable efficient coexistence of the two systems. Targeting MRSS, in physical layer, 6G design shall guarantee efficient resource sharing with 5G system. 6GR shall be non-backward compatible with 5G. Different signal/channel designs, such as SSB, RO, etc., will be introduced to meet the new use cases and requirements. If 6G devices are required to monitor both 5G and 6G signals/channels on the shared spectrum, the 6GR design will be over complicated and too restrictive. Therefore, MRSS should not consider signal/channel sharing between 5G and 6G unless clear and significant benefits are justified.

Proposal 13: Do not consider signal/channel sharing between 5G and 6G unless clear and significant benefits are justified.

To further enhance the efficiency of MRSS, it is important to ensure that 5G and 6G operate on aligned numerology and TDD grid. By adopting common or compatible subcarrier spacing and aligning OFDM symbol boundaries as well as DL/UL switch points, both systems can minimize cross-link interference and guard period overhead. This approach allows efficient spectrum utilization while maintaining the flexibility of 6G to design new signals and channels independently.

Proposal 14: Support 5G–6G aligned numerology and aligned TDD grid for efficient MRSS operation.

Bandwidth adaption

For 5G BWP design, the following three usage scenarios were identified to be supported.

- Usage scenario #1: Supporting reduced UE BW capability

- Usage scenario #2: Supporting reduced UE energy consumption by means of bandwidth adaptation

- Usage scenario #3: Supporting FDM of different numerologies

For 6G design, if each cell is assumed to have only one SCS, the usage scenario #3 will no longer hold. However, since 6G is expected to serve diverse types of UEs (e.g., IoT devices) and there will still be a demand for reducing UE energy consumption by bandwidth adaptation, BWP remains necessary in 6G Day-1.

The current BWP switching mechanism enables the gNB to adjust UE bandwidth according to traffic demand: when traffic is high, the UE can be instructed to operate on a wide bandwidth for data transmission, while during low traffic period, the UE can be restricted to a narrow bandwidth, thereby saving energy by reducing the bandwidth of RF and baseband processing.

Nevertheless, the current BWP mechanism also has several drawbacks:

- Signaling overhead due to per-BWP configuration: Many physical-layer parameters are configured at the BWP level. If a cell supports four BWPs, the gNB must configure parameters for each of them individually. Even when many parameters are identical across BWPs, they are still redundantly configured, and the signaling overhead grows with the number of configured BWPs.

- Complexity and latency of BWP adaptation: In addition to BWP reconfiguration, NR supports BWP switching among multiple BWPs. Once the gNB instructs the UE to switch BWPs, the UE reloads the entire set of parameters for the target BWP. This requires additional processing at the UE side. When many BWPs are configured, this process becomes more complex and increases the switching latency. In addition, multiple BWP switching options are specified, but not all of these mechanisms are proven to be useful in practical deployment.

Proposal 15: The following lessons are learned from 5G NR BWP framework:

- Too many physical layer parameters are per-BWP configured, which are redundant;

- Per-BWP configuration of physical layer parameters increases the overhead, complexity of UE implementation, and probably the latency of BWP switching;

- Multiple BWP adaptation options have been introduced but not all are useful in practical.

Therefore, for 6G Day-1, it is important to study how to enhance BWP design. The main objectives should be:

- Reducing signaling overhead by avoiding redundant parameter configurations across BWPs.

- Simplifying UE complexity during BWP switching, so that latency and processing burden are minimized.

- Preserving the benefits of BWP (support for low-capability devices and energy savings) while ensuring efficient support of control and data channels.

In summary, enhancing BWP framework in 6G should focus on simplification, making it more lightweight, scalable, and efficient to meet the diverse requirements of early 6G deployments.

Proposal 16: Study lightweight, scalable, and efficient BWP enhancements to meet the diverse requirements of early 6G deployments.

- Minimize unnecessary per-BWP configuration parameters and simplify BWP switching options.

Conclusions

In this contribution, we provide our views on frame structure and numerology. The proposals are summarized below:

Proposal 1: FDD, Semi-static TDD, HD-FDD, BS-side semi-static SBFD and dynamic TDD should be supported in 6G Day-1.

Proposal 2: Further study on dynamic SBFD and PDCCH enhancement can be considered for BS-side SBFD.

Proposal 3: Support uplink, downlink, flexible and duplex symbol type in 6G frame format configuration in time domain.

Proposal 4: For 6G frequency division on SBFD symbols, 5G frequency configuration on SBFD can be discussed as a starting point.

Proposal 5: For 6G, the TDD frame format could be configured via one pattern or multiple (including more than two) patterns combination with a duplex symbol type to enable more flexible TDD and SBFD configurations.

Proposal 6: Support a TDD pattern period longer than 10ms in 6G design to meet the requirements of both NTN and TN.

Proposal 7: For 6G frame structure configuration, support cell common and UE-specific semi-static configurations as in 5G, while excluding dynamic SFI to avoid additional signaling overhead, UE blind detection burden, and specification complexity.

Proposal 8: Support mini-slot, slot and multi-slot scheduling.

Proposal 9: In 6G, sensing resources should be configured as reserved for non-sensing UEs, enabling a transparent design while supporting sensing-specific symbol/slot structures.

Proposal 10: Not support additional subcarrier spacing for SSB for FR2-1.

Proposal 11: RF and baseband capability impact, channelization and CA options, power consumption and complexity trade-off should be the main focus of the study on whether and how to enable UE to support 400 MHz bandwidth.

Proposal 12: For 6GR numerology design, support extended CP in NTN scenario.

Proposal 13: Do not consider signal/channel sharing between 5G and 6G unless clear and significant benefits are justified.

Proposal 14: Support 5G–6G aligned numerology and aligned TDD grid for efficient MRSS operation.

Proposal 15: The following lessons are learned from 5G NR BWP framework:

- Too many physical layer parameters are per-BWP configured, which are redundant;

- Per-BWP configuration of physical layer parameters increases the overhead, complexity of UE implementation, and probably the latency of BWP switching;

- Multiple BWP adaptation options have been introduced but not all are useful in practical.

Proposal 16: Study lightweight, scalable, and efficient BWP enhancements to meet the diverse requirements of early 6G deployments.

- Minimize unnecessary per-BWP configuration parameters and simplify BWP switching options.

Reference

- RP-251881, New SID: Study on 6G Radio, Prague, Czech Republic, June 9-13, 2025.