3GPP TSG RAN WG1 #123		R1-2508991

Dallas, USA, Nov 17th – 21st, 2025

Source:	HONOR

Title:	Views on air interface for Device 2b/C for Ambient IoT

Agenda Item:	10.3.2

Document for:	Discussion and Decision

- Introduction

A new ambient IoT study item has been approved in the RAN plenary #108 meeting [1]. One of the objectives is to study necessary and feasible changes to the Rel-19 A-IoT specifications to support device 2b and device C in outdoor scenarios. In this contribution, candidate enhancements are discussed to support the new devices for outdoor scenarios.

- R2D

Frequency hopping

In RAN1#122 meeting, the following agreement was captured [2].

Agreement Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types: - Block-level repetitions - Bit-level Type-2 repetitions - Chip-level repetitions (in case of M=1 with FEC if applied)

Block-level repetitions can help R2D transmission fully collect time diversity gain, and thus improve R2D coverage. Since Rel-20 SID assumes that A-IoT readers are deployed on the same sites as existing outdoor NR macro BSs and A-IoT uses FR1 licensed spectrum in FDD in-band to NR and in standalone bands, there would be unignorable interference suffered at A-IoT devices from the NR DL transmission which occupies frequency resource near that of A-IoT R2D transmission. To handle this interference problem, we prefer the support of frequency hopping on the basis of block-level repetitions.

Specifically, the block-level repetitions can cross different R2D messages. The frequency resource locations of the repetitions are the same within one R2D message, but are frequency hopped when within the other. In this way, the frequency diversity gain can be collected and the influence from NR can be alleviated.

This method of frequency hopped repetitions can also be applied to D2R transmission, considering that in Rel-20 A-IoT, devcie2b/C can adjust the LO towards a desired frequency.

Proposal 1: Study to support frequency hopping for block-level repetitions.

Timing offset

In RAN1#122 meeting, the following agreement was captured [2].

Agreement At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g., - timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

As discussed in subsection 2.1, cross-message block-level repetitions are candidate scenarios where there are consecutive R2D/D2R transmissions for the same device.

For R2D, since A-IoT device requires a certain amount of time to process a single R2D message, a minimum timing offset between consecutive R2D messages should be introduced to ensure the successful R2D reception. For D2R, it is expected that the available duration of device 2b and device C is much less than that of device 1. After device 2b/C sending a single D2R message, there may be the case where the remaining energy of the device is insufficient to support sending subsequent D2R messages. This issue can be taken into account when designing the timing offset between consecutive D2R messages.

Observation 1: Cross-message block-level repetitions are candidate scenarios where there are consecutive R2D/D2R transmissions for the same device.

FDM(A)

In RAN1#122bis meeting, the following agreement was captured [3].

Agreement Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects: - complexity of device to support FDM - how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

The coverage objective of 500m in Rel-20 A-IoT is significantly larger than that of Rel-19. With device density remaining unchanged, inventory delay would increase substantially. Supporting R2D FDM allows for faster transmission of Msg2, which is necessary to reduce inventory delay.

In Rel-19 A-IoT study, it was observed that device 2b with IF-ED/ZIF receivers can support R2D FDM. FDM in R2D as a possible enhancement for a subsequent release is captured in TR 38.769 [4]. According to the definition of device 2b and device C in Rel-20 SID below, both device 2b and device C support IF-ED/ZIF receiver. So R2D FDM is feasible for Rel-20 A-IoT.

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19.

Observation 2: R2D FDM is necessary and feasible for Rel-20 A-IoT.

Calibration signal

In RAN1#122bis meeting, the following agreement was captured [3].

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

As discussed in subsection 2.3, R2D FDM is necessary and feasible for Rel-20 A-IoT. With R2D FDM, it is necessary for device 2b/C to calibrate its LO before it receives R2D messages via IF-ED/ZIF receivers, such that the LO can be aligned as closely as possible to the center frequency of the R2D message, thereby reducing the required guard-band between FDMed R2D messages and improving the spectrum efficiency. In addition, since D2R FDM has been supported in Rel-19 A-IoT, LO calibration is also needed before device sending D2R messages.

By considering FDMed transmission in both directions, we prefer time-location solutions of CFO calibration signal that can satisfy calibration requirements for both of R2D and D2R. If a single R2D contains both L1 control and PRDCH, putting two LO calibration signals within a single R2D, e.g. one located before CAP for R2D calibration, and another located at the end of PRDCH for subsequent D2R calibration, is straightforward and simpler to achieve, which could be Option 1 + Option 2.

Proposal 2: For the time location(s) of CFO calibration signal, if a single R2D contains both L1 control and PRDCH, support Option 1 + Option 2.

- D2R

Frequency resource determination

In RAN1#122bis meeting, the following issue was discussed but no agreement has been reached [5].

Proposal Regarding how a device perform D2R transmission, study followings: - Alt.1: Tuning to a device specific target frequency (i.e., no base-band small frequency shift). - Alt.2: To achieve a device-specific target frequency, use base-band small frequency shift to achieve device-specific frequency offset, and tuning to a carrier frequency.

In Rel-19 A-IoT, D2R FDM is achieved by small FS with R factor. Unlike using external CW, device 2b and device C in Rel-20 generate the carrier via LO. According to the evaluation assumption in AI 10.3.1 [3], CFO in LO after calibration is worse than 10ppm, which yields ±9kHz CFO under a center frequency of 900MHz. Take D2R Manchester coding-based 2SB transmission bandwidth 15kHz as an example: the main lobe of D2R spectrum at R = 1 overlaps with that of D2R spectrums at R = 2 and R = 4 under ±9kHz CFO, which highly limits the available set of R value. In addition, 1SB cannot be supported by small FS, and spectrum efficiency is low. Hence, we prefer tuning to a device specific target frequency (i.e., no base-band small FS).

Proposal 3: For D2R transmission, support tuning to a device specific target frequency (i.e., no base-band small frequency shift).

Power control

In RAN1#122bis meeting, the following agreement was captured [3].

Agreement Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios - Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123 Continue studying feasibility of the following open loop power control methods considering device complexity: - Methods of open loop power control - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader - Required information at the device side at least includes reader Tx power, received target power at the reader, etc. - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range. - The RSRP range is based on RSRP threshold(s) configured by reader or predefined - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s) - FFS details (e.g., required measurements, required information) Continue studying the necessity and feasibility of closed loop power control for D2R.

Since processing capability and energy storage is limited for A-IoT devices, it may be a burden for devices to do calculations by predefined formulas as in Option 1. On the other hand, it could be flexible enough to cover the outdoor use cases of A-IoT if the RSRP threshold(s) and the D2R Tx power in Option 2 are configurable. Hence, we prefer Option 2 for open loop power control.

Considering that the available duration of device 2b and device C is significantly shorter than device 1 and is highly related to the D2R transmitting power, we tend to also support closed loop power control to prevent the device from going to OFF unexpectedly during D2R transmission when its remaining power is low.

Proposal 4: For D2R open loop power control, support Option 2.

Proposal 5: Support to study the closed loop power control.

- Support of DO-A

In RAN1#122bis meeting, the following agreement was captured [3].

Agreement For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility: - Option 1: necessary information provided by broadcast information - Option 2: necessary information provided by a paging-like R2D message - Option 3: necessary information derived from a synchronization signal and provided by broadcast information

To support at least event-triggered DO-A traffic, DO-A related resource configuration info should be provided to devices, and T/F sync is needed for the device to transmit when DO-A traffic is triggered.

If the necessary information for the first D2R transmission for DO-A is provided by a paging-like R2D message, it appears to be DO-DTT traffic, because it seems like the first D2R transmission is trigged by this paging-like R2D message. Compared to Option 1, Option 3 is more flexible because the configuration and T/F sync can be done via separate R2D transmissions, as shown in Fig. 1.

Fig. 1 Separate DO-A resource configuration and T/F sync.

With separate R2D transmissions, the configuration info in one R2D transmission indicates the timing offset between T/F sync (another R2D transmission) and the start of the following first DO-A RO.

Proposal 6: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, support

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information.

- Solution for unavailability

During Rel-19 A-IoT standardization process, direction 1 was tended to address the unavailability issue for device 1, so there was no spec impact. However, based on Rel-19 study, the unavailability has a greater impact on device 2b and device C, which have higher transmission power and have more time unavailability due to energy harvesting. Therefore, solution under direction 2 for unavailability may be needed to standardize in Rel-20 A-IoT.

Direction 1: Reader does not provide information to a device regarding when the device may become available/ unavailable. Direction 2: Reader can provide information to a device based on which the device may become available/ unavailable.

Proposal 7: Study solutions under direction 2 to address unavailability.

- Positioning/Proximity

During Rel-19 A-IoT study, two solutions has been proposed to address Proximity determination [4].

Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader. Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side. Proximity determination based on device-side measurements is not considered.

In outdoor scenarios, devices are more widely distributed, so objective on positioning/proximity determination is listed in the Rel-20 study scope.

In our view, we support to study positioning/proximity determination under solution 2 in Rel-20.

Proposal 8: Study positioning/proximity determination under solution 2.

- Conclusions

In this contribution, we provide our views on candidate enhancements to support the new devices for outdoor scenarios. The following observations and proposals are given:

Observation 1: Cross-message block-level repetitions are candidate scenarios where there are consecutive R2D/D2R transmissions for the same device.

Observation 2: R2D FDM is necessary and feasible for Rel-20 A-IoT.

Proposal 1: Study to support frequency hopping for block-level repetitions.

Proposal 2: For the time location(s) of CFO calibration signal, if a single R2D contains both L1 control and PRDCH, support Option 1 + Option 2.

Proposal 3: For D2R transmission, support tuning to a device specific target frequency (i.e., no base-band small frequency shift).

Proposal 4: For D2R open loop power control, support Option 2.

Proposal 5: Support to study the closed loop power control.

Proposal 6: For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, support

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information.

Proposal 7: Study solutions under direction 2 to address unavailability.

Proposal 8: Study positioning/proximity determination under solution 2.

# References

- RP-251884, Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, RANP#108, June, 2025.

- Chair notes RAN1#122 (10.3 R20 SI on Ambient IoT)

- Chair notes RAN1#122bis (10.3 R20 SI on Ambient IoT)

- TR 38.769, Study on solutions for ambient IoT (Internet of Things)

- R1-2508175, FL summary #9 for 10.3.2 “Study of air interface for Device 2b/C”