3GPP TSG-RAN WG1 Meeting #123			R1-2509230

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	11.2

Title:	Evaluation assumptions for 6GR air interface

Source:	Qualcomm Incorporated

Document for:	Discussion/Decision

# Introduction

For the 6GR study, RAN1 will need to study and evaluate many aspects of radio [1]:

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

Furthermore, there is an interim assessment milestone for delivering a subset of evaluation results for fundamental PHY structure aspects:

TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h)

In this contribution, we discuss the proposals needed to perform the 6GR evaluations needed for the study, prioritizing those required for the interim assessment milestone.

# General Evaluation Scenarios and Assumptions

## Deployment scenarios

In 3GPP RAN1#122bis meeting, the following was agreed for system level simulations for 6GR evaluation [2].

Agreement

The following configurations for system-level simulations could be used for 6GR evaluation:

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated BW | Follow system bandwidth per carrier frequency in TR 38.914 as 1) Around 700 MHz: Up to 60 MHz 2) Around 2GHz: Up to 200 MHz 3) Around 4GHz: Up to 300 MHz 4) Around 7GHz: Up to 400MHz 5) Around 15GHz: Up to 400MHz 6) Around 30GHz: Up to 1GHz |  |  |  |  |
| Simulation BW | Around 700 MHz: 20MHz, 60MHz |  |  |  |  |
|  | Around 2 GHz: 20MHz, 100MHz, 200MHz |  |  |  |  |
|  | Around 4 GHz: 20MHz, 100MHz, 200MHz, 300MHz |  |  |  |  |
|  | Around 7 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 15 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 30GHz: 100MHz, 400MHz, 800MHz |  |  |  |  |
|  | Note: other simulation BW could be considered. |  |  |  |  |
| Note: The layout for each scenario will be separately discussed, including the carrier frequency combination for single layer and/or two layers. |  |  |  |  |  |

Some parameters, e.g., total transmission power per BS, are still open. The following FL proposal was discussed during RAN1#122bis meeting without consensus [8].

(FL4) Proposal on BS Tx power – Afteroffline (unstable)

The following total transmit power per BS for system-level simulations can be used for 6GR evaluation:

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 15GHz | BS transmit power is 23 dBm per 20 MHz, EIRP not exceed 58dBm | Macro BS: BS transmit power is 40 dBm per 20 MHz, EIRP not exceed 73dBm Micro BS: 33 dBm per 20 MHz, EIRP not exceed 68dBm |  | Macro BS: BS transmit power is 43 dBm per 20 MHz, EIRP not exceed 78dBm Micro BS: 33 dBm per 20 MHz, EIRP not exceed 68dBm | BS transmit power is 43 dBm per 20 MHz, EIRP not exceed 78dBm |
| Around 30GHz | BS transmit power is 23 dBm per 100 MHz, EIRP not exceed 58dBm | Micro BS: 33 dBm per 20 MHz, EIRP not exceed 68dBm |  | Micro BS: 33 dBm per 20 MHz, EIRP not exceed 68dBm |  |
| Note: BS Tx power scales up with bandwidth proportionally. Note: The maximum BS Tx power for each scenario will be defined. FFS values. |  |  |  |  |  |

One issue with the above proposal is the assumption of total Tx power per BS for 4GHz and 7GHz in Rural/Dense urban/Urban Macro. In the above table the same total Tx power per BS as 2GHz is assumed, e.g., 49dBm per 20MHz. However, in previous NR evaluation, total Tx power per BS for 4GHz is assumed to be 3dB lower than 2GHz due to the fact that 4GHz is typically deployed as a TDD band and strict regulatory limit is usually assumed for TDD considering shared spectrum and coexistence. It is because TDD system relies more on beamforming and antenna gain rather than more transmission power for improving overall system performance. For example, according to TR 38.830, 33dBm@MHz PSD is assumed for TDD 4GHz in Rural while 36dBm@MHz is used for FDD 700MHz and 2GHz frequency in Rural. For enabling fair comparison with 5G NR, it is suggested to follow the same assumption for 6GR evaluation that TDD band has 3dB lower total transmit power than FDD for the same deployment.

The maximum allowed EIRP value should be also considered. For the deployment at 7GHz, due to larger system bandwidth and more antenna elements, the EIRP value is significantly increased compared to 5G NR as summarized in the following table. It can be seen that the EIRP value can be up to around 97 dBm for 2048 total antennas elements much larger than the existing EIRP limit of 73dBm assumed for urban macro scenario. Therefore, it should be studied whether the existing EIRP limit can be reused for 6GR evaluation and what are the maximum allowed EIRP values for 7GHz deployment.

Table 1 Example of EIRP values for 7GHz deployment in Urban Macro

|  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| Channel bandwidth (MHz) | 400 | 400 | 400 | 400 |
| Downlink PSD (dBm/MHz) | 33 | 33 | 33 | 33 |
| Total Tx power for carrier bandwidth (dBm) | 59 | 59 | 59 | 59 |
| Number of transmit antennas | 256 | 1024 | 1536 | 2048 |
| Antenna element gain (dBi) | 8 | 8 | 8 | 8 |
| Total antenna gain (dB) | 32 | 38 | 39.8 | 41.1 |
| Cable loss (dBi) | 3 | 3 | 3 | 3 |
| EIRP (dBm) | 88 | 94 | 95.8 | 97.1 |

Proposal 1: Consider the following assumption on total transmit power per BS for 6GR evaluation.

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | N/A | Macro BS: 44 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: 44 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: 41 dBm per 20 MHz | 46 dBm per 20 MHz | Macro BS: 46 dBm per 20 MHz | 46 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: 41 dBm per 20 MHz | 46 dBm per 20 MHz | Macro BS: 46 dBm per 20 MHz | 46 dBm per 20 MHz |
| Around 30GHz | 23 dBm per 100 MHz | 40 dBm per 100 MHz |  | 40 dBm per 100 MHz |  |
| Note: BS Tx power scales up with bandwidth proportionally. Note: The maximum allowed EIRP for each scenario will be defined. FFS values. |  |  |  |  |  |

## Antenna modeling

For BS antenna model, the antenna configurations for 700MHz, 2GHz, 4GHz, 7GHz and 30GHz have been agreed in last RAN1#122bis meeting. The remaining issue is the BS antenna configuration for 15GHz band. Based on RAN1#122bis discussion, there are two directions for 15GHz deployment. One is to follow the lower band deployment using large number of TXRUs, and the other is to have a small number of TXRUs and analog beamforming similar to FR2 30GHz band.

We think the latter one makes more sense considering 15GHz is often treated as part of the extended mmWave range and used for short-range deployment. If there is interest in deploying 15GHz band co-sited with existing FR1, then more TXRUs are needed for improving coverage, e.g., using 64 to 256 TXRUs. To reduce cost and complexity and improve power efficiency, hybrid beamforming can be considered. Therefore, we propose to support hybrid beamforming with large number of TXRUs for outdoor deployment and analog beamforming with small number of TXRUs for indoor deployment for 15GHz band. The possible BS antenna configurations are listed in Table 2.

Table 2 BS antenna configurations for 15GHz deployment

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 128 | 8 | (8, 8, 2, 1, 1; 1, 1) | (0.5, 0.5)λ |
| Combination 2 | 512 | 32 | (8, 8, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 1024 | 128 | (32, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Combination 2 | 2048 | 256 | (16, 8, 2, 4, 2; 4, 4) | (0.5, 0.5)λ |

Proposal 2: Consider BS antenna configurations in Table 2 for 15GHz band

For the UE antenna model and antenna element gain pattern, two alternatives were discussed. One is to model the UE antennas as ULA with half wavelength spacing and isotropic radiation pattern, and the other is Rel-19 defined “handheld UT antenna model” in TR 38.901. The omnidirectional antenna has been the default model assumed for UE antenna(s) in previous 5G NR evaluation; however, it is too optimistic and does not correctly reflect the practical UE antenna design. The 6GR study should use other more realistic UE antenna model and antenna radiation patterns as the default in system-level simulations.

In the newly defined “handheld UT and CPE antenna model” in TR 38.901, candidate locations for UE antenna placement (i.e., 8 for handheld UT and 9 for CPE) are defined and each antenna is oriented in different directions using a directional radiation pattern. Rel-19 Handheld UT antenna model allows randomized per-antenna loss modeling and independent antenna imbalance modeling between uplink and downlink, which enables more accurate MIMO performance evaluation.

Figure 1 shows performance comparison between the legacy omnidirectional antenna and Rel-19 Handheld UT antenna model. Detailed evaluation assumption can be found in the appendix. From the figure it can be seen that Rel-19 antenna model leads to worse DL throughput compared to the omnidirectional antenna and the potential reason is that using Omni may be too optimistic in terms of e.g. achievable rank of the channel as shown in Figure 1.

Figure 1: Comparison of omnidirectional antenna and Rel-19 handheld UT antenna on DL performance

For CPE, 9 candidate antenna locations are defined assuming the array is 20cm x 20cm. However, for outdoor CPE mounted on rooftops or poles, larger array is feasible. In addition, when 16 antennas are considered for CPE, the current model may not support it unless the 16 antennas are 16 single-pol units. Therefore, further extension of CPE antenna model for larger array and more than 8 antennas may be needed.

Based on the above, we propose to use the Rel-19 defined handheld UT and CPE antenna model as the default in system level simulation. For link level simulation, e.g., channel coding or waveform evaluation using TDL channel model, the legacy omnidirectional antenna modeling can still be used.

Proposal 3: The Rel-19 defined handheld UT and CPE antenna model should be used as the default in system-level simulations. Legacy omnidirectional antenna could be used for link level simulation.

Proposal 4: Study extension of CPE antenna model for larger array and more than 8 antennas.

Regarding UE antenna configuration, in previous NR evaluation we usually assume 2Rx for <2GHz and 4Rx for >=2 GHz for handheld UT. Antenna configuration for CPE is typically double (so 4Rx for <2GHz and 8Rx >=2GHz). For 6GR, we can follow the same assumption for 700MHz, 2GHz and 4GHz as the default. On the other hand, we could also consider more UE antennas to make 6G better. For the new band 7GHz and 15GHz, more antennas should be supported as required for coverage improvement so up to 8Rx can be assumed for handheld UT and up to 16Rx for CPE. Table 3 below summarizes UE antenna configuration proposed for 700MHz, 2GHz, 4GHz, 7GHz and 15GHz, and the 30GHz antenna configuration is provided in Table 4.

Table 3: UE antenna configurations for 6GR evaluation

|  | 700 MHz | 2GHz | 4GHz | 7 GHz | 15 GHz |
| --- | --- | --- | --- | --- | --- |
| IoT device |  |  |  |  |  |
| Combination 1 | 1T1R | 1T1R | 1T1R | N.A. | N.A. |
| Combination 2 | 1T2R | 1T2R | 1T2R | N.A. | N.A. |
| Handheld UT |  |  |  |  |  |
| Combination 1 | 1T2R (baseline) | 2T4R (baseline) | 2T6R (baseline) | 4T8R (baseline) | 4T8R (baseline) |
| Combination 2 | 1T4R (optional) | 4T4R (optional) | 4T8R (optional) | 4T4R (optional) | 4T16R (optional) |
| CPE |  |  |  |  |  |
| Combination 1 | 2T4R | 4T4R (baseline) | 4T8R (baseline) | 8T8R (optional) | 8T8R (optional) |
| Combination 2 |  | 4T8R (optional) | 8T8R (optional) | 8T16R (baseline) | 8T16R (baseline) |

Table 4: 30GHz UE antenna configurations for 6G evaluation

| UE antenna configuration | Values |
| --- | --- |
| # of antenna elements | Up to 32 elements, 8 elements per panel (M, N, P) = (2, 2, 2) |
| # of panels | Config 1 can be 2 panels on front and back Config 2 can be 4 panels on 4 edges |
| # of TXRUs | 2T2R per panel |

Proposal 5: Consider UE antenna configuration in Table 3 and Table 4 for 6GR evaluation

# Traffic models

## Extension of FTP traffic model

In the RAN1#122bis, the following agreements on studying extension to FTP model 1/3 traffic models were made.

Agreement

Study extensions to FTP Model 1/FTP Model 3 to incorporate the following:

- Multiple packet sizes and associated time-domain behaviors (e.g., inter arrival time)

  - FFS number of packet sizes (e.g., 2 or 3).

  - FFS whether to have fixed or variable packet size and packet arrival rate for a given UE.

  - FFS applicability of multiple packet sizes to only one or both of FTP Model 1/FTP Model 3.

  - FFS packet size and arrival rate characteristics.

- Packet delay budget (PDB) related parameters

  - FFS PDB applicability to packets (e.g., one PDB parameter for only one traffic flow or different PDB parameters for different traffic flows).

  - FFS how to consider the PDB, e.g., whether to drop packets when exceeding the budget, PDB aware metric.

- Note consider the following for PDB:

  - Applicability to the extension to FTP Model 1/ FTP Model 3 with one packet size.

  - Applicability or not to the extension to FTP Model 1/ FTP Model 3 with multiple packet sizes

Full buffer and FTP based traffic models have been widely adopted in 3GPP evaluations from LTE through 5G NR. Full buffer model assuming infinite packet size is very effective for evaluating high-capacity scenario and link level performance. FTP models, typically defined with fixed file sizes (e.g., 0.5MB) and dynamic inter-arrival time based on exponential or Poisson distribution are more suitable for simulating dynamic traffic patterns. For 6G evaluation, these models remain useful. Considering new 6G requirements, parameter extensions of the existing FTP traffic model can be considered. For example, larger file sizes (e.g., 2-5 MB) can be considered for high data rate services, such as high-resolution video streaming. Also, shorter inter-arrival time may be needed to simulate ultra-dense traffic.

One reason to extend FTP Model 1 and 3 is to better reflect the variety of traffic types seen in real smartphone usage. Mobile traffic is often bursty and includes a mix of services, messaging, video etc. all generating packets of different sizes within the same burst. But in FTP Model 3, the current setup assumes every UE uses the same packet size and arrival rate, which doesn’t capture this diversity.

A straightforward way is to let each UE have its own packet size and arrival rate as shown in Figure 2. These values could be randomly selected from a predefined set or based on a probability distribution. This setup allows the network to simulate a mix of traffic types across different UEs. However, it still limits each UE to a single packet size, which isn’t ideal. In practice, a single UE might be running multiple apps or services simultaneously, each with different traffic characteristics.

Figure 2: Extended FTP model 3 with mixed packet sizes across UEs

Hence, it makes sense to allow packet sizes to vary over time even for the same UE. For the extended model illustrated in Figure 3, packets still arrive according to a Poisson process, and each packet’s size is independently selected. This approach may better model the mixed-service traffic and enable more accurate performance evaluation.

Figure 3: Extended FTP model 3 with mixed packet sizes per UE

However, the above model may not fully support multiple QoS flow bursty traffic where different QoS flows have different data rate requirements and their packets often arrive around the same time rather than in a TDM manner. To simulate more realistic bursty traffic models in 6G, it would be useful to consider a general burst traffic where each burst contains multiple series of packets having variable packet sizes.

As illustrated in Figure 4, bursts arrive according to a Poisson process. Each burst includes a number of packets with mixed sizes. Within each burst, packets are separated by a short interval T, typically between 0.5 and 1 ms. The number of packets per burst could be fixed for a given UE and the burst size (i.e., the sum of packet sizes within a burst) may vary over time. If all UEs have the same number of packets per burst (e.g., N1 = N2 in Figure 4), then the model resembles the extended FTP model in Figure 3 except that the individual packet in Figure 3 is replaced by a burst with multiple series of packets. If the number of packets per burst differs across UEs (e.g., N1 ≠ N2 in Figure 4), then the average burst size will vary accordingly, reflecting different traffic profiles used for UEs as the model in Figure 2.

Therefore, the proposed model offers a flexible and scalable way to evaluate general burst traffic, and a single parameter (i.e., the number of packets per burst) could control the UE common or specific traffic profile evaluation

Figure 4: Extended FTP model 3 for general burst traffic with multiple packets having variable sizes

To implement the extended FTP model 3, the first step is to define multiple packet sizes. There are two main approaches to this:

- One option is to treat packet size as a random variable drawn from a predefined distribution—such as a truncated Gaussian, which is commonly used in XR traffic models.

- The other approach is to define a fixed set of packet sizes (e.g., small, medium, and large) and assign a selection probability to each.

While the first method introduces variability, it can be difficult to control the proportion of small and large packets during evaluation. The truncated Gaussian distribution produces packet sizes that are relatively close together, which limits its usefulness when trying to simulate a wide range of traffic types. The second method, on the other hand, aligns more closely with the existing FTP model, where packet sizes are predefined. It also makes it easier to control the offered traffic by adjusting the number of UEs per cell and the selection probabilities for each packet size.

Next is to define the packet (or burst) arrive rate for the UE. The packet (or burst) arrival rate can be a single value shared across all packet sizes or vary depending on the size. While it’s reasonable to expect smaller packets to arrive more frequently than larger packets, using multiple arrival rates adds complexity to the evaluation. Specifically, it becomes harder to determine the right combination of arrival rates to achieve a target load level.

Currently, in FTP Model 3, the number of UEs is fixed, and the packet arrival rate is tuned through trial simulations to meet the desired load. Introducing multiple arrival rates would significantly increase the number of combinations to test, making the simulation setup more complicated and time-consuming. Therefore, we prefer using a single packet arrival rate across all packet sizes to keep the model manageable while still capturing key aspects of traffic diversity.

It is noted that with the single packet arrival rate, the same approach used by the existing FTP model 3 can be reused to determine the packet arrival rate for the target load level. That is, we could firstly calculate the average packet size according to the predefined probabilities for multiple packet sizes, i.e., S=ipiSi. For trial simulations we use  S for all the UEs to find the desired packet arrival rates for the target offered load. This could significantly reduce the efforts to find the required packet arrival rates.

Proposal 6: For extension of FTP model 3 for enabling mixed packet sizes, consider the following modeling

- Total number of UEs per cell is fixed, e.g., 10

- Each UE has multiple bursts that arrive according to a Poisson process. A single arrival rate (or mean inter-arrival time between bursts) is used for all the UEs, determined based on the target offered load (resource utilization ratio).

- Each burst contains N≥1 packets with mixed and variable sizes where N can be common for all UEs, or different based on the setting.

- Two or three packet sizes are predefined. The packet sizes are chosen so that the large packet size is much larger than the smaller packet size. For example, the large packet size is 0.5 Mbytes and the smaller packet size is several hundred Kbits.

- Packets within the burst are separated by a short interval T, typically between 0.5 and 1 ms. The interval can be fixed during the simulation

- For N packets within a burst, UE chooses the packet size independently for each packet according to a predefined probability αi, i.e., iαi=1. The same probability distribution is used for all UEs.

## Extension of XR traffic model

Following was agreed on traffic models for immersive communication services in RAN1#122bis.

Agreement

Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services,

- XR traffic models (in TR 38.838) are considered as starting point.

  - FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc.

- FFS how many models need to be defined and the corresponding representative use cases.

- FFS how to incorporate haptics traffic (TR26.854).

Send LS to SA4 requesting input if any on the relevant traffic characteristics, RAN1 can continue the study before SA4 potential response.

In the existing XR traffic models defined in TR 38.838, video frames are assumed to arrive at approximately 60 frames per second (fps), with each frame size following a truncated Gaussian distribution. To simulate XR applications involving multiple QoS flows—such as video, audio, and haptic feedback—multi-stream downlink traffic models are also supported. However, these models currently treat each stream independently, without accounting for inter-stream dependencies.

Modeling the relationships between multiple streams directly is hard, e.g., defining cross stream synchronization and temporal alignment. To address this challenge, the proposed general burst traffic model in section 3.2 can be modified for better supporting multi-QoS flow as required for immersive communication evaluation.

Similar to video frame arrival in existing XR model, bursts containing mixed packet sizes arrive roughly every 16.67 milliseconds (i.e., with random jitter). Each burst includes a combination of multiple large and small packets: large packets represent video data, while smaller packets correspond to audio or haptic streams. Multiple packets in a burst arrive back-to-back with short inter-arrival times (e.g., 0.5–1 ms). The number of packets within a burst will be upper limited by a number. The short interval between packets and the upper limit are used to emulate tight cross flow synchronization. For example, for a burst with 10 packets and 1ms packet interval, the largest delay is 10ms, which can meet 12 ms latency requirement for haptic stream specified in TR 26.854.

Therefore, we propose the following extension of XR traffic model for immersive communication service evaluation.

Proposal 7: For modeling of immersive communication services, the XR traffic model is extended as the following

- Burst with mixed packet sizes arrive periodically roughly at 60fps with random jitter

- Each burst consists of multiple packets including both small and large packet sizes. FFS the packet sizes

- The packets within the burst arrive back-to-back with short inter-arrival time (e.g., 0.5-1msec)

- The number of packets per burst can be fixed or random with an upper limit. The distribution of different packet sizes within the burst can be determined based on a predefined probability.

## Traffic models for AIML services

Following was agreed on traffic models for AIML services in RAN1#122bis.

Agreement

For the study traffic model(s) for 6GR AI/ML services:

  - A representative AI/ML service is the generative AI, e.g., as defined in TR22.870.

Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services.

Note: RAN1 is discussing the following options for the model:

  - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget.

    - Token is the minimum unit of data generated in the application layer.

    - How to associate Tokens to PHY layer packets.

    - How to reflect the variable importance of tokens.

    - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay.

  - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc.

  - Option-1c: reusing or extending the FTP-3/XR traffic model.

  - FFS other models/options need to be defined for other AI/ML services.

Emerging AI/ML-derived services may introduce traffic patterns different from legacy FTP/XR model. One example is token-based communications in generative AI as discussed in [9] and [10]. Tokens represent semantic units of text, images, audio or video signals that are processed or generated by AI models. Token based communication moves from raw data delivery (bits or packets) to token sequences representing semantic meaning for intelligent information exchange between the user and the AI model, or between distributed AI agents.

In last RAN1 meeting, there were lots of discussions on the traffic models used for evaluating the 6G AI/ML services. The main debate is whether to introduce a token-based traffic model where token size, arrival rate and delay budget are explicitly defined. Some companies view that the existing FTP/XR model (or extended model) are packet based, which may not be sufficient to evaluate token-based communications, e.g., high token error tolerance and differentiated importance for different tokens.

According to [9] [10], each token may have few bits (e.g., 12 bits for image token) representing an index in a large codebook that is built during tokenizer training. Tokens may arrive periodically or a burst dependent on the use cases. For example, for single video streaming, around 512 or 1024 tokens arrive every 1/60 or 1/120 seconds.

Since tokens are application layer data, they cannot be transmitted directly in raw form over the wireless network. Doing so is neither technically feasible nor resource and power efficient. As one example, if we assume 512 video tokens are uniformly generated every 1/60 second, then an average of 32 tokens arrive per millisecond. If each token is individually mapped to one physical channel for transmission, total 32 parallel physical channels need to allocate within that 1ms for transmitting these tokens over the air. In practice, it is not possible to schedule such large number of parallel transmissions within one subframe. If multiple tokens can be aggregated and mapped to one physical channel for transmission, then what is the difference to the packet-based modeling?

Moreover, dedicating a physical channel for transmitting just 10-20 bits per token is highly inefficient in terms of both power and resource utilization, especially the volume of tokens scales up. This inefficiency would make direct transmission of token over physical channel impractical.

In practice, tokens are typically encrypted and encapsulated within standard protocol layers, e.g., embedded into RLC/MAC frames. Furthermore, tokens of varying types (e.g., video, haptic) or tokens with differing importance can be mapped to distinct QoS flows for optimizing transmission of essential tokens.

The traffic model for tokenized generative AI applications can be effectively characterized using the general burst traffic model with mixed packet sizes, as outlined in Sections 3.2 and 3.3. In this framework, bursts represent tokenized multimodal information and may arrive either periodically or according to a Poisson process. Each token is mapped to a packet within the burst, and multiple tokens can be encapsulated into one packet, with its size being an integer multiple of the token size.

Packet sizes within a burst can be uniform or variable, depending on the specific application requirements. We can still assume the packets within the burst arrive back-to-back with short inter-arrival time since they may belong to the same semantic information

Observation 1: Tokens cannot be transmitted directly in raw form over the network. Neither technically feasible nor resource and power efficient

Observation 2: Tokens are typically encrypted and encapsulated within standards protocol layers, e.g., embedded in RLC/MAC frames for transmission over the air

Observation 3: A traffic model parameterized by Token is not necessary.

Proposal 8: The traffic model for tokenized generative AI applications can be effectively characterized using the general burst traffic model with mixed packet sizes

- Bursts represent tokenized multimodal information and may arrive either periodically or according to a Poisson process.

- Each token is mapped to a packet within the burst, and multiple tokens can be encapsulated into one packet, with its size being an integer multiple of the token size

- Packet sizes within a burst can be uniform or variable. The packets within the burst arrive back-to-back with short inter-arrival time (e.g., 0.5-1msec)

# Additional assumption for MIMO evaluation

The 6GR study strives to focus on practical user experience with acceptable performance/complexity [1].  To achieve this goal, the evaluation assumptions should be able to capture the real-world impairments.  For MIMO evaluation, based on the features of interest, several modeling aspects should be considered.

## Link- vs system-level evaluation

Traditionally, link-level simulation is mainly used for the performance assessment of reference signal designs, while the system-level simulation is widely used for evaluating the benefits of various MIMO operation schemes.  However, the modeling of channel estimation and the MIMO receiver in system-level simulation is significantly simplified.  Overly simplified models may not be able to correctly capture some important effect, e.g., the aliasing due to low reference signal density in frequency-domain, and thus shadow the impact caused by some designs.

Proposal 9: Adopt link-level simulation for evaluating MIMO designs, at least for reference signals and CSI feedback.

## DL MIMO

In NR evaluation, linear MMSE receivers or linear MMSE IRC receivers are typically assumed in the system-level evaluation.  The advanced receivers, e.g., the reduced complexity maximum likelihood (R-ML) receivers [4], have been widely available in the real-world UE devices.  The 6G design and performance evaluation should take the commercially available advanced receivers into account.

Proposal 10: Assume UE is equipped with the R-ML receiver in both link- and system-level evaluation.

Coherent joint transmission (CJT) has been studied in both LTE and NR; however, commercial deployment is very rare. Considering the cost for dense deployment of multiple TRPs, we believe the primary consideration should be outdoor scenario based on co-site deployment on macro-cell existing grid.

Proposal 11: The primary scenario for CJT evaluation should assume outdoor co-site deployment on macro-cell existing grid, rather than dense multi-TRP deployment.

In the past, it was assumed that even with non-ideal backhaul, the multiple TRPs can be perfectly synchronized, with the aid of certain UE feedback.  However, it won’t be the case in the real-world where there will also be some residual error after the calibration/compensation.  To objectively evaluate the performance benefits and the design complexity, the residual synchronization error and delay offset should be properly modeled.

Proposal 12: For CJT evaluations, the residual synchronization error and delay offset should be properly modelled.

To get meaningful and unbiased evaluation results, the use of channel model matters.  Essentially, the channel model needs to provide sufficient “degrees-of-randomness” with respect to the designs/KPIs to be evaluated.  For example, to the open-loop schemes, it is important to make sure that the 1st order statistics of the channel (e.g., the distribution of the channel amplitude at any given time instance) is properly modeled (e.g., follows Rayleigh distribution).  When evaluating the impact of channel aging on closed-loop schemes, the 2nd order statistics (e.g., temporal correlation of the fading channel) needs to be correctly captured by the channel model.

For time-domain CSI extrapolation/prediction, it is critical to properly capture the stochasticity across time.  There are two ways to model fading process with certain time-domain correlation:

- model the fading process as the sum-of-sinusoids; and

- filter white Gaussian process by a Doppler spectrum shaping filter.

The former is known as the “deterministic process” approach which has been adopted in the CDL model, while the latter creates a stochastic process.  In a deterministic process, each ray is modeled by a sinusoid of which the amplitude, frequency, and initial phase are deterministic functions of the parameters of the ray, including AoA, AoD, ZoA, ZoD, power, initial phase, etc. of the ray and the field response at both end of the link.  Once these parameters are determined, the sum-of-sinusoids become a deterministic function.  Hence, the channel state in any given time instance is predetermined by these parameters.  So, one can determine the fading coefficient at any time instance once these parameters are determined.  If an extrapolation model can fit such deterministic functions, one can easily compute rather than “predict” the channel state at any time instance including that far beyond the channel coherence time.  This is not the case in the real world.  On the other hand, the filter-based approach retains randomness because of the white Gaussian input.  To avoid overestimating the benefits of time-domain prediction, the stochastic channel models must be used.

Proposal 13: For time-domain CSI extrapolation, deterministic channel models (e.g., CDL-like) shall not be used.  The stochastic channel models (filtering white Gaussian process by a Doppler spectrum shaping filter) shall be used to avoid overestimating the potential of time-domain prediction.

In general, the evaluation methodology itself should avoid being overfit to or favoring a specific type of design.  This applies to both AI and non-AI schemes.

Proposal 14: The evaluation methodology shall avoid being overfit to or favoring a specific type of design.

In Rel-19 channel modeling study item [5], a near-field channel model extension was developed.  Considering the practical deployment scenarios, especially the minimum distance between UE and basestation and the feasible antenna array size, clearly in most cases UEs are still operating in the far-field region.  It is worth noting that the Fraunhofer distance shall not be the only metric to decide whether near-field model should be adopted or not.  This is because the near-field effect becomes less significant rapidly as the scatters are away from the boundary between the reactive region and the Fresnel region and in real world it is quite rare to have scatters anywhere within the reactive region of a basestation antenna.

Proposal 15: Far-field channel models are the baseline for MIMO evaluation.  The use of near-field channel models must be well-justified by the practical deployment scenarios.

## UL MIMO

In LTE and NR evaluation, isotropic radiation pattern and co-located cross-polarized antennas are assumed when using CDL channels.  The real-world UE antennas are quite different from such assumption.  The handheld devices operating at frequencies below 10 GHz are typically equipped with discrete antennas (e.g., inverted-F antennas).  Each of these discrete antennas exhibits a non-isotropic radiation pattern due to its unique shape, the housing, and the neighboring components.  It is also impractical to make co-located cross-polarized antennas available in such frequency range in handheld devices.

For FDD system, the same antenna is used for both UL and DL.  In some bands, the duplexing gap between the UL and DL can be as large as several hundreds of MHz, e.g., band n66.  For such bands, the radiation pattern for a given antenna at the UL frequency and DL frequency may be different.  If this is not properly modeled, we may overestimate the benefit of DL measurement-based antenna selection for UL MIMO.

In addition, the insertion loss for each antenna could be different due to the RF design.  As a sequel, the network may observe different RSRP for each antenna and thus leads to different channel estimation performance for each antenna.  This may impact the MIMO operations based on UL measurement.

If a MIMO operation assumes UE side Tx-Rx reciprocity, the proponent should clarify the requirements of the maximum allowable difference of relative phase and power errors between Tx and Rx.  RAN1 shall study the feasibility of such requirements to avoid making agreement based on infeasible requirement assumptions.

Proposal 16: UL MIMO evaluation needs to properly model UE sub-optimality, e.g., antenna imbalance between UL and DL, insertion loss imbalance among Tx chains, Tx-Rx calibration error, etc.

# Additional assumption for NTN evaluation

In [1], the scope of NTN in 6GR is captured as “Aim at a harmonized 6G Radio design for TN and NTN, including their integration”. In this section, we capture our initial view on evaluation assumptions for NTN.

One of the major hurdles during the NTN work in NR was the lack of a proper description of the antenna model at the satellite. The existing definitions in TR 38.821 do not model several limitations and flexibility of satellite payloads, especially those equipped with antenna arrays. These limitations were brought up in a later stage to 3GPP and resulted in some non-backward compatible changes (beam hopping). In our view, 6GR should not repeat the same mistake, and should consider the following gaps:

- The models in TR 38.821 do not include the limitation on total power / number of RF chains of the satellite. This limitation was studied in Rel-19, but the study was very limited and there is no proper documentation in a technical report of these limitations.

- The antenna modeling in 38.821 does not include any flexibility to allow the satellite to perform “beam shaping” (i.e., create beams of different sizes). The issue of wide/narrow beam was discussed in Rel-19 without conclusion, mostly caused by the lack of consensus on a proper model of the antenna array.

Both issues above would be solved by taking antenna arrays as a baseline for the satellite node, and characterizing both the TXRU virtualization model, number of Tx / Rx chains and power limitations. The simplified model based on Bessel function in TR 38.811 can be reused for simplified evaluations. Therefore, we propose the following:

Proposal 17: For NTN, RAN1 to model the satellite antenna as an array.

- TXRU virtualization model Option-2B in TR 36.897 (full-connection model) can be used as baseline.

  - The virtualization matrix W may include both amplitude and phase coefficients.

- Antenna array parameters:

  - Mg = Ng = 1.

  - Other values FFS (may be different for different satellite types)

- Total and per-beam transmit power (or PFD, when applicable) should be defined.

- For simplified evaluations, the Bessel function in 38.811 can be used.

Although RAN plenary has not agreed yet on the NTN bands to be considered for 6GR, there should be consensus on studying the MSS FR1 bands already specified in NR. For these bands and following the guidance in [1] to aim at a harmonized design, we propose to reuse the UE antenna models for TN in a similar frequency range.

Proposal 18: For NTN in FR1 (e.g. L/S-band), the UE antenna models are reused from TN “around 2GHz or 4GHz”.

Since the introduction of the satellite sets in TR 38.821 (more than 5 years ago), the space industry has evolved: advances in launch technology have reduced the cost of building a constellation, and the increase in demand for direct-to-mobile services is leading to satellites with more capabilities. For 6GR, RAN1 should reconsider the satellite parameters taken as baseline, studying the inclusion of more powerful satellites and new orbit types.

Proposal 19: For 6GR-NTN, RAN1 to discuss new satellite types, including the following:

- Inclusion of orbits lower than 600km (VLEO)

- Larger antenna gains than existing satellite types in TR 38.821

# Link budget for coverage evaluation

For 6G, it is important to take a coverage-first approach and focus on delivering exceptional coverage right from the first release. 6G new band (6-8 GHz) is anticipated to achieve comparable UL coverage to 5G mid-band (~3.5 GHz) with co-site deployment. RAN#109 agreed that link budget is used as the evaluation methodology for coverage, and in last RAN1 meeting two candidates of link budget template was agreed, where the first template is based on link budget template from TR 38.830 and the second one is a simplified version from TR 38.913 without consideration of antenna gain, cable loss, shadow fading margin etc.

One remaining issue on link budget template is the definition of MCL, and whether the link budget metric for coverage evaluation can be based on the MCL or other metrics like MPL/MIL.

In TR 38.830, MCL/MIL/MPL are defined as the following.

- MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2).

- MIL = Total transmit power – Receiver sensitivity – Tx loss – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain

- MPL = MIL – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gains.

From the above, it can be seen that the difference between MCL and MIL is how to include antenna gain. Currently for MCL, only beamforming gain by BS side TXRU is considered, and while MIL includes all BS and UE antenna gain. Tx and Rx cable loss are also included in the MIL. According to TR 38.830, BS antenna gain component 3 denotes analog beamforming gain (i.e., mapping from TXRU to antenna element) while BS antenna gain component 4 is element gain.

For antenna element gain, common values are proposed for both 5G NR and 6GR (e.g., 8 dBi). The potential issue is on analog beamforming gain, which may be dependent on various factors such as TXRU to antenna mapping, BS antenna downtilt, the cluster direction of angle etc. There is no agreement in the Rel-17 NR coverage enhancement study on how to compute BS antenna gain component 3 and it is concluded with companies reporting the values. Although there are divergent values for BS antenna gain component 3 reported by companies, the difference should not be too large since it is only analog beamforming gain. For coverage evaluation of co-site deployments between 5G NR and 6GR, it is expected that the same or similar value is used for both 5G NR and 6GR especially when the TXRU is mapped to the same number of elements. According to the agreements reached for the BS antenna configurations for 4GHz and 7GHz, the typical setting is one TXRU mapped to 4 vertical elements for both 5G NR and 6GR. Therefore, the impact of the BS antenna gain component 3 value will be small. It is expected that if comparable MIL values are observed for 5G NR and 6GR for co-site deployment, then the MCL values would be also comparable.

In last RAN1 meeting there was discussion to update the MCL to contain all BS antenna components 2+3+4. We think it is not needed since in such case the MCL will be more like MIL and defining two similar performance metrics is confusing.

Proposal 20: Reuse the MCL definition in TR 38.830 for 6GR link budget template

- MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2)

In current link budget template, some parameters are still open. In the following table, we provide our views on these parameter values.

Table 5 Updated Link budget templates with proposed values for 6GR coverage evaluation

| System configuration |  |
| --- | --- |
| Channel for evaluation | All DL and UL channels |
| Scenarios and Carrier frequency (GHz) | Prioritize 4GHz and 7GHz in Urban macro, Rural and dense Urban |
| BS antenna heights (m) | Reuse the assumption in TR 38.901 v19.0.0 |
| UT antenna heights (m) | Reuse the assumption in TR 38.901 v19.0.0 |
| Cell area reliability (%) | Reuse the assumption in TR 38.830 |
| Lognormal shadow fading std deviation (dB) | Reported by companies |
| Tx Diversity | Reported by companies |
| Number of SSB | Reported by companies |
| Transmitter |  |
| (1) Number of transmit antenna elements | Following the agreements for antenna configuration |
| (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink | Following the agreements for antenna configuration |
| (2a) Number of transmit chains modelled in LLS | For CDL, equal to number of TXRUs For TDL, reported by companies |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | Following the agreements for general SLS assumption |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | Following the agreements for general SLS assumption |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: no PSD constraint for uplink |  |
| (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) |  |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ((3c) /1000000) (dBm) |  |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) |  |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ((1) / (2)) (dB) for downlink, and = (4c) + 10 log ((1) / (2a)) (dB) for uplink |  |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) | Reported by companies (using 0dB for calibration purpose) |
| (4c) Gain of antenna element (dBi) | Reuse the assumption in TR 38.901 v19.0.0 |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink |  |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log((2)/(2a)) (dB) Note: zero for uplink |  |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | Reported by companies (using 0dB for calibration purpose) |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | Reported by companies |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antenna elements | Following the agreements for antenna configuration |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | Following the agreements for antenna configuration |
| (10b) Number of receive chains modelled in LLS |  |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) |  |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ((10)/(10a)) (dB) for uplink = (11c) + 10 log ((10)/(10b)) (dB) for downlink |  |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) | Reported by companies (using 0dB for calibration purpose) |
| (11c) Gain of antenna element (dBi) | Following the assumption in TR 38.901 v19.0.0 |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink |  |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log((10a)/(10b)) (dB) Note: zero for downlink |  |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | Reported by companies (using 0dB for calibration purpose) |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | Reported by companies |
| (13) Receiver noise figure (dB) | Following the agreements for general SLS assumption |
| (14) Thermal noise density (dBm/Hz) | -174 |
| (15) Receiver interference density (dBm/Hz) | Reported by companies |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) |  |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) |  |
| (19) Required SNR (dB) |  |
| (20) Receiver implementation margin (dB) | Reported by companies |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS | Reported by companies |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) |  |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) |  |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) |  |
| Calculation of available pathloss |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) | Reported by companies |
| (26) BS selection/macro-diversity gain (dB) | Reported by companies |
| (27) Penetration margin (dB) | Reported by companies |
| (28) Other gains (dB) (if any please specify) | Reported by companies |
| (29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB) |  |
| Range/coverage efficiency calculation |  |
| FFS: (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) |  |

Proposal 21: RAN1 discusses and agrees on parameters values in link budget template for 6GR coverage evaluation

# Conclusion

For general evaluation scenarios and assumptions, we have the following proposals:

Proposal 1: Consider the following assumption on total transmit power per BS for 6GR evaluation.

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | N/A | Macro BS: 44 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: 44 dBm per 20 MHz | 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: 41 dBm per 20 MHz | 46 dBm per 20 MHz | Macro BS: 46 dBm per 20 MHz | 46 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: 41 dBm per 20 MHz | 46 dBm per 20 MHz | Macro BS: 46 dBm per 20 MHz | 46 dBm per 20 MHz |
| Around 30GHz | 23 dBm per 100 MHz | 40 dBm per 100 MHz |  | 40 dBm per 100 MHz |  |
| Note: BS Tx power scales up with bandwidth proportionally. Note: The maximum allowed EIRP for each scenario will be defined. FFS values. |  |  |  |  |  |

Proposal 2: Consider BS antenna configurations in Table 2 for 15GHz band

Proposal 3: The Rel-19 defined handheld UT and CPE antenna model should be used as the default in system-level simulations. Legacy omnidirectional antenna could be used for link level simulation.

Proposal 4: Study extension of CPE antenna model for larger array and more than 8 antennas.

Proposal 5: Consider UE antenna configuration in Table 3 and Table 4 for 6GR evaluation

For traffic models, we have the following observations and proposals:

Proposal 6: For extension of FTP model 3 for enabling mixed packet sizes, consider the following modeling

- Total number of UEs per cell is fixed, e.g., 10

- Each UE has multiple bursts that arrive according to a Poisson process. A single arrival rate (or mean inter-arrival time between bursts) is used for all the UEs, determined based on the target offered load (resource utilization ratio).

- Each burst contains N≥1 packets with mixed and variable sizes where N can be common for all UEs, or different based on the setting.

- Two or three packet sizes are predefined. The packet sizes are chosen so that the large packet size is much larger than the smaller packet size. For example, the large packet size is 0.5 Mbytes and the smaller packet size is several hundred Kbits.

- Packets within the burst are separated by a short interval T, typically between 0.5 and 1 ms. The interval can be fixed during the simulation

- For N packets within a burst, UE chooses the packet size independently for each packet according to a predefined probability αi, i.e., iαi=1. The same probability distribution is used for all UEs.

Therefore, we propose the following extension of XR traffic model for immersive communication service evaluation.

Proposal 7: For modeling of immersive communication services, the XR traffic model is extended as the following

- Burst with mixed packet sizes arrive periodically roughly at 60fps with random jitter

- Each burst consists of multiple packets including both small and large packet sizes. FFS the packet sizes

- The packets within the burst arrive back-to-back with short inter-arrival time (e.g., 0.5-1msec)

- The number of packets per burst can be fixed or random with an upper limit. The distribution of different packet sizes within the burst can be determined based on a predefined probability.

Observation 1: Tokens cannot be transmitted directly in raw form over the network. Neither technically feasible nor resource and power efficient

Observation 2: Tokens are typically encrypted and encapsulated within standards protocol layers, e.g., embedded in RLC/MAC frames for transmission over the air

Observation 3: A traffic model parameterized by Token is not necessary.

Proposal 8: The traffic model for tokenized generative AI applications can be effectively characterized using the general burst traffic model with mixed packet sizes

- Bursts represent tokenized multimodal information and may arrive either periodically or according to a Poisson process.

- Each token is mapped to a packet within the burst, and multiple tokens can be encapsulated into one packet, with its size being an integer multiple of the token size

- Packet sizes within a burst can be uniform or variable. The packets within the burst arrive back-to-back with short inter-arrival time (e.g., 0.5-1msec)

For MIMO evaluations, we have the following observations and proposals:

Proposal 9: Adopt link-level simulation for evaluating MIMO designs, at least for reference signals and CSI feedback.

Proposal 10: Assume UE is equipped with the R-ML receiver in both link- and system-level evaluation.

Proposal 11: The primary scenario for CJT evaluation should assume outdoor co-site deployment on macro-cell existing grid, rather than dense multi-TRP deployment.

Proposal 12: For CJT evaluations, the residual synchronization error and delay offset should be properly modelled.

Proposal 13: For time-domain CSI extrapolation, deterministic channel models (e.g., CDL-like) shall not be used.  The stochastic channel models (filtering white Gaussian process by a Doppler spectrum shaping filter) shall be used to avoid overestimating the potential of time-domain prediction.

Proposal 14: The evaluation methodology shall avoid being overfit to or favoring a specific type of designs.

Proposal 15: Far-field channel models are the baseline for MIMO evaluation.  The use of near-field channel models must be well-justified by the practical deployment scenarios.

Proposal 16: UL MIMO evaluation needs to properly model UE sub-optimality, e.g., antenna imbalance between UL and DL, insertion loss imbalance among Tx chains, Tx-Rx calibration error, etc.

For NTN evaluations, we have the following proposals:

Proposal 17: For NTN, RAN1 to model the satellite antenna as an array.

- TXRU virtualization model Option-2B in TR 36.897 (full-connection model) can be used as baseline.

  - The virtualization matrix W may include both amplitude and phase coefficients.

- Antenna array parameters:

  - Mg = Ng = 1.

  - Other values FFS (may be different for different satellite types)

- Total and per-beam transmit power (or PFD, when applicable) should be defined.

- For simplified evaluations, the Bessel function in 38.811 can be used.

Proposal 18: For NTN in FR1 (e.g. L/S-band), the UE antenna models are reused from TN “around 2GHz or 4GHz”.

Proposal 19: For 6GR-NTN, RAN1 to discuss new satellite types, including the following:

- Inclusion of orbits lower than 600km (VLEO)

- Larger antenna gains than existing satellite types in TR 38.821

For link budget template, we have the following proposals:

Proposal 20: Reuse the MCL definition in TR 38.830 for 6GR link budget template

- MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2)

Proposal 21: RAN1 discusses and agrees on parameters values in link budget template for 6GR coverage evaluation

# References

RP-251881 “Study on 6G Radio”, NTT DOCOMO (Moderator), RAN #108.

Chair notes RAN1#122bis, Prague, Czech Republic, October 13 - 17, 2025, Aug 25th – 29th, 2025.

TR 38.914 “Study on 6G scenarios and Requirements (Release 20)”. V0.1.0

TS 38.306 “NR; User Equipment (UE) radio access capabilities (Release 18)”, v18.6.0.

TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19)”, v19.0.0.

TR 38.830 “Study on NR coverage enhancements (Release 17)”, v17.0.0.

RP-251881 “Study on 6G Radio”, NTT DOCOMO (Moderator), RAN #108.

R1-2507956, “FLS#4 on evaluation assumptions for 6GR air interface”, Moderator (Huawei), RAN1#122bis

R1-2507058, “Evaluation assumptions for 6GR air interface”, Huawei, HiSilicon, RAN1#122bis

R1-2507176, “Evaluation assumption for 6GR air interface”, OPPO, RAN1#122bis

# Appendix

Table: DL SLS Assumptions

| Parameter | Value |  |
| --- | --- | --- |
| Scenario | UMa (ISD = 500 m) |  |
| Carrier frequency | 3.5 GHz |  |
| Simulation BW | 100MHz (system BW) / 20MHz (simulation BW) |  |
| SCS | 30KHz |  |
| gNB down tilt angle (degree) | 96° |  |
| gNB antenna array | 16x8x2 = 256 antennas => 4x8x2 = 64 TxRUs |  |
| UE antenna layout | Omni antenna, 4Rx with M =1, N = 2, P = 2 | Rel-19 handheld UE antenna model, 4Rx at the 4 corners of the handheld UT |
| Traffic Models | Full buffer |  |
| Beamforming | SRS based eigen BF |  |
| UE ratio | 80% indoor (3 km/h), 20% outdoor (30 km/h) |  |