3GPP TSG RAN1 WG #124	 	                                                      R1- 2600935

Gothenburg, SE, Feb 9-13, 2026

Source:	Intel Corporation

Title:	LDPC extension for data rate beyond NR range

Agenda item:	10.3.1

Document for:	Discussion and Decision

# Introduction

In RAN1#122bis, the following working assumptions and agreements were made on data channel coding for 6G.

Working Assumption - Study 6G data channel coding for higher throughput than 5G with acceptable performance-complexity tradeoff for both NW side and UE side, - Target peak data rate is assumed to be 2 times of the target peak data rate defined in TR38.913 Agreement - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G LDPC - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed

In RAN1#123, the following agreement was made on LDPC extension.

Agreement For the study of LDPC extension for data rate beyond NR range with acceptable performance-complexity tradeoff, - To provide the initial version of LDPC BG(s) and PCM(s) in the excel spreadsheet by RAN1#124 - To provide the required SNR and complexity for target BLER, and the evaluation assumptions of the decoding algorithm - The definition of complexity will be further discussed - FFS: other metrics

In this contribution, we discuss LDPC extension for data rate beyond NR range, including LDPC structure and LDPC BG and PCM for data rate beyond NR range.

# Basic considerations on LDPC structure

NR LDPC was designed with balanced performance and computational/implementation complexity. Specifically, the following properties of NR LDPC enable efficient encoding and decoding:

- Quasi-cyclic structure. This naturally facilitates layered decoding (which is area/memory efficient) and parallelized row processing within each layer (which improves latency).

- Double-diagonal structure in the first 4 rows of core parity columns of BG, as illustrated in Figure 1 taking BG1 as an example. This structure enables efficient calculation of parity bits in encoding.

- Orthogonality between adjacent rows in the extended check rows. This property enables parallel processing of multiple layers in layered decoding for reducing decoding time.

- Identity submatrix in the extended parity check region. This allows flexible coding rate and efficient HARQ operation.

- Lift size is a multiplication of 2, i.e., Z=a×2j. This allows efficient implementation scalability of LDPC.

Figure 1 illustration on the double-diagonal structure in the first 4 rows of core parity columns of BG, taking BG1 as an example

In 6G, with the target for supporting higher data rate than 5G, it is crucial to continue emphasizing low computational/implementation complexity encoding and decoding in the LDPC design. Therefore, the quasi-cyclic structure and the double-diagonal structure shall continue to be applied in LDPC extension design for 6G.

Proposal 1: For LDPC extension beyond NR data rate, continue to have the following properties in code structure as in NR LDPC:

- Quasi-cyclic structure

- Double-diagonal structure in the first M rows of core parity columns of BG, in which the value M is up to further discussion.

- Orthogonality between adjacent rows in the extended check rows.

- Identity submatrix in the extended parity check region.

- Lift size is a multiplication of 2, i.e., Z=a×2j

# LDPC extension beyond NR date rate range

As discussed in previous RAN1 meetings, techniques to improve throughput for LDPC include:

- Increase number of parallelly processed CBs => This approach requires adding LDPC encoding and decoding modules. The LDPC code design can be the same as 5G NR.

- Increase lift size => This approach requires adding computing resources for parallelly conducting row operations (check node operation) within each LDPC encoding and decoding module. The BGs can be the same as 5G NR.

- Reduce number of edges in BG => This approach reduces computing time per iteration in decoding. A new BG design is required.

- Design a fast-converging code => This approach reduces number of iterations in decoding. A new BG design is required.

All above-mentioned techniques can achieve the peak data rate expected in 6G. The difference mostly lies in 1) whether new LDPC design is needed; 2) additional computing resource needed (in addition to 5G LDPC implementation).  Table 1 compares these techniques.

Table 1 Comparison between techniques for improving LDPC throughput

| Techniques | Comparison |
| --- | --- |
| Increase the number of parallelly processed CBs | - No new LDPC design and minimal additional development cost - Performance is expected to be the same as 5G NR - Requires more encoding/decoding modules for parallel CB processing (vs. 5G NR). - Computing resources for encoding/decoding can be shared between 5G and 6G. |
| Increase max lift size, reuse NR BG1 | - Does not require new LDPC BG design - Requires new PCM design with new lift sizes - Requires more computing resources for parallel row processing (vs. 5G NR). |
| Reduce number of edges in BG | - Requires new BG design and PCM design - May require increase on lift size - Requires new encoder/decoder modules for the new BG (vs. 5G NR) |
| Design a fast-converging code | - Requires new BG design and PCM design - Number of iterations decreases, while computing time per iteration tends to increase - Requires new encoder/decoder modules for the new BG (vs. 5G NR) |

When choosing which techniques to apply for LDPC extension beyond NR data rate, we prefer the ones that do not require redesign of the BG, i.e., directly reuse NR BG1 or use a subgraph of the NR BG and leverage parallel processing techniques to improve throughput. This results in the following schemes for supporting beyond NR data rate:

- Scheme 1: Increase the number of parallelly processed CBs. Reuse NR LDPC as is.

- Scheme 2: Use lift size 768 for the data rate region beyond NR range. Develop a new PCM for lift size 768 using NR BG1 or a subgraph derived from BG1.

Scheme 1 is the most straightforward scheme and is our top preference. If the required computing resource of Scheme 1 exceeds the acceptable system dimension, we can resort to Scheme 2. The choice of the lift size 768 is based on the following considerations:

- The high data rate region corresponds to high TBS values. Assuming 2 TBs per 14-symbol TTI of 0.5ms, for 60 bit/s/Hz spectral efficiency at 400 MHz channel bandwidth and 40% overhead, the TB size is 3.6×106 bits.

- For high TBS values, the max lift size is used. Figure 2 showed the lift size values vs. TBS. Three cases were shown:

  - The existing NR case, i.e., max CBS = 8448 and max lift size = 384

  - A case with doubled max lift size and doubled max CBS, i.e., max CBS = 16896, max lift size = 768.

  - A case with doubled max lift size and same max CBS as NR, i.e., max CBS = 8448, max lift size = 768.

For both Case b and Case c, the following lift sizes beyond NR values were assumed: [416   448   480   512   576   640   704   768]. It can be observed that for all cases, as TBS increases, only max lift size is used. In the NR case (Case a), for TBS ≥ 0.078 Mb, the max lift size of 384 is used. In Case b, for TBS ≥  0.078 Mb, the max lift size of 768 is used. In Case c, for TBS ≥  0.171 Mb, the max lift of 768 is used. Note that assuming 0.5 ms TTI, TBS = 0.078 Mb corresponds to 156 Mbps data rate and TBS = 0.171 Mb corresponds to 342 Mbps data rate, which are both well within NR data rate range.

Figure 2 Lift size vs. TBS

Based on the above observations, we believe the lift size 768 alone is sufficient for enabling data rates beyond NR. Regarding the PCM design for lift size 768, we can either reuse BG1 or use a subgraph of BG1 for higher mother code rate.

The above discussion also brought up a question on how to decide when to apply LDCP extension that requires either PCM change or BG change for the data rate beyond NR range. In NR, the applications of BG1 and BG2 are differentiated by CBS and coding rate. For 6G, since CB1 can already cover coding rate up to 11/12 and from spec perspective, there are no constraints for CB1 to achieve beyond NR data rate (as long as sufficient number of parallelly processed CBs are implemented), it is not clear when to use LDPC extensions that requires either PCM change or BG change. Based on our earlier discussions, one option is to use TBS to decide when to apply LDPC extensions, i.e., if TBS is greater than a threshold, LDPC extensions can be used.

Observation 1: If LDPC extension is based on lift size increase, using a lift size of 768 alone is sufficient to support data rates beyond the NR range.

Observation 2: The criterion on when to apply LDPC extensions needs to be discussed.

Proposal 2: Use following schemes for data rates beyond NR range with Scheme 1 as the primary option and Scheme 2 as an optional alternative.

- Scheme 1: Increase the number of parallelly processed CBs. Reuse NR LDPC BGs and PCMs as is.

- Scheme 2: Develop a new PCM for lift size 768 using NR LDPC BG1 or a subgraph derived from BG1.

Proposal 3: Define a TBS threshold, above which LDPC extension for data rate beyond NR range can be applied.

# Conclusion

In this contribution, we presented our views on LDPC extension for data rate beyond NR range. Based on the discussion we draw the following observations and proposals:

Observation 1: If LDPC extension is based on lift size increase, using a lift size of 768 alone is sufficient to support data rates beyond the NR range.

Observation 2: The criterion on when to apply LDPC extensions needs to be discussed.

Proposal 1: For LDPC extension beyond NR data rate, continue to have the following properties in code structure as in NR LDPC:

- Quasi-cyclic structure

- Double-diagonal structure in the first N rows of core parity columns of BG, in which the value N is up to further discussion.

- Orthogonality between adjacent rows in the extended check rows.

- Identity submatrix in the extended parity check region. Lift size is a multiplication of 2, i.e., Z=a×2j

Proposal 2: Use following schemes for data rates beyond NR range with Scheme 1 as the primary option and Scheme 2 as an optional alternative.

- Scheme 1: Increase the number of parallelly processed CBs. Reuse NR LDPC BGs and PCMs as is.

- Scheme 2: Develop a new PCM for lift size 768 using NR LDPC BG1 or a subgraph derived from BG1.

Proposal 3: Define a TBS threshold, above which LDPC extension for data rate beyond NR range can be applied.

# References

- RAN 1 #122bis Chairman’s Notes

- RAN 1 #123 Chairman’s Notes