3GPP TSG RAN WG1 #122			R1-250XXXX

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.13

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 8.13 Maintenance on LTE-based 5G Broadcast Phase 2

Document for:	Discussion, Decision

## Maintenance on LTE-based 5G Broadcast Phase 2

[122-R19-Broadcast] Email discussion on Rel-19 LTE-bad 5G Broadcast Phase 2 – Assunta (EBU)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Time-frequency interleaver design for MBMS dedicated cells

R1-2505232	Maintenance for LTE based 5G broadcast	Huawei, HiSilicon

R1-2505505	Discussion on Time-frequency interleaver design for MBMS dedicated cells	ZTE Corporation, Sanechips

R1-2505559	Time-frequency interleaver design for MBMS dedicated cells	Samsung

R1-2505645	Maintenance on LTE-based 5G Broadcast	Xiaomi

R1-2506194	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

R1-2506499	Feature lead summary #1 on TFI for LTE-based 5G Terrestrial Broadcast (maintenance)	Moderator(EBU)

Agreement

Adopt the following text proposal for TS 36.211:

******** start of TP *************

6.5.2 Frequency Domain interleaving

The input to the interleaver is the block of complex-valued symbols yp0, …, yp(Msymbap-1)  defined in clause 6.3.4.1 and the output from the interleaver is a block of complex-valued symbols yp0, …, yp(Msymbap-1).

<<unchanged text>>

Permute the rows , if RK-K+1>0, such that row ri is moved to row i where

ri=ri-1+dri-1≤R-d-1idRotherwise
d=RRRK-K+1

******** end of TP *************

Note: it can be discussed in a future meeting whether an essential correction is needed for the case where RK-K+1≤0.

Comeback for CR cover sheet

Agreement

The contents to be included in the MBMS-ROM-Info-19 are as follows:

MBMS-ROM-Info-r19 ::= SEQUENCE { mbms-ROM-Freq-r19 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r19 ENUMERATED {kHz15, kHz7dot5, kHz2dot5, kHz1dot25}, mbms-Bandwidth-r19 ENUMERATED {n6, n15, n25, n30, n35, n40, n50, n75, n100}

Agreement

To maintain backwards compatibility with legacy non-time-frequency-interleaved PMCHs associated an MCCH, frequency interleaving is not applied to MCCH and MSI.

Agreement

Additional contents to be included in the MBMS-ROM-Info-19 is as follows:

β and Nsoftref as described in TS 36.212, for MCH with time-interleaving