3GPP TSG RAN WG1 #122			R1-250XXXX

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.4

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 8.4 Maintenance on Solutions for Ambient IoT (Internet of Things) in NR

Document for:	Discussion, Decision

## Maintenance on Solutions for Ambient IoT (Internet of Things) in NR

[122-R19-A-IoT] Email discussion on A-IoT – Jingwen (CMCC)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2505159	Maintenance for Rel-19 Ambient IoT	Spreadtrum, UNISOC, TCL

R1-2505224	Maintenance on Rel-19 Ambient IoT	Huawei, HiSilicon

R1-2505245	Maintenance on Solutions for Ambient IoT (Internet of Things) in NR	Ericsson

R1-2505292	Remaining issues on Rel-19 A-IoT	CATT

R1-2505377	Text proposals for TS 38.291	vivo

R1-2505434	Discussion on remaining issues for Ambient IoT in NR	Xiaomi

R1-2505541	Maintenance issues on Rel-19 A-IoT	Samsung

R1-2505594	Discussion on remaining issues of Rel-19 Ambient IoT	ZTE Corporation, Sanechips

R1-2505662	Maintenance of AIoT	Ofinno

R1-2505697	Discuss on the remaining issues for Rel-19 A-IoT	TCL

R1-2505729	Text proposals for Rel-19 Ambient IoT	OPPO

R1-2505842	Maintenance on Rel-19 Ambient IoT	LG Electronics

R1-2505922	Maintenance on Solutions for Ambient IoT in NR	NEC

R1-2506008	Maintenance on Solutions for Ambient IoT in NR	Sharp

R1-2506077	TP for A-IoT physical layer functions for TS 38.300	Rapporteur (CMCC)

R1-2506435	TP for A-IoT physical layer functions for TS 38.300	Rapporteur (CMCC)

R1-2506078	Discussion on remaining issues of Ambient IoT (Internet of Things) in NR	CMCC

R1-2506277	Maintenance on solutions for Ambient IoT in NR	NTT DOCOMO, INC.

R1-2506340	Maintenance on Ambient IoT in NR	ASUSTeK

R1-2506486	Summary #1 for maintenance on solutions for A-IoT in NR	Moderator (CMCC)

Agreement

RAN1 adopts text proposal #3.1 for TS 38.291 Clause 6.1.2.1 and 6.2.2.1.

Text proposal #3.1

| Reasons for change | The subscript of the first CRC parity bit is missing. |
| --- | --- |
| Summary of change | The CRC parity bits π,π1,…, πL-1 is changed to π0,π1,…, πL-1. |
| Consequences if not approved | Undefined variable π in CRC attachment. |
| Text proposal | #### 6.1.2.1 CRC attachment Error detection is provided for the transport block through a CRC. The entire transport block shall be used to calculate the CRC parity bits. Denote the bits in a transport block delivered to L1 by a0,a1,…, aA-1, and the parity bits by ππ0,π1,…, πL-1, where A is the payload size and L is the number of parity bits. The lowest order information bit a0 is mapped to the most significant bit of the transport block as defined in clause 6.1.1 of TS 38.391 [3]. The parity bits shall be computed and attached to the transport block according to clause 8.1, by setting L to 16 bits if A>24; and by setting L to 6 bits otherwise. […] #### 6.2.2.1 CRC attachment Error detection is provided for the transport block through a CRC. The entire transport block is used to calculate the CRC parity bits. Denote the bits in a transport block delivered to L1 by a0,a1,…, aA-1, and the parity bits by ππ0,π1,…, πL-1, where A is the payload size and L is the number of parity bits. The lowest order information bit a0 is mapped to the most significant bit of the transport block as defined in clause 6.1.1 of TS 38.391 [3]. The parity bits are computed and attached to the transport block according to clause 8.1, by setting L to 16 bits if A>24; and by setting L to 6 bits otherwise. |

Agreement

RAN1 adopts text proposal # 3.4-v2 for TS 38.291 Clause 6.2.6.

Text proposal #3.4-v2

| Reasons for change | Mathematical notation for abbreviating a series of entries in Clause 6.2.6 is not consistent with other parts in the spec. |
| --- | --- |
| Summary of change | Change Chips χ=NSIP, …, MchipR2D-1 to Chips χ=NSIP, NSIP+1,…, MchipR2D-1 |
| Consequences if not approved | Mathematical notation for abbreviating a series of entries is not consistently written. |
| Text proposal | ### 6.2.6 Mapping to OFDM symbols Chips χ=0, 1,…,NSIP-1 are mapped in sequence to OFDM symbols lRE=χ/4. Chips χ=NSIP, NSIP+1, …, MchipR2D-1 are mapped in sequence to OFDM symbols lRE= χ-NSIP/Mchipsymb+2 in increasing order. |

Agreement

RAN1 endorses text proposal #7.1-v2 for TS 38.300 Clause 16.x.3.2.

Text proposal #7.1-v2

| Reasons for change | The current version of TS 38.300 does not include description of R2D postamble. |
| --- | --- |
| Summary of change | Add sub-section and description of R2D postamble. |
| Consequences if not approved | The spec is not complete and lack of R2D postamble content. |
| Text proposal | #### 16.x.3.2 R2D ##### 16.x.3.2.3 Postamble An R2D postamble is transmitted immediately after the PRDCH. |

R1-2506522	Draft LS on Ambient IoT Stage-2 TP

Agreement

The draft LS to RAN2 in R1-2506522 is endorsed. Final LS in R1-2506523.

Agreement

RAN1 adopts text proposal #4.2 for TS 38.291 Clause 7.1.3.

Text proposal #4.2

| Reasons for change | Based on RAN1 agreement a device should use the same modulation scheme for an inventory/command round. The case for command following CFA is missing. |
| --- | --- |
| Summary of change | Clarify that the device should refer to a corresponding PDRCH transmission for CFA for determining the modulation scheme in section 7.1.3. |
| Consequences if not approved | Unclear device behavior for modulation scheme determination. |
| Text proposal | ### 7.1.3 Device procedure for modulation scheme determination To determine the modulation scheme for the entire D2R transmission, the device shall: - if the PDRCH is for transmitting Msg1 or corresponds to a contention-free random access procedure - determine according to its implementation to use either OOK modulation or BPSK modulation - otherwise - use the same modulation as determined for transmitting the preceding PDRCH. |

Agreement

RAN1 adopts text proposal #4.4 for TS 38.291 Clause 7.2.2:

#### Text proposal #4.4

| Reasons for change | Device will use NTBSD2R to generate D2R transmission, which is not defined in Clause 7.1.1. For the timeline determination, only Toffset4, which is the time interval from the end of a R2D transmission to the starting time of the corresponding D2R time domain resource except for Msg1 and Msg3 transmission, needs TBS for calculating. However, NTBSD2R in clause 7.1.2 is introduced under the main-bullet that “If the D2R transmission is for a Random ID message (Msg1) or corresponds to a Random ID Response message (Msg2)”, which is contrary to the agreement, and can not be used for calculating Toffset4. |
| --- | --- |
| Summary of change | Add NTBSD2R in the last of parameters provided by higher layers in Clause 7.1.1. |
| Consequences if not approved | The spec regarding device procedure to determine the transmission time is not clear. |
| Text proposal | ### 7.1.1 Device procedure for D2R generation A device shall generate the D2R transmission using the following parameters provided by higher layers: <Unchanged parts are omitted> - the D2R transport block size in bytes, NTBSD2R <Unchanged parts are omitted> ### 7.1.2 Device procedure for transmission time determination A device shall upon receiving a PRDCH intended for the device in an R2D transmission ending in chip χendR2D=MchipR2D-1, perform a corresponding D2R transmission with chip χD2R=0 starting an amount of time TR→D after the end of chip χR2D= χendR2D according to the configuration received from higher layers. If the D2R transmission is for a Random ID message (Msg1) or corresponds to a Random ID Response message (Msg2) - the device shall determine the D2R transmission starting time using the following parameters determined by higher layers: - the set of NSFS≥1 potential small frequency shift factors Ri,SFS', i=1, 2, …, NSFS NTBSD2R - Tchip' is equal to the largest value among TbitD2R/(2×Ri,SFS') otherwise - Tchip'=TchipD2R. <Unchanged parts are omitted> |

Agreement (updated on Tuesday with cover sheet)

The following TP is endorsed:

| Reasons for change | Subscript j to index the whole set of D2R OOK and BPSK modulated symbols in the first sentence is not aligned with the range in the following text where j=0,1 only |
| --- | --- |
| Summary of change | Change xj to x in Clause 8.4.1 and 8.4.2. |
| Consequences if not approved | D2R modulated symbols generation is ambiguous. |
| Text proposal | ### 8.4.1 OOK In case of OOK modulation for small frequency shift factor RSFS, a single element vi is mapped to modulation symbols x xj according to: xj+2K=2j-1vi+1-j j=0, 1 K=0, 1, …, RSFS-1. ### 8.4.2 BPSK In case of BPSK modulation for small frequency shift factor RSFS, a single element vi is mapped to modulation symbols x xj according to: xj+2K=22j-1vi+1-2j j=0, 1 K=0, 1, …, RSFS-1. |

R1-2506487	Summary #2 for maintenance on solutions for A-IoT in NR	Moderator (CMCC)

Agreement

Task the TS38.291 editor to add a description of R2D OOK modulation.

Conclusion

The text proposal in section 3.6.1 of R1-2506487 is not pursued.

Conclusion

The two text proposals in section 3.7.1 of R1-2506487 are not pursued.

Conclusion

The text proposal in section 3.9.1 of R1-2506487 is not pursued.

Conclusion

The text proposal in section 3.11.1 of R1-2506487 is not pursued.

Conclusion

The text proposal in section 3.12.1 of R1-2506487 is not pursued.

Conclusion

The text proposals 1, 2, 3, 4, 5, 7 in section 4.5.1 of R1-2506487 are not pursued.

R1-2506488	Summary #3 for maintenance on solutions for A-IoT in NR	Moderator (CMCC)