3GPP TSG RAN WG1 #122			R1-250XXXX

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.7

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 8.7 Maintenance on channel modelling for Integrated Sensing And Communication (ISAC) for NR

Document for:	Discussion, Decision

## Maintenance on channel modelling for Integrated Sensing And Communication (ISAC) for NR

[122-R19-ISAC] Email discussion on channel modelling for ISAC – Yingyang (Xiaomi)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2505215	Maintenance for ISAC channel modelling	Huawei, HiSilicon

R1-2505293	Maintenance on channel modelling enhancements for ISAC	CATT, CICTCI

R1-2505384	Maintenance on ISAC channel modeling	vivo

R1-2505436	Text proposals to TR 38.901 on ISAC CM	Xiaomi, AT&T

R1-2505548	Remaining issues on channel modelling for Integrated sensing and communication in NR	Samsung

R1-2505651	Maintenance for ISAC channel model in TR 38.901	InterDigital, Inc.

R1-2505728	Maintenance on ISAC channel modelling	OPPO

R1-2505883	Maintenance on channel modelling for ISAC for NR	Apple

R1-2505974	Discussion on R19 ISAC Channel Model Maintenance	SK Telecom

R1-2506107	Calibration results for ISAC channel modelling	Sony

R1-2506125	TP for ISAC channel modeling	ZTE Corporation, Sanechips

R1-2506186	Maintenance on channel modelling for ISAC	Qualcomm Incorporated

R1-2506224	Draft 38.901 TP for updated Rel-19 ISAC Calibration Results	AT&T, Xiaomi

R1-2506225	ISAC channel model calibration results	Moderator (AT&T)

R1-2506462	ISAC channel model calibration results	Moderator (AT&T)

R1-2506252	Text proposal to TR 38.901 on ISAC CM	NIST, Xiaomi, AT&T

R1-2506261	Maintenance on ISAC Channel Model	Ericsson (China)

R1-2506473	Summary #1 on channel modelling for ISAC	Moderator (Xiaomi)

Agreement

TP #4.1-1 in section 4.1 of R1-2506473 is endorsed for TR 38.901 v19.0.0.

Agreement

TP #4.3-2-rev2 in section 4.3.1 of R1-2506473 is endorsed for TR 38.901 v19.0.0

Agreement

Update parameter component names in the Tables in Section 7.9.6.2 to align with the parameter component names in 7.9.2.1 i.e. respectively replacing A, B1, B2 with σM, σD, σS.

Agreement

TP #4.4-2-rev1 in section 4.4.1 of R1-2506473 for TR 38.901 v19.0.0.

Agreement

TP #4.4-4 in section 4.4.2 of R1-2506473 for TR 38.901 v19.0.0?

Agreement

TP #4.4-5 in section 4.4.3 of R1-2506473 for TR 38.901 v19.0.0.

Agreement

Add the missing square in the parameter cd in Section 7.9.4.2 step 4, i.e., maxd3D-cd2+(ch-hTX)2c,0

Agreement

TP #4.6-12 and #4.6-13 in section 4.6.5 of R1-2506473 for TR 38.901 v19.0.0

Agreement

TP #4.7-1 in section 4.7.1 of R1-2506473 for TR 38.901 v19.0.0

Agreement

The value for best sensing Tx-Rx pairs to be selected for the target should be set to

- N=2 for TRP-TRP monostatic

- N=1 for TRP-TRP bistatic

Add in the Table 7.9.6.3-1 the following row:

| STX/SRX selection | Best N = 4 STX-SRX pairs to be selected for the target, with the following exceptions for FR1: - For TRP-TRP monostatic sensing, N=2 - For TRP-TRP bistatic sensing, N=1 |
| --- | --- |

Agreement

TP #4.9-1 in section 4.9.1 of R1-2506473 for TR 38.901 v19.0.0.

Agreement

TP #4.8-7-rev2 in section 4.8.4 of R1-2506473 for TR 38.901 v19.0.0.

R1-2506474	Summary #2 on channel modelling for ISAC	Moderator (Xiaomi)

R1-2506475	Summary #3 on channel modelling for ISAC	Moderator (Xiaomi)

Agreement

The TP below is endorsed for TR 38.901 v19.0.0.

<Start of Changes> #### 7.9.5.5 Lower power clusters In addition to the clusters/rays in background channel generated in Clause 7.9.4.2, more clusters/rays using NLOS propagation condition can be generated between a sensing Tx and a sensing Rx for bi-static sensing or between a sensing Tx and a reference point r, where r=0, 1, 2 for mono-static sensing to simulate the weak rays in the background channel. The following procedure is applicable for bistatic sensing or for monostatic sensing. Step 1: Generate a first background channel Hu,sbk1τ,t for bi-static sensing and Hu,sbk1,rτ,t for referenc point r for mono-static sensing according to Clause 7.9.4.2. Step 2: Generate a second background channel Hu,sbk2τ,t for bi-static sensing and Hu,sbk2,rτ,t for reference point r for mono-static sensing according to Clause 7.9.4.2 including utilizing the same DS, ASA, ASD, ZSA, ZSD, Cθ, Cϕas used for the first step, if applicable, with the following revised parameters: 1) The number of clusters (N in Clause 7.5) is increased to 360; 2) The number of ray per cluster (M in Clause 7.5) is reduced to 1; 3) If LOS condition is assumed in Step 1, use NLOS condition to generate DS, ASA, ASD, ZSA, and ZSD values. Step 3: The power of the clusters/rays in the second background channel is scaled down by PmaxS110G10, and is added to the first background channel, where, G=-25 dB, PmaxS1 is the maximum power of the NLOS clusters in the first background channel for bi-static sensing and PmaxS1,r is the maximum power of the NLOS clusters for reference point r in the first background channel. The final background channel Hu,sbkτ,t is 7.9.5-15a for bi-static sensing mode and 7.9.5-15b for mono-static sensing. Hu,sbkτ,t=Hu,sbk1τ,t+PmaxS1 10G10 P1S110G10∙Hu,sbk2τ,t (7.9.5-15a) Hu,sbkτ,t=Hu,sbk1τ,t+r=02PmaxS1,r 10G10∙Hu,sbk2,rτ,t (7.9.5-15b) <End of Changes>

Agreement

The following TP is endorsed for TR 38.901 v19.0.0.

#### 7.9.4.2. Background channel <omitted text> Step 2: Generate 3 reference points (RPs) for the STX/SRX. Draw the 2D distance between the STX/SRX and each RP and the height of each RP respectively by ΓGαd,βd+cd and ΓGαh,βh+ch with parameters defined in Table 7.9.4.2-1/2. Note that Gαd,βd and Gαh,βh are random values generated by the shape-rate parameterized Gamma distribution fx¦α,β=βαΓ(α)xα-1e-βx. With uniform distribution within range -π,π, draw the LOS AOD (ϕLOS,AOD) between the STX/SRX and the first RP. The LOS AOD is further rotated by 23π and 43π to respectively derive the LOS AOD from the STX/SRX to the second and third RPs. Consequently, the 3D location of each RP can be calculated. <omitted text>

Working assumption

The following TP is agreed for TR 38.901 v19.0.0

--------------------------------------- Start of TP for TR 38.901 --------------------------------------- Step 4: Generate the channel between the STX/SRX and each of the 3 RPs using Step 3 to Step12 of Clause 7.5 and Clause 7.6.10 with parameters derived by Table 7.9.3-3, with updates as follows. - In Step 7 of Clause 7.5, the arrival angles ϕn,m,AOA,θn,m,ZOA are respectively equal to the departure angles ϕn,m,AOD, θn,m,ZOD. The rays in a cluster with θn,m,ZOA less than 50, 80 and 90 degrees respectively for scenario UMi, UMa and RMa are dropped. The dropping is not applicable to other sensing scenarios. - Step 8 of Clause 7.5 is not applicable, except that the random coupling of AOD angles n,m,AOD with ZOD angles θn,m,ZOD within a cluster n or within a sub-cluster in the case of two strongest clusters. Apply the same coupling in the corresponding arrival angles ϕn,m,AOA,θn,m,ZOA. - The absolute time of arrival in clause 7.6.9 is applied with d3D replaced by maxd3D-cd+(ch-hTX)2c,0, where hTX is the height of the STX/SRX. Three ∆τ are independently generated and respectively applied to the 3 channels between the STX/SRX and the 3 RPs. NOTE: In the UT monostatic sensing in UMa and UMi scenario, the ZOD offset should be set as 0. --------------------------------------- End of TP for TR 38.901 ---------------------------------------

Agreement

TP #4.5-2 in section 4.5.2 of R1-2506474 is endorsed for TR 38.901 v19.0.0, with the deletion of the last “and (n,m)≠(n',m')”

Agreement

The following TP is endorsed for TR 38.901 v19.0.0

--------------------------------------- Start of TP for TR 38.901 ---------------------------------------

Step 3: Assign NLOS propagation condition to the channel between the STX/SRX and each of the 3 RPs. Note: O2I is not applicable.

--------------------------------------- End of TP for TR 38.901 ---------------------------------------

Agreement

The TP below is endorsed for TR 38.901 v19.0.0

--------------------------------------- Start of TP for TR 38.901 --------------------------------------- 7.9.1 Scenarios <Unrelated part omitted> Table 7.9.1-3: Evaluation parameters for Human (indoor and outdoor) sensing scenarios Parameters; Indoor Values; Outdoor Values. Minimum 3D distances between STX/SRX and sensing target; For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 0 m For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858); For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distances between STX/SRX and the sensing target are 10 m for UMi scenarios and 35 m for UMa scenarios For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858). Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 1m; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, 1m. NOTE 1: N=0 may be considered for the evaluation of false alarm. --------------------------------------- End of TP for TR 38.901 ---------------------------------------

Agreement

Replace “between pairs of STX/SRX and sensing target” by ‘between pairs of STX/SRX and sensing target’ in TR38.901.

Agreement

In Table 7.9.6.1-3, the minimum distance between STX/SRX and sensing target should be:

- 	Min distances based on min TRP/UE distances defined in TR37.885

Agreement

The TP below is endorsed for TR 38.901 v19.0.0.

--------------------------------------- Start of TP for TR 38.901 --------------------------------------- 7.9.6 calibration Table 7.9.6.1-2. Simulation assumptions for large scale calibration for Human sensing targets Parameters; Indoor Values; Outdoor Values. Minimum 3D distances between STX/SRX and sensing target; For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 0 m For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858); For TRP monostatic and TRP-TRP bistatic sensing modes, the minimum 2D distances between STX/SRX and the sensing target are 10 m for UMi scenarios and 35 m for UMa scenarios For UE monostatic and UE-UE bistatic sensing modes, the minimum 2D distance between STX/SRX and the sensing target is 1 m (as specified in TR 38.858). <Unrelated part omitted> --------------------------------------- End of TP for TR 38.901 ---------------------------------------.

Agreement

The reference for calibrations R1-2504950 is replaced by R1-2506518 in TR38.901.

Comeback for draft CR incorporating all the endorsed TPs – Yingyang (Xiaomi)