3GPP TSG RAN WG1 #122		                                   R1-2506421

Bengaluru, India, Aug 25th – 29th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.11

Agenda Item:		9.11

Document for: 	Endorsement

  - UE features for MCE for NR Phase 3

Agreement:

No prerequisite FG is defined for FG 66-1 and 66-2

Agreement:

Update FG 66-1/2 as follows, i.e.:

- Any of the following carrier type and/or SCS is not supported for the set of cells reported in FG 66-1/2 (i.e., FG 66-1 and 66-2 are updated as follows:):

  - FR1 unlicensed band

  - FR2-2

- No more (carrier type, SCS) is supported for the scheduling cell in FG 66-1/2

- Confirm [and/or different carrier type] from FG name and component 2

- Remove “FFS: Whether this FG can indicate two sets of (carrier type, SCS) for the cells in the set as (FR2, 60kHz) and (FR2, 120kHz)” from FG 66-1 and 66-2

| 66-1 | Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS [and/or different carrier type] | 1. UE supports monitoring DCI format 1_3 for DL scheduling with different SCS [and/or different carrier type] for the cells in the set 2. Supported applicable combinations of the following from the band combination: - Two [or three] sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} - A set of (carrier type, SCS) for scheduling cell: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} FFS: Other set of (carrier type, SCS) for the cells in the set and/or scheduling cell FFS: Whether this FG can indicate two sets of (carrier type, SCS) for the cells in the set as (FR2, 60kHz) and (FR2, 120kHz) FFS: Whether/how to define separate FG for reporting three sets of (carrier type, SCS) for the cells in the set FFS: Whether/how to define separate FG for reporting unlicensed band in the sets of (carrier type, SCS) for the cells in the set FFS: Other component(s) |
| --- | --- | --- |
| 66-2 | Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS [and/or different carrier type] | 1. UE supports monitoring DCI format 0_3 for UL scheduling with different SCS [and/or different carrier type] for the cells in the set 2. Supported applicable combinations of the following from the band combination: - Two [or three] sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} - A set of (carrier type, SCS) for scheduling cell: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} FFS: Other set of (carrier type, SCS) for the cells in the set and/or scheduling cell FFS: Whether this FG can indicate two sets of (carrier type, SCS) for the cells in the set as (FR2, 60kHz) and (FR2, 120kHz) FFS: Whether/how to define separate FG for reporting three sets of (carrier type, SCS) for the cells in the set FFS: Whether/how to define separate FG for reporting unlicensed band in the sets of (carrier type, SCS) for the cells in the set FFS: Other component(s) |

Agreement:

Regarding the number of sets of (carrier type, SCS) for the cells in the set,

- Confirm 66-1/2 are for two sets of (carrier type, SCS) for the cells in the set, i.e.,

  - Remove “or three” in FG 66-1/2

  - Remove “FFS: Whether/how to define separate FG for reporting three sets of (carrier type, SCS) for the cells in the set” in FG 66-1/2

- Define separate two FGs (e.g., FG 66-1a and 66-2a, for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling, respectively), for the support of three sets of (carrier type, SCS) for the cells in the set

  - FG 66-1a and 2a have FG 66-1 and 2 as a prerequisite FG, respectively

Agreement:

Adopt the following update:

- Re-index FG 66-4y to 66-4

- Confirm “one or” in component 1 of FG 66-3 and 66-4

Agreement:

Regarding “FFS: 2. supported HARQ enhancement. details for component 2” in FG 66-3, adopt the following updates:

FFS: 2. supported HARQ enhancement. details for component 2 2. Type 1 and Type 2 HARQ codebook for supporting multi-cell multi-PDSCH scheduling with single DCI

Agreement:

Regarding “FFS: Whether/how to define component to report the supported maximum total number of schedulable PDSCHs (or PUSCHs) per DCI”, adopt Alt-2

- Alt-1: remove the whole FFS from FG 66-3 and 66-4

- Alt-2: add the following component in FG 66-3 and 66-4:

  - Add “Maximum number of PDSCHs that can be scheduled by a DCI format 1_3 for multi-cell multi-PDSCH scheduling” in FG 66-3

  - Add “Maximum number of PUSCHs that can be scheduled by a DCI format 0_3 for multi-cell multi-PUSCH scheduling” in FG 66-4

  - Candidate value for each of the components: {8, 12, 16, 32}

Regarding “FFS: Whether/how to define component to report the supported number of maximum schedulable PDSCHs (or PUSCHs) in a cell”, adopt Alt-1

- Alt-1: remove the whole FFS from FG 66-3 and 66-4

R1-2505197	NR Multi-carrier Enhancements Phase 2 UE features	Nokia

R1-2505251	Discussion on UE features for MCE	Spreadtrum, UNISOC

R1-2505349	UE features for Rel-19 Multi-carrier enhancements	Huawei, HiSilicon

R1-2505403	UE features for MCE for NR Phase 3	vivo

R1-2505450	Discussion on UE feature for Rel-19 Multi-carrier enhancements	Xiaomi

R1-2505570	UE features for multi-carrier enhancements	Samsung

R1-2505692	UE features for multi-carrier enhancement for NR Phase 3	Ericsson

R1-2505725	Discussion on UE feature for multi-cell scheduling with a single DCI	OPPO

R1-2505900	Views on UE features for Rel-19 MCE	Apple

R1-2506027	MCE UE features Phase 3	MediaTek Inc.

R1-2506128	Discussion on UE features for MCE for NR Phase 3	ZTE Corporation, Sanechips

R1-2506204	UE features for MCE for NR Phase 2	Qualcomm Incorporated

R1-2506292		Discussion on UE features for multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.