3GPP TSG RAN WG1 #122	                                   	R1-2506415

Bengaluru, India, Aug 25th – 29th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.8

Agenda Item:		9.8

Document for: 	Endorsement

  - UE features for NTN for NR Phase 3

Agreement:

Update FG 65-1-1 as follows, i.e., :

- Remove [FFS: Other components] in Components column

- Remove FFS in Prerequisite FGs column

- Confirm “and 5.2.3-1” in Note column

| 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial [cell selection or access] [FFS Other components] | FFS | FFS | N/A | FFS: UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial access | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 | Optional [with/without] capability signaling [FFS: A UE that supports Rel-19 NR-NTN must support this FG] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

For Component 1 in FG 65-1-1, support Alt-1 below:

- Alt-1: Confirm “cell selection”, i.e., “1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial [cell selection or access]”

- Alt-2: Confirm “access”, i.e., “1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial [cell selection or access]”

Agreement:

Regarding “Consequence if the feature is not supported by the UE” column in FG 65-1-1, adopt Alt-2 below

- Alt-1: UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial access

- Alt-2: UE does not support an additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection

- Alt-3: UE is not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160ms

Proposal 4.1.1-4:

- Remove “FFS: A UE that supports Rel-19 NR-NTN must support this FG” in “Mandatory/optional” column

- Confirm “optional with capability signalling”

Proposal 4.1.2-1:

Update FG 65-1-2 as follows.

- Confirm “of searchSpaceZero configured within MIB pdcch-ConfigSIB1” in Component 1

- Remove [and 5.2.3-1] from Note column

- Confirm “only” in Note Column

- Remove [FFS: Applicabilitiy of this FG to TN] from Note column

| 65-1-2 | PDCCH repetition for Type0 PDCCH CSS | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS [of searchSpaceZero configured within MIB pdcch-ConfigSIB1] FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition | FFS | FFS | N/A | PDCCH repetition for Type0 PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable [only] for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 [FFS: Applicabilitiy of this FG to TN] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4.1.2-2:

- Remove “FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS” from FG 65-1-2

- Define FG 65-1-3 with the following updates

| 65-1-3 | PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS | 1. Support reception of PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS | FFS | FFS Yes | N/A | FFS PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable [only] for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 [FFS: Applicabilitiy of this FG to TN] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4.1.2-3:

Update FG 65-1-2 as follows, i.e., merge FG 65-1-2 with FG for SIB1 PDSCH repetition

- FG name: Update such that “PDCCH repetition for Type0 PDCCH CSS and SIB1 PDSCH repetition within 20ms duration”

- Components column: Add “Support reception of SIB1 PDSCH repetition within 20 ms duration”

- Consequence column: “Either PDCCH repetition for Type0 PDCCH CSS or SIB1 PDSCH repetition within 20ms duration is not supported”

Proposal 4.1.2-4:

Update FG 65-1-2 and 65-1-3 as follows.

- Remove FFS from Prerequisite column

Proposal 4.1.2-5:

Update FG 65-1-2 as follows.

- Put “NoYes” in “Need for the gNB to know if the feature is supported” column

- Confirm “Optional without capability signalling” in mandatory/optional column

Proposal 4.1.2-A (TEI: common PDCCH repetition for TN):

Introduce the following FG (under TEI19)

| 67-X | PDCCH repetition for Type0 PDCCH CSS in TN [Common_PDCCH_rep_TN] | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1 in TN | FFS | FFS | N/A | PDCCH repetition for Type0 PDCCH CSS in TN is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2-1 in TS 38.101-1 | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 67-(X+1) | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS in TN [Common_PDCCH_rep_TN] | 1. Support reception of PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS in TN | FFS | Yes | N/A | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS in TN is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2-1 in TS 38.101-1 | Optional with capability signaling |

Proposal 4.1.5-1:

Introduce the following FG for the support of Msg4 PDSCH repetition for Rel-19 NR-NTN:

| 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition 2. Support of repetition factors 2 and 4 3. | [FFS] | YES | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | A UE that includes [XXX (signaling to be defined in 38.321)] must support FG 65-1-5 Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4.1.5-2 (unstable):

Companies are encouraged to share views on the following alternatives on prerequisites of FG for Msg4 PDSCH repetition:

- Alt-1: One of {5-17a and 16-2b-5}

- Alt-2: None

Proposal 4.2-1:

Update FG 65-2-1 as follows:

- Confirm FG name, Component 2, the third Note, and the fifth Note as they are

- Remove “FFS: whether to divide this FG into two separate FGs between case 3 and case 4”

- Remove “FFS: A UE that supports FG 28-3 in NTN band must support this FG”

| 65-2-1 | Handling collision Case 3 [and collision Case 4] for half-duplex FDD operation for (e)RedCap UE | 1. Support handling collision Case 3 [2. Support handling collision Case 4] [FFS: whether to divide this FG into two separate FGs between case 3 and case 4] | FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band |  | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission [Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission] Note: this FG is for FR1 FDD only Note: collision Case 3 handling is for RRC CONNECTED and INACTIVE states[, and collision Case 4 is for RRC CONNECTED state only] | Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4.3-1:

Update FG 65-3-1 as follows

- Type column is per band

Proposal 4.3-2:

Companies are encouraged to share views on the following alternatives on component 4:

- Alt-1: Confirm Component 4 as it is

- Alt-2: Remove Component 4

- Alt-3: Merge Component 4 with Component 5, i.e., remove the current component 4 and revise component 5 as follows:

  - 5. Supported OCC length during which UE is able to maintain power consistency and phase continuity to support inter-slot OCC for PUSCH over consecutive slots

Proposal 4.3-3:

Confirm “Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5”, i.e., FG 65-3-1 is applicable only to FR1-NTN bands

Proposal 4.3-4:

Companies are encouraged to share views on the following alternatives on whether to separate FG 65-3-1 between GSO and NGSO:

- Alt-1: Not support GSO/NGSO differentiation

- Alt-2: Support GSO/NGSO differentiation

  - Alt 2-1: Supported OCC length is the same between GSO scenarios and NGSO scenarios

  - Alt 2-2: Supported OCC length can be different between GSO scenarios and NGSO scenarios

R1-2505196	NR-NTN phase 3 UE features	Nokia

R1-2505281	On UE features for NR-NTN Phase 3	Ericsson

R1-2505340	Discussion on UE features for NTN for NR Phase 3	CATT

R1-2505345	UE features for NTN for NR phase 3	Huawei, HiSilicon

R1-2505401	UE features for NTN for NR Phase 3	vivo

R1-2505448	Discussion on UE features for NR NTN phase 3	Xiaomi

R1-2505506	Discussion on the UE feature for NR-NTN Phase-3	ZTE Corporation, Sanechips

R1-2505567	UE features for NR NTN Phase 3	Samsung

R1-2505622	Discussion on UE features for NTN	Spreadtrum, UNISOC

R1-2505708	Discussion on UE features for NTN for NR Phase 3	OPPO

R1-2505898	Views on UE features for Rel-19 NR-NTN	Apple

R1-2506037	Discussions on UE Features NR NTN Ph3	MediaTek Inc.

R1-2506087	Discussion on UE features for NTN for NR Phase 3	CMCC

R1-2506202	UE features for NR NTN Phase 3	Qualcomm Incorporated

R1-2506290	Discussion on UE features for R19 NR NTN	NTT DOCOMO, INC.