3GPP TSG-RAN WG1 Meeting #122bis	  R1-2508032

Prague, Czech Republic, October 13 – 17, 2025

NNTT

Agenda Item:	10.1.1.2

Source:	Moderator (Huawei)

Title:	FL summary#5 for other aspects of CSI compression

Document for:	Discussion and Decision

# Introduction

In RAN#108, Rel-20 WID on AI/ML for NR air interface enhancements [1], including the following objectives for CSI compression was approved.

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

## [Rd1] Contact info

Before going to the discussions in the following sections, companies are invited to type in the contact person information into the following table.

Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Jianfeng Wang | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| Qualcomm | Chenxi Hao | chenxih@qti.qualcomm.com |
| Huawei, HiSilicon | Keyvan Zarifi | Keyvan.zarifi@huawei.com |
| Ericsson | Jingya Li, Xinlin Zhang | Jingya.li@ericsson.com Xinlin.zhang@ericsson.com |
| MediaTek | Pedram Kheirkhah Sangdeh Reubengeorge Stephen | Pedram.kheirkhah@mediatek.com Reubengeorge.stephen@mediatek.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| Xiaomi | Qin Mu Min Liu | muqin@xiaomi.com liumin10@xiaomi.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Panasonic | Tetsuya Yamamoto Hidetoshi Suzuki Xuan Tuong Tran (Henry) | yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com xuantuong.tran@sg.panasonic.com |
| CMCC | Yi Zheng Jiazhen Zhang | zhengyi@chinamobile.com zhangjiazhen@chinamobile.com |
| LGE | Yechan Chun | Yechan.chun@lge.com |
| TCL | Pu Yuan Tianqi Wu | Pu.yuan@tcl.com tianqi1.wu@tcl.com |
| NEC | Peng Guan Shafivulla Sayyed Yi Jiang | guan_peng@nec.cn Sayyed.Shafivulla@india.nec.com y-jiang_ct@nec.com |
| NTT DOCOMO | Xin Wang | wangx@docomolabs-beijing.com.cn |
| Apple | Huaning Niu Yuan Zhu | huaning_niu@apple.com yuan_zhu3@apple.com |
| ZTE | Wenfeng Liu | liu.wenfeng@zte.com.cn |
| InterDigital | Afshin Haghighat | afshin.haghighat@interdigital.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Sony | Sam Atungsiri Yingshuang Bai | sam.atungsiri@sony.com yingshuang.bai@sony.com |
| Toyota | Konstantinos Dimou | konstantinos.dimou@toyota.com |
| Southeast University | Jiajia Guo Chengyong Jiang | jiajiaguo@seu.edu.cn chengyongjiang@seu.edu.cn |

# Data collection

## Summary of company proposals

### 2.1-1: Generic

| Company | View |
| --- | --- |
| FUTUREWEI | Observation 1: AI/ML model training (for supervised learning in particular) requires high-quality training samples, and the AI/ML model can only learn from the information present in its training data. |
| NTT DOCOMO | Proposal 6: - Support the synchronized data collection between NW and UE. Proposal 7: Support the notification of the pairing ID for the synchronized data collection. |
| Samsung | Proposal 1: For two-sided model-based CSI compression, prioritize precoder matrix as Target CSI type for inference reporting, training data collection, inter-vendor training collaboration and monitoring. Observation 3: The network may provide the pairing-related ID, e.g., Associated ID, in the data collection configurations to align data categorization between the UE-side and NW-side with respect to the NW-side additional conditions, e.g., deployment scenario (indoor/outdoor TRPs), TxRU mapping, antenna layout, etc. Observation 4: For Direction C, both NW-side and UE-side may train their respective actual models to be compatible with the fully specified reference encoder and decoder, respectively. Alignment on data categorization through the pairing-related ID, e.g., Associated ID, enables training without NW-side and UE-side data distribution mismatch. Observation 5: For Direction A, NW-side may share the pairing-related ID(s), e.g., Associated ID, with reference encoder (Opt. 3a-1) or dataset (Opt. 4-1). UE-side may train its respective actual encoder based on the shared reference encoder/dataset and UE-side data (Target CSI) collected with the same Associated ID(s). Alignment on data categorization through the Associated ID enables training without NW-side and UE-side data distribution mismatch. Proposal 3: Supported the configuration of Associated ID (or named as a pairing ID) in the measurement and reporting configurations for NW-side and UE-side data collection. For the data collection, with the same Associated ID the UE assumes the same data distribution with respect to the NW-side additional conditions, e.g., deployment scenario, antenna layout assumed for corresponding decoder training. Proposal 5: For NW-side and UE-side data collection report configurations, consider - CPU counting/occupancy - CSI processing timeline - Interpretation of other configurations, e.g., CBSR, RI restrictions, if supported. |
| ZTE Corporation, Sanechips | Proposal 1: Spec enhancements of other aspects should be compatible to all to be specified inter vendor training collaboration options (i.e., Direction A options 3a-1 and 4-1, Direction C) including the associated scalability design. Proposal 2: Spec enhancements specified for AI/ML beam management and CSI prediction use cases should be reused for AI/ML CSI feedback to reduce spec efforts and simplify the spec design, when applicable. |

### 2.1-2: NW side data collection

| Company | View |
| --- | --- |
| Apple | Proposal 1: For NW side data collection, data format for L3 report uses scalar quantization. 8bits are used for quantizing a real part or an imaginary part of a scalar. |
| CATT | Proposal 1: For AI/ML based CSI compression Case 0, regarding the target CSI format in NW-side data collection, consider PMI based on Rel-19 eType II codebook as the starting point. - If necessary and justified, consider new parameter combinations and/or enhanced quantization for amplitude/phase coefficients Proposal 2: For AI/ML based CSI compression Case 0, additional content can be optionally included in NW-side data collection report: - to distinguish the layer/rank - to indicate data quality - to facilitate precoding matrix reconstruction from a specific target CSI codebook configuration |
| CMCC | Proposal 1: In CSI compression using two-sided model use case, regarding the target CSI format for NW side data collection, Option 0 and Option 4 can be supported. - Option 0: Reusing legacy e-Type II codebook - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) Proposal 2: In CSI compression using two-sided model use case, for target CSI reporting for NW-side data collection, the rank of the target CSI is configured by gNB and RI reporting in CSI is not needed. Proposal 3: In CSI compression using two-sided model use case, regarding target CSI reporting for NW-side data collection, CQI reporting in CSI is not needed. - FFS: whether CRI or LI is needed for reporting or not |
| Ericsson | Observation 1: For NW-side data collection with the listed data format options, Option 1 is not feasible for NW-side data collection for 128 CSI-RS ports. Options 2 and 4 are both feasible, but Option 4 is more efficient. Observation 2: For NW-side data collection with the listed data format options, Option 4 provides a better payload vs accuracy trade-off, compared to Options 0, 1, and 2. Proposal 7: Support Option 4 as the target CSI format for NW-side data collection, due to its feasibility for 128 ports, and better payload vs accuracy trade-off. Proposal 8: Pairing ID is not needed for configuring NW-side data collection, and UE shall log at least cell ID, measurement configuration ID and target CSI samples per CSI logged measurement configuration per cell. |
| ETRI | Proposal 1: For AI/ML-based CSI compression, consider NW-side data collection under various environments and UE implementations. Proposal 2: For AI/ML-based CSI compression, consider the use of pairing ID and data format indication as assistant information for NW-side data collection. |
| Fujitsu | Proposal 11: For target CSI quantization in NW-side data collection, - Support eT2-like codebook-based quantization. - If scalar quantization is considered, its quantization scheme should be derived from data distribution based on fading channels in field. |
| FUTUREWEI | Proposal 1: In AI/ML-based CSI compression using two-sided model Case 0, at least for NW-side ground-truth data collection for model training, consider the following options for high-resolution Target CSI format: - Rel-16 eType II CB based quantization with new parameters (preferred) - Scalar quantization with higher number of bits for amplitude and phase (FFS on actual number of bits) |
| Google | Proposal 1: For network side data collection, with regard to UE’s memory constraints, options that require large overhead should be precluded. - Continue the study based on Option 2 and Option 3 |
| Huawei, HiSilicon | Proposal 1: For the NW side training data collection, for the quantization of Target CSI, consider the following EVM: - Regarding the benchmark, FP32 is taken as the upper-bound. - Regarding the metric, consider the following two alternatives, wherein Alt.2 is considered as baseline. - Alt.1: SGCS is calculated between the Target CSI of FP32 (CSIFP32) with its quantized version (CSIQT). - Alt.2: SGCS is calculated between Target CSI of FP 32 (CSIFP32) as the model input and the recovery CSI (CSIOut) as the model output of the model trained with Target CSI with quantized version (CSIQT). - Regarding the EVM for Option 1, the normalization method should be aligned. For example, for a batch of S reported data samples each of which includes the Target CSIs for L layers/Rx antennas, N3 subbands, and 2*N1*N2 Tx ports, the real/imaginary value of each floating element is quantized in the range of (0, 1) relative to the strongest element over the S, L, N3, 2*N1*N2 floating elements. - Regarding the EVM for Option 4, consider the enhanced parameters as: L= 8, 10, 12; pv = 0.8, 0.9, 0.95; beta = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits. Proposal 2: For the NW side training data collection, for the quantization of Target CSI, consider to support scalar quantization (Option 1) or eType II like CB with new parameter values (Option 4). Proposal 3: For the NW side training data collection, Pairing ID may be optionally indicated to UE. - UE reports Pairing ID along with the corresponding Target CSI data in the higher layer reporting. |
| KT Corporation | Observation 1: Wideband beam selection in the e-Type II codebook requires on the order of OO1∙O2∙N1N2LN3NT operations, making the computational burden a challenge for performance monitoring where timely feedback is required. Observation 2: If channel matrix is to be supported as a target CSI type, the enhanced Type II codebook may face challenges, since inter-column power relationships must be preserved and the larger magnitude variation of channel matrix entries can increase quantization error. Observation 3: Reliance on enhanced Type II codebook for NW-side data collection may limit the practical adoption of AI/ML-based CSI compression due to its limited commercialization rate. Proposal 1: Support scalar quantization for training, since accuracy is the critical requirement and latency and overhead can be mitigated by applying MDT as the data collection mechanism. |
| Lenovo | Observation 2: Other than data transfer for data-collection, transmission of quantized ground-truth CSI is required for model monitoring and model updating/tuning phase, which need lower data transfer overhead and require less latency than that data collection for model training. Proposal 7: Support possibility of using different options for transmission of Target-CSI for data-collection for model training, model monitoring, model update/tunning, and for dataset exchange between the NW-side and the UE-side for UE-side encoder training. Observation 3: For fixed feedback overhead cases, there exist a trade-off between the number of samples that can be feedback and the resolution of each sample in the feedback data. Therefore, when analysing the gain of transmitting a dataset with higher sample resolution, it should be compared with the case of not increasing the sample resolution but instead send more samples. Observation 4: Although it is desirable to have larger CSI dataset for training, due to the overhead associated with data collection, it is important to determine the proper size of the CSI dataset which provides enough information for training without imposing large overhead on the network. Observation 5: It is desirable to include CSI samples with more informative for model training or updating to reduce the CSI dataset size, based on the statistics of the input, e.g., lower information content of some CSI samples or high correlation between one CSI sample (a group of samples) and another CSI sample (group of samples). Proposal 8: Support specification of procedures/signaling enabling transmission of subset of CSI samples among the set of measured/collected CSI samples from the environment. Observation 6: Transmission of additional information such as the level of distortion when measuring a sample, quality indicator, or how frequently a particular representative sample occurs, can help the NW side to better train the model. Observation 7: As the information content of CSI samples depends on the experienced distortion level, the UE may use distortion level or a quality indicator to determine which samples should be transmitted to the NW or at least send this information along the sample itself. The NW may be able to use this information further during the training of the model. Proposal 9: Support specification of procedures/signaling enabling transmission of subset of CSI samples based on the experienced distortion level or a quality indictor. Proposal 10: Support specification of procedures/signaling for transmission of additional information such as sample-group size, quality indicator, distortion level along the transmission of the sample itself. Observation 8: Knowledge of the conditions/additional conditions under which the samples of the training data have been collected is needed can be used to train a Specialized model. Proposal 11: Support specification of procedures/signaling enabling UE to report UE-side additional conditions and also the NW conditions/additional conditions under which the data/samples have been collected. Observation 9: The applicability of a model can be determined based on the combined additional information of all dataset(s) used for training with associated IDs, which represents the conditions of UE and NW to measure and collect the samples. Proposal 12: Support associating additional information with the dataset/model parameters generated by the NW-side and indicating that to the UE-side along the exchange of the dataset/model parameters. Proposal 13: Study the necessary of the ID associated (i.e., associated ID) with the conditions to generate and tune the dataset/model parameters. |
| LG Electronics | Proposal 4: For target CSI format, support legacy e-Type II codebook and scalar quantization. Proposal 5: For data collection for training, study how to efficiently construct dataset (e.g. data omission/drop, filtering) |
|  | Proposal 2: Deprioritize option 4 for NW-side data collection. Proposal 3: Support option 0 and 1 for NW-side data collection. Proposal 4: It is up to UE that how many CSI samples can be accumulated and reported to NW for training purpose at a time. Proposal 5: Discuss the assistant information that UE needs to include in reporting CSI samples to NW for training purpose. |
| NEC | Proposal 1: For target CSI format, support legacy CSI codebooks (i.e., Option 0). In addition, higher resolution eType II like codebook with new parameter values (i.e., Option 4) can be considered. |
| Nokia | Observation 2: UEs supporting AIML-enabled CSI compression feature are expected to support legacy e-Type II codebook-based CSI feedback as a fallback option. Proposal 4: RAN1 to adopt Option 0 as baseline for NW-side data collection. Observation 3: Initial results show that the SGCS performance of e-Type II codebook with new values of paramCombination-r16 improves as compared to existing values of paramCombination-r16. Proposal 5: Study the performance vs complexity of e-Type II codebook with new parameters (Option 4) for NW side data collection. - FFS: the values of the parameters (e.g., L, ρv, β, amplitude, phase). Observation 4: One of the major concerns for Option 0 is its higher computational complexity at UE. Proposal 6: As a complexity-friendly alternative to Option 0, study the required feedback overheads (in terms of the number of feedback bits needed to satisfy the E2E performance requirement for data collection when applying elementwise scalar quantization). Proposal 7: Do not support any study on Options 2 and 3. Observation 5: For the NW side data collection, - On the feedback overhead, Option ‘0’ is the least one followed by Option ‘4’, and the overhead is large for Option ‘1’. - On the computational complexity, Option ‘1’ is the minimum followed by Option ‘0’, but Option ‘4’ appears to have the maximum computational load among these three options. Proposal 8: RAN1 to consider both the computational complexity as well as feedback overhead in the study to further down select the options for NW side data collection. Proposal 9: For NW-sided data collection, when adopting elementwise uniform SQ applied to Magnitude/Phase, for the given total number of allocation bits, allocate ⌊total number of allocation bits/2⌋ bits to the magnitude part, and allocate the remaining bits to the phase part. Proposal 10: For NW-sided data collection, RAN1 needs to investigate E2E performance with the limited bit allocation (e.g., 5 bits per element with 2 bits for magnitude, 3 bits for phase) for elementwise uniform SQ scheme. - SB-wise sampling (reporting every 2 or 3 SBs) can be tested as a means for overhead reduction. |
| NTT DOCOMO | Proposal 3: - For NW side data collection, - Support a high-resolution data format for the target CSI. - Support a data quality indicator report for the NW side data collection. Proposal 4: - Support Option 1 as the quantization scheme for target CSI in the data collection. Proposal 5: - Support a unified quantization approach for ground truth report for performance monitoring, NW-side data collection, and dataset exchange. - For reports that require less accuracy and a smaller payload size, UE can obtain the bit sequence by ignoring partial bits from the high-resolution version of target CSI bits. |
| OPPO | Observation 1: Performance and scalability issues are not fully addressed for target CSI type with precoding matrix in beam-delay or beam-frequency domain. Proposal 1: Only support CSI eigenvector in spatial-frequency domain as the target CSI type. - Whether the input of precoding matrix in spatial-frequency domain is transformed into other forms can be left to UE-side encoder implementation. Proposal 2: Support Option 1 with linear normalization and uniform scalar quantization on real and imaginary part as baseline solution for NW-side data collection. - Determination of quantization bits, xmin and xmax can be further discussed. Proposal 3: For NW-side data collection from serving cell, pairing ID is not needed. Proposal 4: For NW-side data collection not from serving cell, whether pairing ID assigned from previous cells is valid for serving cell requires further study. |
| Panasonic | Proposal 3: On data sample type / format for ground-truth CSI reporting, high resolution codebook-based format e.g., legacy codebook (e.g., eType II codebook) with potential enhancements such as to extend more configurations in some parameters, should be considered. Proposal 4: For NW-side data collection, the necessity and feasibility of UE reporting UE-side additional conditions to network should be studied. Instead of informing actual configuration, UE-side associated ID is necessary. |
| Qualcomm | Proposal 1: For NW side data collection, the data format discussion should consider performance and UE complexity / power consumption as high priority. The incurred over-the-air overhead is not a critical issue. Observation 1: For data format used for training data collection, element-wise quantization and spatial compression only achieve similar performance, and spatial compression only achieves similar performance to enhanced eT2 at with lower number of SD bases. Observation 2: For data format used for training data collection, element-wise quantization (e.g., floating point) and spatial compression only has much lower complexity than enhanced eT2. Proposal 2: Adopt one from the following data format for NW side training data collection - Option 1: Element-wise quantization of target CSI directly (e.g., floating32, floating16 or even lower precision with A-bit for amplitude + B-bit for phase) - Option 2: Target CSI with spatial compression only. - TypeII-like, W=W1*W2 with L=6 beams and element-wise quantization of W2 in subband-by-subband manner (floating point - like, even lower precision with A-bit for amplitude + B-bit for phase). - Note: this is equivalent to eT2 format with β=1 and p=1. Observation 3: eT2 enhancement is a generic problem for CSI enhancement, and is orthogonal to the discussion on data collection or monitoring. Observation 4: 5G data collection and performance monitoring framework are informative examples of 6G study. Proposal 3: For data collection and monitoring of Rel-20 CSI feedback with two-sided model, consider a more generic data format with low complexity rather than using a specific format used for 5G CSI feedback. - Do not support eT2 or eT2 enhancement as data format for NW side monitoring or data collection. |
| Samsung | Observation 1: For Target CSI in NW-side data collection, for Option#3 and Option#4 below, Option#3 provides a slightly better accuracy than Option#4 while both options provide more than 90% overhead reduction from benchmark (Float32). - Option#3: principal component analysis (PCA)-based enhanced method for overhead saving by reporting the principal components (left and right eigenvectors and eigenvalues) of the spatial-frequency precoding matrix. - Option#4: eType II with new parameter combination, L, ρv, β, with Rel-16 amplitude and phase quantization. Observation 2: For two-sided model-based CSI compression, for Target CSI in NW-side data collection, - Option#0: eType II PC=8 reduces data samples overhead by 98.7% at the expense of considerable performance degradation as compared to a model trained by dataset quantized by Float32. - Option#1: SQ with int8 reduces data samples overhead by 75.0% with similar performance as compared to a model trained by dataset quantized by Float32. - Option#3: Principal component analysis (PCA)-based enhanced method for overhead by reporting the principal component of spatial-frequency precoding matrix, reduces data samples overhead by 94.53% at the expense of marginal performance degradation (0.38%) as compared to a model trained by dataset quantized by Float32. - Option#4: eType II with new parameter combination, ρv=1/2, β=3/4, reduces data samples overhead by 96.1% at the expense of marginal performance degradation as compared to a model trained by dataset quantized by Float32. Note: above evaluation is on rank=1 dataset captured by RAN4. Proposal 2: For two-sided model-based CSI compression, for Target CSI in NW-side data collection, deprioritize, Option 0: Reusing legacy e-Type II codebook. Proposal 4: For NW-side data collection, consider the following additional information reporting - Rank information - Data quality information, e.g., explicit quantities including eigenvalues, SINR, etc., or implicit quantities such as SGCS, CQI. |
| Sharp | Proposal 1: For CSI compression, for NW side data collection for training, support the use of Rel-16 eTypeII codebook-based data format for target CSI. Proposal 2: For CSI compression, for NW side data collection for training, support that NW can indicate rank information for target CSI reporting. |
| Spreadtrum, UNISOC | Proposal 1: For network side data collection, UE reports ground-truth CSI with legacy Rel-16 eTypeII codebook design for model training. Proposal 2：Whether to configure rank/layer can be discussed after the model training method is determined. |
| TCL | Proposal 1: The quality information such as CSI-SINR and CQI can be provided along with training data or the quality threshold can be configured by NW for the reliability of training dataset. |
| vivo | Observation 1: There is no or only minor performance loss in the option 1quantization Target CSI if quantization precision of at least int4 Proposal 4: Down-select the Target CSI format for inter-vendor training collaboration from Options 0, 1, and 4. |
| Xiaomi | Proposal 5: Taken Rel-16 eType2 codebook as baseline for target CSI reporting for NW side data collection. |
| ZTE Corporation, Sanechips | Proposal 7: For NW side data collection for training, support R16 eType II CB with new parameter as the target CSI type for better tradeoff between reporting overhead and inference performance. Proposal 8: For NW side data collection of CSI compression, the content of the collected data can include pairing ID which is beneficial for both NW side and UE side. Proposal 9: To enable high-quality data collection from the UE to the NW, further study the following options: - The UE reports data quality-related information to the NW, e.g., SINR, CQI, positioning information - The NW configures a data quality threshold to the UE and the UE only reports qualified data |
| Southeast University | Proposal 1-1: Further discuss the possibility of alleviating data collection requirements by introducing CSI-free unsupervised CSI compression model training scheme. Proposal 1-2: Study whether CSI-free unsupervised CSI compression model training scheme can co-exist with ordinary CSI compression model training scheme. Proposal 1-3: Study how to reduce data collection requirements for training two-side CSI compression model. Proposal 1-4: Study the dataset selection and dataset distillation-based method to further reduce the data collection signaling overhead. |

### 2.1-3: UE side data collection

| Company | View |
| --- | --- |
| Apple | Proposal 2: For UE side data collection, when a UE initiates a data collection procedure for training purposes, it may also recommend a CSI-RS-related configuration such as CSI-RS density and pairing ID, to optimize the quality of the collected data. |
| CATT | Proposal 3: For AI/ML based CSI compression Case 0, optionally UE can request the specific configuration for UE-side data collection. - Multiple candidate configurations for UE-side data collection can be configured by NW - UE can indicate preferred configuration from candidate configurations for UE-side data collection |
| CMCC | Proposal 4: In CSI compression using two-sided model use case, to enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report - Pairing ID can be optionally indicated in CSI-ReportConfig |
| Ericsson | Proposal 9: Pairing ID can be optionally configured for UE-side data collection, if needed. |
| ETRI | Proposal 3: For AI/ML-based CSI compression, consider the use of the pairing ID as assistant information for UE-side data collection. |
| FUTUREWEI | Proposal 2: In AI/ML-based CSI compression using two-sided model Case 0, for UE side data collection, consider including the following additional information in the CSI-ReportConfig: - Pairing ID that is associated with exchanged dataset or exchanged model parameters or standardized reference model in the inter-vendor training collaboration procedure - reportQuantity that reflects no report is needed and is for Rel-20 CSI compression via two-sided model, e.g., “none-CSI-r20” |
| Google | Proposal 2: Reuse the same mechanism of data collection for Rel-19 AI/ML based CSI prediction for the UE-side data collection for AI/ML based CSI compression. |
| Huawei, HiSilicon | Proposal 4: Pairing ID(s) can be optionally indicated in CSI-ReportConfig for UE side training data collection. |
| Lenovo | Proposal 14: To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose with Pairing ID indicated. |
| MediaTeK | Proposal 1: For UE-side data collection, CSI-ReportConfig is used for configuring the resources for data collection purpose without CSI report; the corresponding indication is that reportQuantity is set to ‘none-r20-CSI’. |
| OPPO | Proposal 5: For UE-side data collection, pairing ID is required and can be configured in CSI-ReportConfig. |
| Panasonic | Proposal 1: For UE-side training data collection, in order to identify the scenario / configuration, configuration of ID, which is same ID as paring ID, is necessary at least for the case that UE-side data collection is based on OTA approach. - For the case that UE-side data collection is non-OTA approach, it would not be necessarily required to use the same ID as paring ID. Proposal 2: For UE-side data collection at least based on non-OTA approach, the necessity and feasibility of UE reporting UE-side additional conditions to network should be studied. Instead of informing actual configuration, UE-side associated ID is necessary. - For the case that UE-side data collection is OTA approach, UE-side additional condition can be added when OTA data is sent to UE-side server. |
| Qualcomm | Proposal 4: For UE side data collection for two-sided CSI compression use case, configure measurement CSI-RS using CSI report config without a report. The CSI report configuration should include a paring ID. |
| Sharp | Proposal 3: For data collection for UE side model, reportQuantity is set to a dedicate value to notify UE of the data collection for CSI compression. |
| Spreadtrum, UNISOC | Proposal 3: The pairing ID can be used for UE to collect UE-side target CSI for model training and fine-tuning. |
| TCL | Observation 1: The UE side data collection for training can be performed in both an opportunistic and proactive manner. |
| vivo | Proposal 2: The pairing ID is indicated in CSI-ReportConfig for UE-sided data collection. Proposal 5: Support to indicate the following information in CSI-ReportConfig used for UE side data collection - Pairing ID - The supported configuration(s) of pairing ID (e.g., multiple port configuration, multiple subband configuration, and multiple payload size configuration) |
| Xiaomi | Proposal 6: The paring ID is configured in CSI-ReportConfig for UE side data collection Proposal 7: UE side data collection based on UE request is supported. |
| ZTE Corporation, Sanechips | Proposal 10: For UE-side training data collection, pairing ID can be configured to address data distribution mismatch issues regarding NW-side/UE-side additional conditions. FFS other necessary information. |

## Discussions

## Issue#2-1 NW side data collection - Quantization format of Target CSI

Brief summary:

Supporters of quantization methods:

- Option 0 (legacy eT2 codebook): vivo (down-selection among 0/1/4), Xiaomi, CMCC, NEC, Tejas, Sharp, MediaTek

  - Not support: Samsung

- Option 1 (Scalar quantization): Huawei, HiSilicon, Qualcomm, vivo (down-selection among 0/1/4), Samsung, Nokia (study as an alternative to Option 0), Apple, OPPO, NTT DOCOMO, MediaTek, Futurewei, KT

- Option 2 (Type II like codebook with scalar quantized W2): Qualcomm, Google

  - Not support: Nokia

- Option 3 (reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix): Samsung, Google

  - Not support: Nokia

- Option 4 (eType II like codebook with new parameter): Huawei, HiSilicon, Ericsson, vivo (down-selection among 0/1/4), Nokia, ZTE, CATT, CMCC, NEC, Fujitsu, Futurewei

  - Ericsson: due to its feasibility for 128 ports, and better payload vs accuracy trade-off

  - Not support: MediaTek

Overhead issue:

- Ericsson: Option 1 is not feasible for NW-side data collection for 128 CSI-RS ports (v=4 layers, N3=19 subbands)

- Lenovo, LGE: transmission of subset of CSI samples/filtered CSI samples among the set of measured/collected CSI samples

  - based on the experienced distortion level or a quality indictor

Baseline solution:

- Option 0 (legacy eT2 codebook): Nokia, Xiaomi, CATT, Tejas

Evaluation related issues:

- Metric:

  - Alt.1: SGCS is calculated between the Target CSI of FP32 (CSIFP32) with its quantized version (CSIQT) Huawei, HiSilicon,

  - Alt.2: For a model trained with Target CSI with quantized version (CSIQT), SGCS is calculated between Target CSI of FP 32 (CSIFP32) as the model input and the recovery CSI (CSIOut) as the model output Huawei, HiSilicon, Qualcomm, Ericsson, Nokia, Samsung

- Benchmark:

  - FP32 as upper-bound: Huawei, HiSilicon,

- Payload size of Target CSI (2N1N2 ports, N3 subbands, v layers):

  - Option 1 (K bits per complex coefficient, each complex coefficient includes real and imaginary):

    - Huawei, HiSilicon: 2N1N2*K bits per subband per layer

    - Ericsson: 2N1N2* N3*K*v bits

  - Option 2 (2L selected spatial basis):

    - Ericsson (M selected frequency basis): 2LMvK+log2N1N2 L+log2N3 M

  - Option 4 (2L selected spatial basis, M selected frequency basis, β percent non-zero coefficients):

    - Ericsson: 2LMvKβ+log2N1N2 L+log2N3 M+2LMv

- Option specific assumptions

  - Option 0:

  - Option 1:

    - Huawei, HiSilicon: Align the normalization of Option 1: For a batch of S reported data samples each of which includes the Target CSIs for L layers/Rx antennas, N3 subbands, and 2*N1*N2 Tx ports, the real/imaginary value of each floating element is quantized in the range of (0, 1) relative to the strongest element over the S, L, N3, 2*N1*N2 floating elements.

    - OPPO: The real number of each sample (denoted as x) is linearly normalized to [0,1] (denoted as y) as y=x-xminxmax-xmin, wherein xmin and xmax is the minimum and maximum value among the whole dataset. Quantization related parameters, including quantization bits, xmin and xmax can be specified, or configured by NW, or reported by UE along with target CSI.

    - Apple: 8bits to real or imaginary

    - Qualcomm: A-bit for amplitude + B-bit for phase

    - Nokia: Scalar quantization to real and imaginary value, or magnitude and phase value

    - Nokia: Consider subband-wise down sampling (every 2 or 3 subbands) for overhead reduction

  - Option 2:

    - Candidate parameter values:

      - Qualcomm: W=W1*W2 with L=6, 8, 10 beams and element-wise quantization of W2 in subband-by-subband manner (floating point, or even lower precision with A-bit for amplitude + B-bit for phase). L=6, 6bit-phase, 5-bit amplitude

      - Ericsson: L= 8, 10, 12; M=8,10,12; amplitude = 4bits; phase = 4bits.

  - Option 3:

    - Samsung: Perform SVD to the eigen vector (with dimension of Tx port * Subband Num) constructed matrix of H=c1,1c1,2c2,1c2,2⋯c1,NSB⋯c2,NSB⋮⋮cNport,1cNport,2⋱⋮⋯cNport,NSB, deriving left singular vectors (Nport length) and right singular vectors (Nsubband length). Then, further perform scalar quantization (amplitude, phase) to each element of left singular vectors and right singular vectors, with total number of coefficients of d*( Nport + Nsubband), where d is selected singular value which achieves a compression of NportNSBd(Nport+NSB).

      - Candidate parameter values: d=2,3,4; amplitude=4 bits; phase=4 bits

  - Option 4:

    - Candidate parameter values:

      - Huawei, HiSilicon#1/Nokia#1: L=10, p=0.9, beta=0.31, reference amplitude: 6 bits, differential amplitude: 4 bits, phase quantization: 6 bits

      - Huawei, HiSilicon#2/Nokia#2: L=12, p=0.8, beta=0.5, reference amplitude: 6 bits, differential amplitude: 4 bits, phase quantization: 6 bits

      - Qualcomm: L=10, M=10, beta=1, 6bit-phase, 5-bit amplitude

      - Ericsson#1/2: L = 10, M = 8/10 (N3=13), 𝛽 = 0.50. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

      - Ericsson#3/4/5: L = 10, M = 8/10/12 (N3=13), 𝛽 = 0.25. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

      - Ericsson#6: L = 12, M = 8 (N3=13), 𝛽 = 0.50. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

      - Ericsson#7/8: L = 12, M = 10/12 (N3=13), 𝛽 = 0.25. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

      - Samsung#1: L= 8; pv=0.75, 1; beta = 0.75. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

      - Samsung#2/3: L= 10; pv=0.75; beta = 0.5/0.75. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

      - Samsung#4: L= 10; pv=1; beta = 0.75. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

      - Samsung#5: L= 16; pv=1; beta = 1. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively).

Moderator note: From supporters referring to “Brief summary”, more companies support Option 1 and Option 4. Some company have concern on Option 0, 2, and 3. Let’s see if we can further make down selection between 1 and 4. Regarding Option 0, it is Moderator’s understanding that it can be achieved with legacy L1 report, so there seems no need to report via higher layer signaling?

### [Rd1] (Closed) Proposal 2.1 (down selection):

For NW side data collection with higher layer reporting, further discuss and perform potential down selection between Option 1 and Option 4

- Note: NW side data collection via Option 0 can be achieved with legacy L1 report.

### [Rd2] (Closed) Proposal 2.1a (down selection):

For NW side data collection with higher layer reporting, further discuss and perform potential down selection between Option 1 and Option 4

- Note: NW side data collection via Option 0 can be achieved with legacy L1 report.

| Company | View |
| --- | --- |
| vivo | OK |
| Huawei, Hisilicon | OK and we can support both Option 1 and Option 4. For target CSI quantization, we think the most important issue is the accuracy since the quantized Target CSIs will be used as label for NW model training. Overhead should be also considered but not as the main critera since the data transfer is over higher layers and there is no oparticular latency requirement for NW side data collection. Among listed options, evaluations show that both Option1 and Option4 provide excellent SGCS accuracy (considerably better than legacy Option 0 specally for higher layer indeces). Between Option 1 and Option 4, Option 4 has lower overhead in general. However, Option 1 has the benefit of less complexity since there is no need to perform matrix projection or SVD/EVD operations. Also, eType II CB is only optionally supported by legacy UEs, thus, a UE with limited processing capabilities may not support eType II-based Option-4. In addition, both Option1 and Option4 have much less specification effort compared to Option2/3: There is no CB design for Option1 and Option4 can be specified essentially by adding a new row to eTypeII parameter combination table. |
| Apple | Support to further down-select between option 1 and 4. From ground truth accuracy and generalization’s point of view, we see that option 1 is better than option 4. If payload size is a concern for rank 4, collecting data layer by layer (or subband by subband) is also doable as there seems to be no urgency for transmitting the collected data over one shot. Option 4 seems limiting the collected data for the channel that the eTypeII codebook is designed for. For those channels, e.g., near field channels, apparently eTypeII codebook may have large quantization error compared to Option 1 which is more generally applicable. On the other hand, for legacy UEs, eTypeII L=6 and rank 3/4 is still a UE capability. Option 4 is likely to further demand a higher end UE which may have limited availability. |
| NTT DOCOMO | Fine with the main proposal, but suggest removing the note because it implies that Option 0 is a valid solution for NW-side data collection. There are two issues with this note. The CSI accuracy is poor with Option 0, and the Layer 1 CSI reporting is overqualified for the data collection purpose. |
| Qualcomm | Don’t see a strong need to down-select at this early stage. We think at least option 2 should be on the table. It addresses the pain point of option 1 and 4. Compared to option 1, the overhead is saved by dimension reduction via SD bases. Compared to option 4, the complexity is reduced by not taking further bases transformation and sorting in delay domain. Note that option 2 is not a new codebook, it’s just a intermediate step of T2 and eT2 codebook processing and perform a coefficient quntization (same resolution of eT2 coefficient quantization can be considered, e.g., 4-bit phase, 3-bit amplitude). |
| Lenovo | We are fine with the downselection. |
| Samsung | First of all, the note is not required. If L1 signaling for data collection, does it mean transparent or the purpose is indicated to the UE? If the purpose is indicated, are we going to follow a different timeline and processing unit management from legacy eType II reporting? We have seen some benefits from Option#3. Thus, it is beneficial to compare it with the other options, especially if higher number of ports, e.g., 128 ports, to be supported. Before down selection, we suggest to first set some ballpark number/range for the expected payload sizes given the 9Kbyte budget, e.g., 1000-2000bits per layer. |
| OPPO | Prefer Option 1 with affordable UE simplicity. The advantage of Option 4 over overhead is not critical for NW-side data collection For the note bullet, it means that NW can use the L1 CSI report for NW-side data collection whenever NW wants? It seems not proper from UE perspective. |
| Futurewei | We are ok with the proposal and we can down-select from Option 1 and Option 4. |
| CMCC | Support. |
| CATT | Generally fine with the direction. We also suggest to rmove the note which is controversial. |
| NEC | To our understanding, Option 0 is the basis. But we can accept Option 4. Option 1 overhead seems too high. We are open to consider it after the overhead discussion. |
| Ericsson | We support to futher downselection between Option 1, Option 2, and Option 4. Agree with Docomo, Samsung and CATT that the note is not needed. |
| ZTE | Support to further down select between Option 1 and Option 4, considering the performance-complexity tradeoff. Besides, the note can be deleted since there is no spec impact for L1 report and we shall focus on high layer report for NW side data collection. |
| Spreadtrum | We prefer Option1 since its less complexity. |
| InterDigital | Support |
| Panasonic | We are fine with the proposal to down select between Option 1 and Option 4. |
| LGE | We are fine with FL’s proposal. Both option 1 and option 4 can achieve good reconstruction accuracy compared to Option 0. In our view, we prefer Option 1 over Option 4 due to computational complexity on the UE. Since model training requires a large amount of data, it is important to generate target CSI using a computational efficient method for the UE. |
| Fujitsu | We prefer Option 4 since the good balance can be achieved between the feedback overhead and the quantization resolution. If Option 1 can meet the payload size restriction of RRC message, we can live with both Option 4 and Option 1. |
| ETRI | Support to further consider Option 1 and Option 4. |
| HONOR | Support. We believe one option is enough. Considering UE complexity and UE capability, Option 1 is preferred. |
| Toyota | Support the proposal to downselect between Option 1 and Option 4. There is a preference for Option 4, since this option serves the overall goal of the AI/ML CSI compression, which is to reduce the uplink signaling overhead due to CSI reporting. |

Moderator note: Option 1 needs a definition for further study. Some company raise the method to perform normalization (e.g., how to determine max value and min value, and formula for normalization), and some company raise the method to perform quantization (quantize to real/imaginary parts, or amplitude/phase parts, and whether to adopt same or different bits between real and imaginary or amplitude and phase).

### [Rd1] (Closed) Proposal 2.2 (definition of Option 1):

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantzation), consider the following definition:

- Normalization: for a number of S Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which incorporates overall S*2N1N2*N3*v*2 floating elements, any floating element (namely x) is linearly normalized to y within the range of [0, 1], relative to a minimum value of xmin (corresponding to 0) and a maximum value of xmax (corresponding to 1).

  - FFS the normalization formula, e.g., y=x-xminxmax-xmin.

  - FFS how to determine xmin and xmax. E.g., selected among S samples or within per sample.

- Quantization: real part (or amplitude part) is quantized to k1 bits and imaginary part (or phase part) is quantized to k2 bits. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part.

  - FFS k1 and k2 values.

  - FFS whether to quantize to real value and imaginary value, or quantize to amplitude value and phase value.

- Note: If each complex element is quantized to k1+k2 bits, the overall payload size per data sample (Target CSI with 2N1N2 ports, N3 subbands, v layers for precoding matrix) is 2N1N2*N3*(k1+k2)*v bits.

Moderator note: Reformulate the proposal. Define from three aspects: content, normalization, and quantization.

Regarding vivo question on the normalization scope, indeed it needs to be further studied: normalized within or across layers/subbands/ports/samples/real and imag parts. Make it as an FFS. To Moderator’s understanding, normalizing across dimensions will preserve the correlation information.

Regarding QC and DCM comments that normalization is implemention, make the bullet more generic “in normalized representation”; whether to specify the normalization formula can be FFS. But, at least NW should know or align with UE on the normalization scope? If one UE normalizes the data per layer, while another UE normalizes the data across layers, their data distributions will be different?

### [Rd2] (Closed) Proposal 2.2a (definition of Option 1):

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantzation), consider the following definition:

- Content: for a number of S Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall S*2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2.

  - FFS whether to quantize to real value and imaginary value, or quantize to amplitude value and phase value.

- Normalization: x1 is in forms of normalized representation within the range of [T1min, T1max], and x2 is in forms of normalized representation within the range of [T2min, T2max].

  - FFS whether/how to specify the normalization formula, e.g., y=x-xminxmax-xmin.

  - FFS the range of [T1min, T1max] and [T2min, T2max]

  - FFS the normalization is performed within or across S samples/ 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts

  - FFS how to determine xmin and xmax. E.g., selected among S samples or within per sample.

- Quantization: x1 real part (or amplitude part) is quantized to k1 bits and x2 imaginary part (or phase part) is quantized to k2 bits. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part.

  - FFS k1 and k2 values, and whether they are equal.

  - FFS whether to quantize to real value and imaginary value, or quantize to amplitude value and phase value.

- Note: If each complex element is quantized to k1+k2 bits, the overall payload size per data sample (Target CSI with 2N1N2 ports, N3 subbands, v layers for precoding matrix) is 2N1N2*N3*(k1+k2)*v bits.

| Company | View |
| --- | --- |
| Mod | Please provide your views for at least following questions. Q1: For Option 1, whether it is needed to define normalization and quantization, and what other aspects need to be defined? Q2: For normalization, provide your views on the formula and how to determin xmin and xmax Q3: For quantization, whether to quantized to real/imaginary parts, or amplitude/phase parts. Whether to adopt same or different number of bits between real and imaginary, or between amplitude and phase |
| vivo | Firstly, we are curious why each sample cannot be quantized individually, and instead S samples are quantized together. Secondly, compared to normalization based on the maximum value, are there any additional benefits to mapping from [xmin, xmax] to [0,1]? Lastly, if quantizing the real and imaginary parts, should the quantization range be from [-1 to +1]? |
| Huawei, Hisilicon | Overall, supportive of the proposal direction. Q1: Both normalization and quantization step/quantization method (Real/Imaginary vs. Phase/amplitude) should be specified to have comparable data samples for each UE and across different UEs. Q2: Assuming quantization is over Real/Imaginary parts, the normalization should be independently done on the real and imaginary part of each element so that the dynamic range of both real and imaginary parts cover the whole interval of [0,1]. As for the question whether the normalization should be per sample or across all S samples, we think using the same normalization across all S sample Target CSIs has the advantage of preserving the relative channel strength across different time instances. On the other hand, if one sample Target CSI corresponds to a much better channel condition than other samples, using the same normalization across all samples results in close-to-zero values for the entries of all other samples and, subsequently, using only a fraction of the quantization dynamic range to quantize these samples; resulting in quantization distortion. As such, we think the best compromise is to normalize per sample. Q3) As for real/imaginary vs. amplitude/phase normalization, we prefer real/imaginary normalization as it is simpler for implementation and lends itself better to subsequent linear processing. Also, it does not have the discontinuity problem of phase quantization around ±𝜋. Finally, when real/imaginary part is used, it makes sense that K1=K2. Based on simulations, it seems that 8 bit scalar quantization for each or real and imaginary part provides a good accuracy/overhead compromise. |
| Apple | In principle we are fine with 8 bits scalar quantization for real or imaginary part. Detailed normalization schemes can be FFS. |
| NTT DOCOMO | For Q1, the quantization codebook/look-up tables should be specified, with which the normalization is up to UE implementation. For Q2, following the answer for Q1, the x_mina and x_max are implied in the codebook or look-up tables for the specified quantization. Therefore, it is up to UE implementation about whether and how to normalize the target CSI according to the specified quantization. For Q3, the same bit width should be adopted for real and imaginary parts, but a different bit width can be adopted for amplitude and phase, if supported. |
| Qualcomm | I don’t think we need to describe these coefficients are “floating points”, we should just say per complex coefficients quantization, with k1 bits for amplitude, k2 bits for amplitude (or k1 bits for real part, k2 bits for imaginary part). Nomarlization is implementation, but not in the spec. All we need is a quantization alphabet for amplitude / phase, or real / imaginary part. In our evaluation, 4-bit for phase, 4-bit for amplitude provides sufficiently good performance. |
| Lenovo | We are generally fine with the direction, but the details of normalization can be FFS. Specially we think the current suggested per-entry normalization may change the ultimate direction of the input vector. |
| Samsung | Generally fine with the proposal. |
| OPPO | Fine with this proposal. To Q1: the normalization and quantization needs to be defined so that NW-side and UE-side can align the format of target CSI reporting for different samples. To Q2: we support to use the formula of y=x-xminxmax-xmin. The value of xmin and xmax can be repoted by UE along with target CSI, or configured by NW. To Q3: support to quantize real/imaginary parts directly with the same number of bits, e.g., 4 bits for real and 4 bits for imag. |
| Futurewei | In general, we are ok with the proposal. For Q1, the normalization and quantization have to be defined so companies are aligned. For Q2, we are ok with the definition. For Q3, we are fine with using same number of bits for real and imaginary parts. |
| CATT | Generally OK with the proposal. Q1: For Option 1, normalization and quantization resolution are needed Q2: regarding how to determin xmin and xmax, same normalization among S samples is preferred Q3: For quantization, quantization over real/imaginary parts is preferred. |
| Ericsson | Genearlly find with this proposal. The selected quantization and normalization methods shall be justified by proper evaluations on the target CSI accuarcy performance. The evalucation shall cansider different CSI reporting configurations (e.g., different number of antenna ports, number of subbands, CSI payload sizes). |
| ZTE | Normalization may help reduce the quantization errors at the cost of additional UE complexity. Given that we didn’t evaluate the impact of normalization in previous release, more evaluations are needed to justify the necessity of normalization and facilitate the selection between real/imaginary quantization and amplitude/phase quantization. |
| LGE | We are fine with FL’s proposal. For Q1, defining normalization and quantization is needed for NW/UE alignment. For Q2, we support to configure independent xmax and xmin values between imaginary and real, or between amplitude and phase. These values can be reported per sample or configured by NW. For Q3, we are fine with same number of bits between real and imaginary, or between amplitude and phase. |
| Fujitsu | For SQ based data collection, one issue is reconstructing x form it normalized value y which requires the values of xmin and xmax. To realize a high quantization resolution, xmin and xmax should be determined and feedbacked for each sample. Also, it may need a higher resolution to represent xmin and xmax , e.g. scalar-16/32. We suggest adding one FFS to study the feedback overhead for the de-normalization in SQ. |
| ETRI | Q1 : needed Q2 : OK with the current definition. Maximum and minimum values can be determined by the UE implementation. Q3 : OK with same numbers for real/imag parts |
| HONOR | Fine with the direction |
| Qualcomm | We don’t see the need of bullet of normalization. Taking eType II amplitude quantization as example, we only need to define the relative the amplitude to the strongest coefficients in the quantization alphabet. How to find these amplitude the values is upto UE implementation. UE just reports the coefficients with the values defined in the alphabet. |
| Toyota | Ok with the direction proposed. |

Moderator note: Ericsson brought up a point that for Option 1, one single Target CSI sample (matrix of 2N1N2 ports, N3 subbands, v layers, and real+imaginary (or amplitude+phase) parts, with each floating element quantized to k1 or k2 bits) may be infeasible to be carried within one RRC message for up to 128 ports.

Moderator tries to calculate (in following tables) for several combinations of paramters of N1N2, N3 and v, with different k1+k2 values. It looks when k1+k2=6bis, all combinations can fall below 9KBytes.

Other solutions such as limiting the number of layers/subbands, or splitting one Target CSI sample into multiple RRC messages, are also raised for discussion.

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| FP32 (k1+k2=64bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 3328 | 6656 | 13312 |
| 2N1N2=32, N3=19 | 4864 | 9728 | 19456 |
| 2N1N2=64, N3=19 | 9728 | 19456 | 38912 |
| 2N1N2=128, N3=19 | 19456 | 38912 | 77824 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| FP16 (k1+k2=32bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 1664 | 3328 | 6656 |
| 2N1N2=32, N3=19 | 2432 | 4864 | 9728 |
| 2N1N2=64, N3=19 | 4864 | 9728 | 19456 |
| 2N1N2=128, N3=19 | 9728 | 19456 | 38912 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| Scalar8 (k1+k2=16bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 832 | 1664 | 3328 |
| 2N1N2=32, N3=19 | 1216 | 2432 | 4864 |
| 2N1N2=64, N3=19 | 2432 | 4864 | 9728 |
| 2N1N2=128, N3=19 | 4864 | 9728 | 19456 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| Scalar4 (k1+k2=8bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 416 | 832 | 1664 |
| 2N1N2=32, N3=19 | 608 | 1216 | 2432 |
| 2N1N2=64, N3=19 | 1216 | 2432 | 4864 |
| 2N1N2=128, N3=19 | 2432 | 4864 | 9728 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

| Para | Payload size per Targe CSI sample (Bytes) |  |  |
| --- | --- | --- | --- |
| Scalar3 (k1+k2=6bits) | v=1 | v=2 | v=4 |
| 2N1N2=32, N3=13 | 312 | 624 | 1248 |
| 2N1N2=32, N3=19 | 456 | 912 | 1824 |
| 2N1N2=64, N3=19 | 912 | 1824 | 3648 |
| 2N1N2=128, N3=19 | 1824 | 3648 | 7296 |
| Note: Other assistance info to indicate configuration/normalization may be additionally needed. |  |  |  |

### [Rd1] Proposal 2.3 (overhead for Option 1):

For NW side data collection with higher layer reporting, regarding Option 1 (scalar quantzation), further study the solution to limit the overhead per sample within 9KBytes (max payload size per RRC message) for up to 128 ports, e.g.,

- Alt.1: Using lower quantization resolution per floating element, e.g., using k1+k2=8 bits or 6 bits.

- Alt.2: Limit the number of layers (for precoding matrix) or subbands of the Target CSI sample (i.e., reduce N3, v values).

- Alt.3: Report one Target CSI sample via multiple RRC messages.

  - This alternative needs RAN2 to confirm

| Company | View |
| --- | --- |
| Mod | Please provide your preference on the alternatives |
| vivo | Based on our evaluation result, k1+k2<8 bits lead to significant performance loss. Therefore, if option 1 is adopted, k1+k2 should not be less than 8bits. - The SGCS performance of different quantization method. Quantization level of training data; Inference SGCS on quantization data; Inference SGCS on float test data; gain to model trained with no quantization data. No quantization; 0.73913; 0.75066; Baseline. Int8; 0.73282; 0.74880; -0.25%. Int6; 0.72933; 0.75025; -0.05%. Int5; 0.72865; 0.75664; 0.80%. Int4; 0.68628; 0.75264; 0.26%. Int3; 0.54039; 0.69630; -7.24%. Additionally, using a smaller N3 (e.g., N3<=13) might be a good solution, as the performance metric gaps between different subbands are minimal. - SGCS(s) for different subbands and/or different payloads 10/11/13/15subband mixed training; 10 sub-b; 11 sub-b; 13 sub-b; 15 sub-b. 59bits SGCS/Loss to upper bound; 0.683 (-1.3%); 0.704 (-1.2%); 0.695 (-1.5%); 0.688 (-1.1%). 104bits SGCS/Loss to upper bound; 0.750 (-2.0%); 0.772 (-1.2%); 0.762 (-1.0%); 0.758 (-0.7%). 252bits SGCS/Loss to upper bound; 0.863 (-0.8%); 0.872 (-0.3%); 0.861 (0.2%); 0.851 (+0.4%). |
| Huawei, HiSilicon | This is a valid problem and we support discussing solutions to limit the overhead per sample within 9Kbytes. Alt.1 and Alt.2 are both feasible. Assuming that the feasibility of Alt.3 will be confirmed by RAN2, a simple way to implement it is to split Target CSI along different layers/Rx-antennas. For instance, if k1+k2=16bits, 2N1N2=128, N3=19, v=2, 2 RRC messages carry Target CSI where the first/second RRC message correspond to the first/second layer. Among the three alternatives, Alt.2 is not our preferred solution since it essentially causes that the AI/ML-based CSI compression not support more than one layer for 2N1N2=128, N3=19 if (k1+k2=16bits) or more than two layers for 2N1N2=128, N3=19 if (k1+k2=8bits). Finally, we think it is important that the provided compromise in alt1/alt2/alt3 to be applicable only to the problematic cases. For instance, RAN1 can agree on Scalar8 (k1+k2=16bits) but, only for the three combinations of (2N1N2=64, N3=19, v=4), (2N1N2=128, N3=19, v=2), (2N1N2=128, N3=19, v=4), k1+k2=6bits is used. Using this combined solution, the quantization accuracy of Target CSI for lower number of Tx ports will not be compromised. |
| Apple | Alt.3 seems to be an easy solution for this problem pending RAN2’s confirmation (assuming 8bits quantization for a real or imaginary part). |
| Qualcomm | We don’t need to spend much time on this proposal, let us first study and conclude the data format first, and of course we will try to send one sample within RRC message, if not let us transmit them in multiple messages, not a concern. |
| Lenovo | While minimising overhead is ideal, it's unclear why restricting it to one message is necessary, especially since we are in the data collection phase and overhead or delay is not a major concern. If necessary, such as for higher ranks, the first two layers may be included in the initial RRC message, while the subsequent two layers can be included in the following RC message. |
| Samsung | Alt1. One RRC message should support multiple multi-rank samples. Otherwise, the cost of data collection would be very high from UE’s point of view. We believe the per-layer payload size should not exceed few thousands of bits (hundreds of bytes). |
| OPPO | Prefer Alt 1.This proposal should be studied after proposal 2.2 |
| CATT | Alt1 and Alt 3 (if confirmed by RAN2) are more promising alternatives. |
| Ericsson | We are ok to include Alt1 and Alt3 in the study scope for Option 1. Alt 2 is a different option as compared to the definition of Option 1, hence, Alt2 shall be removed. |
| ZTE | We think the easiest way may be that the maximum payload size in MDT framework is extended to over 9KBytes, which is to be confirmed by RAN2 based on RAN1 requirement. If RAN2 has some outcomes, RAN1 can further down select one option considering the trade-off between performance and overhead per sample. Per our understanding, Alt.1 would reduce the label accuracy and result in model performance degradation. Alt. 2 would limit the applicable scenario of AI/ML CSI feedback. Alt. 3 potentially bring many issues, for instance, the target CSI portions would occupy UE buffer for a relatively long time, and it’s not guaranteed that all target CSI portions can be reported since it’s also related to UE power state. |
| LGE | Similar view with apple. Reporting the target CSI via multiple messages seems to be the easiest solution among the three options. Regarding the overhead issue, criteria-based sub-sampling can be used to reduce the reporting overhead. |
| Fujitsu | As we mentioned in Proposal 2.2, NW-side need the values of xmax and xmin to reconstruct the target CSI for AI/ML training. The overhead of this part should be counted in the payload size as well. On the other hand, if one data sample must be transmitted via multiple RRC messages, we need to consider how long it takes, and whether it is feasible for a UE in high mobility status. Thus, Alt 3 is not feasible. Alt2 may face the risk fo model performance degradation, since a part of layers/subbands are not used for AI/ML training. |
| HONOR | In our understanding, to balance overhead and performance, Alt.1 is the most straightforward way. For Alt.2, further evaluation of its impact on model generalization is required. If RAN2 confirms support for Alt.3, there will be no need to discuss Alt.1 and Alt.2. Therefore, it is recommended to postpone the discussion of this issue until it is determined whether Alt.3 can be supported. |

Moderator note: Regarding Option 4, the majority companies consider L as 8, 10, 12; pv ranging from 0.6 to 1; beta ranging from 0.25 to 1; amplitude and phase follow legacy, or increase 1 or 2 bits on top of legacy. To achieve convergency on the parameter combination, some specific combinations are selected from companies’ Tdocs (also refer to “Brief summary”).

### [Rd1] (Closed) Proposal 2.4 (parameters for Option 4):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following enhanced parameter combinations with potential down selection.

- L= 10, pv=0.6, Beta = 0.5. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 10, pv=1, Beta = 0.75. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 10, pv=0.75, Beta = 1. Reference amplitude = 5 bits; differential amplitude = 5 bits; phase = 6 bits

- L= 12, pv=0.8, Beta = 0.5. Reference amplitude = 6 bits; differential amplitude = 4 bits; phase = 6 bits

Moderator note: Updated based on QC and Samsung comments. Make the ranges more diverse.

### [Rd2] (Closed) Proposal 2.4a (parameters for Option 4):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following enhanced parameter combinations with potential down selection.

- L= 6, pv=1, Beta = 1. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 10, pv=0.6, Beta = 0.5. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 10, pv=1, Beta = 0.75. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 10, pv=0.75, Beta = 1. Reference amplitude = 5 bits; differential amplitude = 5 bits; phase = 6 bits

- L= 12, pv=0.8, Beta = 0.5. Reference amplitude = 6 bits; differential amplitude = 4 bits; phase = 6 bits

- L= 16, pv=0.75, Beta = 1. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

Moderator note: Updated by testing different values of one factor at each bullet.

### [Rd3] (Closed) Proposal 2.4b (parameters for Option 4):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following enhanced parameter combinations with potential down selection.

- L= 6/12/16, pv=1, Beta = 1. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 16, pv=0.75, Beta = 1. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 1210, pv=0.6/0.8, Beta = 0.5. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 12, pv=0.8, Beta = 0.5/0.8. Reference amplitude = 6 bits; differential amplitude = 4 bits; phase = 6 bits

| Company | View |
| --- | --- |
| Mod | 4 PC seem too much - maybe down select to one or two PC? |
| Huawei, Hisilicon | We agree that only one additional PC is enough. As the first step, we think the fourth Option corresponding to L=12 may be removed as it has a considerably higher overhead than some other options with L=10 while having very similar SGCS performance. For instance, based on our simulations, compared to {L=10, p=0.9, beta=0.31, reference amplitude: 6 bits, differential amplitude: 4 bits, phase quantization: 6 bits}, Option 4 has 60% more overhead with less than 0.1% SGCS gain. |
| Qualcomm | The only setup we could consider for option 4 is pv=1 and beta=1. Further operation after SVD is not needed, especially for the compression in frequency domain with FD bases searching and ranking. |
| Samsung | For Option#4, we need to further clarify whether - SD basis selection is per layer or across layer (as in legacy). - Whether to relax the KNZP when rank>1 e.g., relax 2K0 in legacy to vK0. - For fair comparison, companies to determine and report the values for L, pv and Beta considering the same ballpark payload range as the other options, e.g., Option#1 Are the candidate values assuming 2N1N2<=32? If we go with a higher number of ports, the value for L should be relaxed, e.g., - L= 16, pv=3/4, Beta = 1. Reference amplitude = 6 bits; differential amplitude = 4 bits; phase = 6 bits Moreover, we may either preclude update on the bit width of amplitude or phase quantization or leave it up to companies reporting. |
| OPPO | No need to discuss this proposal now. If option 4 is supported, we can further discuss it. |
| CATT | OK to limit to 1 additional PC for option 4. The detailed paramters for this additional PC can be FFS. |
| ZTE | Ok in general. |
| Spreadtrum | let’s focus on proposal 2.1，and perform potential down selection between Option 1 and Option 4. |
| Fujitsu | We think 1 or 2 new PC is fine for supporting high resolution target CSI. We can have agreement on this level first. Regarding the exact number of each parameter, it can be disussed later. |
| ETRI | Same view with OPPO. |
| Qualcomm | If intention is to list all alternatives for down-selection, we propos the following L= 6 / 8 / 10, pv=1, Beta = 1. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3 / 4 / 5bits, 4 / 6bits, respectively) Basically, as long as pv=1 and beta=1, we are open to other L values and quantization resolution. |
| Toyota | This proposal would be more relevant once Option 4 is accepted. The motivation for this proposal is understood and welcome: to limit the number of parameters combinations and get an estimate of the total codebook payload. |
| ZTE | Agree with HW that L=10 should be kept considering it’s better performance-complexity tradeoff over L=12. Additionally, we prefer pv=3/4 which is more aligned with the current PC value defined for eType II. - L= 12/10, pv=0.6/0.75/0.8, Beta = 0.5. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively) - L= 12/10, pv=0.75/0.8, Beta = 0.5/0.8. Reference amplitude = 6 bits; differential amplitude = 4 bits; phase = 6 bits |
| KT | We agree with Qualcomm’s comment. By setting pv = 1 and beta = 1, we can observe the upper bound of Option 4 performance for a given L. Our proposal is to fix pv=1 and beta = 1, and then focus on introducing new L values or increasing bit budgets for amplitude/phase if necessary. |

Moderator note: To discuss the FFS on evaluations assumptions, following proposal is provided. Regarding CSI payload size, it is Moderator’s understanding that at least a large payload size (with comparable payload size to PC6/8) should be there to justify the high-resolution quantized Target CSI.

### [Rd1] (Closed) Proposal 2.5 (Evaluation assumption):

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer

- Metric is SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

- For Option 1, consider k1+k2=4, 6, 8, 16, 32

- Other assumptions follow R18 study EVM.

Moderator note: To clarify, evaluating large CSI payload is to check how much high resolution (e.g., values of k1, k2) is needed and impact the performance. If we only consider small CSI payload size, e.g., 32 bits, then small k1/k2 values can provide good performance (even legacy eT2 with PC6 may serve as label), but for large payload size, small k1/k2 values may not commit a good performance.

Tx port and subband number assumptions are added. Intention is to at least consider large Target CSI payload size case.

### [Rd2] (Closed) Proposal 2.5a (Evaluation assumption):

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Metric is SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

- For Option 1, consider k1+k2=4, 6, 8, 16, 32

- Other assumptions follow R18 study EVM.

Moderator note: Updated with QC and Ericsson comments.

### [Rd3] (Closed) Proposal 2.5b (Evaluation assumption):

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

  - rank-common layer-common with payload scalable design is considered

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Metric#1: is SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

- Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

- SGCS results are reported per layer

- For Option 1, consider k1+k2=4, 6, 8, 16, 32

- Other assumptions follow R18 study EVM.

| Company | View |
| --- | --- |
| vivo | Could clarfy why CSI payload sizes are restricted to Z ≥ 230 bits for EVM? Based on our unserstanding, the performance gains from AI become more limited as the payload size increases. |
| Huawei, HiSilicon | Support |
| Qualcomm | Layer-common and rank-common model should be considered as baseline. Multiple payload configurations, low / medium / high, should be considered with certain scalability options, e.g., multiple adaptation layers or truncation, etc. |
| Samsung | Does it mean we restrict the number of ports to 16 and 32? |
| OPPO | Same question to vivo about the Z? |
| Futurewei | We have similar question regarding the size of Z. |
| CATT | OK in general. Same question as vivo about payload Z |
| Ericsson | Same question as Vivo, we think the study shall consider different CSI reporting configurations (e.g., # antenna ports, # subbands, #CSI payload bits). Regarding the fouth bullet, if using the proposed metric, then, different AI models used by different companies may lead to very different conclusions, and it is difficult to do cross check of companies’ results. Shouldn’t the target CSI accuracy, i.e., SGCS for specific layer(s) between Target CSI of FP32 and quantized Target CSI with a specific quantization option, be considered as a metric as well? |
| ZTE | Prefer to have evaluations for different CSI payload sizes. |
| Qualcomm | At this stage, let us align with the agreement on the payload determination principle – rank-common layer-common with payload scalable design is considered. Otherwise, all these study are not practical. |
| Toyota | Similar view with Qualcomm: once the payload determination principle is agreed, this proposal would be more concise and easier to agree upon. |
| KT | We agree with Ericsson that the target CSI accuracy should be considered as a key performance metric. Meanwhile, we have a question to the FL. Will the decision between Option 1 and Option 4 be based soely on the evaluation results? From an operator’s perspective, the eTB codebook is not a feature that has been widely deployed. Our concern is that supporting Option 4 would require heavily reliance on another feature that has not yet been widely commercialized. Based on the above, we believe that the decision should consider not only the evaluation results, but also practical aspects such as implementation complexity, backward compatibility, and deployment feasilbility. |

## Issue#2-2 NW side data collection – Additional information indicated by NW/reported by UE

Brief summary:

Additional information configured by NW:

- Data quanlity threshold:  ZTE, TCL

- Rank/layer info:  CMCC, Sharp

Additional information reported by UE:

- Data quality related info: CATT, Samsung, ZTE, NTT DOCOMO, Lenovo, Tejas, TCL

  - RSRP, SINR, CQI: Samsung, ZTE, Tejas, TCL

  - positioning information: ZTE

  - SGCS: Samsung

  - Distortion level: Lenovo, Tejas

- Rank/layer info:  CATT, Samsung

  - No need to report RI/CQI: CMCC

- Configurations to generate the Target CSI/CSI feedback, including Tx ports and subband configurations CATT, Ericsson

Moderator note: Regarding assistance information for NW side data collection:

Some companies point out that the number of layers for the Target CSI should be configured by NW rather than determined by UE – seems fair?

Some companies point out the data quality related information is reported together with the logged data – seems fair; it can serve as marking the quality of the Target CSI (e.g., CSI quality measured from low SNR is generally worse than that from high SNR).

Some companies raise that cell ID and configuration information is reported together with the logged data, following R19 AI BM as captured in latest 38.331 (R2-2506530) and shown in below.

UE report:

NW configuration:

### [Rd1] (Closed) Proposal 2.6:

For NW side data collection with higher layer reporting,

- Target CSI (for precoding matrix) is generated following the number of layers configured by NW

- UE additionally reports data quality related information, e.g., CQI, RSRP, SINR, etc.

- UE additionally reports the cell ID and configuration ID (e.g., CSI-LoggedMeasurementConfigId) of the corresponding NW side data collection configuration for generation of Target CSI, reusing the R19 AI BM NW side data collection rule.

Moderator note: Updated according to Huawei, Apple comments by adding FFS on whether to attach quality indicator per data or across data.

Updated according to DOCOMO, Lenovo comments by adding FFS on the definition.

To Samsung: It is Moderators understanding that RAN2 particulary created a new RRC IE to distinguish from the CSI-ReportConfig (which is used for L1 CSI reporting), since some IEs under CSI-ReportConfig are not applicable for NW side data collection.

### [Rd2] Proposal 2.6a:

For NW side data collection with higher layer reporting,

- Target CSI (for precoding matrix) is generated following the number of layers configured by NW

- UE additionally reports data quality related information

  - FFS definition of data quality related information, e.g., CQI, RSRP, SINR, etc.

  - FFS data quality related information is per data sample or across data samples

- UE additionally reports the cell ID and configuration ID (e.g., CSI-LoggedMeasurementConfigId) of the corresponding NW side data collection configuration for generation of Target CSI, reusing the R19 AI BM NW side data collection rule.

| Company | View |
| --- | --- |
| Huawei, HiSilicon | Support. For data quality information, we think it is required not only to prune out unreliable data from cell edge UEs, but also to inform NW about the relative channel strength experienced by the same UE over different Target CSI sampling times if SQ with per-sample normailization is used. Therefore, we think L1-RSRP or SINR should be appended to each reported Target CSI sample. |
| Apple | If indeed necessary, UE’s reporting one data quality related information, e.g., RSRP, seems sufficient. |
| NTT DOCOMO | We generally support the directions. For quality information, we suggest changing the e.g. part to “FFS the definition of quality-related information”. The link quality information like CQI/RSRP/SINR is only a part of the data quality. Other UE operations like normalization and quantization also impact the data quality. |
| Qualcomm | Defer the discussion. RAN2 has a framework for data collection with configured condition, they will take the lead. |
| Lenovo | We generally support the proposal. We also believe we can further discuss the exact definition of the qualiy indicator, “FFS the definition of quality-related information” We think that in some instances, quality reflects how informative a sample is. For example, a sample with high SNR may not be considered informative if similar samples have already been reported. |
| Samsung | We need to first discuss on our assumption of configurations for data collection of CSI compression. Given several CSI-ReportConfig related parameters are needed for data collection in a CSI compression, e.g., number of port, CBSR, paramCombination, rank restriction, etc, and considering both UE-side and NW-side data collection, we believe the CSI-LoggedMeasurementConfig can be linked with a CSI-ReportConfig where the later provides such parameters. This way the same CSI-ReportConfig can be utilized for the network-side and UE-side data collection. This is efficient considering UE’s processing resources utilization. |
| Futurewei | We are ok with the proposal. |
| CMCC | Ok with first bullet. The 3rd bullet can be discussed by RAN2. |
| CATT | OK |
| Ericsson | Support in general |
| ZTE | Support in general. Regarding the 2nd bullet, we suggest adding “positioning information” in the examples, since positioning information may also enable the network to assess the data quality and determine whether and how to utilize it for training. The 3rd bullet can be determined by RAN2 since the data collection framework is designed there. |
| Xiaomi | We think a data quality threshold can be configured by NW, and the UE only reports qualified data. |
| Panasonic | We generally support the directions. |
| LGE | We are fine with FL’s proposal. Reporting quality-related information and configuration/cell ID can be helpful for NW-side data categorization and training. |
| Fujitsu | For additional information in NW side data collection, we can first agree on data quality related information. Other two may need further discussions. |
| TCL | OK |
| Toyota | Support for the general direction suggested. With regards to the first bullet, we think it would be beneficial if the UE reports the total amount of layers that the UE measures. |
| Southeast University | Support. |

## Issue#2-3(Onhold) UE side data collection – additional information

Brief summary:

Other information provided in CSI-reportConfig for UE side data collection

- Multiple candidate configurations within one CSI-reportConfig, e.g., multiple port number/subband number subject to a maximum port/subband. vivo

- Tx antenna port or subband information ZTE

Other information requested by UE for UE side data collection

- UE can indicate preferred configuration: Apple (CSI-RS density and pairing ID), CATT (same as AI BM)

Moderator note: Inputs are limited. Onhold until more inputs are collected.

## Issue#2-4 UE processing related issues for NW/UE side data collection

Brief summary:

Consider UE processing related issues for NW side data collection and UE side data collection Samsung

- CPU counting

- CSI processing timeline

- Interpretation of other configurations, e.g., CBSR, RI restrictions, if supported

Moderator note: CPU value for NW side data collection can be discussed later, depending on the quantization format which may impact the UE complexity to generate the Target CSI.

CPU value for UE side data collection may reuse R19 BM and R19 CSI prediction?

### [Rd1] (Closed) Proposal 2.7:

For CPU counting of UE side data collection,

- OCPU=1

- The CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

| Company | View |
| --- | --- |
| vivo | Support |
| Huawei, HiSilicon | Support. The same CPU counting/occupancy time from R19 BM/CSI prediction is applicable for UE side data collection for CSI compression. |
| Qualcomm | CPU counting should be applied to NW side data collection as well. |
| Samsung | Ok. |
| OPPO | Ok. |
| Futurewei | We are ok with the proposal. |
| MediaTek | Support |
| CMCC | Ok. |
| CATT | OK |
| ZTE | OK |
| Xiaomi | Support |
| Spreadtrum | Support |
| Panasonic | Support |
| LGE | Support |
| Fujitsu | OK. |
| HONOR | Support. |
| TCL | OK |
| Toyota | Support |

## Issue#2-5 Other solutions or issues

### [Rd1] Question 2.8:

For NW/UE side data collection, any other solutions or issues are missed?

| Company | View |
| --- | --- |
| Mod | Pairing ID for UE side data collection refer to Sec 4. |
| vivo | Agreement: Support the configuration of following parameters in a CSI-ReportConfig with the higher layer parameter reportQuantity set to'none-bm-r19': - TimeGap-r19, i.e., the time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance. - nroftimeinstance-r19, i.e., number of predicted time instances for BM-Case 2. Agreement: For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.. According to BM agreement and inference report configuration, separate configuration of subband, Tx port and payload size from pairing ID can be configured |
| Panasonic | We think the necessity and feasibility of UE reporting UE-side additional conditions for UE-side data collection and/or NW-side data collection should be discussed. |

# Monitoring

## Summary of company proposals

### 3.1-1: Generic

| Company | View |
| --- | --- |
| CATT | Proposal 11: For AI/ML based CSI compression Case 0, regarding performance monitoring, support L1 signaling via CSI report. |
| ETRI | Proposal 4: For AI/ML-based CSI compression, study both NW-side and UE-side performance monitoring to leverage their respective advantages. |
| Lenovo | Proposal 17: For monitoring of CSI-compression, configure the UE with additional report-config instructing it how to generate the additional data needed for monitoring of the model, e.g., the quantized version of the RAW CSI for NW-side monitoring or computed performance metric for UE-assisted monitoring. Proposal 19: Further specify the required procedure/signalling for model updating, switching or model fall-back as a result of a) model monitoring outcome, or b) request from the UE, or c) change of some configuration parameters. |
| NEC | Observation 2: Both NW and UE side performance monitoring are feasible and a combination of NW and UE side monitoring can be supported. Proposal 8: Study the mechanism and specification impact of identifying root causes for performance degradation, enabling accurate diagnosis across different root causes with corresponding impacts on LCM actions. |
| NTT DOCOMO | Observation 1: - NW-sided monitoring: The performance can be monitored by the ground truth (Target CSI) report from the UE with some UL overhead. - UE-sided monitoring: - Based on the proxy model or direct KPI estimation: Inaccurate and biased because the UE does not know the CSI reconstruction implementations, and it has limited capability. - Based on the RS from the NW: Introduce DL overhead and UE complexity for additional measurements and CSI processing. - The UE-sided monitoring based on the proxy model or direct KPI estimation is not suitable for standalone usage, but it can serve as a trigger for the NW-sided monitoring. Proposal 1: - Support the NW-side performance monitoring with UE-reported ground truth. - The UE-side performance monitoring with proxy model or direct KPI estimation can be supported given the following conditions, - The NW-side performance monitoring is supported, and the ground truth report can be event-trigger-based. - The UE-side performance metric serves only as a trigger for the ground truth report. |
| OPPO | Proposal 6: Regarding the performance monitoring for AI/ML based CSI compression, support - UE-side monitoring Case 2-1/Case 2-2 - NW-side monitoring according to UE capabilities. Possible combination of UE-side monitoring Case 2-1 and NW-side monitoring. |
| Panasonic | Observation 1: There are at least two purposes for performance monitoring. The monitoring purpose 1 is to check new untested model / parameter behavior. The monitoring purpose 2 is to check current model / parameters are suitable to current environment. Observation 2: If AI/ML models are trained by UE-side and AI/ML model update cycle is non-real time, UE vendor specific monitoring in offline for monitoring purpose 1 sufficiently work. Observation 3: If AI/ML models are trained by network side and AI/ML model update cycle is non-real time, A/B test can be used, and data collected for non-real time performance monitoring for monitoring purpose 1 can also be used for model training. Observation 4: - For monitoring purpose 1, the monitoring mechanisms to detect degradation could be sufficient. - For monitoring purpose 2, both monitoring mechanisms to detect degradation and to identify the cause of degradation would be needed. Observation 5: - As the monitoring mechanism to detect degradation, the following is useful. - NW-side or UE-side eventual KPI-based monitoring - UE-side intermediate KPI monitoring based on the output of the CSI reconstruction model at the UE - As the monitoring mechanism to identify the cause of degradation, the following is useful. - NW-side intermediate KPI monitoring based on the target CSI reported by the UE via legacy eTypeII codebook or eTypeII-like codebook. - UE-side intermediate KPI monitoring based on the output of the CSI reconstruction model indicated by NW. |
| Qualcomm | Observation 5: There are spec-transparent performance monitoring mechanisms, such as throughput comparison, CQI comparison with legacy CSI codebooks, or obtaining ground-truth CSI using existing CSI report with existing codebook for NW side monitoring. Proposal 5: Spec-transparent monitoring can be used; the necessity of specifying dedicated monitoring methods needs justification. - If needed, any standardized performance monitoring for two-sided AIML CSF should consider UE complexity and require additional capability than the feature itself. |
| Samsung | Observation 8: For two-sided model-based CSI compression, the following monitoring types were identified - Type 1: UE-assisted monitoring: UE calculates monitoring metric - Type 1-1: UE-assisted monitoring without precoded CSI-RS - Type 1-2: UE-assisted monitoring with precoded CSI-RS - Type 2: NW-side monitoring: NW calculates monitoring metric from report ground-truth CSI Proposal 8: Study the benefits and cost of the following monitoring types after some progress in the reference model discussion and Target CSI format - Type 1: UE-assisted monitoring: UE calculates monitoring metric - Type 1-1: UE-assisted monitoring without precoded CSI-RS - Type 1-2: UE-assisted monitoring with precoded CSI-RS - Type 2: NW-side monitoring: NW calculates monitoring metric from report ground-truth CSI - Note: cost analysis should include at least measurement and feedback overhead, UE complexity and network complexity. |
| vivo | Proposal 9: Similar to CSI predication, periodic, semi-persistent, and aperiodic CSI-RS are supported, and semi-persistent, and aperiodic monitoring report type are supported for the monitoring of CSI compression. |

### 3.1-2: NW side monitoring

| Company | View |
| --- | --- |
| Apple | Proposal 5: For CSI compression using two-sided model, for NW side performance monitoring, UE report CSI feedback and target CSI pair using immediate MDT with logging framework. The MDT framework can be used for root cause detection following UE side performance monitoring. |
| ASUSTeK | Proposal 3: For NW side performance monitoring, UE transmits assistance information of UE-side encoder’s performance along with ground truth reporting. |
| CATT | Proposal 12: For AI/ML based CSI compression Case 0, support NW-side monitoring based on the target CSI reported by the UE via high resolution codebook. - Monitoring report content is the target CSI, i.e., SF-domain eigenvector - Legacy Rel-19 eType II codebook can be considered as the starting point Proposal 13: Regarding AI/ML based CSI compression Case 0, for NW-side monitoring based on target CSI reporting, monitoring report is separately configured from inference report. - Association between monitoring report and inference report is managed by gNB in a specification transparent manner |
| CMCC | Proposal 13: For CSI compression using two-sided model use case, support L1 signaling for NW side monitoring, and the content is Target CSI only - Quantization format of Target CSI is the same as for NW side data collection. |
| Ericsson | Observation 3: Eventual KPI based monitoring has low complexity, low overhead, and can capture network MU-MIMO performance. The NW can perform frequent monitoring of eventual KPIs via NW implementation-based solutions and use it as a first step for detecting potential AI/ML feature/functionality failure. Observation 4: It is necessary to specify UE reporting high resolution target CSI to enable NW-side intermediate KPIs based performance monitoring. Observation 5: It is necessary to specify UE reporting target CSI and CSI feedback pair to enable performance degradation error cause detection for the two-sided CSI-compression use case. Observation 6: NW-side monitoring of the two-sided CSI-compression model based on target CSI reporting is expected to be executed infrequently (e.g., event triggered or periodically with a large periodicity), hence, the monitoring data collection overhead for this model monitoring method is in general not an issue. Proposal 10: Support UE reporting multiple target CSI and CSI feedback pairs via high-layer signaling for NW-sided performance monitoring: - Reuse the target-CSI format defined for NW-side training data collection - Enhance the higher-layer signaling mechanism designed for NW-side training data collection to support UE reporting target CSI and CSI feedback pairs |
| ETRI | Proposal 5: For AI/ML-based CSI compression, for NW-side performance monitoring, consider L1 signaling for low latency, and consider report config ID or pairing ID for linkages between AI/ML-based CSI feedback and the ground-truth CSI. |
| Fujitsu | Proposal 1: Support NW-side performance monitoring based on target CSI reporting from UE, by using the format of eT2-like codebook with new parameters. Proposal 2: Regarding the content of CSI report for NW-side performance monitoring, at least L1 report is supported. - Support the reporting of a paired Target CSI and CSI feedback in one CSI report. |
| FUTUREWEI | Observation 2: For NW-side monitoring, using L1 for reporting Target CSI may have the benefit of allowing NW to receive performance report in a timelier manner compared to L3 reporting. Proposal 3: In AI/ML-based CSI compression using two-sided model Case 0, at least for NW-side monitoring, consider using L1 signaling for Target CSI reporting and adopting the following option for the content: - Option 2: Target CSI only (assuming NW obtains CSI feedback via inference CSI report) |
| Google | Proposal 3: For performance monitoring, support the network to configure the UE to report an additional W2 based on the eType2 codebook for an inference report. |
| HONOR | Proposal 1: For L1 performance monitoring, reuse the legacy codebook without introducing new codebook parameters. Observation 1: For inference reports with rank > 2, the high-resolution codebook with PC#8 cannot be used for monitoring purposes, which impact monitoring accuracy. Proposal 2: Based on the legacy PC table, for the inference reports with rank > 2, partial layers of the Target CSI can be reported by UE via higher-resolution codebook with PC#8. Proposal 3: For NW-side monitoring using higher layer signaling, support Option2, only reporting Target CSI, and further study the pairing rule for target CSI and CSI feedback. Proposal 4: For L1 signaling-based target CSI reporting, further study how to reduce overhead, such as reporting only a portion of the target CSI. |
| Huawei, HiSilicon | Proposal 9: For NW side monitoring, reuse the same Target CSI quantization format(s) that are supported for NW side data collection. Proposal 10: For NW side monitoring, support inclusion of the following content in the report: - Paired Target CSI and CSI feedback if L3 reporting is supported - Target CSI if L1 reporting is supported - Note: In this case, NW obtains the associated CSI feedback via inference CSI report. |
| InterDigital | Observation 2: NW-side monitoring can have a significant impact on signaling overhead. Proposal 2: For NW-side model monitoring, study low overhead signaling mechanisms, e.g., use of predefined vector sets, redundancy elements NW-side monitoring with |
| KT Corporation | Proposal 2: Support scalar quantization for performance monitoring, with further study to reduce overhead by leveraging sparsity in angular-delay domain. |
| Lenovo | Proposal 18: Support the following combinations for the AI-output CSI-report and the type of monitoring report. Monitoring report type AI-output report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support. |
| LG Electronics | Proposal 3: For NW-side monitoring, support target CSI reporting via L1 signaling, paired with CSI feedback. |
| MediaTek | Proposal 6: Consider Rel 16 eType II’s ParComb 6 and/or 8 for ground truth CSI reporting for NW-side monitoring. |
| NEC | Proposal 5: For NW-side monitoring, for reporting compressed CSI and reporting (corresponding) target CSI, discuss whether it is still necessary to report other information besides PMI (e.g., RI, CQI) for the latter report in the time domain. Proposal 6: Support both L1 signaling and L3 signaling for target CSI reporting. Proposal 7: NW can configure target CSI reporting based on the SGCS results, with MAC CE to indicate whether target CSI reporting needs to be performed. |
| Nokia | Proposal 11: Prioritize NW-side performance monitoring for CSI compression using two-sided model. - RAN1 to focus on addressing OTA feedback overhead and UE complexity issue associated with ground-truth target CSI reporting. Proposal 12: RAN1 to agree to use the same data format for ground-truth target CSI reporting, for NW side data collection and for NW side performance monitoring. Observation 6: For NW side performance monitoring, the required feedback overhead for GT target CSI reporting can be reduced by adjusting its resolution with respect to the resolution of the AIML-enabled CSI acquisition scheme under monitoring. Lower resolution of the CSI inference report being monitored requires lower resolution of the ground truth target CSI. Proposal 14: For NW-side performance monitoring, to reduce the feedback overhead of reporting the ground truth target CSI, introduce a definition of ground truth target CSI with an adjustable resolution with respect to the resolution of the CSI inference report being monitored. |
| NTT DOCOMO | Proposal 2: - If the high-layer signalling is adopted for the ground truth reporting, the signalling selection is up to RAN2. RAN1 should further study the following issue, - The latency requirements, report occasions, and payload size of the report, which help RAN2 to select the reporting mechanism. - The periodicity of the report, including the support of event-triggered reporting. - The association between the high-layer ground truth report and Layer 1 inference report. |
| OPPO | Observation 2: Regarding NW-side monitoring Case 1-1, two concerns from overhead and UE capability aspects should be addressed. Proposal 7: Regarding the CSI report configuration for NW-side monitoring, consider: - Option 1: one CSI report configuration for both of inference and monitoring report on the same CSI report resource. - New reportQuantity or combination of two PMIs in reportQuantity is required to indicate UE reports both of inference from AI/ML based CSI and target CSI with legacy codebook for monitoring - Option 2: dedicated CSI report configuration for monitoring report on dedicated CSI report resource different from inference report. - Codebook type and parameter combinations for target CSI calculation should be configured. |
| Qualcomm | Observation 6: SGCS statistics or average SGCS across multiple inference samples are needed to determine the data distribution drift. Observation 7: High monitoring accuracy may not matter much in monitoring decision, the dominating factor is the natural SGCS variation / statistics across multiple samples. Proposal 6: SGCS statistics or average SGCS across multiple inference samples should be taken into account for the study of the necessity and feasibility of NW side monitoring. Observation 8: Performance monitoring is not time-critical and requires multiple target CSI samples to identify data drift, so there is no fundamental difference between NW side monitoring and NW side data collection. L3 signalling is a more proper solution that allows UE to log multiple target CSI in a flexible manner with lower cost than L1 signalling. Proposal 7: For Rel-20 CSI feedback with two-sided model, support L3 signalling and mechanism for target CSI reporting for NW side monitoring. Framework for NW training data collection can be a starting point. Proposal 8: For NW side monitoring, study the SGCS variation in a dataset A vs. SGCS variation in a dataset B where dataset A is considered as “in-distribution” and dataset B is considered as “out-of-distribution”. Define false-alarm and miss-detection event using dataset A and B. Observation 9: Increasing number of samples used in averaging the SGCS measurement improve the monitoring decision accuracy. Observation 10: Using existing parameter-combinations of eType II codebook is sufficient to achieve good monitoring results. Observation 11: If NW data collection framework is used for NW side performance monitoring, the data format used for data collection can be reused for monitoring. Proposal 9: Further optimization on data format for NW performance monitoring is not needed. Data format for model training can be reused if L1 mechanism is supported for NW side performance monitoring. |
| Shanghai Jiao Tong University, Toyota | Observation 1: For two-sided model-based CSI compression, Signaling to carry the reported data - Type 1: L1(L1 signaling with legacy eType II) - Type 2: Higher layer, with MDT mechanism (same as NW side data collection) Observation 2: For two-sided model-based CSI compression, Content of the reporting - Type 1: Paired Target CSI and CSI feedback - Type 2: Target CSI only Observation 3: Performance monitoring exposes a signaling trade-off—while L1 signaling can be overly aggressive for enhanced eType-II implementations, L3 signaling does not provide bounded timeliness (no guaranteed latency), and therefore may fail to satisfy the latency requirements critical for effective network-side performance monitoring. Proposal 1: Adopt a hybrid monitoring flow: perform lightweight, low-latency monitoring via eventual-KPI checks, SRS based or legacy eType II over L1 with configured thresholds; when a threshold is exceeded the NW instructs the UE to record multiple sets of Paired Target CSI (higher-resolution format) and corresponding CSI feedback per configuration, then upload them via L3 signaling with MDT mechanism for network-side high-precision evaluation, root-cause analysis and subsequent model processing. |
| Sharp | Proposal 4: For NW side monitoring, target CSI reporting via eTypeII codebook can be supported as baseline. - Support target CSI reporting with configured rank by network - FFS: whether/how to support eTypeII codebook with higher-resolution parameters with UE capability Proposal 5: For NW side monitoring, support to reuse CSI framework for monitoring report in L1 signalling. - For monitoring report type, semi-persistent on PUSCH and aperiodic CSI report are supported. - FFS: whether some contents in existing quantity can be omitted and whether a new report quantity is configured. |
| Sony | Proposal 1: RAN1 to adopt Approach 3 for performance monitoring of AI/ML-based CSI compression, where: - the UE transmits both compressed CSI and ground truth CSI on configured resources, and the gNB performs the comparison Proposal 2: RAN1 to consider performance monitoring frequency, including: - Periodic monitoring - Event-triggered monitoring based on eventual KPIs measured by the NW, such as BLER or throughput thresholds, to minimize overhead while ensuring timely detection of model degradation. Proposal 3: RAN1 should adopt the same intermediate KPI (SGCS) for monitoring that was adopted for CSI prediction for CSI compression. |
| Spreadtrum, UNISOC | Proposal 4: For performance monitoring, support NW-side monitoring based on the ground-truth CSI reported by UE with legacy Rel-16 eType II codebook. Proposal 5：L1 signaling is preferred that allows NW to quickly monitor model performance and trigger fallback if necessary. |
| TCL | Proposal 2: For the NW side end-to-end performance monitoring, the label acquisition could be based on the existing CSI report framework, e.g., both target CSI and CSI feedback are configured as the report quantities. Proposal 4: The request of target CSI, encoder output and quantizer output could be based on the existing CSI report framework for the root cause analysis. |
| vivo | Proposal 6: Target CSI and CSI feedback pair report can be supported for NW side monitoring and error cause identification - The CSI feedback is the output derived from the recovery precoding matrix by target CSI (e.g., quantizing Target CSI) |
| Xiaomi | Proposal 8: Higher layer reporting for performance monitoring at NW side is supported. Proposal 9: The report content including pairs of target CSI and CSI feedback Proposal 10: The paring ID is indicated to UE for higher layer reporting for performance monitoring at NW side |
| ZTE Corporation, Sanechips | Proposal 11: For AI/ML CSI feedback, support NW-side performance monitoring based on the target CSI reporting with CSI codebook enhancement via higher resolution parameter combination. |

### 3.1-3: UE side monitoring

| Company | View |
| --- | --- |
| Apple | Observation 1: For UE side performance, NW implicitly transmit output CSI using precoded CSI-RS or DMRS to the UE, provides a simple and low-overhead solution. In addition, comparing with legacy codebook based on defined KPIcodebook and KPIactual can help to determine whether fall back decision is required. Proposal 3: For CSI compression using two-sided model, support UE side performance monitoring with pre-coded RS using output CSI. Define monitoring KPI using precoding gain difference between KPIcodebook and KPIactual. Proposal 4: For CSI compression using two-sided model, for UE side performance, support RLF/BFD like mechanism. UE transmit fallback request via the applicability report procedure with UAI. |
| CATT | Proposal 14: For AI/ML based CSI compression Case 0, performance monitoring at UE side based on reference model or proxy model can be deprioritized. Proposal 15: For AI/ML based CSI compression Case 0, support UE-side monitoring based on precoded RS (e.g., CSI-RS) transmitted from NW with RS precoded by the output of the CSI reconstruction model (i.e., decoder output). - Consider monitoring metrics (i.e., report quantity for monitoring report) including SGCS, RSRP/differential RSRP. Proposal 16: Regarding AI/ML based CSI compression Case 0, for UE-side monitoring based on decoder output precoded RS, reuse performance monitoring mechanism via CSI report for AI BM and AI CSI prediction in Rel-19: - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link inference report configuration and monitoring report configuration - For the time behavior of monitoring report, semi-persistent and aperiodic reporting are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For time domain behavior of monitoring RS resources, semi-persistent and aperiodic resources are supported. Legacy rule on CSI report should be applied for allowed time domain behavior combination of RS resources and monitoring report. |
| CMCC | Proposal 11: For CSI compression using two-sided model use case, it is proposed to deprioritize UE side monitoring with proxy model / direct estimator of KPI / reference decoder. Proposal 12: For CSI compression using two-sided model use case, it is proposed to deprioritize UE side monitoring with precoded RS. |
| Ericsson | Proposal 11: Supported UE-side performance monitoring based on UE reporting an intermediate-KPI based performance metric (e.g., a SGCS value range indicator), at least the following spec impacts are identified: - The format of the performance monitoring metric (e.g., a SGCS range indicator) - L1 signaling for UE reporting the monitoring metric (e.g., a SGCS together with the CSI feedback via inference CSI report) Proposal 12: To ensure the quality of UE reported monitoring metric, at least the following spec impacts are identified: - RAN4 testing of the quality of UE reported monitoring metric - Mechanisms (e.g., RRC-message based methods) to support UE reporting target CSI, encoder output, together with the associated intermediate-KPI based performance metric to the NW. Observation 7: UE-side monitoring based on precoded RS transmission from NW introduces RS signaling overhead, latency and processing complexity at both UE and NW, without clear benefit. Proposal 13: UE-side performance monitoring based on precoded RS transmission from NW is not supported. |
| ETRI | Proposal 6: For AI/ML-based CSI compression, for UE-side performance monitoring, consider L1 signaling to report performance metric such as SGCS. |
| Fujitsu | Proposal 3: Regarding UE-side performance monitoring, AI/ML-based performance monitoring is not supported unless how to guarantee the monitoring model’s performance can be clarified/justified. Proposal 4: Study on the standard impact for UE-side performance monitoring is suggested to be deprioritized. |
| HONOR | Proposal 5 Deprioritize UE-side performance monitoring, if supported, precoded RS based monitoring can be considered. |
| Huawei, HiSilicon | Proposal 11: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring. Proposal 12: Precoded RS based monitoring should not work as the only option without NW side monitoring. |
| InterDigital | Observation 1: For UE-side model monitoring, - Both time-based trigger and event-based trigger can be used for reporting monitoring metrics. - Inappropriate monitoring metrics may lead to unnecessary model updating or switching. Proposal 1: For UE-side model monitoring, study the following aspects, - Details of reporting mechanism for the monitoring metrics with both time/event-trigger based - Appropriate UE-side monitoring metric other than SCGS, e.g., out-of-distribution metrics, performance gap between the actual and encoded CSI, etc. |
| Lenovo | Observation 10: Model monitoring based on metric calculated at the UE has been specified for BM use case. This confirmed the possibility of having UE-sided monitoring without the need for extra NW-side monitoring for checking the reliability of UE-side monitoring reports. Proposal 15: Conclude that the local decoder model (or proxy model) that is trained using data shared in Direction A sub-option 4-1 and Direction C has a good match with the NW-side decoder model and it can be used for UE-side monitoring and UE-side root-cause determination. Proposal 16: Conclude that UE-sided model monitoring can be used at least for inter-vendor collaboration options based on Direction A sub-option 4-1 and Direction C. |
| MediaTeK | Proposal 7: If UE-side monitoring supported, discuss the following options to specify how UE reports a monitoring event to NW: - UE can calculate and report from a set of specified monitoring metrics. - UE can report specified monitoring output (i.e., occurrence) cause by a monitoring event. Proposal 8: Deprioritize UE-side monitoring due to additional burden on UE with no significant benefit. |
| NEC | Proposal 3: For UE-side monitoring, support SGCS as performance metric, SGCS is calculated based on target CSI and reconstructed CSI. Proposal 4: For UE-side monitoring, support event triggered performance monitoring report. The event is defined based on the SGCS lower than a threshold value for a time duration. |
| OPPO | Observation 3: Regarding UE-side monitoring Case 2-1/Case 2-2 there is no overhead and UE capability concerns. Proposal 8: UE-side monitoring Case 2-1 and Case 2-2 with SGCS report can be supported according to UE capability. Proposal 9: Regarding SGCS report format in UE-side monitoring, SGCS1 and SGCS2 agreed in CSI prediction agenda can be reused, wherein: - SGCS1 denotes the SGCS between target CSI and proxy model output in Case 2-1 or the direct output of SGCS estimator in Case 2-2. - SGCS2 denotes the SGCS using legacy codebook configured by NW-side within the CSI report configuration. Proposal 10: Regarding the CSI report configuration for UE-side monitoring, consider: - Option 1: one CSI report configuration for both of inference and monitoring report on the same CSI report resource. - New reportQuantity or combination of PMI and SGCS in reportQuantity is required to indicate UE reports both of inference from AI/ML based CSI and SGCS monitoring result - Option 2: dedicated CSI report configuration for monitoring report on dedicated CSI report resource different from inference report. Codebook type and parameter combinations for SGCS2 calculation should be configured. Proposal 11: Support UE-triggered NW-side monitoring to reduce the reporting overhead of target CSI and improve the accuracy of UE-side monitoring. |
| Qualcomm | Proposal 10: Do not support UE side monitoring. |
| Samsung | Observation 9: For two-sided model-based CSI compression, for Type 1-1: UE-assisted monitoring without precoded CSI-RS, the UE may calculate performance metric, e.g., SGCS, using - Alt1: Specified reference decoder - Alt2: NW-shared proxy decoder - Alt3: direct metric estimator |
| Sharp | Proposal 6: For UE side monitoring, further study and down-select the following options of UE-side monitoring options: - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook |
| Spreadtrum, UNISOC | Proposal 6: For performance monitoring, support UE-side monitoring based on the recovery CSI indicated by NW or precoded RS transmitted form NW. |
| TCL | Observation 2: The UE side end-to-end performance monitoring can be performed by UE implementation. Proposal 3: The UE needs to report the monitoring metric values or monitoring outcome if the final decision-making is performed at NW side. |
| vivo | Proposal 7: UE side monitoring can be supported for CSI compression with proxy model based solution. - SGCS can be a report format of monitoring Proposal 8: For the definition of SGCS, - For a given layer l∈{1,…,v}, subband n3∈{1,…,N3}, and data instance n4∈1,…,N4，SGCS is defined as SGCSn4,ln3=wn4,ln3Hwn4,ln3wn4,ln3wn4,ln32 where wn4,ln3 is the reconstructed precoder by NW-side decoder of l-th layer and n3-th subband, n4-th data instance, and wn4,ln3 is the corresponding precoder represented by PMI used for ground-truth target CSI for the l-th layer, n3-th subband, n4-th data instance - SGCS is calculated by - wideband frequency granularity - one or multiple data instances configured by NW - per layer |
| Xiaomi | Proposal 11: Precoded RS based solution is preferred comparing with model-based solution for UE side performance monitoring. Proposal 12: The configuration for UE side performance monitoring is linked with a CSI report configuration for inference which including a paring ID, if UE side performance monitoring is supported. |
| ZTE Corporation, Sanechips | Proposal 12: For AI/ML CSI feedback, prioritize NW-side performance monitoring and take UE-side performance monitoring as second priority. |
| Southeast University | Proposal 2-1: Specify UE-side performance monitoring based on a lightweight proxy decoder, with requirements including architecture, training, and performance metrics. Proposal 2-2: Define the evaluation metrics as normative performance requirements for UE-side proxy monitoring, specifying the calculation method and minimum/maximum allowed values for each: average SGCS prediction gap, classification precision and recall, and UE-NW reconstruction alignment. Proposal 2-3: Specify signaling and reporting mechanisms to support the proxy decoder framework. The NW shall signal the proxy decoder’s configuration (e.g., architecture ID, distillation parameters) to the UE via RRC signaling during model activation. The UE shall report the proxy’s predicted SGCS (or a SGCS range indicator) to the NW alongside the CSI feedback to enable joint performance analysis. |

## Discussions

## Issue#3-1 (Onhold) Prioritization between NW side monitoring and UE side monitoring

Brief summary:

Prioritize NW side monitoring/Deprioritize UE side monitoring: ZTE, CMCC, Qualcomm, Nokia, LGE, MediaTek, Fujitsu, [HONOR]

- Qualcomm Any implementation related to UE side monitoring seems to be additional cost without clear benefit.

## Issue#3-2 NW side monitoring

Brief summary:

Quantization format of Target CSI

- Follow the same format as NW side data collection: Huawei, HiSilicon, Nokia, CMCC, NTT DOCOMO

- eType II(-like) format with higher resolution parameter combination: ZTE, Tejas

- legacy eType II codebook: CATT, LGE, Tejas, Sharp, MediaTek, HONOR, Spreadtrum

Content for reporting:

- Option 1: Paired Target CSI and CSI feedback

  - Support: Huawei, HiSilicon (for L3 reporting), Ericsson, vivo, Apple, Fujitsu, TCL, Sony

- Option 2: Target CSI only (assuming NW obtains CSI feedback via inference CSI report)

  - Support: Huawei, HiSilicon (for L1 reporting), CMCC (for L1 reporting), LGE (for L1 reporting), Futurewei (for L1 reporting), Futurewei (for L1 reporting), HONOR

  - Not support: Huawei, HiSilicon (for L3 reporting) Xiaomi

    - Huawei, HiSilicon, Ericsson: NW would not know the logged Target CSI corresponds to which reported resources during inference

    - Xiaomi: CSI omission may happen for CSI feedback at inference CSI report

- Other views

  - Introduce monitoring Target CSI which is adjustable resolution to the CSI feedback of the associated inference CSI report Nokia

  - UE may not be needed to report RI, CQI NEC

  - NW to configure the rank Sharp

Signaling to carry the reported data:

- L1: Qualcomm (L1 signaling with legacy eType II), Nokia, CATT (with legacy eType II), Huawei, HiSilicon, [OPPO], [Lenovo], LGE, NEC, Sharp, Futurewei, ETRI, Fujitsu, HONOR (with legacy eType II), Spreadtrum (with legacy eType II)

  - Separate CSI report configuration for monitoring from inference CATT, Huawei, HiSilicon, OPPO, Lenovo

    - Association between monitoring report and inference report is managed by gNB in a specification transparent manner CATT, Huawei, HiSilicon

    - Linkage between monitoring and inference by report ID/pairing ID ETRI

  - One CSI report for both inference and monitoring report OPPO

- L3/higher layer: Ericsson, Qualcomm, Apple, Xiaomi, Huawei, HiSilicon, NEC

- UE requested NW side monitoring: OPPO

Moderator note: From inputs of companies, L1 signaling for NW side monitoring seems bundled with Target CSI of eType II codebook, especially legacy eType II. Higher layer for NW side monitoring may work with Target CSI of higher resolution, similar to data collection.

For L1 signaling, there seems no additional spec impact (an individual CSI report to carry legacy eT2 CB is configured, and the association to the inference report is managed by NW), so, let’s try to make a conclusion.

### [Rd1]  Proposed conclusion 3.1 (L1 signaling):

For NW side monitoring, it is NW implementation to configure a dedicated monitoring CSI report with legacy eType II codebook via L1 signaling feedback for monitoring the R20 inference CSI report.

- Note: association between monitoring CSI report and inference CSI report is managed by gNB (e.g., by configuring the same RS resources) and transparent to UE.

- Note: other implementation based solutions can also be adopted for monitoring, e.g. eventual KPI based, SRS based, etc.

| Company | View |
| --- | --- |
| vivo | OK |
| Huawei, HiSilicon | We agree that, for L1 signalling, the association between monitoring CSI report and inference CSI report can be managed by gNB. However, since both CSI feedback and Target CSI will be sent on two different UCIs, one or both UCIs may be (partially) omitted and, as a consequence, the performance metric (eg, SGCS) at the NW side may not be correctly calculated. One simple solution might be not using (CSI feedback, Target CSI) pair for monitoring if at least one of the target CSI or CSI feedback is (partially) omitted. Another advantage of this solution is that the L1 signalling-based NW side monitoring can still be supported based on implementation and without any specification. |
| Apple | We need to be aware that NW side monitoring is costly. As such, operator should avoid running it too often. UE side performance monitoring provides the best insights on when NW should run NW side monitoring to find the root cause of performance degradation. Thus we prefer UE side performance monitoring to be discussed and decided together. |
| Qualcomm | We don’t need this proposal. In Rel-19, we concluded that monitoring via legacy CSI codebook is feasible. This conclusion is sufficient. It implies that it’s upto NW to configure a CSI report to acquire a target CSI using legacy eT2 format. From UE perspective, it’s just a CSI report, nothing else. There is no spec impact. If a conclusion is needed, we should just say “NW side monitoring can be achieved using legacy CSI report without specification impact.” Or “NW side monitoring can be achieved by NW implementation.” |
| Lenovo | First we agree with Apple on the complexity and cost of NW-based monitoring. Secondly, this independent configuration of monitoring data requires unnesecray additional measurement at the UE. Alternatively, including the pairing-ID in the configuration message allows the UE to perform a single measurement for both CSI feedback calculation (inference) and generating the monitoring report (etype-2 report). |
| Futurewei | We are not sure this proposal has any specification impact as it can be done by NW implementation. Some clarification would be helpful. |
| CMCC | Fine with the proposal but there seems no specification impact. |
| CATT | OK |
| NEC | OK. It is more suitable to have L1 report for monitoring. |
| ZTE | OK in general. Regarding the last note, we suggest removing the e.g. to be generic, since we didn’t justify the feasibility of the two methods. For example, even if eventual KPI is bad, it may not be impacted by AI model only, but also some other factors, e.g., channel variation, scheduling, etc. While for SRS based method, we are not sure it may work due to the weak reciprocity. |
| Xiaomi | The proposal seems not necessary as it is gNB implementation and transparent to UE. |
| Spreadtrum | OK It is transparent to UE to report the CSI with legacy eType II codebook via L1 signaling. |
| Panasonic | If the association between monitoring CSI report and inference CSI report is managed by gNB and transparent to UE, UE may change UE-side additional condition (such as Rx filter assumptions) even if RS resource configuration is same. Do we need to consider such situation? |
| LGE | First of all, our preference has not been captured properly. We prefer option 1 for reporting contents, since a dedicated monitoring report requires unnecessary additional measurement at the UE (e.g., buffering, CPU), as Lenovo mentioned. However, Option1 seems to require more spec effort compared to option 2. If dedicated monitoring report is supported, it can be further discussed whether some information other than PMI (e.g., CQI) still needs to be reported, which may have a spec impact. |
| Fujitsu | We think the ground truth for NW side monitoring should be feedbacked by eType II like codebook with new parameters via L1 signaling. |
| Sony | OK |
| HONOR | It is fine, but there seems no specification impact. |
| Toyota | Some clarifications on this proposal are necessary. If the understanding is correct and the goal of this proposal is to state that there is no explicit or implicit association of the configurations/resource allocations for monitoring CSI and for inference CSI, then, the proposal is not necessary. |

Moderator note: For higher layer signaling to report Target CSI, it seems the report of Target CSI only may not be workable: one thing is that CSI feedback from inference may be an omitted version; another issue is that gNB/UE may miss the UCI/DCI, so that the pairing between CSI feedback (in L1 inference CSI report) and Target CSI (in higher layer reporting) may not be ensured.

### [Rd1]  Proposal 3.2 (higher layer signaling):

For NW side monitoring, support the higher layer signaling for reporting.

- Regarding the reporting content, consider paired Target CSI and CSI feedback

  - FFS CSI feedback is before or after UCI omission.

- Quantization format of Target CSI is the same as for NW side data collection

Moderator note: Updated the UCI omission part. The intention is to distinguish from L1 CSI feedback which considers UCI omission.

Regarding Lenovo, CMCC, CATT comments, if we adopt high-resolution (e.g., Option 1) for Target CSI format, how is it possible to be carried at L1 signaling? Or, what you have in mind is just reusing legacy eType II CB with NW implementation?
Regarding ZTE comment, for NW side data collection, indication of time stamp is precluded by RAN2. It is Moderator’s feeling that the same thing may happen for monitoring via higher layer.

### [Rd2] Proposal 3.2a (higher layer signaling):

For NW side monitoring, support the higher layer signaling for reporting.

- Regarding the reporting content, at least consider paired Target CSI and CSI feedback

  - Note: FFS CSI feedback is the one before or after UCI omission.

- Quantization format of Target CSI is the same as for NW side data collection

| Company | View |
| --- | --- |
| vivo | Support |
| Huawei, HiSilicon | Support the proposal. If CSI feedback is paired with Target CSI in a higher layer signaling, there won’t be any UCI (partial) omission issue. |
| NTT DOCOMO | We are generally fine with the higher-layer reporting. For the first bullet, we think the approaches without reporting the CSI feedback should not be precluded at this moment. Some methods, like UE reports an index with CSI feedback for the association purpose, can be studied to reduce the overhead. For the second bullet, we suggest the following, - Quantization format of Target CSI is the same as or a truncated/low-resolution version of that for NW side data collection The accuracy requirement of performance monitoring may not be as high as that of training data, but the latency and UL overhead are more important. Using the truncated/low-resolution version, UE does not need to implement two different quantization schemes, so the complexity is almost the same. |
| Qualcomm | For reporting content, since it is at early stage, let us say “at least paired… is reported”, to make it open for other possibilities. |
| Lenovo | It is currently unclear whether support is required for both L1 and higher-layer functions, and for monitoring purposes, L1 signalling appears to be more suitable. |
| MediaTek | Support L3 signaling for NW-side monitoring |
| CMCC | Similar as R19 AI based BM, L1 signaling is preferred for NW-side monitoring. |
| CATT | Not support. When the spec transparent L1 NW-side performance monitoring is already there, the motivation is not clear why we want to further introduce a L3 NW-side performance monitoring with spec impact. |
| Ericsson | Support |
| ZTE | Per our understanding, the UCI missing or omission happens infrequently. Given the CSI feedback is already available at the NW side by L1 reporting, we are not sure whether it’s still worthy to report it repeatedly by high layer signaling. The pairing can be easily done at the NW side, assuming some assistant information such as time stamp can be reported along with the target CSI. |
| Xiaomi | OK The FFS part seems not necessary as UCI omisson won’t happen in higher layer reporting. |
| Panasonic | Support |
| Fujitsu | Fine in general. |
| Sony | Support |
| HONOR | Support. Share same view as Huawei. For NW-side monitoring via L3 signaling, when the reported content includes paired Target CSI and CSI feedback, it is not unnecessary to consider UCI omission. |
| TCL | OK |
| Qualcomm | The not should be FFS, not sure UE is able to log the CSI feedback before omission. The simplest way it just log the entire UCI payloads. |
| Toyota | Support |

## Issue#3-3 UE side monitoring

Brief summary:

Supporters to UE side monitoring Case 2-1/2-2:

- Case 2-1 (based on UE side decoder): vivo, Samsung, Lenovo (UE trained decoder under Direction A, or specified decoder), OPPO, NEC, Tejas, TCL, Sharp

  - Spec impact:

    - vivo: Format (e.g., SGCS) of the metric.

      - Reuse the definition of SGCS for CSI prediction

      - How to generate the metric (e.g., SGCS) is purely UE implementation

    - Samsung: Specify the proxy decoder, or NW to share the proxy decoder

    - NEC: Format (SGCS) of the metric. Event triggered report.

- Case 2-2 (direct estimation of intermediate KPI): Ericsson, Samsung, OPPO, TCL, Sharp, ETRI

  - Spec impact:

    - Ericsson: Format (e.g., a SGCS range indicator) and reporting signaling (L1) of the metric.

    - Ericsson: RAN4 testing of the quality of UE reported monitoring metric

    - Ericsson: Mechanisms (e.g., RRC-message based methods) to support UE reporting target CSI, encoder output, together with the associated intermediate-KPI based performance metric to the NW

- Conditionally support: NTT DOCOMO (serves to trigger NW-side performance monitoring)

- Not support: Huawei, HiSilicon, CATT, CMCC

Content of the reporting for Case 2-1/2-2:

- SGCS: Ericsson (range indicator), vivo (between Target CSI and recovered Target CSI), Samsung

- SGCS1 and SGCS2: OPPO (SGCS1 between Target CSI and recovered Target CSI, or estimated SGCS;  SGCS2 for legacy CB)

Supporters to UE side monitoring based on precoded RS: CATT, Samsung, Apple

- Option 1: Reusing legacy eventual KPI reporting with precoded CSI-RS, e.g., CQI report subject to “cri-RI-CQI”, i.e., no additionaly spec impact. Huawei, HiSilicon

- Option 2: SGCS between vTarget(T0) and vPrecoded(T0) CATT

  - FL assessment: UE needs to perform channel measurement on another non-precoded CSI-RS to obtain HT1, and then extracts vPrecoded(T0)

- Option 3: Gap of precoding gain between HT0 vTarget(T0)2 and HT0 vPrecoded(T0)2, or HT0 vTarget(T0)2 and HT1 vPrecoded(T0)2. Samsung

  - FL assessment: Former solution: UE needs to perform channel measurement on another non-precoded CSI-RS to obtain HT1, and then extracts vPrecoded(T0), and mutiplies it with HT0. Latter solution: Seems easy, but there is gap between HT0 and HT1, which may cause inaccuracy.

- Option 4: Gap of precoding gain between HT1 vPrecoded(T0)2 and HT1 vPMI2, where a second precoded RS is transmitted at T1 precoded with non-AI PMI/legacy CB of vPMI. Apple, CATT

  - FL assessment: UE needs to feedback another non-AI based PMI/legacy CB, i.e.,  vPMI, to NW at T0, and perform channel measurement on another precoded CSI-RS precoded by vPMI.

- Note: T0 is the time for channel measurement for CSI feedback. T1 is the time when precoded RS is transmitted. HT0 is the measured channel at T0. HT1 is the measured channel at T1. vTarget(T0) is the precoder based on Target CSI at T0. vPrecoded(T0) is the precoder based on recovered Target CSI, and applied to the precoded RS.

- Not support: Ericsson, CMCC

  - Argument 1: UE-specific CSI-RS overhead scaled with UE number

  - Argument 2: Additional latency between CSI feedback and precoded CSI-RS

- Other views:

  - Apple: support RLF/BFD like mechanism, where UE transmit fallback request

Signaling and RS type for UE side monitoring

- L1 signaling: Ericsson, vivo, CATT, Lenovo, ETRI

- Support P/SP/AP CSI-RS resource types vivo

- Support SP/AP CSI reporting types vivo CATT, Lenovo

Linkage between monitoring report and inference CSI report Xiaomi, CATT

- Use pairing ID to indicate the linkage Xiaomi

- Inference CSI-reportConfigId is configured for monitoring, which is analogous to Rel-19 AI BM monitoring report and AI CSI prediction monitoring report CATT

Moderator note: No need to support multiple solutions for UE side monitoring – let’s see if we can down select to at most one option.

Option 1: Case 2-1/2-2. As majority companies’ view, there is no need to specify how UE generates the SGCS. It serves more like a ‘quality indicator’, which provides soft information on the generalization performance of the model.

Option 2: 4 candidate sub-options are provided as discussed in the last meeting. In this meeting, only a few companies elaborate the flavor in Tdoc, so companies can provide the views in FL summary. The intension is to select only one sub-option. The prices on latency/UE complexity for sub-option 2-2, 2-3, 2-4 are assessed by FL referring to the “Brief summary” part.

In addition, from a number of companies’ view, the signaling for reporting the metric can follow the R19 BM/CSI prediction, i.e., configure with a dedicated monitoring CSI report associated with an inference CSI report.

### [Rd1] Proposal 3.3:

For UE side monitoring, study the following two options, and perform down selection to at most one option:

- Option 1 (i.e., Case 2-1/2-2): UE reports estimated SGCS in monitoring CSI report via L1 signaling.

  - How to generate the estimated SGCS is UE implementation from RAN1 perspective.

  - FFS definition of estimated SGCS, e.g., between Target CSI and reconstructed Target CSI

  - FFS format of estimated SGCS, e.g., statistic value, mean value, or range.

  - FFS association of the monitoring CSI report with the inference CSI report

- Option 2 (i.e., monitoring based on precoded RS): UE reports a metric representing the accuracy or precoding gain of the recovery CSI, via L1 signaling. Select one sub-option in below:

  - Option 2-1: Reusing legacy eventual KPI reporting with precoded CSI-RS, e.g., CQI report subject to “cri-RI-CQI”, i.e., no additionaly spec impact.

  - Option 2-2: SGCS between vTarget(T0) and vPrecoded(T0)

  - Option 2-3: Gap of precoding gain between HT0 vTarget(T0)2 and HT0 vPrecoded(T0)2, or HT0 vTarget(T0)2 and HT1 vPrecoded(T0)2.

  - Option 2-4: Gap of precoding gain between HT1 vPrecoded(T0)2 and HT1 vPMI2, where a second precoded RS is transmitted at T1 precoded with non-AI PMI/legacy CB of vPMI.

  - Note: T0 is the time for channel measurement for inference CSI feedback. T1 is the time when precoded RS is transmitted. HT0 is the measured channel at T0. HT1 is the measured channel at T1. vTarget(T0) is the precoder based on Target CSI at T0. vPrecoded(T0) is the precoder based on recovered Target CSI, and applied to the precoded RS.

  - FFS for Option 2-2, 2-3, and 2-4, association of the monitoring CSI report with the inference CSI report.

| Company | View |
| --- | --- |
| Mod | Please provide your preference between Option 1 and Option 2, and the preferred sub-option under Option 2. |
| vivo | We prefer option 1 to report estimated SGCS. |
| Huawei, HiSilicon | We are not supportive of Option-1 which relies on a proxy decoder model at the UE side. In our view, the use of proxy model has two main challenges: - Generalization performance of the proxy model: Since the proxy CSI reconstruction model is different from the actual NW part CSI reconstruction model, e.g., with a smaller size/simpler architecture and weaker learning capability, the generalization performance of the proxy model will be typically worse than the actual NW part CSI reconstruction model. - Additional LCM of the UE side proxy model: Considering the proxy model is used to monitor the performance of actual model, then a subsequent issue is how to monitor and manage the proxy model. Since the proxy model is one-sided model at the UE side, one potential way is that it is up to UE implementation with level x collaboration and, therefore, its performance can hardly be monitored or trusted by the NW side. Option 2 relies on precoded CSI-RS. This option is overhead friendly for UL report, as it may avoid delivery of Target CSI from UE to NW. In addition, it is more reliable than proxy model-based monitoring in option 1. On the other hand, precoded RS based monitoring may reflect the eventual performance, but may not reflect the intermediate KPI (e.g., SGCS) of the model. However, the eventual performance may not only be impacted by the accuracy of the model but also other factors such as scheduling, MU pairing, channel status, etc. Moreover, this method is applicable only when the channel state does not change rapidly. Otherwise, considering there is a time gap between the time slot when the UE feeds back the Target CSI and the time slot when UE calculates the KPI, the accuracy of the calculated KPI may be degraded if the channel state changes significantly in between. We think, compared with proxy model-based monitoring, precoded RS based monitoring may be a more reliable method for UE side monitoring, but it cannot work in a standalone manner without NW side monitoring. Overall, we think RAN1 should first support NW side monitoring and, if necessary, a variation of Option2-based UE side monitoring can be further supported. |
| Apple | We are not supportive of option 1 for the same reasons mentioned by Huawei/HiSilicon. For option 2, instead of reporting a precoding gain, e.g., L1 signaling, UE can trigger an event for NW side performance monitoring. This makes UE side performance monitoring and NW side performance monitoring complementary to each other. |
| Qualcomm | In general, we don’t support UE side monitoring as NW side monitoring with target CSI reporting is needed anyway. Any other operation at UE side is additional complexity without justified benefit. Even if it is supported, we don’t support L1 reporting of the monitoring metric. Let us first make a decision between NW and UE side monitoring, we don’t need two solutions. If NW monitoring is prioritized based on majority, we don’t need to spend time on this topic. |
| Lenovo | We are supporting option-1 as - It is more accurate than NW-side monitoring - It has less overhead and less delay than NW-side monitoring. Regarding the accuracy of the proxy model: Considering Option 4-1 and Option 3a-1 with target CSI, one common method for developing the encoder is to first train the “nominal decoder” and then use this decoder for training the encoder. Assuming correct training, the UE-side (UE) has access to a accurate Encoder and “nominal decoder” after the training phase. Now for monitoring, we can use the developed “nominal decoder” (which we are sure that it is a good model) as the “proxy model. No need for development of other proxy model and, therefore, the issues regarding the accuracy of the proxy model is not valid (we know that the nominal decoder model is a good match with the NW-side decodor model). The rationale for employing a “nominal decoder” as the proxy model addresses concerns about generalization capability and mitigates the need for additional LCM requirements. In summary, if a "nominal decoder" is present on the UE side, it addresses concerns related to accuracy, generalization, and the LCM requirements for UE-side monitoring. |
| OPPO | We share the same view to Lenovo, and support option 1. UE-side monitoring using proxy decoder (Case 2-1) can provide adequate monitoring accuracy and address the concerns on overhead in NW-side monitoring. We can reuse the similar framework to UE-side assisted monitoring in CSI prediction by reporting SGCS. Therefore, the UE-side monitoring Case 2-1/Case 2-2 should be studied and specified |
| MediaTek | Share same view as QC. If the UE-side monitoring should be complemented by NW-side monitoring, the question will be why UE-side monitoring is needed at the first place. |
| CMCC | We don’t support UE side monitoring since the monitoring accuracy of UE sided monitoring based on proxy model at UE side is lower than NW sided monitoring. Besides, if there is an additional proxy model for monitoring specific on top to CSI generation part for inference at UE side, NW/UE may suffer from more burdens on model storage, complexity, power consumption, and management. |
| CATT | We are not supportive for Opt 1 for the same reasons mentioned by HW. We support Opt 2. |
| Ericsson | Ok to further study and down select between option 1 and option 2. We are not supportive of option 2. For Option 2, the NW needs to first use its decoder to generate the reconstructed CSI and then derive a precoding matrix to precode a RS, and then, transmit the RS to the UE. By measuring on the precoded RS, the UE may derive some link-level KPIs like CQI, BLER, etc., but the UE cannot derive intermediate KPI. This method cannot support a UE to obtain accurate information of the model performance, since only the effective channel and not the precoder is visible to the UE, yet it introduces additional latency, RS signalling overhead, and processing complexity at both UE and NW side. In addition, this method does not scale if considering multiple UEs in the network and many monitoring samples are required per UE to derive a monitoring result, since each UE required dedicated precoded CSI-RS which can result in unacceptable CSI-RS overhead. Moreover, the effect of channel aging can impact the accuracy and reliability of the monitoring result. |
| ZTE | We don’t think option 1 can work. The proxy model shows significant generalization performance loss in terms of monitoring accuracy according to TR 38.843, and it may impose huge burden on the network side to additionally identify/monitor/manage the UE-side proxy model. Additionally, if the estimated SGCS is purely UE implementation, the SGCS accuracy across different UEs varies. It cause confusing for the gNB for the interpretation of the reported metric, and false alarming or miss detection may happen. |
| Xiaomi | We share similar view with HW and apple. Considering the issues on accuracy, generation and LCM requirement for proxy model, we are not supportive of Option-1. If the proxy model is trained as normal decoder to mimic the decoder at NW side, it means that the normal decoder must be trained at UE side for inter-vendor collaboration training Option4-1/3a-1 or Option1. However, based on discussion in AI 10.1.2, UE side does not have to train normal decoder. Thus, Option 1 reqires the restriction on inter-vendor collaboration. For Option 2, we are fine to down selection one option. Considering the monitoring performance and calculation complexity, Opiton2-3 or Opiton 2-4 is preferred. |
| InterDigital | Support |
| Panasonic | Our preference is Option 1 and Option 2-1. Monitoring based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW can be used for calculating eventual KPI (L1-RSRP, hypothetical BLER, etc.). Intermediate KPI-based monitoring based on the output of the CSI reconstruction model at the UE (Case 2-1) can be used periodically during normal operation for each check of AI/ML model. |
| LGE | We have similar view with QC that first make a decision between NW and UE-side monitoring. In our view, UE-side monitoring is not needed. Since UE-side monitoring options have many limitations, such as montitoring accuracy, computional complexity, latency. If UE-side monitoring is supported, since precoded RS based monitoring have channel aging and latency issue, we prefer option1. |
| Fujitsu | For Option 1, how to monitor the performance of proxy models should be clarified first. For Option 2, CSI aging is a big challenge for its reliability. Thus, NW side monitoring should be considered as high priority. |
| ETRI | We shares similar view with Lenovo. For intervendor training option 4-1, the nominal decoder could be developed and served as the proxy model for deriving SGCS for UE-side performance moniroting. In addition, for intervendor trianing direction C, UE-side can acquire the actual standardized decoder. |
| HONOR | Since there is no additionaly spec impact, Option 2-1 is preferred. |
| TCL | support the proposal and prefer Option 1. |
| Toyota | No support for UE side monitoring, due to the latency and higher signaling overhead it generates. |
| Southeast University | We prefer option 1. |

## Issue#3-4 Other solutions or issues

### [Rd1] Question 3.4:

For NW/UE side monitoring, any other solutions or issues are missed?

| Company | View |
| --- | --- |

# Model pairing

## Summary of company proposals

| Company | View |
| --- | --- |
| Apple | Proposal 6: The paring ID is used for NW side data collection for performance monitoring configuration and report. Proposal 7: When the paring ID is used for UE side data collection for training, the pairing ID is configured together with resourcesForChannelMeasurement (e.g., part of CSI-ResourceConfig). Proposal 8: For model pairing procedure for inference, if different paring ID is used for different dataset and NW uses a mixing of datasets for NW side model training with continuous learning: - A pairing ID list is used in inference configuration as part of the applicability reporting procedure; and - UE will report applicable if one of its pairing IDs matches within the pairing ID list. |
| ASUSTeK | Proposal 1: Support both Option A and Option B of Rel-19 applicability report procedure. Proposal 2: If Pairing ID(s) is included in step-3, no need to report pairing ID in step-4. |
| CATT | Proposal 4: For AI/ML based CSI compression Case 0, for model pairing procedure for inference, pairing ID(s) indicated in Step 3 of applicability report is(are) the same as pairing ID(s) in inter-vendor training collaboration. - For Direction A Opt 3a-1, the pairing IDs are received at UE side during model parameter exchange - For Direction A Opt 4-1, the pairing IDs are received at UE side during dataset exchange - For Direction C, the pairing IDs can be generated at the same time when a pair of encoder and decoder reference models is specified Proposal 5: For AI/ML based CSI compression Case 0, inference model pairing in Step 4 of applicability report can be achieved via UE reporting of configuration ID(s) in which the supported pairing ID(s) and inference related parameters/configuration are configured. Proposal 6: For AI/ML based CSI compression Case 0, for model pairing procedure for inference, support both Option A and Option B for applicability report. - For Option A, entire CSI-ReportConfig for inference configuration is configured in Step 3 - For Option B, the following sets of inference related parameters for applicability check only are configured in Step 3: - pairing ID(s) - subband configuration - Tx port configuration - Payload size configuration Proposal 7: For AI/ML based CSI compression Case 0, regarding the applicability report, a pairing ID can be associated with multiple configurations of subband, Tx port and payload size. Proposal 8: For AI/ML based CSI compression Case 0, do not support configuration of pairing ID during either NW-side data collection or UE-side data collection. Proposal 9: For AI/ML based CSI compression Case 0, regarding the pairing procedure, do not support additional configuration of associated ID. Proposal 10: For AI/ML based CSI compression Case 0, it is unnecessary to configure the pairing ID in performance monitoring report. |
| CMCC | Proposal 5: With inter-vendor collaboration Option 4-1 and 3a-1, the exchanged dataset or model parameters may be assigned with Pairing ID(s) during the dataset/model parameters exchange. Proposal 6: In CSI compression using two-sided model use case, the Pairing ID can be optionally indicated in CSI-ReportConfig for inference configuration. Proposal 7: In Step 4 of applicability reporting procedure, UE reports applicability for CSI-ReportConfig and/or set(s) of inference related parameters, and Pairing ID(s) is not needed to report. Proposal 8: For applicability reporting procedure, both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only) can be supported. Proposal 9: Inference related parameters for Option B at least include the following parameters: - Pairing ID - this doesn’t imply the Pairing ID is always present - Configuration of subband - Configuration of Tx port - Configuration of payload size Proposal 10: In CSI compression using two-sided model use case, the Pairing ID is not used for monitoring configuration. |
| Ericsson | Proposal 1: The ID assignment for Direction A, option 3a-1 and 4-1 shall be studied in RAN2 to ensure the uniqueness of the ID across different NW vendors. Proposal 2: In Direction C, a fully standardized reference model is associated with an ID for pairing related discussion, and the pairing ID is specified together with the reference model. Proposal 3: For CSI compression use case, pairing ID (i.e., the ID associated with an exchanged dataset/model-parameters or a fully standardized reference model) is sufficient for ensuring consistency between model training and inference. Do not support associated ID for this use case. Proposal 4: For model pairing procedure for inference, support only Option B (sets of inference related parameters for applicability report only). Proposal 5: For model pairing procedure for inference, only Pairing ID(s) is/are indicated in Step 3. - A Pairing ID is associated with an exchanged dataset of Option 4-1, a set of exchanged model parameters of Option 3a-1, or a fully standardized reference model of Direction C. - A Pairing ID indicated in Step 3 implicitly indicates other inference related configuration parameters that were sent in the dataset exchange for the same pairing ID (e.g., number of Tx ports, number of subbands, and CSI payload size configurations) Proposal 6: For model pairing procedure for inference, for the content of UE reporting in Step 4, support one from the following two options: - Option 1: applicable pairing ID(s) only. - Option 2: applicable pairing ID(s) and applicable (sub)set of other inference related configurations (e.g., number of Tx ports, number of subbands, and CSI payload size configurations) per pairing ID. |
| ETRI | Proposal 7: For AI/ML-based CSI compression, consider both Option A and Option B in the model pairing procedure for AI/ML-based CSI compression. Proposal 8: For AI/ML-based CSI compression, support multiple configurations and payload lengths under a single pairing ID within the model pairing procedure for inference. |
| Fujitsu | Proposal 5: Pairing information exchange between NW and UE is essential for two-sided models, which is indicated by an ID obtained from inter-vendor training collaboration. Proposal 6: The ID is shared from NW to UE along with the exchanged dataset/model parameters, which is an indicator of the NW-side reconstruction model used to generate the exchanged dataset. Proposal 7: Support pairing-ID-based UE applicability checking and reporting for CSI compression, including: - NW provides paired ID(s) in Step 3 with Option A. - UE reports its applicable paired ID in Step 4. Proposal 8: In UE’s applicability report of Step 4, - UE can report a paired ID to indicate its preferred model for inference among the multiple paired IDs obtained from NW in Step 3. - UE can report inapplicability as the response of indicated paired ID(s) in Step 3, if UE judges its AI-based CSI compression function is not applicable. - UE can confirm applicability as the response of indicated paired ID(s) in Step 3, if UE judges its AI-based CSI compression function is applicable. Proposal 9: Pairing ID is separately provided from the configurations for TX ports, sub-bands, and payload size. Proposal 10: There is no need to introduce associated ID for CSI compression. |
| FUTUREWEI | Proposal 4: In AI/ML-based CSI compression using two-sided model Case 0, consider reusing procedures agreed in Rel-19 Beam Management use case as much as possible and additionally consider the following: - Indicate Pairing ID in the procedure (applicable for both Option A and Option B in Step 3) - Adopt at least Option A in Step 3 of the procedure |
| Google | Proposal 4: Support the network to configure a list of model IDs in a CSI report configuration - Different models can be associated with different payload size - UE reports CSI corresponding to a model ID that fits for the requirement of the maximum payload size to avoid CSI omission |
| Huawei, HiSilicon | Proposal 5: Support indicating the Pairing ID in Step 3 of applicability report procedure to enquire if the UE part model that is associated with the Pairing ID is applicable with the provided inference configuration (Option A) or set of inference related parameters (Option B). - The Pairing ID refers to the exchanged dataset/exchanged model parameters/standardized reference model in the inter-vendor training collaboration. - For Direction C, it can be specified with a fixed Pairing ID. Proposal 6: Regarding the model pairing procedure for inference, support both Option A and Option B. Proposal 7: Regarding the model pairing procedure for inference, one Pairing ID can be indicated per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B. - Reuse Rel-19 rule where the applicability state reported in Step 4 is per CSI-ReportConfigId for Option A and per ApplicabilitySetConfigId for Option B. Proposal 8: Regarding the model pairing procedure for inference/monitoring: - No need to additionally indicate associated ID to UE in Step 3. - No need to additionally indicate Pairing ID for monitoring. |
| InterDigital | Observation 3: The UE-side model management complexity may be high in case of purely numeric IDs that carry no implicit information. Proposal 3: Study mechanism to manage ID for model pairing with reduced complexity. Observation 4: ID structures that convey (implicit) dataset information may help reduce the UE-side model management complexity. Proposal 4: Support ID structures that convey implicit information to enable reducing the UE-side model management complexity. Observation 5: When the ID associated with the exchanged dataset or the model parameters is also used for the performance target in Options 3a-1 and 4-1, the same ID can be used for monitoring configuration. Proposal 5: Support using the Pairing ID for monitoring configuration. |
| Lenovo | Observation 1: For Options 3a-1 and 4-1 of Direction A, associating an ID for pairing information during the exchange of dataset/model parameters between the UE side and NW side may disclose the information about the identity of the UE-vendor and/or NW-vendor during the inference phase. Proposal 1: Consider specification of procedure/signaling enabling the UE/gNB to identify/select a correct paired encoder/decoder without disclosing proprietary information of the other side, i.e., gNB/UE. Proposal 2: Consider that the Pairing ID associated with dataset/model parameters is unique within the coverage area of the node assigning the pairing ID (e.g. an OAM node/function assigning the pairing ID). Proposal 3: Specify required procedures/signalling enabling the node/function in the network responsible for dataset/model parameters exchange to associate a Pairing ID with datasets/model parameters that need to be exchanged between the NW-side and the UE-side. Proposal 4: Further clarify that the associated ID for the exchanged dataset/model parameters can be used for further NW-side data collection (if needed) but is not needed for the initial data collection for initial construction of the decoder model (and the corresponding dataset/model parameters). Proposal 5: Further study the procedure to initiate development of an encoder model for a particular ID (i.e., dataset/model parameters) that the UE/UE-side has not yet developed the corresponding encoder model. Proposal 6: Pairing ID can be used in Step 3 to indicate the exchanged dataset/model parameters/standardized reference model in the inter-vendor collaboration. |
| LG Electronics | Proposal 1: For applicability report of CSI compression, consider both option A and option B. Proposal 2: Information on the supported combinations of Tx ports, sub-bands and payload sizes can be reported in addition to pairing ID during applicability report. FFS on which step includes that information. |
| NEC | Observation 1: Model pairing procedure can be different based on the methods used for inter-vendor collaboration. But the ID used for model paring can be the same for different procedures. Proposal 2: Support applicability/inapplicability reporting of inference configuration with the pairing ID and support the inapplicability report to trigger parameter/dataset transfer for model pairing if needed. |
| Nokia | Proposal 1: For Direction C, Pairing ID shall be uniquely determined based on the reference models defined in the spec. Proposal 2: In the applicability reporting procedure, Paring ID(s) are not reported in Step 4. Observation 1: NW-shared inference parameters and pairing ID guide UE model selection, including applicability reporting. Proposal 3: In Step 3 of the applicability procedure, support only Option A. |
| OPPO | Proposal 12: Support Option B only for applicability report for AI/ML CSI compression: - Pairing ID is indicated in the sets of inference related parameters in Step 3 - UE to check whether the pairing ID is pre-exchanged during inter-vendor collaboration and has been associated with an encoder. The applicability of pairing ID is explicitly reported in Step 4 |
| Panasonic | Proposal 5: Pairing ID should be allocated by MNO. Proposal 6: The same ID as pairing ID can be used for UE to collect UE-side target CSI for UE-side training at least for OTA approach. - For the case that UE-side data collection is non-OTA approach, it would not be necessarily required to use the same ID as paring ID. Proposal 7: The same ID as pairing ID, i.e., ID associated with exchanged dataset or the model parameters, can be used for applicability inquiry and reporting. Proposal 8: For inference configuration, the same ID as pairing ID, i.e., ID associated with exchanged dataset or the model parameters, can be used. A paring ID can be associated with multiple inference configurations within the range where scalability is ensured. |
| Samsung | Observation 6: For Direction C, fully specified reference model-based approach, there are four different deployment cases - Case 0: Both UE and NW deploy the specified reference encoder and decoder, respectively - Case 1: NW deploys finetuned decoder, UE implements the specified reference encoder. - Case 2, UE deploys finetuned encoder, NW deploys the reference decoder. - Case 3: Both UE and NW deploy finetuned encoder and decoder, respectively Proposal 6: Consider a unified pairing-related ID for Direction A and Direction C with the pairing-related ID consisting of two parts. - Part 1 indicates a specified ID, e.g., specified reference model. - Part 2 indicates network-assigned ID to ensure consistency between training and inference. Both Part 1 and Part 2 or just Part 2 may be associated with exchanged dataset/model parameters in Direction A, or data collection-related configurations in Direction A and Direction C. Observation 7: The implementation dependency between UE-side beam prediction model and NW-proprietary (undisclosed) beam information is analogous to the dependency of UE’s CSI encoder and the NW-proprietary (undisclosed) actual decoder. Thus, the Associated ID framework adopted for UE-side model-based beam prediction can be reused. Proposal 7: For pairing and applicability reporting, reuse the Associated ID framework adopted in UE-side model-based beam prediction using UE-side model - With the same pairing-related ID, e.g., Associated ID, the UE assumes the same decoder. - FFS: additional information, if necessary, for UE-side to train common encoder across cells. - FFS: additional information subject to inter-vendor collaboration options. |
| Spreadtrum, UNISOC | Proposal 7：In step 4,the UE should report the pairing ID received from gNB, and optionally included supported inference configurations when multiple configurations share the same pairing ID. Proposal 8: Support both option A and option B to configure the pairing ID for applicable functionality reporting. Proposal 9：It is necessary to ensure the uniqueness of the pairing ID across different NW vendors. Details is up to RAN2 discussion. Proposal 10：For CSI compression, there is no need to indicate the pairing ID for monitoring procedure. |
| TCL | Observation 3: The paring ID can be exchanged along with the model parameters (Option 3a-1) or dataset (Option 4-1) for model identification. Proposal 5: The paring ID can be used for model paring procedure for inference, UE-side data collection and NW-side data collection. Proposal 6: It is recommended to introduce a unified ID to streamline the ID-related configuration. |
| vivo | Proposal 1: Study whether a dataset (direction 4-1), model parameters (direction 3a-1) and direction C can be associated with same pairing ID - If yes, the additional ID also needs to be indicated in inference configuration to indicate which method or which reference model is used Proposal 3: For applicability check, option B should be supported for checking supported pairing ID, and the following inference related parameters are provided along with pairing ID. - Payload size - Subband number and/or port number |
| Xiaomi | Proposal 1: CSI-ReportConfig for inference configuration is adopted for the step 3 of applicability report. Proposal 2: For inter-vendor collaboration direction A, the paring ID is exchanged together with the dataset and/or model parameters, the paring ID is linked with the model generated by the exchanged dataset and/or model parameters. Proposal 3: For inter-vendor collaboration direction C, the model ID is linked with the fully standardized reference model, the paring ID is additionally indicated to align the dataset feature in UE and NW side. Proposal 4: The parings ID(s) included in the applicable CSI-reportConfig ID(s) is reported in Step4. |
| ZTE Corporation, Sanechips | Proposal 3: To enable the UE to select an encoder compatible with the decoder used by the gNB, pairing ID shall be defined which may be in the form of specified ID, dataset ID, or model ID, depending on the inter-vendor training collaboration options. Proposal 4: For the applicability report for AI/ML CSI feedback use case, reuse the applicability reporting procedures defined for AI/ML beam management as a starting point. Proposal 5: For the applicability report for AI/ML CSI feedback, support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only). Proposal 6: The CSI feedback specific inference configuration or inference related parameters provided to the UE shall at least include configurations of subband, Tx port and payload size. |

## Discussions

## Issue#4-1 Model pairing procedure

Brief summary:

Support of Option A and Option B

- Support both: Huawei, HiSilicon, ZTE, CATT, CMCC, LGE, ETRI, Spreadtrum

- Option B only: Ericsson, vivo, OPPO

- Option A only: Nokia, Xiaomi, Futurewei, Fujitsu, Futurewei

  - Nokia: applicability reporting can already be performed based on the full inference configuration, and following the path of Option B seems unnecessary for CSI compression

  - Xiaomi: option B need to define additional IEs which would require more RAN1 and RAN2 work

  - Concern on Option A only: vivo, Huawei, HiSilicon, ZTE

    - vivo: 1) it is challenge to indicate the pairing ID directly in CSI-ReportConfig if NW side has multiple models and does not know which one the UE supports 2) Subset of preferred configuraitons can be replied to NW for checking, rather than configure multiple CSI-ReportConfig for applicability checking

    - Huawei, HiSilicon: For Option A, 1) if gNB cannot confirm whether the UE can support a CSI-reportConfig in the first place but blindly configure with the CSI-reportConfig, it may suffer the consequence of UE reporting ‘inapplicable’. 2) If the UE replies with ‘inapplicable’, then it means the gNB cannot immediately receive the CSI report, and it needs to blindly try another CSI-reportConfig, causing longer CSI report latency.

    - ZTE: since there is no priori information on UE-side applicability status, many of the inference configurations provided in Step 3 may become useless and result in a waste of air interface resources.

How to indicate Paring ID(s) in Step 3

- For Option A:

  - one Pairing ID can be indicated per CSI-ReportConfigId: Huawei, HiSilicon, OPPO, Futurewei

- For Option B:

  - Alt.1: One Pairing ID can be indicated per ApplicabilitySetConfigId: Huawei, HiSilicon,

  - Alt.1’: One Pairing ID can be indicated within the set of inference related parameters: OPPO

  - Alt.2: NW directly indicates one or multiple pairing ID(s) Ericsson

    - Ericsson: each pairing ID already implicitly indicates the sets of other inference related parameters

Whether/how Paring ID(s) are reported in Step 4

- For Option A:

  - No need for explicit report, since applicability state reported in Step 4 is per CSI-ReportConfigId which includes all inference configurations: Huawei, HiSilicon, Nokia, ZTE, Xiaomi, CATT, CMCC

  - UE reports applicable Pairing ID Fujitsu

- For Option B:

  - Alt.1: No need for explicit report, since applicability state reported in Step 4 is per ApplicabilitySetConfigId: Huawei, HiSilicon, ZTE, CATT, CMCC

  - Alt.2: report applicable Pairing ID(s) in Step 4: Ericsson, OPPO

  - Alt.3: Reports applicable Pairing ID(s) as well as (sub)set of other inference related configurations, if more than one set of configurations is represented by a Pairing ID  Ericsson

CSI compression specific inference configuration for Option A:

- Antenna port geometry: N1 (number of array rows), N2 (number of array columns) Nokia, Xiaomi

- Bandwidth configuration: number of subbands and location of subbands (i.e., contiguous or some other pattern) Nokia, Xiaomi

- Latent vector dimension Nokia

- Quantization scheme Nokia

- Carrier frequency Nokia

- CSI payload size Xiaomi

CSI compression specific inference related parameters for Option B:

- Payload size: vivo, ZTE, CATT (d, Q, L), CMCC, LGE (d, Q, L)

- Quantization information: LGE (VQ/SQ, layer/rank specific/common)

- Subband number: vivo, ZTE, CATT, CMCC, LGE

- Tx port number: vivo, ZTE, CATT, CMCC, LGE

Moderator note:  Discuss the FFS on whether/how Paring ID(s) are reported in Step 4, whether to support both Option A and Option B, and CSI compression specific inference configuration or inference related parameters.

Applicability report procedure for R19 AI BM in latest 38.331 (R2-2506530)

Step 3-Option A: CSI-reportConfig => CSI-ReportConfigId

…

Step 3-Option B: OtherConfig-v19xy => ApplicabilityReportConfig-r19 => applicabilitySetConfigId-r19

…

Step 4: Option A and Option B: RRCReconfigurationComplete or UAI => ApplicabilityReportList. In ApplicabilityReportList => ApplicabilityInfoReport, UE reports the applicability status per CSI-ReportConfigId (for Option A), and per ApplicabilitySetConfigId (for Option B).

Regarding whether to support Option A and B, to Moderator’s understanding, Option A and Option B may be applicable to different scenarios: for initial applicability check (e.g., a UE newly camps on the cell), Option B is better as it can avoid gNB blindly configuring CSI-reportConfig. After UE feedback applicability via UAI or RRCReconfigurationComplete and gNB is somehow aware of the applicability at UE, Option A may be applicable.

Regarding whether to report Pairing ID in Step 4, it is Moderator’s understanding that for Option A and Option B, UE will feedback applicability status per CSI-reportConfigId and ApplicabilitySetConfigId which already refer to Pairing ID configured in Step 3, so there is no need to explicitly report Pairing ID.

Regarding inference configuration for Option A, it seems straightforward that all the sub-IEs under CSI-reportConfig are relevant, while for Option B, some IEs are proposed based on companies’ inputs.

### [Rd1] Proposal 4.1 (Support of Option A/Option B):

For model pairing procedure for inference, support both Option A and Option B.

- For Option A: In Step 3, Pairing ID(s) is indicated in CSI-reportConfig. In Step 4, UE reports the the applicability status per CSI-reportConfigId (which refers to the information of both Pairing ID and other inference configurations).

  - Inference configuration includes the sub-IEs under CSI-reportConfig.

- For Option B: In Step 3, Pairing ID(s) is indicated in CSI-reportConfig. In Step 4, UE reports the the applicability status per ApplicabilitySetConfigId (which refers to the information of both Pairing ID and other inference related parameters).

  - Inference related parameters for Option B may include at least: payload size related information (e.g., d, Q, L), subband configuration, and Tx port configuration. FFS quantization codebook.

Moderator note:  It seems some companies are not interested in the signaling details. Then, at least we can provide the inference related parameter list for Option B. Otherwise, the same story of CSI prediction may happen on CSI compression, that RAN2 may draw conclusion to preclude Option B just attributing RAN1 did not provide parameters.

### [Rd2] Proposal 4.1a (Support of Option A/Option B):

For model pairing procedure for inference, support both Option A and Option B.

- For Option A: In Step 3, Pairing ID(s) is indicated in CSI-reportConfig. In Step 4, UE reports the the applicability status per CSI-reportConfigId (which refers to the information of both Pairing ID and other inference configurations).

  - Inference configuration includes the sub-IEs under CSI-reportConfig.

- For Option B: In Step 3, Pairing ID(s) is indicated in CSI-reportConfig. In Step 4, UE reports the the applicability status per ApplicabilitySetConfigId (which refers to the information of both Pairing ID and other inference related parameters). Inference related parameters for Option B may include at least: payload size related information (e.g., d, Q, L), subband configuration, and Tx port configuration. FFS quantization codebook.

| Company | View |
| --- | --- |
| vivo | Support both Option A and Option B |
| Huawei, HiSilicon | Support. We think both Option A and Option B have their own advantages and, similar to Rel-19, RAN1 should support both options. For Option A, the full inference configurations are all provided within CSI-reportConfig in Step 3, and as long as UE reports ‘applicable’ state in RRCReconfigurationComplete message at Step 4 (for each CSI-reportConfigId), gNB can continue to further activate/schedule the corresponding CSI report with MAC CE/DCI. Therefore, there is no need for UE to wait for another RRCReconfiguration for CSI reporting configuration in Step 5. However, for Option A, if gNB cannot confirm whether the UE can support a CSI-reportConfig in the first place but blindly configure with the CSI-reportConfig, it may suffer the consequence of UE reporting ‘inapplicable’ with RRCReconfigurationComplete, and thus the CSI-reportConfig is wasted. Furthermore, if gNB wishes to reconfigure AI/ML based CSI reporting, it needs to always configure a new CSI-reportConfig without knowing whether the UE will support it or not in advance. If the UE replies with ‘inapplicable’, then it means the gNB cannot immediately receive the CSI report, and it needs to blindly try another CSI-reportConfig, causing longer CSI report latency. In contrast, for Option B, gNB may configure all candidate combinations of preferred parameters and receive their applicability status in one hand-shake. |
| Qualcomm | Pairing ID discussion in RAN1 is finished, this proposal should be discussed in RAN2. |
| Lenovo | Both option A and option B can be maintained; however, according to the definition of option B in BM, it involves sending a list of for example Pairing-IDs whiting a configuration message that may not be complete configuration message. These messages are then followed by a complete configuration message for the selected model. This distinction between Option A and Option B is not reflected in the current description of the proposal. So some adjusmtne is needed. |
| OPPO | Support Option B only. It would be better to discuss this in RAN2. |
| Futurewei | We are ok with the proposal in general. However, the actual spec work is in RAN2 |
| MediaTek | The proposal should be handled by RAN2 |
| CMCC | We support both Option A and Option B, the signaling design can be up to RAN2. |
| CATT | support |
| NEC | OK |
| Ericsson | Agree with QC, Oppo and Futurewei that it is better to discuss this proposal in RAN2. |
| ZTE | Support both Option A and Option B, considering the different applicable scenarios of Option A and Option B. Note that both Option A and Option B are proposed for the applicability reporting of AI/ML beam management and got supported finally. |
| Xiaomi | RAN1 still needs to discuss the list of parameters for Option B, although the final spec work is under RAN2 discssion. |
| Spreadtrum | Generally fine with the proposal，details is up to RAN2 discussion. |
| Panasonic | Support both Option A and Option B. |
| LGE | Support |
| Fujitsu | Generally fine with the proposal. |
| TCL | support |

Brief summary:

Whether/how to link the Pairing ID with the inter-vendor collaboration

- Pairing ID refers to the exchanged dataset/exchanged model parameters/standardized reference model in the inter-vendor training collaboration Huawei, HiSilicon, Ericsson, Samsung, ZTE, Xiaomi, CATT, CMCC, OPPO, Lenovo, Fujitsu, Futurewei

- Definition/UE assumption of Pairing ID:

  - with the same Associated ID the UE assumes the same data distribution with respect to the NW-side additional conditions, e.g., deployment scenario, antenna layout assumed for corresponding decoder training Samsung

  - With the same pairing-related ID, e.g., Associated ID, the UE assumes the same decoder Samsung

- Study multiple-to-one mapping between pairing ID and reference model/delivered parameters/delivered dataset: vivo

- Linkage to inter-vendor collaboration Direction C:

  - With a specified fixed ID: Huawei, HiSilicon, Ericsson, Nokia, ZTE

Moderator note:  Discuss the FFS on whether/how to link the Pairing ID with the inter-vendor collaboration.

### [Rd1] Proposal 4.2 (Link to inter-vendor collaboration):

For model pairing procedure for inference, Pairing ID refers to the exchanged dataset (for Option 4-1 if applicable)/exchanged model parameters (for Option 3a-1 if applicable)/standardized reference model (for Direction C if applicable) in the inter-vendor training collaboration.

- For Direction C if applicable, the corresponding Pairing ID is specified as a fixed value.

| Company | View |
| --- | --- |
| Huawei, HiSilicon | Support |
| NTT DOCOMO | Fine with a clarification on the bullet - For Direction C if applicable, the corresponding Pairing ID is specified as a fixed value per reference model. In case RAN4 defines multiple reference models. |
| Qualcomm | We don’t see a need of this proposal. What is inter-vendor training collaboration? What is missing if we don’t have this proposal? Pairing ID is assigned and exchanged during training collaboration, but it is a separate procedure than applicability enquiry process, we don’t have to link them. Moreover, this collaboration can be via standardized procedure or proprietary method, thus it’s hard to describe where this pairing ID comes from. |
| Lenovo | Support |
| Samsung | In our contribution, we elaborated the pairing related ID for Direction C is associated with both a specified reference decoder as well as network-assigned ID for data collection - For Direction C if applicable, the corresponding Pairing-related ID may consist of a specified as a fixed value and network assigned ID for data collection. |
| OPPO | Support. |
| Futurewei | We are ok with the proposal. |
| MediaTek | Support |
| CMCC | Support |
| CATT | support |
| Ericsson | Support this proposal with Docomo’s modification. |
| ZTE | ok |
| Xiaomi | Support |
| Spreadtrum | Support |
| InterDigital | ok |
| Panasonic | Support |
| LGE | Support |
| HONOR | Support. |
| TCL | support |
| Toyota | Support this proposal with DoCoMo’s modification. |

## Issue#4-2 Pairing ID for LCM procedures

Brief summary:

Inclusion of Pairing ID in LCM procedures:

- NW side data collection:

  - Support: Huawei, HiSilicon (optional), vivo, ZTE, Apple, TCL, ETRI

    - Argument: Pairing ID is anyway indicated to UE for model pairing purpose, it would not cause any harm if it is additionally indicated for NW side data collection purpose Huawei, HiSilicon

  - Not support: Ericsson, CATT, CMCC, OPPO, Lenovo (not needed for initial data collection)

    - Argument 1: UE logs and reports the target CSI samples together with the associated measurement configuration ID and the cell ID, which is sufficient for NW to categorize data by implementation Ericsson

    - Argument 2: NW side can manage the dataset categorization and aggregation in an implementation specific manner CATT

- UE side data collection:

  - Support: Huawei, HiSilicon (optional), Qualcomm, Ericsson (optional), vivo, Samsung, ZTE, Apple, Xiaomi, CMCC, OPPO, Tejas, TCL, Panasonic, ETRI, Spreadtrum

    - Argument 1: If the same Pairing ID is associated with an exchanged model#A/dataset#A (with certain NW side additional condition) during inter-vendor training collaboration, UE side may categorize the newly collected data into the dataset for model#A/dataset#A Huawei, HiSilicon, Qualcomm, Samsung

    - Argument 2: as Pairing ID is anyway indicated to UE for model pairing purpose, it would not cause any harm if it is additionally indicated for UE side data collection purpose Huawei, HiSilicon

    - Argument 3: For direction 3A-1 and direction C, it is possible that no Target CSI is shared, then UE side must collect data by itself vivo

  - Not support: CATT

    - Argument: UE fine-tuning happens when UE has identified that there is potential performance degradation for a certain model

  - vivo: how to configure the pairing ID, especially when two models exist under the same scenarios or deployments

- Monitoring:

  - Support: Tejas, InterDigital

  - Not support: Huawei, HiSilicon, CMCC, Spreadtrum

Need of additional associated ID in LCM procedures:

- Needed: Xiaomi

- Not needed: Huawei, HiSilicon, Ericsson, CATT, Fujitsu

Assignment of Pairing ID

- ID assignment shall be studied by RAN2: Ericsson,

- A unified structure for Direction A and Direction C – part 1 is specified ID referring to a standardized reference model, part 2 indicates NW assigned ID associated with exchanged dataset/parameters: Samsung,

Moderator note:  For indicating Pairing ID(s) for NW side data collection, proponents/opponents are half-half. For indicating Pairing ID(s) for UE side data collection, majority are fine to have it. For indicating Pairing ID(s) for monitoring, more companies have concern.

### [Rd1] Proposal 4.3 (Inclusion of Pairing ID in LCM procedures):

Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS whether to configure Pairing ID for NW side data collection

- No need to configure Pairing ID(s) for monitoring

| Company | View |
| --- | --- |
| vivo | We support indicating pairing ID for UE side data collection, and prefer to remove ‘optionally’ since it is already mandatory for inference or applicability checking. |
| Huawei, HiSilicon | Support. Regarding the FFS, we think indicating Pairing ID for NW side data collection can simplify NW side data management. For example, assuming UE logs the data for a long time before it reports, after NW side receives a batch of reported data, for any specific data, it does not know when it is generated, and what is the NW side additional condition at that historic time, if there is no tag to describe the data. |
| Apple | PairingID should be mandatory for UE side data collection based on NW first training. Otherwise, NW should train a NW side decoder first. |
| NTT DOCOMO | Support. |
| Qualcomm | We think it is mandatory, not optional. Otherwise, the UE has no idea how to use the collected data for model training. It is worth noting that the dataset exchanged in training collaboration is used to establish a private decoder which mimics the actual decoder employed by the NW, then the data collected by the UE data collection is used to train the actual encoder against the established private decoder. In this procedure, UE needs to be aware of which exchanged dataset the collected UE side data is associated with. The above training procedure is more meaningful for new UE devices/vendors where their data has not yet collected by the NW. For these new UEs, if pairing ID assigned to UE data collection, they would be able to train their model against an existing pairing ID, without necessarily going through a new round of NW side data collection and dataset exchanged with a new pairing ID. |
| Lenovo | We agree with the proposal's first part, but as mentioned earlier, we believe pairing-ID should be included in the monitoring configuration. - No need to configure Pairing ID(s) for monitoring |
| Samsung | Same view as Apple. Pairing ID is mandatory for UE-side data collection. |
| OPPO |  |
| Futurewei | We are ok with the proposal. |
| MediaTek | UE-side data collection is always associated with a model/dataset that has been already assigned a pairing ID for. So, UE needs to provide the pairing ID anyway and it is not optional |
| CMCC | Support. |
| CATT | We don’t think pairing ID is needed for either UE side or NW side data collection. For NW-side data collection before initial inter-vendor training collaboration, the pairing ID does not even exist. For NW-side data collection after initial inter-vendor training collaboration, the NW side can manage the dataset categorization and aggregation in an implementation specific manner. For UE-side data collection, UE fine-tuning happens when UE has identified that there is potential performance degradation for a certain model, UE should know which model is the target model for fine-tuning and the pairing ID is not needed. In case of a new UE devices/vendors appear in the NW, it should follow the initial NW data collection and inter-vendor training collaboration as all other devices/vendors have done before. |
| Ericsson | Support |
| ZTE | Support. For NW side data collection, we think the content of the collected data can include pairing ID which is beneficial for both NW side and UE side. From the NW perspective, a pairing ID is pre-assigned to the UE based on some determined rules (e.g., antenna port distribution or other NW-side settings). Then, the reporting of pairing ID along with the target CSI help relief NW-side efforts on data categorization for subsequent model training. From the UE perspective, the to-be-reported training data labeled by pairing ID can also be used for the offline engineering at the UE side. |
| Xiaomi | We think pairingID should be mandatory for UE side data collection, not optional. |
| Spreadtrum | Similar view with vivo. |
| InterDigital | Support |
| Panasonic | Support. Assuming the pairing ID can be associated with what is shared from the NW-side to the UE-side and the related configuration parameters, such as the NW-side additional conditions, the same ID can be used for UE to collect UE-side target CSI for UE-side training, i.e., for the configuration of data collection for UE-side training at least if data collection for UE-side training is based on OTA approach. If data collection for UE-side training is non-OTA approach, the relationship between “what is shared from the NW-side to the UE-side and the related configuration parameters” and “the configuration of data collection for UE-side training” is known by NW-side, and then it would not be necessarily required to use the same ID. |
| LGE | Support. For NW-side data collection, in order to provide a pairing ID for NW-side additional conditions, further discussion seems to be needed on which additional condition can be inferred from pairing ID. |
| ETRI | Support |
| HONOR | Support. |
| TCL | support |
| Toyota | Pairing ID should be used for CSI-Configuration, NW side data collection and for monitoring. |

Moderator note:  Regarding whether associated ID is additionally introduced for model pairing, from the last meeting as well as inputs for this meeting, majority of companies are not convinced to have it.

### [Rd1] Proposed conclusion 4.4:

For model pairing procedure, other than Pairing ID, no need to additionally indicate associated ID in Step 3 for representing NW side additional conditions

| Company | View |
| --- | --- |
| vivo | We can postpone the discussion as the definition of associated ID remains unclear, and further discuss the detailed pairing ID for all the inter vendor collaboration direction and finetuning. |
| Huawei, HiSilicon | Support. Pairing ID represents NW side additional condition as well and there is no need for an additional ID. |
| NTT DOCOMO | Support. |
| Samsung | As long as we agree on the following, we are fine on terminologies. - For Direction C if applicable, the corresponding Pairing-related ID may consist of a specified as a fixed value and network assigned ID for data collection. |
| Futurewei | We support this proposal, no need to indicate associated ID for pairing purpose. |
| MediaTek | Support. Associate ID is a redundant concept here in presence of pairining ID |
| CMCC | Support. |
| CATT | support |
| Ericsson | Support |
| ZTE | Support. |
| Xiaomi | We think the associated ID is needed at least for Direction C considering model finetuning. |
| Spreadtrum | Support，pairing ID is enough for model pairing procedure. |
| Panasonic | Support |
| LGE | Support. Associated ID is not needed in step 3 for representing NW-side additional conditions, since Pairing ID which can represent NW-side additional condition is already in step 3. |
| Fujitsu | Support |
| ETRI | In our understanding, associated ID is not cleared defined yet. So we don’t need this proposal at this stage, in our view. |
| HONOR | Support. |
| TCL | support |
| Toyota | Support |

## Issue#4-3 Other solutions or issues

### [Rd1] Question 4.5:

For model pairing, any other solutions or issues are missed?

| Company | View |
| --- | --- |

# [Place holder] Other issues

# Potential proposals for GTW/offline

## 13 Oct (Mon)

## 14 Oct (Tue) -offline

### [Rd2] Proposal 2.2a (definition of Option 1):

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantzation), consider the following definition:

- Content: for a number of S Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall S*2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2.

  - FFS whether to quantize to real value and imaginary value, or quantize to amplitude value and phase value.

  - FFS value of v across samples

  - Take S=1, same k1/k2/ T1min / T1max …as a baseline for evaluations.

- Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part.

  - FFS whether/how to perform the normalization, and whether it is normalized within or across [S samples]/ 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts, with evalutions.

  - FFS k1 and k2 values, and whether they are equal.

- Normalization: x1 is in forms of normalized representation within the range of [T1min, T1max], and x2 is in forms of normalized representation within the range of [T2min, T2max].

  - FFS whether/how to specify the normalization formula, e.g., y=x-xminxmax-xmin.

  - FFS the range of [T1min, T1max] and [T2min, T2max]

  - FFS the normalization is performed within or across S samples/ 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts

  - FFS how to determine xmin and xmax. E.g., selected among S samples or within per sample.

- Quantization: x1 real part (or amplitude part) is quantized to k1 bits and x2 imaginary part (or phase part) is quantized to k2 bits. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part.

  - FFS k1 and k2 values, and whether they are equal.

  - FFS whether to quantize to real value and imaginary value, or quantize to amplitude value and phase value.

- Note: If each complex element is quantized to k1+k2 bits, the overall payload size per data sample (Target CSI with 2N1N2 ports, N3 subbands, v layers for precoding matrix) is 2N1N2*N3*(k1+k2)*v bits.

## 14 Oct (Tue) -online

### [Rd2] Proposal 2.2a (definition of Option 1):

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantzation), consider the following definition:

- Content: for a number of S Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall S*2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2.

  - FFS whether to quantize to real value and imaginary value, or quantize to amplitude value and phase value.

  - FFS whether to ensure the same value of v across samples if S>1.

- Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part.

  - FFS whether/how to perform the normalization, and whether it is normalized within or across [S samples]/ 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts, based on evalutions.

  - FFS k1 and k2 values, and whether they are equal.

- Take S=1, and fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values and normalization method adopted in simulation.

- Note: If each complex element is quantized to k1+k2 bits, the overall payload size per data sample (Target CSI with 2N1N2 ports, N3 subbands, v layers for precoding matrix) is 2N1N2*N3*(k1+k2)*v bits.

### [Rd1] Proposal 4.2 (Link to inter-vendor collaboration):

For model pairing procedure for inference, Pairing ID refers to the exchanged dataset (for Option 4-1 if applicable)/exchanged model parameters (for Option 3a-1 if applicable)/standardized reference model (for Direction C if applicable) in the inter-vendor training collaboration.

- For Direction C if applicable, the corresponding Pairing ID is specified as a fixed value.

### [Rd1] Proposal 4.3 (Inclusion of Pairing ID in LCM procedures):

Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS whether to configure Pairing ID for NW side data collection

- No need to configure Pairing ID(s) for monitoring

## 15 Oct (Wed) -online

### [Rd3] Proposal 2.4b (parameters for Option 4):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following enhanced parameter combinations as basline with potential down selection.

- L= 6/12/16, pv=1, Beta = 1. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 12, pv=0.6/0.8, Beta = 0.5. Reference amplitude, differential amplitude, and phase follow legacy (4bits, 3bits, 4bits, respectively)

- L= 12, pv=0.8, Beta = 0.5/0.8. Reference amplitude = 6 bits; differential amplitude = 4 bits; phase = 6 bits

- Other combinations can be considered and reported by companies.

### [Rd3] Proposal 2.5b (Evaluation assumption):

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

  - rank-common layer-common with payload scalable design is considered

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

- Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

- SGCS results are reported per layer

- For Option 1, consider k1+k2=4, 6, 8, 16, 32

- Other assumptions follow R18 study EVM.

### [Rd1] Proposal 4.2 (Link to inter-vendor collaboration):

For model pairing procedure for inference, Pairing ID refers to the exchanged dataset (for Option 4-1 if applicable)/exchanged model parameters (for Option 3a-1 if applicable)/standardized reference model (for Direction C if applicable) in the inter-vendor training collaboration.

- For Direction C if applicable, the corresponding Pairing ID is specified as a fixed value.

### [Rd1] Proposal 4.3 (Inclusion of Pairing ID in LCM procedures):

Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS whether to configure Pairing ID for NW side data collection

- No need to configure Pairing ID(s) for monitoring

## 16 Oct (Thu) - offline

### [Rd3] Proposal 2.4b (parameters for Option 4):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection.

- L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits

  - Company to report the quantization step for reference amplitude for 6bits.

- No restriction to the total number of non-zero coefficients for a specific rank.

- [Note: determine up to 2 combinations of parameters]

- Tx port number is 32, 64, or 128

- Other combinations can be considered and reported by companies.

### [Rd3] Proposal 2.5b (Evaluation assumption):

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Performance metric:

  - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

  - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

  - SGCS results are reported per layer

- Alt1: Note: Overhead and complexity should not be ignored.

- Alt2: Note: Overhead and complexity should be analyzed [with quantitative values]

- For Option 1, consider k1+k2=4, 6, 8, 16, 32

- Other assumptions follow R18 study EVM

### [Rd1] Proposal 4.2 (Link to inter-vendor collaboration):

For model pairing procedure for inference, Pairing ID refers to the exchanged dataset (for Option 4-1 if applicable)/exchanged model parameters (for Option 3a-1 if applicable)/standardized reference model (for Direction C if applicable) in the inter-vendor training collaboration.

- For Direction C if applicable, the corresponding Pairing ID is specified as a fixed value.

### [Rd1] Proposal 4.3 (Inclusion of Pairing ID in LCM procedures):

Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS whether to configure Pairing ID for NW side data collection

- No need to configure Pairing ID(s) for monitoring

## 16 Oct (Thu) - online

### [Rd3] Proposal 2.4b (parameters for Option 4):

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection.

- L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits

  - Company to report the quantization step for reference amplitude for 6bits.

- No restriction to the total number of non-zero coefficients for a specific rank.

- Tx port number is 32, 64, or 128

- Other combinations can be considered and reported by companies.

### [Rd3] Proposal 2.5b (Evaluation assumption):

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Performance metric:

  - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

  - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

  - SGCS results are reported per layer

- Alt1: Note: Overhead and complexity should not be ignored.

- Alt2: Note: Overhead and complexity should be analyzed [with quantitative values]

- For Option 1, consider k1+k2=4, 6, 8, 16, 32

- Other assumptions follow R18 study EVM

### [Rd1] Proposal 4.2 (Link to inter-vendor collaboration):

For model pairing procedure for inference, Pairing ID refers to the exchanged dataset (for Option 4-1 if applicable)/exchanged model parameters (for Option 3a-1 if applicable)/standardized reference model (for Direction C if applicable) in the inter-vendor training collaboration.

- For Direction C if applicable, the corresponding Pairing ID is specified as a fixed value.

### [Rd1] Proposal 4.3 (Inclusion of Pairing ID in LCM procedures):

Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS whether to configure Pairing ID for NW side data collection

- [No need to configure Pairing ID(s) for monitoring]

## 17 Oct (Fri) - online

### [Rd1] Proposal 2.7:

For CPU counting of UE side data collection,

- OCPU=1

- The CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

### [Rd1] Proposal 4.3 (Inclusion of Pairing ID in LCM procedures):

Pairing ID(s) can be optionally configured in CSI-ReportConfig for UE side data collection

- FFS whether to configure Pairing ID for NW side data collection

- [No need to configure Pairing ID(s) for monitoring]

# Agreements

## Agreements for #122bis

Agreement:

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation:

- Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2.

  - Note: Companies to report whether  real value and imaginary value are quantized, or amplitude value and phase value are quantized.

- Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed.

  - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts.

- Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values.

- Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation.

Agreement:

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection.

- L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits

  - Company to report the quantization step for reference amplitude for 6bits.

- No restriction to the total number of non-zero coefficients for a specific rank.

- Other combinations can be considered and reported by companies.

Agreement:

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Performance metric:

  - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

  - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

  - Overhead per sample and per layer SGCS results are reported per layer

- Note: Complexity should be analysed.

- For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32

- Other assumptions follow R18 study EVM

Agreement

For CPU counting of UE side data collection,

- OCPU=1

- FFS: The CPU occupancy duration

## Agreements for #122

Agreement:

For model pairing procedure for inference, consider the applicability report procedure of Rel-19 AI/ML beam management for UE side model as a starting point.

- Pairing ID(s) (i.e., ID for pairing related discussion) is indicated (e.g., in CSI-reportConfig) in Step 3.

  - FFS whether/how to link the Pairing ID with the inter-vendor collaboration.

  - FFS whether/how Paring ID(s) are reported in Step 4.

- FFS whether to support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only).

  - FFS CSI compression specific inference configuration or inference related parameters.

Agreement:

For NW side data collection of CSI compression, discuss the content and format of the collected data by assuming higher layer based signalling.

- Details on signaling is up to RAN2.

Agreement:

To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report.

- P-CSI-RS and /SP CSI-RS are supported.

Agreement:

For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,:

- Option 0: Reusing legacy e-Type II codebook

- Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI.

- Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization.

- Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix.

- Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase)

- Note: the quantization format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling.

- Note: At least take precoding matrix as Target CSI type. FFS channel matrix.

- FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

# Potential issues for the next meeting

Following aspects may be at least discussed in the next/future meetings:

- NW side data collection

  - Submission of simulation results and analysis for Option 1 and Option 4 based on the EVM agreed at this meeting.

    - For Option 1, SGCS (including layer 1/2/3/4) performance and overhead (whether/how to hold at least one Target CSI sample within one RRC message) analysis are encouraged.

    - For Option 1, please provide your views on the normalization scope (within or across layers/subbands/Tx ports/samples/real-imag), quantize to real+imaginary or amplitude+phase, and values of k1/k2/T1min/T1max/T2min/T2max.

    - For Option 4, please opponent companies analyze the implementation complexity on extending the parameter values – as mentioned in the online session, the more you elaborate, the convincing it is to the proponent side on the challenges.

    - Depending on the performance, overhead, and complexity, we will decide whether to performe down selection.

- Monitoring

  - For NW side monitoring, please input your views on whether/how to support higher layer based monitoring (Issue#3-2).

  - For UE side monitoring (Issue#3-3), the suggestion from FL is to have at most one solution to support it. The major benefit of UE side monitoring is that it can be carried by L1 signaling which is faster than NW side monitoring; otherwise there is no need to support UE side monitoring given that we will probably have NW side monitoring.

- Model pairing

  - Please track the RAN2 specification on applicability report procedure of R19, including Option A and Option B (Issue#4-1), and provide your inputs on the option(s) to support for R20. It is Moderator’s feeling that the two options are applicable to different scenarios and it may not be harmful to have both.

    - No need to discuss Proposal 4.2 – it can be discussed at a later phase or by RAN2.

  - Issue#4-3 did not get a chance to be discussed. Please continue your inputs in the Tdoc.

# References

- RP-251870 “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Prague, Czech Republic, June 9-13, 2025.

- R1-2506744	Discussion on other aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

- R1-2506775	Discussion on other aspects of CSI compression	ZTE Corporation, Sanechips

- R1-2506782	Discussion on other aspects for CSI compression	TCL

- R1-2506793	AI/ML CSI Spatial/Frequency Compression: Other Aspects	InterDigital, Inc.

- R1-2506804	Discussion on AIML for CSI compression other aspects	Spreadtrum, UNISOC

- R1-2506833	Other aspects of AI/ML for CSI compression	Ericsson

- R1-2506888	Discussion on other aspects for CSI compression	vivo

- R1-2506932	Discussion on other aspects for CSI compression	Huawei, HiSilicon

- R1-2506979	Discussion on other aspects of CSI spatial/frequency compression	Xiaomi

- R1-2507007	Discussion on other aspects of CSI compression	CMCC

- R1-2507080    On Data Collection, Monitoring, and Model Pairing for AI/ML-based CSI Compression Southeast University

- R1-2507109	Discussion on other aspects of AI/ML-based CSI compression	CATT

- R1-2507166	Other aspects for AI/ML CSI compression	OPPO

- R1-2507184	Discussion on other aspects of CSI compression	KT Corp.

- R1-2507206	Other aspects for CSI compression	HONOR

- R1-2507243	Views on other aspects of CSI compression	Samsung

- R1-2507281	Discussion on other aspects of CSI compression	Fujitsu

- R1-2507303	Discussion on other aspects of CSI compression	NEC

- R1-2507390	CSI Compression: Other Aspects	Nokia

- R1-2507397	Discussion on other aspects for CSI compression	LG Electronics

- R1-2507416	Discussion on other aspects for AI/ML-based CSI compression	Panasonic

- R1-2507444	Other aspects for CSI compression	Lenovo

- R1-2507478	Other aspects	Tejas Network Limited

- R1-2507497	Discussion on other aspects of CSI compression	ETRI

- R1-2507518	Other Aspects for AI/ML based CSI Compression	Google

- R1-2507587	Discussion on other aspects for CSI compression	Sony

- R1-2507617	CSI spatial/frequency compression without temporal aspects (“Case 0”)- Other aspects	MediaTek Inc.

- R1-2507664	On other aspects for AI based CSI spatial/frequency domain compression	Apple

- R1-2507711	Specification of other aspects of AIML CSI compression	Qualcomm Incorporated

- R1-2507775	Discussions on other aspects for NR air interface enhancements	Sharp

- R1-2507778	Discussion on performance monitoring for CSI compression	Shanghai Jiao Tong University, Toyota

- R1-2507803	Discussion on other aspects of AI/ML CSI compression	NTT DOCOMO, INC.

- R1-2507868	Discussion on AIML based CSI compression	ASUSTeK