3GPP TSG-RAN WG1 Meeting #122bis	R1-2508196

# Prague, Czech, October 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | - | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | X | Radio Access Network |  |  |  |  | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | CR on determination of cyclic shift for PRACH transmission in 2TA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | CATT, Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2025-09-21 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-18 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |  |  |
| Reason for change: |  | In 38.331, for Rel-18 2TA operation, the higher-layer parameter prach-RootSequenceIndex in RACH-ConfigTwoTA is used to indicate PRACH root sequence index for PRACH transmission occasions associated with an additional PCI, this has not been captured in current 38.211. In 38.331, for Rel-18 2TA operation, the higher-layer parameter twoTA-restrictedSetConfig in RACH-ConfigTwoTA is used to configure an unrestricted set or one of two types of restricted sets for cyclic shifts generation of a PRACH transmission occasion associated with an additional PCI. This has not been captured in current 38.211. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the higher-layer parameter prach-RootSequenceIndex in RACH-ConfigTwoTA is used to indicate PRACH root sequence index for PRACH transmission occasion associated with an additional PCI. Clarify that the higher-layer parameter twoTA-restrictedSetConfig in RACH-ConfigTwoTA is used to determine cyclic shift of a logical root sequence for PRACH transmission occasion associated with an additional PCI. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot generate the preamble for PRACH transmission occasion associated with an additional PCI correctly for 2TA operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.3.1	Sequence generation

The set of random-access preambles  shall be generated according to

from which the frequency-domain representation shall be generated according to

where , , LRA=1151, or LRA=571 depending on the PRACH preamble format as given by Tables 6.3.3.1-1 and 6.3.3.1-2.

There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift  of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter prach-RootSequenceIndex or rootSequenceIndex-BFR or by msgA-PRACH-RootSequenceIndex if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213] or by prach-RootSequenceIndex in EarlyUL-SyncConfig if the PRACH transmission is for a candidate cell or by prach-RootSequenceIndex in RACH-ConfigTwoTA if the PRACH transmission is associated with an additional PCI different from serving cell PCI. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic; the logical index 0 is consecutive to LRA-2. The sequence number  is obtained from the logical root sequence index according to Tables 6.3.3.1-3 to 6.3.3.1-4B.

The cyclic shift  is given by

where  is given by Tables 6.3.3.1-5 to 6.3.3.1-7. The type of restricted sets (unrestricted, restricted type A, restricted type B) is given by

-	the higher-layer parameter msgA-RestrictedSetConfig, if provided;

-	or the higher-layer parameter ltm-restrictedSetConfig associated with a candidate cell indicated in Cell indicator field of a PDCCH order, if provided;

- 	or the higher-layer parameter twoTA-restrictedSetConfig associated with an additional PCI indicated in PRACH association indicator field of a PDCCH order, if provided;

-	otherwise, the higher-layer parameter restrictedSetConfig.

Tables 6.3.3.1-1 and 6.3.3.1-2 indicate the type of restricted sets supported for the different preamble formats.

<Unchanged parts are omitted >