3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	10.2

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session notes for AI 10.2 (NR MIMO Phase 6)

Document for:	Discussion, Decision

## NR MIMO Phase 6

Please refer to RP-252936 for detailed scope of the WI.

[122bis-R20-MIMO] Email discussion on Rel-20 MIMO – Rebecca (MTK)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Improvement of SRS capacity and coverage

Including a) Multiple frequency-domain starting positions for SRS repetition, and b) Cross-slot SRS between one U slot and one adjacent S slot.

R1-2506795	NR MIMO Phase 6: SRS Enhancement	InterDigital, Inc.

R1-2506806	Discussion on improvement of SRS capacity and coverage	Spreadtrum, UNISOC

R1-2506836	Improvement of SRS capacity and coverage	MediaTek Inc.

R1-2506845	Discussion on improving of SRS capacity and coverage	TCL

R1-2506890	Discussion on improvement of SRS capacity and coverage	vivo

R1-2506925	Improvement of SRS capacity and coverage	Huawei, HiSilicon

R1-2506981	Discussion on the improvement of SRS capacity and coverage	Xiaomi

R1-2507026	Improvement of SRS capacity and coverage	Tejas Network Limited

R1-2507040	Discussion on improvement of SRS capacity and coverage	ZTE Corporation, Sanechips

R1-2507111	Discussion on improvement of SRS capacity and coverage	CATT

R1-2507168	Discussion on enhancement of SRS capacity and coverage for MIMO phase 6	OPPO

R1-2507207	Discussion on improvement of SRS capacity and coverage	HONOR

R1-2507245	Views on improvement of SRS capacity and coverage	Samsung

R1-2507283	Discussion on improvement of SRS capacity and coverage	Fujitsu

R1-2507315	Discussion on improvement of SRS capacity and coverage	NEC

R1-2507325	Discussion on SRS capacity and coverage improvement	China Telecom

R1-2507406	Improvement of SRS Capacity and Coverage	Nokia

R1-2507438	Improvement of SRS capacity and coverage	Lenovo

R1-2507499	Discussion on improvement of SRS capacity and coverage for NR MIMO Phase 6	ETRI

R1-2507529	Discussion on improvement of SRS capacity and coverage	Panasonic

R1-2507543	Discussion on Improvement of SRS Capacity and Coverage	Rakuten Mobile, Inc

R1-2507589	Further discussions on improvements of SRS capacity and coverage	Sony

R1-2507632	Improvement of SRS capacity and coverage	Transsion Holdings

R1-2507669	On Rel-20 MIMO SRS capacity and coverage improvement	Apple

R1-2507713	SRS enhancements in 5G MIMO Phase 6	Qualcomm Incorporated

R1-2507769	Improvement of SRS capacity and coverage	Sharp

R1-2507805	Discussion on Improvement of SRS capacity and coverage	NTT DOCOMO, INC.

R1-2507880	On Rel-20 improvement of SRS capacity and coverage	Ericsson

R1-2507881	Views on enhancements for Improvement of SRS capacity and coverage	KDDI Corporation

R1-2507908	Discussion on improvement of SRS capacity and coverage	NICT

R1-2507994

Agreement:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop and K=R (i.e., each subgroup includes R/K=1 symbol (or 1 pair of consecutive symbols for 8-Tx SRS with ports8tdm))), the following configuration combinations and basic starting position patterns are supported

- when PF=K=2, support pattern {0,1}

- when PF=4 and K=2, select one of the following patterns:

  - Alt 1: {0,2}

  - Alt 2: {0,1}

- when PF=K=4, select one of the following patterns:

  - Alt 1: {0,2,1,3}

  - Alt 2: {0,1,2,3}

Note: exact pattern(s) are deduced from the basic patterns. FFS details.

Agreement:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop and K<R (i.e., each subgroup includes RK>1 symbol (or >1 pair of consecutive symbols for 8-Tx SRS with ports8tdm)), support the following configuration combinations and basic starting position patterns

- when PF=K=2, select at least one of the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,1,…,1}

  - Alt 2 (non-consecutive mapping): {0,1,,…,0,1}

- when PF=4 and K=2, down select from the following basic patterns:

  - Alt 1-1 (consecutive mapping):{0,…,0,2,…,2}

  - Alt 1-2 (consecutive mapping):{0,...,0,1,…,1}

  - Alt 2-1 (non-consecutive mapping):{0,2,…,0,2}

  - Alt 2-2 (non-consecutive mapping): {0,1,…,0,1}

- when PF=K=4, down select from the following basic patterns:

  - Alt 1-1 (consecutive mapping): {0,…,0,2,…,2,1,…,1,3,…,3}

  - Alt 1-2 (consecutive mapping): {0,…,0,1,…,1,2,…2,3,…,3}

  - Alt 2-1 (non-consecutive mapping): {0,2,1,3,…,0,2,1,3}

  - Alt 2-2 (non-consecutive mapping): {0,1,2,3,…,0,1,2,3}

Note: exact patterns are deduced from the basic patterns. FFS details.

R1-2508045

Agreement

To determine the time-domain location of a cross-slot SRS resource,

- ‘l0’ is interpreted as the starting symbol in the starting slot for an SRS resource.

- ‘l'+l0’ is interpreted as the symbol index in a slot or two consecutive S and U slots.

- The restriction ‘loffset≥NsymbSRS-1’ is removed.

Agreement

- For intra-repetition hopping for SRS repetition symbols within each SRS frequency, support periodic, semi-persistent and aperiodic SRS.

- For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support all SRS usages (i.e., codebook, nonCodebook, antennaSwitching and beamManagement)

Agreement:

- Support cross-slot SRS for all SRS resource types, i.e., for periodic, semi-persistent, and aperiodic SRS.

- Support cross-slot SRS for usages of codebook, non-codebook, antenna switching, beam management

R1-2508087

Agreement:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support Alt-1 from the agreement in RAN1#122bis as the following:

- when PF=4 and K=2, select one of the following patterns:

  - Alt 1: {0,2}

- when PF=K=4, select one of the following patterns:

  - Alt 1: {0,2,1,3}

R1-2508105

Agreement:

- For an aperiodic cross-slot SRS resource set in scenario 1, the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set.

- Note 1: Whether 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) can be supported in scenario 1 is discussed separately.

- Note 2: If the transmission of 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) is supported, for the resource(s) in the second slot, whether/how to utilize the slot offset configured for the SRS resource set is discussed separately.

- Note 3: 0 or more SRS resource with time-domain resource entirely in the first slot (i.e. S slot) can be supported in scenario 1.

Working Assumption:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, support Alt-2:

- when PF=K=2, select at least one of the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,1,…,1}

  - Alt 2 (non-consecutive mapping): {0,1,,…,0,1}

- when PF=4 and K=2, down select from the following basic patterns:

  - Alt 1 (consecutive mapping):{0,…,0,2,…,2}

  - Alt 2 (non-consecutive mapping):{0,2,…,0,2}

- when PF=K=4, down select from the following basic patterns:

  - Alt 1 (consecutive mapping): {0,…,0,2,…,2,1,…,1,3,…,3}

  - Alt 2 (non-consecutive mapping): {0,2,1,3,…,0,2,1,3}

### Enhancing DL CSI acquisition

Including a) Early SRS/CSI/CSI-RS triggering, and b) CSI-RS density reduction for 48, 64, and 128 CSI-RS ports.

R1-2506746	Enhancing DL CSI acquisition	FUTUREWEI

R1-2506759	Configuring CSI-RS Resources with Different Densities	Kyocera Corporation

R1-2506796	NR MIMO Phase 6: DL CSI Enhancement	InterDigital, Inc.

R1-2506807	Discussion on enhancing DL CSI acquisition	Spreadtrum, UNISOC

R1-2506837	Enhancing DL CSI acquisition	MediaTek Inc.

R1-2506838	Moderator summary on enhancing DL CSI acquisition (Round 0)	Moderator (MediaTek Inc.)

R1-2506839	Moderator summary on enhancing DL CSI acquisition (Round 1)	Moderator (MediaTek Inc.)

R1-2506840	Moderator summary on enhancing DL CSI acquisition (Round 2)	Moderator (MediaTek Inc.)

R1-2506844	Discussion on enhancing DL CSI acquisition	TCL

R1-2506891	Discussion on enhancing DL CSI acquisition	vivo

R1-2506926	DL CSI acquisition enhancment	Huawei, HiSilicon

R1-2506953	On DL CSI Acquisition Enhancements for FR1	Nokia

R1-2506982	Discussion on enhancing DL CSI acquisition	Xiaomi

R1-2507027	Enhancing DL CSI acquisition	Tejas Network Limited

R1-2507033	Discussion on Early DL CSI Acquisition Enhancements	Panasonic

R1-2507041	Discussion on enhancing DL CSI acquisition	ZTE Corporation, Sanechips

R1-2507112	Discussion on enhancements for DL CSI acquisition	CATT

R1-2507169	Discussions on Enhancing DL CSI Acquisition	OPPO

R1-2507208	Discussion on enhancing DL CSI acquisition	HONOR

R1-2507246	Views on enhancing DL CSI acquisition	Samsung

R1-2507284	Discussion on enhancing DL CSI acquisition	Fujitsu

R1-2507304	Discussion on Enhancing DL CSI acquisition	NEC

R1-2507399	Discussion on enhancing DL CSI acquisition	LG Electronics

R1-2507410	DL CSI acquisition enhancements for Rel. 20 MIMO	Fraunhofer IIS, Fraunhofer HHI

R1-2507500	Discussion on enhancing DL CSI acquisition for NR MIMO Phase 6	ETRI

R1-2507542	Discussion on Enhancement of CSI DL Acquisition	Rakuten Mobile, Inc

R1-2507551	Enhancing DL CSI acquisition	Lenovo

R1-2507561	Discussions on enhancing DL CSI acquisition	China Telecom

R1-2507579	Discussion on enhancing DL CSI acquisition	Google

R1-2507590	Further discussions on DL CSI acquisition enhancements	Sony

R1-2507670	On Rel-20 MIMO CSI enhancement	Apple

R1-2507714	DL CSI acquisition enhancements in 5G MIMO Phase 6	Qualcomm Incorporated

R1-2507744	DL CSI Enhancements for NR Rel-20	AT&T

R1-2507757	On Rel-20 enhanced DL CSI acquisition	Ericsson

R1-2507770	Enhancing DL CSI acquisition	Sharp

R1-2507777	Discussion on early DL CSI acquisition design	Fainity Innovation

R1-2507806	Discussion on Enhancing DL CSI acquisition	NTT DOCOMO, INC.

R1-2507830	Discussion on DL CSI acquisition	ITRI, Acer Incorporated

R1-2507898	Views on DL Channel acquisition enhancements	CEWiT

R1-2507909	Discussion on enhancing DL CSI acquisition	NICT

R1-2507945	Discussion on enhancing DL CSI acquisition 	IIT Kanpur

R1-2506838

Agreement:

For early triggering of SRS-AS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field

- FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4)

Agreement:

For early triggering of CSI/CSI-RS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting for the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- The aperiodic CSI reporting is associated with aperiodic CSI-RS for CSI on the non-dormant BWP

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field

- FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

Agreement:

For UE transition from IDLE/INACTIVE to CONNECTED mode, support the followings for aperiodic CSI reporting triggered by MSG4:

- For PMI-based reporting, support the report quantity ‘CRI-RI-PMI-CQI’

  - FFS: ‘CRI-RI-LI-PMI-CQI’

- For PMI-free reporting, support the report quantity ‘CRI-RI-CQI’

Agreement:

For early TRS reception when UE transition from IDLE/INACTIVE to CONNECTED mode, down-select one from the followings in RAN1#123 meeting:

- Alt-1: Support aperiodic TRS triggered by MSG4 only

- Alt-2: Support periodic TRS triggered by MSG4 and aperiodic TRS triggered by MSG4

- Alt-3: Support periodic TRS triggered by MSG4 only

R1-2506839

Agreement

For UE transition from IDLE/INACTIVE to CONNECTED mode, using MAC-CE in the PDSCH for MSG4 to trigger early aperiodic SRS-AS/CSI-RS/CSI

- Note: How to determine the PUSCH for aperiodic CSI reporting is separately discussed.

Agreement

For SCell transition from deactivation to activation, support the followings for aperiodic CSI reporting triggered based on legacy SCell activation command:

- For PMI-based reporting, support the report quantity ‘CRI-RI-PMI-CQI’

  - FFS: ‘CRI-RI-LI-PMI-CQI’

- For PMI-free reporting, support the report quantity ‘CRI-RI-CQI’

Agreement

For SCell transition from dormant-BWP to non-dormant BWP, support the followings for aperiodic CSI reporting triggered via a DCI indicating switching out of SCell dormancy:

- For PMI-based reporting, support the report quantity ‘CRI-RI-PMI-CQI’

  - FFS: ‘CRI-RI-LI-PMI-CQI’

- For PMI-free reporting, support the report quantity ‘CRI-RI-CQI’

Agreement

For early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting) when UE transition from INACTIVE to CONNECTED mode, reuse the procedure for UE transition from IDLE to CONNECTED mode.

Agreement

For early triggering of SRS-AS when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows:

- In Step-1, SIBx provides one or multiple SRS configurations based on one or multiple ‘xTyR’ UE capability assumptions.

  - At least one SRS configuration(s) resource sets for SRS-AS in an SRS configuration is provided for a same ‘xTyR’

  - FFS: Information to be provided in SRS configuration(s)

  - FFS: Whether Configuration information could be SRS configuration or the CSI report configuration for PMI-free report.

  - FFS: Whether SRS configuration can be associated with CSI report configuration for PMI-free report.

- In Step-2, UE reports through MSG3 which/whether the SRS configuration(s) provided in SIBx is/are supported.

  - FFS: How to report which/whether the SRS configuration(s) provided in SIBx is/are supported.

R1-2506840

Agreement:

For early triggering of aperiodic CSI reporting and the associated CSI-RS for CSI when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows:

- In Step-1, SIBx provides one or multiple CSI report configurations based on one or multiple UE capability assumptions

  - Each CSI reporting configuration is associated with a CSI resource configuration for channel measurement.

    - The CSI-RS resource set(s) in the CSI resource configuration for channel measurement is provided with a same number of CSI-RS ports

  - Each CSI reporting configuration is associated with a CSI resource configuration for CSI-IM based interference measurement.

  - FFS: Support of NZP CSI-RS based interference measurement

  - FFS: Information to be provided in a CSI report/resource configuration

- In Step-2, UE reports through MSG3 which/whether the CSI report configuration(s) provided in SIBx is/are supported.

  - FFS: How to report which/whether the CSI report configuration(s) provided in SIBx is/are supported.

R1-2508115

Agreement:

On PUSCH allocation for MSG4-triggered aperiodic CSI reporting associated with aperiodic CSI-RS for CSI, when UE transition from IDLE/INACTIVE to CONNECTED mode, down select one from the following alternatives to determine PUSCH for carrying the aperiodic CSI report by RAN1#123:

- Alt-1: The PUSCH is scheduled by MAC CE in MSG4 along with the aperiodic CSI report triggering.

- Alt-2: The PUSCH is scheduled by a legacy DCI after MSG4

- Alt-3: The PUSCH is configured by SIBx providing resource/reporting configuration for early triggering of aperiodic CSI reporting.

Note: The timeline of PUSCH for aperiodic CSI reporting is separately discussed.

Agreement:

On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for a SCell, based on the legacy SCell activation activating the SCell, down-select one from the followings in RAN1#123 meeting:

- Alt-1 (Implicit mechanism):

  - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to RRC configuration.

  - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to RRC configuration

- Alt-2 (Explicit mechanism):

  - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command.

  - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command.