3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.2

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session notes for AI 8.2 (Maintenance on NR MIMO Phase 5)

Document for:	Discussion, Decision

## 8.2	Maintenance on NR MIMO Phase 5

Note: Maximum one contribution. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- Enhancements for UE-initiated/event-driven beam management

- CSI enhancements

- Support for 3-antenna-port codebook-based transmissions

- Enhancement for asymmetric DL sTRP/UL mTRP scenarios.

[122bis-R19-MIMO] Email discussion on MIMO – Eko (Samsung)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2506790	Maintenance of Rel-19 MIMO	InterDigital, Inc.

R1-2506791	FL Summary Support for 3TX CB-based Uplink; First Round	Moderator (InterDigital, Inc.)

R1-2506797	Remaining issues on NR MIMO Phase 5	Spreadtrum, UNISOC

R1-2506835	Maintenance on NR MIMO Phase 5	MediaTek Inc.

R1-2506872	Maintenance on MIMO phase 5	vivo

R1-2506923	Maintenance of MIMO Phase 5	Huawei, HiSilicon

R1-2506962	Maintenance on NR MIMO Phase 5	Xiaomi

R1-2507034	Maintenance topics on UE-initiated/event-driven beam management 	Panasonic

R1-2507036	Maintenance on NR MIMO Phase 5	ZTE Corporation, Sanechips

R1-2507095	Maintenance on  NR MIMO Phase 5	CATT

R1-2507159	Remaining Issues of NR MIMO Phase 5	OPPO

R1-2507228	Remaining issue on NR MIMO Phase 5	Samsung

R1-2507269	Moderator Summary#1 on Maintenance for Rel-19 CSI Enhancements: Round 1	Moderator (Samsung)

R1-2507270	Moderator Summary#1 on Maintenance for Rel-19 CSI Enhancements: Round 2	Moderator (Samsung)

R1-2507271	Moderator Summary on Offline Session for Rel-19 CSI Enhancements	Moderator (Samsung)

R1-2507278	Remaining issues on NR MIMO Phase 5	Fujitsu

R1-2507316	Remaining issues on NR MIMO Phase 5	NEC

R1-2507346	Maintenance on NR MIMO Phase 5	Ericsson

R1-2507372	Summary on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

R1-2507436	Maintenance on NR MIMO Phase 5	Lenovo

R1-2507453	Maintenance on NR MIMO Phase 5	Ofinno

R1-2507558	Maintenance on Rel-19 CSI enhancements	Tejas Network Limited

R1-2507577	Maintenance on NR MIMO Phase 5	Google

R1-2507628	Maintenance on UE-initiated/event-driven beam management	Transsion Holdings

R1-2507695	Maintenance on NR MIMO Phase 5	Qualcomm Incorporated

R1-2507737	Maintenance of NR MIMO Phase 5	Nokia

R1-2507756	Discussions on Maintenance on MIMO phase5	China Telecom

R1-2507790	Maintenance on NR MIMO Phase 5	NTT DOCOMO, INC.

R1-2507864	Remaining issues on UE initiated beam report	ASUSTeK

### Enhancements for UE-initiated/event-driven beam management

R1-2508014

Agreement:

The TP in Section 5 of R1-2508014 for proposal 3.5 for TS38.214 is endorsed.

Agreement:

The TP in Section 5 of R1-2508014 for proposal 3.6 for TS38.214 is endorsed.

Agreement:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding mode-A, introduce RRC configuration parameter of reportSlotOffsetList in the configuration of the report transmission mode.

R1-2508074

Agreement:

The TP in Section 5 of R1-2508074 for proposal 3.7 for TS38.214 is endorsed.

Agreement:

The TP in Section 5 of R1-2508074 for proposal 3.10B for TS38.214 is endorsed.

R1-2508094

Agreement:

Adopt the following changes in Clause 5.2.1.5.4.1 in TS38.214.

- Reason for change: Regarding the condition of resetting the counter of event instances, the following candidate#2 was agreed to be applied for Event-2, Event-1 and Event-7, i.e., the measured current beam RS is updated based on indicated TCI state. However, it is intuitive that current beam RS of Event-7 is just related to the activated TCI states, rather than the indicated TCI state. Consequently, the condition of resetting the counter for Event-7 has not yet been captured in the current specifications, i.e., the measured current beam RS is updated based on activated TCI states.

- Summary of change: Clarifying the condition of resetting the counter for Event-7, i.e., the measured current beam RS is updated based on activated TCI states.

- Consequences if not approved: It is unclear how to reset the counter for Event-7.

##### 5.2.1.5.4 UE Initiated reporting 5.2.1.5.4.1 UE Initiated CSI reporting <Irrelevant part is omitted> If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated when the UE is configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’ or ‘event1’, or - if the reference signal with the valueOfQ highest L1-RSRP out of the reference signals among the activated TCI states or the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH blocks QCLed with the reference signals among the activated TCI states, which determines event instance(s), is updated when the UE is configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. <Irrelevant part is omitted>

Agreement:

Adopt the following changes in TS38.213 Section 9:

- Reason for change: Reusing the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH for UEI-BR for Mode A was agreed. However, it is not captured in the current specification.

- Summary of change: In TS38.213 section 9, clarify the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A.

- Consequences if not approved: UE behavior is not clear when PUSCH carrying UEI-BR for Mode A overlaps with other uplink channels/RSs.

9 UE procedure for reporting control information < Unchanged parts are omitted > For the remaining of this clause, when a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs, and is provided coresetPoolIndex with a value of 1 for second CORESETs, on active DL BWPs of serving cells, and - is provided sTx-2Panel the UE separately determines and resolves time overlapping among first PUSCH transmissions that use respective first spatial domain filters corresponding to first TCI-State or TCI-UL-State associated with the first CORESETs, and among second PUSCH transmissions that use respective second spatial domain filters corresponding to second TCI-State or TCI-UL-State associated with the second CORESETs. For the remaining of this clause, reference to PUSCH with semi-persistent CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeB’ in the CSI report configuration [6, TS 38.214]. For the remaining of this clause, unless otherwise stated, reference to PUSCH with aperiodic CSI reports includes a PUSCH with UE initiated report when reportTransmissionMode is configured as ‘ModeA’ in the CSI report configuration [6, TS 38.214]. For the remaining of this clause, for a UE operating on an NTN serving cell, the timeline conditions for resolving time overlapping between a PUSCH transmission with repetitions in an OCC group [6, TS 38.214] and PUCCH transmissions are applicable with respect to the first repetition of the PUSCH transmission in the OCC group - if the UE would multiplex UCI from the PUCCH transmissions in the PUSCH, the UE multiplexes the UCI in all repetitions of the PUSCH transmission in the OCC group - if the UE would not transmit a repetition of the PUSCH transmission, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group - the UE does not expect to transmit in different slots more than one PUCCHs that provide HARQ-ACK information or CSI reports and would overlap with the PUSCH transmission in the OCC group When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in clause 9.2.5 and 9.2.6 before resolving the overlapping for PUCCH transmissions without SL HARQ-ACK or the overlapping for PUCCH transmissions and PUSCH transmissions. < Unchanged parts are omitted >

### CSI enhancements

R1-2507269

Agreement

The TP2.A for TS38.214 in R1-2507269 is endorsed.

Agreement

The TP3.B.2 for TS38.214 in R1-2507269 is endorsed.

Agreement

The TP1.A for TS38.214 in R1-2507269 is endorsed.

Agreement

The TP1.B for TS38.214 in R1-2507269 is endorsed. Note that the Clause number for this TP is Clause 5.2.2.5 and not 5.2.1.4.1.

### Support for 3-antenna-port codebook-based transmissions

R1-2506791

Agreement:

Adopt the following corrections for TS 38.211, TS 38.213 and TS 38.214.

Reason for change: Adapt to the agreed terminology.

Summary of change: Correct the relevant RRC parameter to “fourPortSRS-3Tx”.

Consequence if not changed: Incorrect/Incomplete referencing.

- TS 38.211: - 6.3.1.5 Precoding --------------------------------------------Unchanged parts are omitted------------------------------------------ For codebook-based transmission, the precoding matrix depends on the number of antenna ports used for the transmission: - for single-layer transmission on a single antenna port, ; - for transmissions using 2, or 4 antenna ports, is given by Tables 6.3.1.5-1 to 6.3.1.5-7; - for transmissions using 3 antenna ports when fourPortSRS-3Tx 4portSRS_3TX is configured, is given by Tables 6.3.1.5-48 to 6.3.1.5-50; --------------------------------------------Unchanged parts are omitted------------------------------------------ Table 6.3.1.5-48: Precoding matrix for single-layer transmission using three antenna ports with fourPortSRS-3Tx 4portSRS_3TX configured. --------------------------------------------Unchanged parts are omitted------------------------------------------ Table 6.3.1.5-49: Precoding matrix for two-layer transmission using three antenna ports with fourPortSRS-3Tx 4portSRS_3TX configured. --------------------------------------------Unchanged parts are omitted------------------------------------------ Table 6.3.1.5-50: Precoding matrix for three-layer transmission using three antenna ports with fourPortSRS-3Tx 4portSRS_3TX configured. --------------------------------------------Unchanged parts are omitted------------------------------------------ - 6.4.1.4.3 Mapping to physical resources --------------------------------------------Unchanged parts are omitted------------------------------------------ - for an SRS resource in an SRS resource set with the higher-layer parameter fourPortSRS-3Tx 4portSRS_3TX is configured, - otherwise, --------------------------------------------Unchanged parts are omitted------------------------------------------

### Enhancement for asymmetric DL sTRP/UL mTRP scenarios

R1-2507372

Agreement

The TP in proposal 3.1 for TS38.212 in Section 3 of R1-2507372 is endorsed.

Agreement

The TP in proposal 3.2a for TS38.213 in Section 3 of R1-2507372 is endorsed.

Agreement

The TP in proposal 3.2b for TS38.214 in Section 3 of R1-2507372 is endorsed.

Agreement

The TPs in proposal 2.1 for TS38.213 and TS38.212 in Section 3 of R1-2507372 are endorsed.

R1-2508097

Conclusion:

Regarding the first agreement provided in RAN2 LS R1-2506721, whether any action from RAN1 is needed due to this new RRC parameter can be discussed after RAN2 RRC parameters are finalized.