3GPP TSG-RAN WG2 Meeting #131	R2-2505072

Bangaluru, India, 25-29 August 2025

Agenda item:	8.7.5

Source:	Qualcomm Incorporated

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss the remaining issues on RLC enhancements.

# Discussion

## Avoid unnecessary retransmission

[UE capability-4] Whether to have separate or joint UE capability for UE Tx and Rx operations

For transmitter and receiver that are peer RLC entities, we acknowledge that there should be dependency between Tx and Rx discard. But the proper configuration for that can be ensured by network.

In the context of UE capability, our understanding is the following:

- On UL, if UE supports Tx side discard and network enables it, it is network that needs to implement Rx side discard;

- On DL, if UE supports Rx side discard and network enables it, it is up to network implementation whether to enable Tx discard, as DL RLC can still works without Tx discard.

If these understandings are not flawed, then it is not hard to see that if a UE supports only Tx (Rx) side discard but not the Rx (Tx) side discard, its UL (DL) operation still works because network will ensure it performs the required operation on the other hand. Therefore, Tx discard and Rx discard are really two separate UE capabilities with no dependency between them.

In addition to technical feasibility discussed above, having Tx discard and Rx discard as separate UE capabilities can simplify the development and testing of the features.

Proposal 1.	[capability-4] Define separate UE capabilities for Tx discard and Rx discard.

[RLC-10] 	Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard.

Based on the above analysis, it should be quite clear that if a UE is configured with stopReTxObsoleteSDU, it is network’s responsibility to ensure Rx-side discard is properly implemented. As RAN2 specs do not specify network implementations, nothing needs to be captured in the spec.

Proposal 2. 	[RLC-10] Nothing needs to be captured in the spec regarding the dependency between stopReTxObsoleteSDU and Rx side discard at gNB.

[RLC-12] Impact of discard on PDCP SN gap report

At the RAN2#129 meeting, companies discussed the issue with PDCP SN gap report during mobility events. The following agreements were made:

From RAN2 point of view, for Rel-18 alternative 3 should not be supported. RAN2 doesn’t see a big problem to solve for Rel-18. RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem, further discussions are needed on the alternatives.

It is possible that a PDCP SN gap report is lost. For example, an SN gap report can be lost during a mobility event, e.g. due to RLC re-establishment (Details may be found in R2-2500491).

If a SN gap report is lost, the receiver will not be able to know that a missing PDCP SDU within a SN gap has already been discarded. As a result, it will start t-Reordering timer, causing extra delay in the delivery of other SDUs to the upper layers and degrading mobility performance.

We think this issue should be considered in Rel-19, as new RLC enhancements such as avoiding unnecessary retransmissions may increase PDCP discards. Increased discards will lead to more SN gap reports, raising the likelihood that some reports are lost.

There were three different options proposed at last meeting to address the issue:

-	Option 1: The UE sends the PDCP SN gap report again after connecting to the target gNB.

- Option 2: The UE shall re-transmit any PDCP SN Gap report(s) transmitted prior to the PDCP entity re-establishment for which the successful delivery has not been confirmed by lower layers.

- Option 3: UE triggers a PDCP SN gap report upon receiving a status report that includes a negative acknowledgment for an SDU indicated in an earlier gap report

Although Option 1 addresses the issue, we find it very inefficient. Since losing a gap report during mobility events is rare, requiring the UE to send an SN gap report after every mobility event would be too excessive, as most of them are not needed.

As to Option 2, we think it is more efficient than Option 1 but does not completely avoid unnecessary retransmission of SN gap reports. For example, an SN gap report may have been successfully received by gNB. However, it is not acknowledged when handover is performed (RLC entity at source gNB) is released. Therefore, there is no issue at the target gNB. But according to Option 2, UE needs to retransmit the SN gap report to the target gNB, because it did not receive acknowledgement on it before handover.

Based on the above analysis, we believe a good option should be the one that helps UE reliably detect a loss of SN gap reports reliably and avoid unnecessary retransmissions in all scenarios. We think Option 3 can be such a solution. When UE receives a PDCP status report indicating a PDCP SDU is missing but the SDU has already been discarded, it can infer with certainty that the SN gap report covering this PDCP SDU must have lost. It then triggers a new SN gap report. Therefore, this option requires UE to re-transmit a SN gap report only when necessary.

Proposal 3. 	[RLC-12] UE triggers a PDCP SN gap report upon receiving a status report that includes a negative acknowledgment for an SDU indicated in an earlier gap report.

## Timely retransmission

Status report

To enable the full benefits of enhanced polling, it is important to expedite the corresponding status report too. Otherwise, polls triggered by remaining time are still regulated by legacy mechanisms like t-StatusProhibit. This limits the effectiveness of enhanced polling.

It is not a good idea to configure a very small t-StatusProhibit to avoid this issue. For example, when no RLC PDUs have low remaining time, status reports do not need to be sent frequently. They need to be expediated or sent more frequently only when there are time-critical PDUs (those with low remaining times).

Observation 3.	To maximize the effectiveness of the enhanced polls, it is important to expediate the transmission of the corresponding status reports.

There can be a few options for a RLC receiver to do that. For example, the transmitter can inform the receiver that a poll if an enhanced poll triggered based on remaining time, the receiver then sends a status report immediately without the restriction by t-StatusProhibit or apply a shorter prohibit timer.

Proposal 4.	The transmitter includes an extra indication in the RLC PDU header whether the included poll is an enhanced poll (i.e. triggered based on remaining time).

Proposal 5.	Upon receiving an enhanced poll, the receiver sends a status report immediately, ignoring t-StatusProhibit; or apply a shorter t-StatusProhibit for the report.

The receiver may also be configured to trigger a time-critical status report itself. Since the newly introduced timer t-RxDiscard approximates the remaining time of a missing RLC PDU, its remaining time implies whether a missing PDU has become time critical. If so, it should send a status report asap.

Proposal 6.	When the remaining time of t-RxDiscard drops below a configured threshold, the receiver sends a status report immediately, ignoring t-StatusProhibit; or apply a shorter t-StatusProhibit for the report.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Avoiding unnecessary retransmissions

Proposal 1.	[capability-4] Define separate UE capabilities for Tx discard and Rx discard.

Proposal 2. 	[RLC-10] Nothing needs to be captured in the spec regarding the dependency between stopReTxObsoleteSDU and Rx side discard at gNB.

Impact of discard on PDCP SN gap report

Proposal 3. 	[RLC-12] UE triggers a PDCP SN gap report upon receiving a status report that includes a negative acknowledgment for an SDU indicated in an earlier gap report.

Status report

Observation 3.	To maximize the effectiveness of the enhanced polls, it is important to expediate the transmission of the corresponding status reports.

Proposal 4.	The transmitter includes an extra indication in the RLC PDU header whether the included poll is an enhanced poll (i.e. triggered based on remaining time).

Proposal 5.	Upon receiving an enhanced poll, the receiver sends a status report immediately, ignoring t-StatusProhibit; or apply a shorter t-StatusProhibit for the report.

Proposal 6.	When the remaining time of t-RxDiscard drops below a configured threshold, the receiver sends a status report immediately, ignoring t-StatusProhibit; or apply a shorter t-StatusProhibit for the report.