3GPP TSG-RAN WG2 Meeting #131                                                                R2-2505083

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.9.4

Source:	vivo

Title:	Remaining Issues on PWS Support for NB-IoT

Document for:	Discussion and Decision

# Introduction

In RAN2#130 [1], PWS support for NB-IoT was discussed and the following agreements were made.

Agreements: - A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT) - Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case

In RAN2#128 [2], there is a FFS on whether to support the PWS acquisition without acquiring SIB1.

2. We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB-NB acquisition is needed

Besides, according to the post email discussion [3], whether to extend the support of PWS to NB-IoT TN is unclearer.

In the contribution, we will continue to analyse the above open issues and give our views correspondingly.

# Discussion

For PWS support in NB-IoT TN:

In R19 IoT NTN session, we discussed and introduce support of the PWS for NB-IoT NTN. Some companies argue that whether we can also extend the PWS support for NB-IoT TN, since the WID says “The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.”. Note that the support of the PWS for NB-IoT TN can be enabled only when there is no additional spec impacts is needed. However, according to the LS from CT1 [4] and the endorsed CR in SA2 [5], the PWS for NB-IoT TN is not supported, and there is no corresponding requirement on supporting PWS for NB-IoT TN. Since the discussion of the topic is already finished in SA2, it is proposed not to extend the PWS support to NB-IoT TN in R19.

Table M-5: Misc. Clause; Clause title; Support. NB-IoT RAT; CP/UP/Both; UE; NW. 5.8; MBMS (3); (3); (3); (3); (3). 5.10; Multiple PDN support and PDN activation for UEs supporting Attach without PDN connectivity; Yes (1); Both. 5.11; UE capability handling; Yes; Both. 5.12; Warning message delivery; Yes(4); Both; Optional; Optional. 5.13; DRX and UE-specific DRX parameter handling; Yes (2); Both. 5.13a; Extended idle mode DRX; Yes; Both; Optional; Optional. 5.15; RAN information management procedures; No; N/A; N/A; N/A. 5.16; MME-initiated procedure on UE's CSG membership change; No; N/A; N/A; N/A. 5.17; HeNB multicast packet forwarding function; No; N/A; N/A; N/A. NOTE 1: Multiple PDN optional. NOTE 2: UE-specific DRX not supported in NB-IoT RAT. NOTE 3: See Table M-6. NOTE 4: Support is applicable for NB-IoT RAT over NTN..

Proposal 1: Do not support PWS for NB-IoT TN in R19.

For PWS acquisition without SIB1 acquisition:

When we reviewed the spec, we found that the normal procedure for PWS acquisition includes the following steps:

Step 1: UE acquires the PWS notification either via direct indication information or via the paging message;

Step 2: UE acquires the SIB1-NB and decides the PWS scheduling information;

Step 3: UE acquires the corresponding PWS message in the corresponding resource.

Observation 1: PWS acquisition using the current mechanism leads to comparable long latency.

However, considering the urgency of the PWS message, the earlier the UE acquires the PWS message the earlier UE can make the corresponding response. Moreover, due to the long RTT in NTN scenario, the problem will be more serious. Optimizations can be researched to enable the UE to accelerate the PWS acquisition。

The candidate solution can be:

Option 1: NW provides the PWS scheduling information within SIB1-NB even though there is no PWS notification, and UE can avoid reading the SIB1-NB upon acquiring the PWS message.

According to the spec [6], the PWS information change does affect the value tag of the SIB1-NB. Thus for this option, it is totally up to NW implementation, i.e., NW always includes the PWS scheduling information even if there is no PWS notification. From the UE perspective, it can use the stored SIB1-NB directly upon receiving the PWS notification. One may argue there may be some resource consumption, but considering the urgency of the PWS, such a solution is also beneficial. Besides, option 1 can be totally up to implementation, there is no spec impact.

E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, RLOS indication (i.e., rlos-Enabled), regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters, UAC parameters, positioning system information blocks, or satellite assistance information. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

Option 2: NW provides the PWS scheduling information or the PWS message directly within the paging message.

For this option, providing the PWS scheduling information in the paging message has a similar performance as option 1, while providing the PWS message in the paging message may be faster for PWS acquisition by UE. But the spec impact is larger, and it may impact RAN1, i.e.,  the PWS notification must be included in the paging message since the current LTE spec does not support the paging message and the direct indication information is provided at the same time [7]. Moreover, the signalling overhead is larger due to indicating the scheduling information or the PWS message in the paging message.In the following, we make a comparison of the above options. And it is clear the option 1 is better.

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | UE can avoid reading the SIB1-NB to accelerate the PWS acquisition; No spec impacts. | Resource consumption is reserved for PWS. |
| Option 2 | UE can avoid reading the SIB1-NB to accelerate the PWS acquisition. | Spec impacts are required; RAN1 may be involved; Signaling overhead. |

Besides, according to the following description [6], UE can assume the scheduling information in SIB1(-NB) is unchanged when acquiring the SIB31(-NB), that is, NW can provide the corresponding scheduling information in advance. Such a mechanism is similar to option 1. Thus, we think there is no complicated spec impact for option 1.

NOTE 3: When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

Observation 2: The current spec already supports the NW to provide the SIB31(-NB) scheduling information to UE before the SIB31(-NB) is scheduled.

Proposal 2 (RRC-2): To avoid UE reacquire the SIB1-NB upon acquiring the PWS message, the network provides the SIB10-NB/SIB11-NB/SIB12-NB scheduling information within SIB1-NB.

Proposal 3 (RRC-2): When acquiring SIB10-NB/SIB11-NB/SIB12-NB, UE may assume that the scheduling is unchanged.

For PWS acquisition across cells:

In RAN2#130, the support the PWS reception across different cells was discussed and it is agreed that even we support the enhancement in R19, it should be limited in intra-eNB case. However, there is still some issues to support the PWS reception across different cells.

Firstly, one PWS will be splited to multiple PWS messages with each identified with a messagIdentify, and each PWS will be repeated many times.  Further, each PWS message will be segmented to multiple segments. One possible scenario is that the UE receives the segment 3/4 of the first PWS message, then UE reselects to cell 2 and receives the segment 1/2 of the second PWS message. RAN2 need to confirm whether to support the combination of the segment 3/4 of the first PWS message and the segment 1/2 of the second PWS message, and also need to clarify the corresponding UE behaviour.

Moreover, to enable the UE to decide whether the PWS segment can be combined, the eNB may need to provide some information to the UE. Such information can either be indications of the cells within which the PWS combination can be enabled, or an indication whether the cell supports the PWS combination and then UE can decide all cells within the same eNB of the cell supports the PWS combination. Besides, for test purpose, one UE capability on whether UE supports the PWS combination may need to be introduced.

Observation 3: Supporting the PWS acquisition across cells have significant spec impacts, e.g., applicable scenarios of the PWS acquisition across cells, the UE behaviour upon acquisition and combination of the PWS across cells, how to enable the UE to decide across which cell the PWS combination is supported, and UE capability.

Considering the limited time budget of the R19, it is proposed not to support the enhancement in R19.

Proposal 4 (RRC-3): Do not support PWS acquisition across cells in R19.

# Conclusion

In this contribution, we have further discussed the PWS support for NB-IoT. And we make the following observations and proposals:

For PWS support in NB-IoT TN:

Proposal 1: Do not support PWS for NB-IoT TN in R19.

For PWS acquisition without SIB1 acquisition:

Observation 1: PWS acquisition using the current mechanism leads to comparable long latency.

Observation 2: The current spec already supports the NW to provide the SIB31(-NB) scheduling information to UE before the SIB31(-NB) is scheduled.

Proposal 2 (RRC-2): To avoid UE reacquire the SIB1-NB upon acquiring the PWS message, the network provides the SIB10-NB/SIB11-NB/SIB12-NB scheduling information within SIB1-NB.

Proposal 3 (RRC-2): When acquiring SIB10-NB/SIB11-NB/SIB12-NB, UE may assume that the scheduling is unchanged.

For PWS acquisition across cells:

Observation 3: Supporting the PWS acquisition across cells have significant spec impacts, e.g., applicable scenarios of the PWS acquisition across cells, the UE behaviour upon acquisition and combination of the PWS across cells, how to enable the UE to decide across which cell the PWS combination is supported, and UE capability.

Proposal 4 (RRC-3): Do not support PWS acquisition across cells in R19.

# Reference

- Draft_RAN2_130_Meeting_Report_v1.

- Draft_RAN2_128_Meeting_Report_v1.

- https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23130%5D/%5BPost130%5D%5B309%5D%5BR19%20IoT%20NTN%5D%20capability%20CR%20(Qualcomm)

- S2-2504572, LS on stage 1 requirements for the support for PWS over satellite NGRAN in Rel-17.

- S2-2505537, PWS support.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- TS36.212, Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding.