3GPP TSG-RAN WG2 Meeting #131                                                                   R2-2505090

Bengaluru, India, 25th – 29th August, 2025

Source:	CATT

Title:	Leftover Issues on other aspects in SBFD

Agenda Item:	8.11.3

Document for:	Discussion

1. Introduction

RAN1 has sent the latest RRC parameter list in R1-2503242 and the RRC running CR is revised at [Post130][216][SBFD] Running CR for 38.331 (Huawei).

We are going to discuss the leftover issues in the running CR, spotted issues in R1-2503242 and the working assumption which were made at previous meetings in this contribution.

2. Discussion

2.1 RRC-1 RRC signalling design for the NT indicates RO types to the UE

There is no clear conclusion on this issue in [Post130][216][SBFD] Running CR for 38.331 (Huawei)

It has been agreed in previous RAN2 meetings that for determining CBRA RO type (SBFD ROs or non-SBFD ROs) used in the RACH operation, the NT can indicate RO type to the UE directly through RRC signalling. When this signalling is absent, the UE can use RSRP measurement based method or use its implementation to determine the RO type (when the RSRP threshold signalling is also absent). There are two candidate solutions which were discussed:

Proposed resolution:

Option 1:  Use the multiple values (e.g., 2 bits for four values) to assist the UE to select RO type.

Option 2:  Use 1 bit indicating RO type directly (as in the current RRC running CR).

The load issue mentioned in option 1 is not critical especially with RSRP-based methods. So the justification of option 1 is not strong enough.

We propose to support option 2 as majority supported:

Proposal 1: [RRC-1] For the NT indicating RO type, use 1 bit signalling (as in the current RRC running CR).

2.2 RRC-7 Whether to support SBFD with NR-DC

RAN1 made the following agreement according to the LS from RAN1 in R1-2504858, for the question on whether SBFD and DC can be configured simultaneously and whether there is any issue with such configuration for an SBFD-aware UE:

Agreement

- From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier.

- Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly.

- It is up to RAN2 to decide whether or not to support simultaneous configuration of SBFD and DC.

- CC: RAN3, RAN4.

From RAN2’s perspective, the impact of DC can be discussed case by case:

- NR-DC: SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier.

So the restriction mentioned above needs to be captured in stage-2 spec if SBFD and NR-DC which can be configured simultaneously only for inter-band NR-DC is agreed.

- EN-DC, NGEN-DC, NE-DC can be configured SBFD in gNB

It seems the impacts of RAN2 specs on simultaneous configuration of SBFD and DC are not restricted to RAN2, RAN4 should make the decision instead.

We prefer to let RAN4 make the decision on simultaneous configuration of SBFD and NR-DC.

Proposal 2: [RRC-7] From RAN2’s perspective, the simultaneous configuration of SBFD and NR-DC can be supported. Send a reply LS to RAN4 on this understanding.

2.3 Editor’s note in RRC running CR

- FD for field resourcesForChannelCLI

The field description of resourcesForChannelCLI can be polished which was copied directly from RAN1 RRC parameter list.

resourcesForChannelCLI Configures CLI measurement resource set and the list of corresponding references to TCI-States. If the parameter resourcesForChannelCLI is configured, the following legacy parameters should not be configured or should be ignored: resourcesForChannel, csi-IM-ResourcesForInterference, nzp-CSI-RS-ResourcesForInterference, resourcesForChannel2-r17, resourcesForChannelTDCP-r18, applyIndicatedTCI-State-r18, csi-ReportSubConfigTriggerList-r18.

According to the similar FD of other IEs in RRC, the field description can be revised as:

If field is present, the following fields should be ignored by UE:

Proposal 3:	The field description of resourcesForChannelCLI can be revised as:

resourcesForChannelCLI Configures CLI measurement resource set and the list of corresponding references to TCI-States. If field is present, the following fields should be ignored by UE: resourcesForChannel, csi-IM-ResourcesForInterference, nzp-CSI-RS-ResourcesForInterference, resourcesForChannel2-r17, resourcesForChannelTDCP-r18, applyIndicatedTCI-State-r18, csi-ReportSubConfigTriggerList-r18.

2.4 LS to RAN1 on the observed issues in R1-2503242

The range of sbfd-Configuration2-PUSCH-RBOffset should be (0..maxNrofPhysicalResourceBlocks-1), instead of (0..maxNrofPhysicalResourceBlocks) in R1-2503242.

| Tx/Rx enhancement | 38.214 | sbfd-Configuration2-PUSCH-RBOffset | New | Indicates the RB offset to determine the starting PRB for type 2 CG and DG PUSCH transmissions in SBFD symbols for Configuration 2. | (0..maxNrofPhysicalResourceBlocks) |
| --- | --- | --- | --- | --- | --- |

RAN1 should revise the RRC parameter list.

Proposal 4: RAN2 to send an LS to RAN1 to indicate the spotted issue in RRC parameter list:

- The range of sbfd-Configuration2-PUSCH-RBOffset should be (0..maxNrofPhysicalResourceBlocks-1).

2.5 Others

There is a working assumption at RAN2#129bis meeting which has not been confirmed yet.

Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

There is a working assumption at RAN2#130 meeting as well:

Agreements on RRC open issues

Working assumption: RAN2 confirms that the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected.

These WAs can be confirmed since they follow the legacy rules.

Proposal 5: Confirm the working assumption made before:

- the configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

- the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected.

3. Conclusion

Based on the analysis in section 2, the proposals are summarized as following:

Proposal 1: [RRC-1] For the NT indicating RO type, use 1 bit signalling (as in the current RRC running CR).

Proposal 2: [RRC-7] From RAN2’s perspective, the simultaneous configuration of SBFD and NR-DC can be supported. Send a reply LS to RAN4 on this understanding.

Proposal 3:	The field description of resourcesForChannelCLI can be found as:

resourcesForChannelCLI Configures CLI measurement resource set and the list of corresponding references to TCI-States. If field is present, the following fields should be ignored by UE: resourcesForChannel, csi-IM-ResourcesForInterference, nzp-CSI-RS-ResourcesForInterference, resourcesForChannel2-r17, resourcesForChannelTDCP-r18, applyIndicatedTCI-State-r18, csi-ReportSubConfigTriggerList-r18.

Proposal 4: RAN2 to send an LS to RAN1 to indicate the spotted issue in RRC parameter list:

- The range of sbfd-Configuration2-PUSCH-RBOffset should be (0..maxNrofPhysicalResourceBlocks-1).

Proposal 5: Confirm the working assumption made before:

-the configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync.

- the separate Layer 3 measurement report for CSI-RS resources in SBFD symbol is not supported, no RRC spec impact is expected.

4. References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  Huawei

- RAN1 Chair Notes WG1 #121

- RAN2 Chair Notes WG2 #129bis

- RAN2 Chair Notes WG2 #130

- R2-2505364	Summary of [Post130][216][SBFD] Running CR for 38.331  Huawei, HiSilicon