3GPP TSG-RAN2 Meeting#131												R2-2505112

# Bengaluru, India, August 25th - August 29th, 2025

Agenda Item:	5.1.3.2

Source:	OPPO

Title:	Discussion on R4 LS on Mandatory Gap Capability

Document for:	Discussion, Decision

- Introduction

This is to discuss the issue of mandatory gap capability, as stated in R4 LS R4-2508379.

- Discussion

R4 asked that

When UE reports ‘1’ to support a gap pattern in Rel-15 supportedGapPatterns but reports ‘0’ for the same gap pattern in supportedGapPatterns-NRonly-r16, UE shall support the gap pattern which is reported in Rel-15 supportedGapPatterns.

For which the assumption is the gaps supported for NR-only should be a superset of supportedGapPattern, rather than a sub-set, considering the additional applicable scenarios for supportedGapPattern.

| supportedGapPattern Indicates measurement gap pattern(s) optionally supported by the UE for NR SA, for NR-DC, for NE-DC and for independent measurement gap configuration on FR2 in (NG)EN-DC. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [5] and so on. The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2. | UE | CY | No | No |
| --- | --- | --- | --- | --- |
| supportedGapPattern-NRonly-r16 Indicates measurement gap pattern(s) optionally supported by the UE for NR SA and NR-DC when the frequencies to be measured within this measurement gap are all NR frequencies. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3 and so on. The UE shall set the bits corresponding to the measurement gap pattern 2, 3 and 11 to 1. | UE | FD | No | No |

- R2 update 38.306 for supportedGapPattern and supportedGapPattern-NRonly-r16 as requested by R4.

Besides that, some similar issues have been identified, which worth a further check as well.

One is for supportedGapPattern-NRonly-NEDC-r16

| supportedGapPattern Indicates measurement gap pattern(s) optionally supported by the UE for NR SA, for NR-DC, for NE-DC and for independent measurement gap configuration on FR2 in (NG)EN-DC. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [5] and so on. The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2. | UE | CY | No | No |
| --- | --- | --- | --- | --- |
| supportedGapPattern-NRonly-NEDC-r16 Indicates whether the UE supports gap patterns 2, 3 and 11 in NE-DC when the frequencies to be measured within this measurement gap are all NR frequencies. | UE | No | No | No |

I.e., if UE report ‘1’ for pattern 2, 3, or 11, but supportedGapPattern-NRonly-NEDC-r16 is absent, it should assume UE support the corresponding pattern.

- R2 confirm (and if yes, update 38.306) that if UE report ‘1’ for pattern 2, 3, or 11 in supportedGapPattern, but supportedGapPattern-NRonly-NEDC-r16 is absent, it should assume UE support the corresponding pattern.

The other is LTE spec, where

#### 4.3.6.36	measGapPatterns-r15

This field defines whether the UE that supports NR supports gap patterns 4 to 11 in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS38.133 [37].

<Text Removed>

4.3.6.44	measGapPatterns-NRonly-r16

This field indicates whether the UE supports gap patterns 2, 3 and 11 in LTE standalone when the frequencies to be measured within this measurement gap are all NR frequencies.

- R2 confirm (and if yes, update 36.306) that if UE report ‘1’ for pattern 2, 3, or 11 in measGapPatterns-r15, but measGapPatterns-NRonly-r16 is absent, it should assume UE support the corresponding pattern.

And similarly for ENDC case specifically

#### 4.3.6.36	measGapPatterns-r15

This field defines whether the UE that supports NR supports gap patterns 4 to 11 in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS38.133 [37].

<Text Removed>

4.3.6.45	measGapPatterns-NRonly-ENDC-r16

This field indicates whether the UE supports gap patterns 2, 3 and 11 in (NG)EN-DC when the frequencies to be measured within this measurement gap are all NR frequencies.

- R2 confirm (and if yes, update 36.306) that if UE report ‘1’ for pattern 2, 3, or 11 in measGapPatterns-r15, but measGapPatterns-NRonly-ENDC-r16 is absent, it should assume UE support the corresponding pattern.

- Conclusion

Proposal 1	R2 update 38.306 for supportedGapPattern and supportedGapPattern-NRonly-r16 as requested by R4.

Proposal 2	R2 confirm (and if yes, update 38.306) that if UE report ‘1’ for pattern 2, 3, or 11 in supportedGapPattern, but supportedGapPattern-NRonly-NEDC-r16 is absent, it should assume UE support the corresponding pattern.

Proposal 3	R2 confirm (and if yes, update 36.306) that if UE report ‘1’ for pattern 2, 3, or 11 in measGapPatterns-r15, but measGapPatterns-NRonly-r16 is absent, it should assume UE support the corresponding pattern.

Proposal 4	R2 confirm (and if yes, update 36.306) that if UE report ‘1’ for pattern 2, 3, or 11 in measGapPatterns-r15, but measGapPatterns-NRonly-ENDC-r16 is absent, it should assume UE support the corresponding pattern.