3GPP TSG-RAN2 Meeting#131												R2-2505113

# Bengaluru, India, August 25th - August 29th, 2025

Agenda Item:	8.5.2

Source:	OPPO

Title:	Left Issues on On-Demand SSB

Document for:	Discussion, Decision

- Introduction

This is to discuss Objective-1 of R19 NES.

- Discussion

2.1 Issue-1: OD-SSB Activation/Deactivation

An issue is that R1 has concluded to use separate capability for RRC-based and MAC-CE based OD-SSB A/D.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) |  |
| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2. Supported time domain relation between on-demand SSB and always-on SSB |  |
| 61. Netw_Energy_NR_enh | 61-2a | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency between always-on SSB and on-demand SSB | 61-2 |
| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate activation, [adaptation,] and deactivation of on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) 2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indicationImplicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | FFS |
| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency | 1. Support MAC CE based signalling to indicate activation, [adaptation,] and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2.Supported time domain relation between on-demand SSB and always-on SSB 3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | FFS |
| 61. Netw_Energy_NR_enh | 61-4a | On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency | 1. Support MAC CE based signalling to indicate activation, [adaptation,] and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency between always-on SSB and on-demand SSB 2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | 61-4 |

There is a need to differentiate the behavior for UEs supporting 61-1/2/2a and 61-3/4/4a only

- For UE supporting 61-1/2/2a only, it relies on RRC only to set the A/D status of OD-SSB.

- For UE supporting 61-3/4/4a only, it relies on MAC-CE to set the A/D status of OD-SSB, but that is not sufficient, since the OD-SSB parameter setting anyway comes from RRC first, and thus before MAC-CE arrival, UE has to know whether the configured OD-SSB is activated or not.

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-19                     INTEGER (0.. maxNrofOD-SSB-1-r19)                                       OPTIONAL, -- Need R

od-ssb-ActivationStatus-r19            ENUMERATED {true}                                                       OPTIONAL, -- Need R

[…]

...

}

OD-SSB-Config field descriptions

od-ssb-ActivationStatus Indicates the activation status of this OD-SSB pattern upon configuration.

Therefore, it is more reasonable to assume UE supporting 61-3/4/4a (MAC-CE based A/D) relies on RRC to set initial state of OD-SSB, but leave to MAC-CE for the subsequent A/D status setting.

For this reason, R1 indicates that R2 can further discuss different RRC-based capability (in R1-2502961), i.e., the capability for RRC setting initial A/D state and RRC setting initial and subsequent A/D state have to be differentiated.

Note: whether there is a need to define separate UE capabilities for different RRC-based signaling mechanisms (if any) for indication of on-demand SSB transmission is up to RAN2

- For 61-3/4/4a in R1 feature list, R2 confirm UE relies on RRC to set initial A/D state of OD-SSB (i.e., status setting upon OD-SSB configuration, rather than re-configuration), and relies on MAC-CE for subsequent A/D control.

  - Issue-2: L3 Measurement of OD-SSB

During 130, R2 concluded that

(modified) The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated.

When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

It is obvious that measurement behavior would adapt to the (de)activation status of OD-SSB, and to the specific activated OD-SSB configuration.

On the other hand, we had a previous conclusion below, which leads to some confusion, since some always assume a same SMTC window for both serving and neighbor cell(s).

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

If we stick to the ‘same-SMTC’ restriction as in legacy, then

- When AO-SSB and OD-SSB are on the same frequency: A more frequent measurement on neighboring cell would be adopted when a denser OD-SSB is activated. But neighboring cell SSB may not be transmitted as frequently as the denser SMTC, which would further lead to not only unnecessary measurement, but also failure to reach the R4 requirement (since now R4 defines the L3 measurement requirement according to SMTC window periodicity)

- When AO-SSB and OD-SSB are on different frequency(ies): E.g., assume AO-SSB is on F1, while OD-SSB is on F2. On the other hand, both F1 and F2 may have neighboring cell deployed and thus SSB to measure, and those SSB (of neighboring cell) are transmitted independent of OD-SSB (de)activation status of serving cell. And in either F1 or F2, if there is MO configured for neighboring cell measurement, there is always a SMTC to refer to (for F1, it is also the SMTC for serving cell measurement), which should be used for neighboring cell measurement. If UE deviate from this SMTC, the same issue as for same-frequency case above also happens.

- The adaptive SMTC usage for OD-SSB does not affect the SMTC usage for neighboring cell as in legacy.

Another issue is whether inter-frequency OD-SSB would lead to a change for measurement gap design.

In our view, there is no remaining issue for gap necessity, since that no matter OD-SSB is configured as per-cell or per-BWP, it has to stick to the R1 conclusion that AO-SSB and OD-SSB are located in the same BWP. So that the gap necessity can follow legacy rule. I.e., UEcan always refer to the presence of AO-SSB to decide on the need of MG.

- The legacy measurement gap design can be reused considering AO-SSB and OD-SSB are always on the same BWP as agreed by R1.

  - Issue-3: Open Issues in RRC CR

For the open issue as follows (both are for the same issue)

| Fujitsu 006 | – OD-SSB-Config RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.; Field servingCellMO is introduced in IE OD-SSB-Config. In this way it is possible to keep the same field name an hence all the procedural text intact.. [Fujitsu] RAN2 agreed to introduce a new servingCellMO in ServingCellConfig in RAN2#130. Although the current CR may work, we think the original agreement is simpler for implementation, because it can keep the same message structure. We also think even if it is introcuded in ServingCellConfig, the current procedural text is not changed (similar to servingCellMO-r17). We would like to add it into open issue list and ask other company’s views. [Apple] We have same view as Fujisu. | Open issue. |
| --- | --- | --- |
| HW001 | On OD-SSB-Config-R19, currently there is a servingCellMO per OD-SSB-Config. However, we only agreed to introduce one servingCellMO for OD-SSB, it should be per SCell, not per OD-SSB config. [Apple] We have same view as Huawei. | It is only one active since only one OD-SSB is active. If this is added to Servingcell, there needs to be a list of SMTCs where only one can be active(maybe other parameters as well?), one that is corresponding to the currently active OD-SSB. |

We tend to agree with Rapp that essentially there is no difference, i.e., there should be a mapping between SMTC and OD-SSB, while the current specification is clearer since it saves the effort to further clarify how the mapping is to be done, but rather directly contains the SMTC configuration within the OD-SSB configuration.

- For the RRC open issue of Fujitsu006, keep the current CR as it is (i.e., implement the MO within the OD-SSB configuration).

| CATT003 | od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is indicated.. Agreement (RAN1 #121) For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication. According to the conclusion of RAN1, od-ssb-nrofBurst can only be configured by RRC and activated by MAC CE, and RAN2 has not discussed this issue. The od-ssb-nrofBurst is configured in parallel with od-ssb-ActivationStatus in OD-SSB-Config, so od-ssb-nrofBurst should be able to be configured and activated through RRC? We think we need to make a clarification. | Open issue |
| --- | --- | --- |

This part failed to be concluded in R1, since different companies have different proposals regarding how to derive the first OD-SSB occasion, e.g., whether/how to define the processing time upon RRC (re)configuration. Considering the processing time is more of R1-expertise, it seems hard for R2 to take a decision on this issue.

- For the RRC open issue of CATT003, R2 follow R1 conclusion, to clarify in the RRC CR that implicit OD-SSB deactivation cannot co-configured with RRC-based activation.

- Conclusion

Proposal 1	For 61-3/4/4a in R1 feature list, R2 confirm UE relies on RRC to set initial A/D state of OD-SSB (i.e., status setting upon OD-SSB configuration, rather than re-configuration), and relies on MAC-CE for subsequent A/D control.

Proposal 2	The adaptive SMTC usage for OD-SSB does not affect the SMTC usage for neighboring cell as in legacy.

Proposal 3	The legacy measurement gap design can be reused considering AO-SSB and OD-SSB are always on the same BWP as agreed by R1.

Proposal 4	For the RRC open issue of Fujitsu006, keep the current CR as it is (i.e., implement the MO within the OD-SSB configuration).

Proposal 5	For the RRC open issue of CATT003, R2 follow R1 conclusion, to clarify in the RRC CR that implicit OD-SSB deactivation cannot co-configured with RRC-based activation.