3GPP TSG-RAN WG2 Meeting #131	R2-2505114

Bangalore, India  Aug 25th – 29th , 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on functionality management for RRM measurement prediction

Agenda Item:	8.3.2.1

Document for:	Discussion and Decisions

- Introduction

During the RAN#108 meeting, the new WID on “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The WID describes the objective of the work on RRM measurement prediction  as below,

- Specify support for RRM measurement prediction for both UE and network sided models [RAN2]:

- Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction

- Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction

- At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported

Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI.

Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY)

- Inference configuration

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline

Note 3: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation.

This paper discusses the functionality management for RRM measurement prediction, including the applicability procedure, inference configuration, inference report configuration and performance monitoring.

- Applicability procedure

At the RAN2 #130 meeting, the following were agreed for the applicability procedure for AI4PHY and AI4Mob[2],

Agreements for AI4PHY

- (RRC1) Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified)

- (RRC2) Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.  Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)

- (RRC4) When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

- (RRC5) The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable.

- (RRC5) The UE shall report when CSI-ReportConfig becomes not applicable.

- (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations).  FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free).

- (RRC7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

- (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

Agreements for AI4Mob

- Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case)

•	Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress)

•	Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

•	Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

•	Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

•	Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

•	No prohibit timer is introduced.

- Associated ID should be optionally configurable for training and  inference (i.e. it may not mandatorily required for all training/inference configurations).  FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

Based on the last meeting’s agreements, AI4Mob already takes the beam management agreements as the baseline. Given that AI4PHY and AI4Mob share a similar overall framework for functionality management on Option A/B applicability procedure, most of the RRC-level agreements from AI4PHY can be reused in AI4Mob with minimal changes. Below is the mapping:

AI4PHY agreements that can be reused for AI4Mob:

  - (RRC1) Cause of inapplicability

In AI4Mob, the UE may also need to request release of a configuration (e.g., due to model unavailability or resource constraints). Introducing a “release configuration” flag instead of an inapplicability cause streamlines handling and keeps behaviour consistent with AI4PHY, avoiding divergent release trigger mechanisms.

  - (RRC2) Content of otherConfig for enabling reports in UAI

AI4Mob also has scenarios where frequent applicability reporting via UAI is unnecessary. Reusing this flag for both Option A and Option B reduces signalling overhead and keeps the two options aligned in signalling design.

  - (RRC7 & RRC8) Applicability reporting for option B in RRCReconfigurationComplete & Coexistence between Option A and B

For AI4Mob, Option B applicability can also be reported in both RRCReconfigurationComplete and UAI, same as Option A. Allowing Option A and Option B to be configured together in one RRCReconfiguration message with a unified applicability report procedure simplifies implementation.

Proposal 1: Following procedure from AI4PHY is supported for applicability reporting for AI4Mob:

  - Introduce a release-configuration flag for RRM measurement prediction configurations; actual release is network-controlled and UE-initiated release is not supported.

  - OtherConfig shall include a switch to enable/disable applicability reporting via UAI for Option A and Option B.

  - The initial applicability for RRM measurement-prediction is carried in RRCReconfigurationComplete; subsequent updates are carried via UAI when enabled.

  - Option B applicability for RRM measurement-prediction may be included in RRCReconfigurationComplete and UAI, same as Option A.

  - Option A and Option B for RRM measurementprediction may be configured together in one RRCReconfiguration with a unified applicability report procedure, and A/B applicability or inapplicability shall be reported separately.

Items that require adaptation for AI4Mob:

  - (RRC4 & RRC5) Activation of a periodic CSI report config upon change from inapplicable to applicable & Reporting behavior of inapplicable periodic beam prediction configuration

Aligned with AI4PHY, AI4Mob configurations for RRM measurement prediction shall be explicitly released by the network; UE-initiated release is not supported. Until release, the UE continues the configured inference and any required reporting so that the network preserves the necessary context to process predictions. In addition, because AI4Mob relies on the RRM (rather than CSI) framework, AI4Mob is expected to require certain enhancements at the RRC layer to host AI functionality management; specific mechanisms are left to WI.

  - (RRC6) Handling of AIML configurations upon release to IDLE/INACTIVE and during Reestablishment

Similar to AI4PHY, AI4Mob shall take the TS 38.331 legacy behavior as the baseline for whether to release or retain configuration when going to RLF/INACTIVE/IDLE, minimizing specification impacts. In addition, considering the fact that AI4MOB may always work at the cell edge where RLF may happen more frequently than AI4PHY, rapid AI recovery after RLF may be more important. When the gNB has not released the AI-related RRM inference configuration, the UE and NW should support prompt restoration of the AI function upon RRC Reestablishment/Resume without requiring a full reconfiguration. Concrete methods may refer to AI4PHY conclusions (if any) or be further studied in the WI phase.

Proposal 2: Following procedure from AI4PHY is modified for applicability reporting for AI4Mob:

  - UE continues the configured inference and any required reporting until NW explicitly releases the RRM measurement prediction configurations.

  - Enhancements at the RRM measurement configurations are expected to enable AI functionality management and specific mechanisms should be designed in the WI phase.

  - UE and network should support prompt restoration of the AI functionality upon RRC Reestablishment/Resume for RRM measurement prediction. Concrete methods may refer AI4PHY conclusions (if any) or be further defined in the WI phase.

The Associated ID is valuable to ensure consistency between training and inference by binding an AI function to network-side conditions that the UE cannot fully observe. In AI4PHY, there are still open issues on (a) the granularity of the ID (e.g., per cell, cell-group/area, per frequency/multi-frequency) and (b) the UE’s behavior when the ID is absent. Both items depend on the detailed design of the inference configuration (e.g., how Option A/B, Set-A/Set-B, and per-scenario parameters are structured).

For AI4MOB, the inference configuration per scenario/(sub-)use case is not yet finalized. Prematurely fixing the ID’s granularity or absence semantics would risk inconsistencies and later rework. Therefore, it is prudent to track AI4PHY’s discussions, logic, and conclusions for absence semantics, and to specify the ID’s granularity/terminology only after AI4MOB’s per-scenario inference configuration is sufficiently clear in the WI phase.

Proposal 3

- Keep Associated ID optional for RRM measurement prediction, recognizing its role to ensure training–inference consistency under scenarios require such binding.

- Defer defining granularity (per cell / cell-group / frequency / multi-frequency), UE behavior when absent, and terminology until the per-scenario/(sub-)use-case inference configuration is clarified with refering AI4PHY’s discussion and conclusions.

- Inference and report configuration

At the RAN2 #129bis and #130 meeting, the following were agreed for the inference and report configurations [2],

Agreements at #129bis meeting

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

Agreements at #130 meeting

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain

• PW length (for temporal case A)

• Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional).  MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI

• Measured and predicted beam pattern (optional).   MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI

• Measured and predicted frequency (For inter-freq domain prediction)

- Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

The RSRP was considered as the baseline quantity of the study. In practical deployments with multiple frequency bands, the RSRQ is also an important quantity to support the inter-frequency handovers. Therefore, the RSRQ prediction could also benefit the NW performance in measurement overhead reduction or HO performance enhancements. Both RSRP and RSRQ should be considered as the baseline of the study for the specification impacts of RRM measurement predictions, and the choice of RSRP or RSRQ as the report quantity is up to NW implementation.

Proposal 4

  - RSRQ should also be considered as the report quantity baseline for RRM measurement prediction.

At the last meeting, the time domain information i.e. PW length, was agreed as the inference parameters for temporal domain prediction Case A scenario. In fact, the UE should know the corresponding reference signals to be measured and the results to be predicted without the need of measurement in spatial/time/frequency domain for different supported scenarios. This information should be configured to the UE before the activation of the UE-sided model.

Proposal 5

  - The measurements that a UE needs to actually measure and to infer should be configured for RRM measurement prediction in different domains(spatial/time/frequency).

After the UE obtains the predictions, how the UE processes them and reports them should be specified to align the understanding of reported values between the NW and UE. For legacy RRM measurements, a Layer 3 filtering operation is specified and controlled by the NW. After the RRM measurement predictions are considered, the UE has both measured and predicted measurements in hand. How UE processes them to obtain the filtered measurements should be specified to maintain a common understanding of the reported values.

Besides, for sub-use case 1 and sub-use case 3, the Layer 1 measurement results are assumed as the input to AI/ML functionality for measurement prediction. In addition, the inference configuration and reporting for AI/ML mobility are based on RRM measurement framework. How to integrate the Layer 1 measurement results into the RRM measurement framework needs to be specified.

Proposal 6

  - The RRM measurement framework should be enhanced to accommodate the configuration of Layer 1 measurement results as configuration of inference input, and the measured and/or predicted values as inference output.

- Performance monitoring

At the RAN2 #130 meeting, the following were agreed for performance monitoring [2],

Agreements

1	For UE-sided model, NW-sided monitoring, the performance monitor procedure contains

- NW sends monitoring configuration and inference configuration to UE.

- UE reports measurements and inference output based on the corresponding configurations to NW.

- NW performs monitoring and makes decisions, which can be fully NW-implemented.

2	For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains

- NW sends monitoring configuration to UE

- UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data.

- (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision.

- Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW.  FFS which use cases.

- For RRM measurement prediction

  - RSRP differences can be used as the performance metric for monitoring.

For RRM measurement prediction with UE-side models, UE-side monitoring and decision-making have been discussed as a potential approach. However, two concerns arise for mobility use cases:

- Because AI/ML model performance directly affects user experience and network resource scheduling, allowing the UE to unilaterally perform LCM operations can introduce instability and misalignment with network policies due to unexpected UE behavior.

- The network has independent visibility of handover-related information (e.g., HO occasions/opportunities, ping-pong HOs, time-of-stay) without relying on UE reports. Therefore, when mobility issues are detected, the network should have the ability to deactivate or reconfigure the UE-side models, rather than relying solely on UE-decided actions.

Proposal 7

  - Same as AI4PHY, UE-sided decision(Type 2) is precluded for performance monitoring for RRM measurement prediction with UE-sided model.

For UE-side monitoring, the RSRP difference has been agreed as the performance metric. In addition, as noted in Section 3, RSRQ is also important when inter-frequency handover is involved. To mitigate sensitivity to instantaneous fluctuations in RSRP/RSRQ, the metrics should be computed from averaged values (e.g. over an appropriate window), providing a more stable basis for performance monitoring.

Proposal 8

  - RSRQ differences can be used as the performance metric for monitoring.

  - Average RSRP or RSRQ differences can be used as the performance metric for monitoring.

For network-side monitoring in AI4PHY, the monitoring configuration and inference configuration are provisioned independently via CSI-ReportConfig. For AI4Mob, equivalent definitions are required under the RRM framework—i.e., explicit monitoring configuration, inference configuration, and reporting configuration should be specified. For UE-side monitoring in AI4PHY, a metric instance is defined by a single transmission occasion of CSI-RS/SSB for monitoring and a specific prediction time instance for reporting the inference result. Afterwards, the monitoring output is then aggregated over N instances. For AI4Mob, comparable rules are needed.

Proposal 9:

  - Specify explicit RRM-based monitoring, inference, and reporting configurations, and define UE-side metric-instance and aggregation rules for RRM measurement prediction.

- Conclusion

In this contribution, we have the following proposals on functionality management of UE-sided model,

- Applicability procedure

Proposal 1: Following procedure from AI4PHY is supported for applicability reporting for AI4Mob:

  - Introduce a release-configuration flag for RRM measurement prediction configurations; actual release is network-controlled and UE-initiated release is not supported.

  - OtherConfig shall include a switch to enable/disable applicability reporting via UAI for Option A and Option B.

  - The initial applicability for RRM measurement-prediction is carried in RRCReconfigurationComplete; subsequent updates are carried via UAI when enabled.

  - Option B applicability for RRM measurement-prediction may be included in RRCReconfigurationComplete and UAI, same as Option A.

  - Option A and Option B for RRM measurementprediction may be configured together in one RRCReconfiguration with a unified applicability report procedure, and A/B applicability or inapplicability shall be reported separately.

Proposal 2: Following procedure from AI4PHY is modified for applicability reporting for AI4Mob:

  - UE continues the configured inference and any required reporting until NW explicitly releases the RRM measurement prediction configurations.

  - Enhancements at the RRM measurement configurations are expected to enable AI functionality management and specific mechanisms should be designed in the WI phase.

  - UE and network should support prompt restoration of the AI functionality upon RRC Reestablishment/Resume for RRM measurement prediction. Concrete methods may refer AI4PHY conclusions (if any) or be further defined in the WI phase.

Proposal 3

- Keep Associated ID optional for RRM measurement prediction, recognizing its role to ensure training–inference consistency under scenarios require such binding.

- Defer defining granularity (per cell / cell-group / frequency / multi-frequency), UE behavior when absent, and terminology until the per-scenario/(sub-)use-case inference configuration is clarified with refering AI4PHY’s discussion and conclusions.

- Inference and report configuration

Proposal 4

  - RSRQ should also be considered as the report quantity baseline for RRM measurement prediction.

Proposal 5

  - The measurements that a UE needs to actually measure and to infer should be configured for RRM measurement prediction in different domains(spatial/time/frequency).

Proposal 6

- The RRM measurement framework should be enhanced to accommodate the configuration of Layer 1 measurement results as configuration of inference input, and the measured and/or predicted values as inference output.

- Performance monitoring:

Proposal 7

  - Same as AI4PHY, UE-sided decision(Type 2) is precluded for performance monitoring for RRM measurement prediction with UE-sided model.

Proposal 8

  - RSRQ differences can be used as the performance metric for monitoring.

  - Average RSRP or RSRQ differences can be used as the performance metric for monitoring.

Proposal 9

  - Specify explicit RRM-based monitoring, inference, and reporting configurations, and define UE-side metric-instance and aggregation rules for RRM measurement prediction.

References

- Moderator (CATT), RP-251864, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR,” Jun. 2025.

- RAN2 Chair (InterDigital), “RAN2 #130 Chair Notes”, May, 2025.