3GPP TSG-RAN WG2 Meeting #131	R2-2505126

Bangalore, India, 25-29 Aug 2025

Agenda Item:	8.11.2

Source:	NEC

Title:	Remaining issues on random access in SBFD

Document for:	Discussion, Decision

# Introduction

This paper discusses the remaining open issues for random access in SBFD based on the post meeting email discussion in [Post130][217][SBFD].

# Discussion

## Open Issue MAC-3: SBFD RO location derivation in CFRA

During the open issue discussion for the running CR for TS38.321 for Rel-19 SBFD, one company points out that, when network indicates RO type as SBFD RO in CFRA (e.g., RACH-ConfigDedicated, BeamFailureRecoveryConfig), it remains unclear how to derive the locations (in time domain) of the SBFD ROs from the CFRA RA resource set, especially due to the ambiguity on which SBFD RACH configuration option (i.e., Option 1 or Option 2) should be applied in SBFD RO location derivation for CFRA. Different options may lead to different derivation outcomes.

We think it would be a rare case if the network only configures the SBFD RO for CFRA, but not configure the SBFD RO for CBRA. CFRA currently only contains RO type (SBFD RO or legacy RO), and does not contain detailed SBFD RO configuration. Logically, the UE should be able to the acquire the SBFD RO configuration from a cell as legacy RO, where we assume the SBFD RO should be at least applicable to CBRA as a starting point. With this assumption, we do not see a real issue. Then we have the following proposal:

Proposal-1: SBFD RACH configuration is always configured to be applicable to CBRA as the starting point.

## Open Issue MAC-5: Power ramping for RO type switch

Since RAN2 agreed to support switching from SBFD RO to non-SBFD RO type, based on this, RAN1 agreed that for RACH configuration Option 1 and Option 2, the UE can increase the power ramping counter when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure.

RAN1#120bis Agreement For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter.

In addition, RAN2 agreed that RO type switch from legacy RO to additional RO in SBFD symbols is supported. Therefore, UE should also increase the power ramping counter when a SBFD-aware UE switches from legacy-ROs to additional ROs in one RACH procedure.

Proposal-2: Increase the power ramping counter when a SBFD-aware UE switches RO type (i.e., RO type switches from legacy-ROs to additional ROs or from additional ROs to legacy ROs) in one RACH procedure.

At least for RACH configuration option 2, the separate preamble_power_ramping_step for SBFD may be configured. Therefore, upon the RO type switching in PRACH transmission re-attempt in one RACH procedure, similar rules as switching from 2-step RACH to 4-step RACH, a power offset can be introduced to compensate the power ramping difference, which is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP_Legacy).

Proposal-3: For RACH configuration Option 2, when RO type switch is performed in one RACH procedure, a power offset is introduced to compensate the power ramping difference.

Proposal-4: If the proposal 3 is agreed, the power offset is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP_Legacy)

# Conclusion and Proposal

We have the following proposals:

Proposal-1 [MAC-3]: SBFD RACH configuration is always configured to be applicable to CBRA as the starting point.

Proposal-2 [MAC-5]: Increase the power ramping counter when a SBFD-aware UE switches RO type (i.e., RO type switches from legacy-ROs to additional ROs or from additional ROs to legacy ROs) in one RACH procedure.

Proposal-3 [MAC-5]: For RACH configuration Option 2, when RO type switch is performed in one RACH procedure, a power offset is introduced to compensate the power ramping difference.

Proposal-4 [MAC-5]: If the proposal 3 is agreed, the power offset is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP_Legacy)