3GPP TSG-RAN WG2 Meeting #131	R2-2505127

Bangalore, India, 25-29 May 2025

Agenda Item:	8.13.3

Source:	NEC

Title:	Remaining issues for Multi-hop Relay

Document for:	Discussion, Decision

# Introduction

This paper discusses the solutions for some remaining issues for Multi-hop Relay identified by the CR rapporteur.

# Discussion

## Open issue RRC-10: Extension of SFN-DFN Offset Mechanism to Multihop Relay Scenarios

There is an open issue (RRC-10) on the extension of SFN-DFN Offset Mechanism to Multi-hop Relay Scenarios as listed by the email discussion rapporteur [[Post130][407][Relay]].

The current SFN-DFN offset mechanism is defined for single-hop sidelink relay scenarios. In multi-hop configurations, it is not clear whether this mechanism can or should be extended when the Remote UE requests it in such a scenario.

The current assumption in the running CR of TS38.331 made by the CR rapporteur is that the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE can be forwarded by intermediate U2N Relay UEs within the multi-hop relay chain.

However, this behaviour did not count the SFN-DFN offset between child UEs among the relay chain. We doubt if the current assumption can meet the intended timing alignment and positioning requirements for the case of multi-hop relay operation. As positioning is important feature and its accuracy is subject to the accuracy of the timing between the UE and the network. We suggest considering the timing offset between the UEs when populating the SFN-DFN offset along with the multi-hop relay chain.

Proposal-1: When the Remote UE requests the SFN-DFN offset from the connected parent Relay UE, the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE should be compensated by the timing offset between the UEs when it is forwarded by intermediate U2N Relay UEs within the multi-hop relay chain.

## Open Issue RRC-12: FFS Cause Value / Indication Type for Notifications from Intermediate Relay UE

The cause value or indication type used by the Intermediate Relay UE to notify the Remote UE of certain upstream events remains FFS as captured in open issue RRC-12.

As summarized by CR rapporteur, one possible solution is to ask the Intermediate Relay UE to set the indication type to a specific cause value, such as relayUE-RelayReselection, to clearly signal that the notification is due to its own reselection action. However, we think multiple cause value code points should be specified, for example, the potential cause value can include Relay UE RelayReselection, Relay UE PC5 RLF, and RLF notification from upstream node.

Proposal-2: The Intermediate Relay UE sets the indication type to a specific cause value (including Relay UE RelayReselection, Relay UE PC5 RLF, and RLF notification from upstream node.), when sending the sidelink failure notification message to its child UEs.

## Open Issue RRC-13: Fine Tuning of Extended Timer Values Covering Both Uu and PC5 Hops

In the RAN2 # 130 meeting, it was agreed to extend the T300, T301 and T319 timers for multi-hop U2N relay operation such that the UE and gNB will multiply the signalled timer value by the hop count.

In the current running CR, the Remote UE timer values, UE-TimersAndConstantsRemoteUE, which include components from both the Uu and PC5 hops, were considered to be multiplied by the hop count.

As summarized by the CR there are three options to fine tune the timer values:

- Option 1 - Instead of multiplying the timers in UE-TimersAndConstantsRemoteUE with the hop count we multiply the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count.

- Option 2 – We introduce a new timer for a PC5 hop, e.g., t300-PC5/t301-PC5/t319-PC5 and calculate the value as t300 for n-hop relay = t300 + t300-PC5*n.

In our opinion, Option 1 seems simpler but less accurate especially when the hops of the multiple hop sidelink relay goes up, which means Option 1 is not future proof. For Option 2, it introduces new PC5 timers, which seems more future proof.

Proposal-3: Introduce new timer for PC5 hop for T300, T301 and T319 timers and calculate the value as t300 for n-hop relay = t300 + t300-PC5*n.

# Conclusion and Proposal

We have the following proposals:

Proposal-1[RRC-10]: When the Remote UE requests the SFN-DFN offset from the connected parent Relay UE, the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE should be compensated by the timing offset between the UEs when it is forwarded by intermediate U2N Relay UEs within the multi-hop relay chain.

Proposal-2[RRC-12]: The Intermediate Relay UE sets the indication type to a specific cause value (including Relay UE RelayReselection, Relay UE PC5 RLF, and RLF notification from upstream node.), when sending the sidelink failure notification message to its child UEs.

Proposal-3[RRC-13]: Introduce new timer for PC5 hop for T300, T301 and T319 timers and calculate the value as t300 for n-hop relay = t300 + t300-PC5*n.