3GPP TSG-RAN WG2 Meeting #131		R2-2505131

Bengaluru, India, Aug 25th – 29th, 2025

Source:	vivo

Title:	Discussion on UE side data collection

Agenda Item:	8.3.2.3

Document for:	Discussion and Decision

# Introduction

At the RAN2#130 meeting [1], the following agreements were reached regarding UE-side data collection:

Agreements For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

In this contribution, we will further discuss the AI/ML mobility-specific issues related to the UE-side data collection.

# Discussion

## 2.1 RS type

In NR, the reference signal for RRM measurement can be SSB or CSI-RS. In the AI-based beam management use case, these two types of reference signals are supported. However, for AI mobility, SSB was selected as the reference signal for simulation since the mobility is typically based on SSB. Thus, we understand that at least SSB-based measurement prediction should be supported in AI-based mobility.

Furthermore, RAN2 can discuss whether CSI-RS is also considered.

- For the reference signal type in AI/ML-based mobility, SSB is supported as the baseline. RAN2 to discuss whether CSI-RS is also considered.

## 2.2 Data collection configuration framework

According to the agreement in the last meeting, the AI/ML PHY request/configuration framework is used as baseline for the UE-side data collection of AI-based mobility. The specific agreements regarding UE-side data collection in AI/ML PHY include:

RAN2# 129 Agreements for AI/ML PHY Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). RAN2# 129bis Agreements for AI/ML PHY - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In our understanding, the existing AI/ML PHY request/configuration framework for UE-side data collection can be extended to support AI-based mobility.

- For UE-side data collection for UE-sided model training for RRM measurement prediction and measurement event prediction:

- The network can configure whether UE is allowed to initiate the request for data collection configuration, which may include a list of candidate configurations.

- The UE can send a request for data collection configuration via UAI (e.g. start/stop, preferred configuration from the list of candidate configurations). And it is up to UE implementation when to send the request.

- Data collection related configuration(s) and associated ID(s) (if needed) can be included in the data collection configuration.

- The network can decide when to start/stop the data collection by provision of configuration, with or without UE request.

The primary distinction lies in the specific content of the data collection configurations. In the AI-based beam management use case, the Layer 1 CSI measurement framework is used to provide the data collection configuration, including at least:

        - CSI-ResourceConfigId of Set A

        - CSI-ResourceConfigId of Set B

        - The corresponding associated IDs of Set A and Set B.

For AI-based mobility, the RRM measurement resources required for both model training of RRM measurement prediction and measurement event prediction are provided via the legacy L3 RRM measurement framework. Therefore, the RRM measurement framework should be used to provide the UE-side data collection configuration.

- For RRM measurement prediction and measurement event prediction, the L3 RRM measurement configuration is used to provide UE-side data collection configuration from network to UE.

Further, the candidate data collection configuration should include at least the measurement object ID to identify the RRM measurement resource for UE-side data collection. Additionally, as we agreed, the associated ID should be optionally configurable for training purposes, while the details regarding its granularity will be left for further discussion in the WI phase. Therefore, we can reach the following agreement regarding the contents of candidate data collection configuration:

- For RRM measurement prediction and measurement event prediction, candidate UE-side data collection configuration includes at least:

        - measObjectId

        - Associated ID(s) (FFS for WI phase further details)

## 2.3 Contents of UE-side data collection

According to the agreement in RAN2#130, the model input/output is up to UE implementation.

Agreements - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In addition, for AI-based BM, for data collection for UE-sided model, the reportQuantity is set to “none-BM-r19” in CSI-report configuration, which means the contents of UE-sided data collection are up to UE implementation as well.

Therefore, there is no need for the network to configure the contents of collected data for the UE-side data collection.

- For UE-side data collection, there is no need for the network to configure the contents of collected data.

# Conclusion

In the contribution, we have the following observations and proposals:

RS type

- For the reference signal type in AI/ML-based mobility, SSB is supported as the baseline. RAN2 to discuss whether CSI-RS is also considered.

Data collection configuration framework

- For UE-side data collection for UE-sided model training for RRM measurement prediction and measurement event prediction:

- The network can configure whether UE is allowed to initiate the request for data collection configuration, which may include a list of candidate configurations.

- The UE can send a request for data collection configuration via UAI (e.g., start/stop, preferred configuration from the list of candidate configurations). And it is up to UE implementation when to send the request.

- Data collection related configuration(s) and associated ID(s) (if needed) can be included in the data collection configuration.

- The network can decide when to start/stop the data collection by provision of configuration, with or without UE request.

- For RRM measurement prediction and measurement event prediction, the L3 RRM measurement configuration is used to provide UE-side data collection configuration from network to UE.

- For RRM measurement prediction and measurement event prediction, candidate data collection configuration includes at least:

        - measObjectId

        - Associated ID(s) (FFS for WI phase further details)

Contents of UE-side data collection

- For UE-side data collection, there is no need for the network to configure the contents of collected data.

# Reference

- RAN2#130_ChairNotes

- RAN2#129_ChairNotes

- RAN2#129bis_ChairNotes